3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240606

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3515 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 11.3.12 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test |  |  |  |  |  | Date: |  |  | 2024-08-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to 38.523-1 Table 11.3.12.3.2-1, at Step 1 the IP PDU should be transmitted on the DRB associated with the PDU Session with Internet DNN. Current TTCN simply transmits this on the DRB that is configured for the “first” PDU Session. In case the first PDU Session established is for IMS, the TTCN is not conforming to Prose. This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected the TTCN implementation to transmit the IP PDU on DRB associated with first non-IMS PDU session. Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TTCN implementation not in-line with Prose |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.12.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	3

2.1	function fl_TC_11_3_12_TestBody	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D24wk24 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## function fl_TC_11_3_12_TestBody

| Function name | fl_TC_11_3_12_TestBody |
| --- | --- |
| Reason for change | According to 38.523-1 Table 11.3.12.3.2-1, at Step 1 the IP PDU should be transmitted on the DRB associated with the PDU Session with Internet DNN. Current TTCN simply transmits this on the DRB that is configured for the “first” PDU Session. In case the first PDU Session established is for IMS, the TTCN is not conforming to Prose. This needs to be addressed. |
| Summary of change | Corrected the TTCN implementation to transmit the IP PDU on DRB associated with first non-IMS PDU session. Please see below. |
| TTCN module | UAC_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_11_3_12_TestBody() runs on NR5GC_PTC { var template (value) QosFlow_Identification_Type v_QosFlowId := f_NR_GetQosFlowIdentification_ForDRBId(f_NR_GetDefaultDRB_ForFirstPDUSession()); var SDAP_SDUList_Type v_SDAP_Data := { f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()) }; //Create the IP Data <<SKIPPED CODE>>

After Change:

function fl_TC_11_3_12_TestBody() runs on NR5GC_PTC { //WA#WI1325241 11_3_12 COMMENTED OUT var template (value) QosFlow_Identification_Type v_QosFlowId := f_NR_GetQosFlowIdentification_ForDRBId(f_NR_GetDefaultDRB_ForFirstPDUSession()); var template (value) QosFlow_Identification_Type v_QosFlowId := f_NR_GetQosFlowIdentification_ForDRBId(f_NR_GetDefaultDRB_ForFirstNonIMSPDUSession()); //WA#WI1325241 11_3_12 <<SKIPPED CODE>>