3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240609

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3518 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Additional handling of PICS settings in EN-DC RRC test case 8.2.1.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Current implementation does not consider the case when a PICS item is wrongly set to true for a feature the UE does not support and this IE is contained in an omitted data structure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR lists all changes implemented in test case 8.2.1.1.1. Along with the changes listed in this document, updates proposed on R5s240547 for test case 8.1.5.1.1 are also needed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE with invalid parameter configuration may wrongly pass the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.1.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

Overview	2

1	Changes to EN-DC RRC test case 8.2.1.1.1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D24wk32 related to the title of this CR

Contact:	Olivier Genoud
	olivier.genoud@etsi.org

# Changes to EN-DC RRC test case 8.2.1.1.1

Modules NR_CapabilityFunctions and NR_AuxiliaryCapCheckFunctions – Changes listed in R5s240547 for test case 8.1.5.1.1.

Module NR_CapabilityFunctions_MRDC

NOTE: addition marked with label “//NOTE: inconclusive case in Dual and Single mode” is also done in functions:

fl_Check_splitDRB_withUL_Both_MCG_SCG, fl_Check_srb3, fl_Check_sftd_MeasPSCell, fl_Check_sftd_MeasNR_Cell, fl_Check_sftd_MeasPSCell_NEDC,

function fl_Check_splitSRB_WithOneUL_Path(UE_MRDC_Capability p_ReceivedCapabilityMsg)

{ //@sic R5-201761 sic@

var boolean v_GeneralParsMRDC_splitSRB_WithOneUL_Path := false;

var boolean v_FDD_Add_MRDC_splitSRB_WithOneUL_Path := false;

var boolean v_TDD_Add_MRDC_splitSRB_WithOneUL_Path := false;

var UE_MRDC_Capability v_ReceivedCapabilityMsg := p_ReceivedCapabilityMsg;

var GeneralParametersMRDC_XDD_Diff v_GeneralParametersMRDC_XDD_Diff_splitSRB_WithOneUL_Path;

var GeneralParametersMRDC_XDD_Diff v_FDD_Add_UE_MRDC_Cap_splitSRB_WithOneUL_Path;

var GeneralParametersMRDC_XDD_Diff v_TDD_Add_UE_MRDC_Cap_splitSRB_WithOneUL_Path;

if (ispresent(v_ReceivedCapabilityMsg.generalParametersMRDC)) {

v_GeneralParametersMRDC_XDD_Diff_splitSRB_WithOneUL_Path := v_ReceivedCapabilityMsg.generalParametersMRDC;

}

if (ispresent(v_ReceivedCapabilityMsg.fdd_Add_UE_MRDC_Capabilities.generalParametersMRDC_XDD_Diff)) {

v_FDD_Add_UE_MRDC_Cap_splitSRB_WithOneUL_Path := v_ReceivedCapabilityMsg.fdd_Add_UE_MRDC_Capabilities.generalParametersMRDC_XDD_Diff;

}

if (ispresent(v_ReceivedCapabilityMsg.tdd_Add_UE_MRDC_Capabilities.generalParametersMRDC_XDD_Diff)) {

v_TDD_Add_UE_MRDC_Cap_splitSRB_WithOneUL_Path := v_ReceivedCapabilityMsg.tdd_Add_UE_MRDC_Capabilities.generalParametersMRDC_XDD_Diff;

}

if(pc_splitSRB_WithOneUL_Path) {

// check indication of generalParametersMRDC_XDD_Diff

if (ispresent(v_GeneralParametersMRDC_XDD_Diff_splitSRB_WithOneUL_Path.splitSRB_WithOneUL_Path)){

v_GeneralParsMRDC_splitSRB_WithOneUL_Path := true;

}

// check indication of FDD IE

if (ispresent(v_FDD_Add_UE_MRDC_Cap_splitSRB_WithOneUL_Path.splitSRB_WithOneUL_Path)){

v_FDD_Add_MRDC_splitSRB_WithOneUL_Path := true;

}

// check indication of TDD IE

if (ispresent(v_TDD_Add_UE_MRDC_Cap_splitSRB_WithOneUL_Path.splitSRB_WithOneUL_Path)){

v_TDD_Add_MRDC_splitSRB_WithOneUL_Path := true;

}

//NOTE: inconclusive case in Dual and Single mode

if (not (f_NR_AnyOfThree_SignaledbyUE(v_GeneralParsMRDC_splitSRB_WithOneUL_Path, v_FDD_Add_MRDC_splitSRB_WithOneUL_Path, v_TDD_Add_MRDC_splitSRB_WithOneUL_Path)))

{

f_SetVerdict(inconc, __FILE__, __LINE__, "setting of pc_splitSRB_WithOneUL_Path to true is inconsistent with reported capabilities");

}

if ((not f_NR_DualModeCheck()) and ((not v_GeneralParsMRDC_splitSRB_WithOneUL_Path) or v_FDD_Add_MRDC_splitSRB_WithOneUL_Path or v_TDD_Add_MRDC_splitSRB_WithOneUL_Path)) {  //@sic R5s200160 sic@

f_SetVerdict(fail, __FILE__, __LINE__, "Reporting of generalParametersMRDC is inconsistent with pc_splitSRB_WithOneUL_Path on Single mode"); // Fail case on Single mode

}

else if (f_NR_DualModeCheck() and (((v_FDD_Add_MRDC_splitSRB_WithOneUL_Path) and (v_TDD_Add_MRDC_splitSRB_WithOneUL_Path)) or (v_GeneralParsMRDC_splitSRB_WithOneUL_Path and (v_FDD_Add_MRDC_splitSRB_WithOneUL_Path or v_TDD_Add_MRDC_splitSRB_WithOneUL_Path)))) {

f_SetVerdict(fail, __FILE__, __LINE__, "Reporting of fdd/tdd-Add-UE-MRDC-Capabilities IEs is inconsistent with Dual Mode"); // Fail case on Dual mode

}

}

else {  // Error cases if the feature is NOT set: neither the Common IE nor any of FDD/TDD IEs are reported by the UE

if (((ispresent(v_GeneralParametersMRDC_XDD_Diff_splitSRB_WithOneUL_Path.splitSRB_WithOneUL_Path))) or

((ispresent(v_FDD_Add_UE_MRDC_Cap_splitSRB_WithOneUL_Path.splitSRB_WithOneUL_Path))) or

((ispresent(v_TDD_Add_UE_MRDC_Cap_splitSRB_WithOneUL_Path.splitSRB_WithOneUL_Path)))) {

f_SetVerdict(fail, __FILE__, __LINE__, "setting of pc_splitSRB_WithOneUL_Path to false is inconsistent with reported capabilities for Rel-15 and above UE");

}

}

}

/*

* @desc      Function to check PICS within optional groups of fields matching with ifpresent in Rel-17.

*            PICS shall not be set if it is in a group not siganlled by the UE. If there is a mismatch, an inconclusive verdict is raised.

* @param     p_MRDC_Capability

*/

function fl_Check_MRDC_PICSinOptionalGroups_NR_r17 (UE_MRDC_Capability p_MRDC_Capability)

{

var UE_MRDC_Capability v_Received_MRDCCapabilityMsg := p_MRDC_Capability;

if (not isvalue(v_Received_MRDCCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.measAndMobParametersMRDC_v1700.measAndMobParametersMRDC_Common_v1700)) {  // UE_MRDC_Capability_v1700.MeasAndMobParametersMRDC_Common_v1700

if (pc_mn_InitiatedCondPSCellChange_FR1FDD_ENDC_r17 or pc_mn_InitiatedCondPSCellChange_FR1TDD_ENDC_r17 or pc_mn_InitiatedCondPSCellChange_FR2TDD_ENDC_r17) {

f_SetVerdict(inconc,__FILE__, __LINE__, "UE is not signalling MeasAndMobParametersMRDC_Common_v1700, no PICS shall be set in a branch when parent is not present");

}

}

}; //end of fl_Check_MRDC_PICSinOptionalGroups_NR_r17

/*

* @desc      Function to check some Rel 17 UE MRDC capabilities

* @param     p_MRDC_Capability

* @param     p_Configuration_Type

* @status    APPROVED (ENDC)

*/

function fl_MRDCCap_CommonChecks_r17(UE_MRDC_Capability p_MRDC_Capability, NR_Configuration_Type p_Configuration_Type)

{// @sic R5-242328 sic@

var UE_MRDC_Capability v_Received_MRDCCapabilityMsg := p_MRDC_Capability;

fl_MRDCCap_CommonChecks_r16(v_Received_MRDCCapabilityMsg, p_Configuration_Type);

if (p_Configuration_Type == EN_DC){

fl_Check_MRDCCommon_mn_InitiatedCondPSCellChange_FR1FDD_ENDC_r17(v_Received_MRDCCapabilityMsg);

fl_Check_MRDCCommon_mn_InitiatedCondPSCellChange_FR1TDD_ENDC_r17(v_Received_MRDCCapabilityMsg);

fl_Check_MRDCCommon_mn_InitiatedCondPSCellChange_FR2TDD_ENDC_r17(v_Received_MRDCCapabilityMsg);

}

fl_Check_MRDC_PICSinOptionalGroups_NR_r17(v_Received_MRDCCapabilityMsg); // PICS set in optional group of IEs checked in Rel-17

} // fl_MRDCCap_CommonChecks_r17