3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240610

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3519 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC Test case 8.1.5.8.1.NR5GC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Below issues has been found related to Processing Delay test cases: 1 In the current TTCN implementation Step3,Step14a5,Step25 are failing due to CR R5s240284 which modified K1 value of affected steps. Resultant requirement arrised to update DL scheduling as per TDD_DL_UL config and TDL as per the specification. 2 In the current TTCN implementation TC is failing at step26 because of unsuccessful contention resolution. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Corrected DL scheduling and TDL - Implemented variable v_RachProcedureConfigReestablishment after setting the cell2 TC_RNTI |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.8.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk24 related to the title of this CR.

Contact:	Kalahasti, Narendra

Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_TC_8_1_5_8_1_NR5GC_TestBody() |
| --- | --- |
| Reason for change | v_RachProcedureConfigReestablishment must be assigned values after setting RNTI of cell2. Prior assignment of wrong TC_RNTI is leading to unsuccessful contention resolution at step26 |
| Summary of change | Assigned values to v_RachProcedureConfigReestablishment after f_NR_CellInfo_SetRNTI(nr_Cell2, tsc_C_RNTI_Value4); |
| TTCN module | RRC_Others_NR5GC |
| MCC160 Comment |  |

Before Change

function f_TC_8_1_5_8_1_NR5GC_TestBody() runs on NR5GC_PTC { var RNTI_Value v_RNTI_Value; var PhysCellId v_PhysCellId; var ShortMAC_I v_ShortMAC_I; var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell2); //@sic R5s240284 sic@ var NR_SecurityParams_Type v_SecurityParams; var octetstring v_EncodeCapRequestNR; var template (value) DRB_CountMSB_InfoList v_CountList; var template (value) RadioBearerConfig v_RadioBearerConfig; var template (value) RadioBearerConfig v_RadioBearerConfigResume;//@sic R5s220683 sic@ var template (value) CellGroupConfig v_CellGroupConfig; var NR_RadioBearerList_Type v_SS_Config; var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext; var template (value) NR_RRC_MSG_Request_Type v_RrcMsg; var NR_SRB_COMMON_IND v_ReceivedNR_SRB_COMMON_IND; var template (omit) octetstring v_EncodedSCG_Config := omit; var NR_DRBInfoList_Type v_ExistingDRBs; var template (value) NR_RachProcedureConfig_Type v_NR_RachProcedureConfig := f_NR_InitialiseRachProcedure(nr_Cell1); //@sic R5s240284 sic@ var integer v_Max_nPRB := f_NR_CellInfo_GetCarrierBandwidth_DL(nr_Cell1); var NR_ResourceAllocation_Type v_NR_ResourceAllocation := {v_Max_nPRB, 28}; var template DRB_CountInfoList v_CountList_UE; var template (value) NG_NAS_MSG_RequestList_Type v_ServerAcceptMsg; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT0; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1; //@sic R5s220683 sic@ var template (omit) DRB_ToAddModList v_DRB_ToAddModList := omit; var integer i; var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(nr_Cell1); //@sic R5s240284 sic@ var template (value) NR_RachProcedureConfig_Type v_RachProcedureConfigReestablishment := cs_NR_RachProcedureConfig_RRCReestablishment(v_NR_PhysicalParameters, f_NR_CellInfo_GetRNTI(nr_Cell2), 256); //@sic R5s240284 sic@ timer t_Wait; v_CellPowerList_AtT0 :={ cs_NR_CellPower(nr_Cell2, -96, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR2)//@sic R5-237355 sic@ }; v_CellPowerList_AtT1:={ cs_NR_CellPower(nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE),//@sic R5-237355 sic@ cs_NR_CellPower(nr_Cell2, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2)//@sic R5-237355 sic@ }; f_NR_SetCellPowerList(v_CellPowerList_AtT0); //@sic R5s240284 sic@ v_NR_RachProcedureConfig.RachProcedureList[0].ContentionResolution.Msg4_Based.SearchSpaceAndDci.DciInfo.Format.Format_1_0.PdschHarqTimingIndicator.Value := f_NR_DciFormat_1_0_RRCDelayPdschHarqTimingIndicator(v_SCS); f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedureConfig); //@siclog "Step 1" siclog@ //The SS transmits a Paging message including a matched ng-5G-S-TMSI. f_NR_Paging_Def(nr_Cell1, RRC_IDLE); //@siclog "Step 2" siclog@ //The UE transmits an RRCSetupRequest message. f_NR_RRC_SetupRequest_Def(nr_Cell1); //@siclog "Steps 3 - 4" siclog@ //The SS transmits an RRCSetup message. //The SS starts transmitting a periodic DCI from (10*2^numerology+1+TDL) slots after step 3 to schedule PUSCH. //Check: Does the UE transmit an RRCSetupComplete message within (10*2^numerology+1+TDL+K2) slots15 (FDD)/18 (TDD) subframes after successful completion of step 3? v_ReceivedNR_SRB_COMMON_IND := f_NR5GC_RRC_ProcedureDelay(nr_Cell1, cas_NR_SRB0_RrcPdu_REQ(nr_Cell1,cs_TimingInfo_Now,f_NR_38508_RRCSetup(nr_Cell1, tsc_NR_RRC_TI_Def)), car_NR_SRB1_RrcNasPdu_IND(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?,- , *), cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected)), 10, "Step 4" , -, -, true); //@sic R5s240284 sic@ //@siclog "Step 5 - 6" siclog@ //The SS transmits a SecurityModeCommand message. //The SS starts transmitting a periodic DCI from (5*2^numerology+1+TDL) slots after step 5 to schedule PUSCH. //Check: Does the UE transmit a SecurityModeComplete message within (5*2^numerology+1+TDL+K2) 10 (FDD)/13 (TDD) subframes slots after successful completion of step 5? v_SecurityParams := f_NR_Security_Get(); v_SecurityParams := f_NR_Authentication_InitAS (v_SecurityParams, v_ReceivedNR_SRB_COMMON_IND.Signalling.Nas[0].SecurityProtection.NasCount); v_SecurityParams.AS_Ciphering.ActTimeList := f_NR_RRC_CipherActTime_GetInitial(nr_Cell1); f_NR_SS_RRC_EnableIntProt_CiphULandDL(nr_Cell1, v_SecurityParams.AS_Integrity, v_SecurityParams.AS_Ciphering); f_NR5GC_RRC_ProcedureDelay(nr_Cell1, cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_RRCSecurityModeCommand(tsc_NR_RRC_TI_Def, cs_38508_SecurityAlgorithmConfig(v_SecurityParams.AS_Ciphering.Algorithm, v_SecurityParams.AS_Integrity.Algorithm))), car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCSecurityModeComplete(tsc_NR_RRC_TI_Def)), 5, "Step 6" ); f_NR_Security_Set(v_SecurityParams); //@siclog "Step 7 - 8" siclog@ //The SS transmits a UECapabilityEnquiry message. //The SS starts transmitting a periodic DCI with the maximum grant allowed by the carrier bandwidthwith MCS=24 from (80*2^numerology+1+TDL) slots after step 7 to schedule PUSCH. //Check: Does the UE transmit a UECapabilityInformation message within (80*2^numerology+1+TDL+K2) slots after successful completion of step 7? v_EncodeCapRequestNR := bit2oct(encvalue( cs_38508_UECapabilityRequestFilterNR({ cs_NR_BandInformationNR(f_NR_CellInfo_GetBandIndicator(nr_Cell1)) }) )); f_NR5GC_RRC_ProcedureDelay(nr_Cell1, cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_UECapabilityEnquiry(tsc_NR_RRC_TI_Def, nr, v_EncodeCapRequestNR)), car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_UECapabilityInformationAny(tsc_NR_RRC_TI_Def)), 80, "Step 8", cs_NR_FreqDomainSchedulExplicit(-, v_NR_ResourceAllocation.Lrbs), //DCI with the maximum grant allowed by the carrier bandwidth @sic R5-217815 sic@ -,-,24);//@sic R5-243506 sic@ //@siclog "Step 9 - 10" siclog@ //The SS transmits an RRCReconfiguration message to establish SRB2 and DRB #n for the UE f_NR_BuildRRCReconfigExistingDRBs (v_RadioBearerConfig, v_CellGroupConfig, v_SS_Config, nr_Cell1); v_SecurityParams := f_NR_Security_Get(); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); //Reconfigure DRB in SS - existing DRBs are not reconfigured during release f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Config, cs_TimingInfo_Now); // Configure PDCP security v_SecurityParams.AS_Ciphering.ActTimeList := f_NR_RRC_CipherActTime_GetCurrent(nr_Cell1, false); f_NR_SS_RRC_EnableIntProt_CiphULandDL(nr_Cell1, v_SecurityParams.AS_Integrity, v_SecurityParams.AS_Ciphering); v_ServerAcceptMsg := {cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_ServiceAcceptMsg(v_ReceivedNR_SRB_COMMON_IND.Signalling.Nas[0].Pdu.Msg.service_Request.pduSessionStatus))}; v_RrcMsg := cs_NR_RRC_MSG_Request_DCCH(cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, v_RadioBearerConfig, -, -, v_V1530Ext)); f_NR5GC_RRC_ProcedureDelay(nr_Cell1, cas_NR_RrcNasPduList_REQ(nr_Cell1, tsc_NR_RbId_SRB1, cs_TimingInfo_Now, v_RrcMsg, v_ServerAcceptMsg, -, -, cs_RlcBearerRouting_NR(nr_Cell1)), car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete), 10, "Step 10" ); //@siclog "Step 11 - 12" siclog@ //The SS transmits a CounterCheck message. //The SS starts transmitting a periodic DCI from (5*2^numerology+1+TDL) slots after step 11 to schedule PUSCH. //Check: Does the UE transmit a CounterCheckResponse message within (5*2^numerology+1+TDL+K2)10 (FDD)/13 (TDD) subframes slots after successful completion of step 11? v_ExistingDRBs := f_NR5GC_MobileInfo_GetDRBInfoList(); v_CountList := {cs_38508_DRB_CountMSB_Info(v_ExistingDRBs[0].DRB_Id, 1, 1)}; v_CountList_UE := {cr_DRB_CountInfo(v_ExistingDRBs[0].DRB_Id)}; f_NR5GC_RRC_ProcedureDelay(nr_Cell1, cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_CounterCheck(tsc_NR_RRC_TI_Def, v_CountList)), car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_CounterCheckResponse(tsc_NR_RRC_TI_Def, v_CountList_UE)), 5, "Step 12" ); //@siclog "Step 13" siclog@ //Void //@siclog "Step 14a1-14a7" siclog@ if ( pc_inactiveState ){ //@siclog "Step 14a1" siclog@ //The SS transmits an RRCRelease message with suspendConfig. f_NR_RRC_InactiveDef(nr_Cell1); //@siclog "Step 14a2" siclog@ //Wait 1s to let UE enter RRC_INACTIVE state. f_Delay(1.0); //@siclog "Step 14a3" siclog@ //The SS transmits a Paging message including a matched ng-5G-S-TMSI. f_NR_Paging_Def(nr_Cell1, RRC_INACTIVE); v_CellGroupConfig := f_NR_GetSCGConfigResume(nr_Cell1); v_EncodedSCG_Config := bit2oct(encvalue(v_CellGroupConfig)); //@sic R5s220683 sic@ for (i:= 0; i < lengthof(v_ExistingDRBs); i :=i+1) { v_DRB_ToAddModList[i] := cs_NR_DRB_ToAddMod(omit, v_ExistingDRBs[i].DRB_Id, true_, -, omit); } v_RadioBearerConfigResume := cs_38508_RadioBearerConfigDef( {cs_NR_SRB_ToAddMod_ReestablishPDCP(tsc_SRB2)}, v_DRB_ToAddModList); //@siclog "Step 14a4" siclog@ //The UE transmits an RRCResumeRequest message SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, mt_Access))); f_NR_BuildRRCReconfigExistingDRBs (v_RadioBearerConfig, v_CellGroupConfig, v_SS_Config, nr_Cell1); f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Config); //Activate security on newly configured DRB f_NR_SS_AS_ActivateSecurity_NoRefresh(nr_Cell1, cs_TimingInfo_Now); //@siclog "Step 14a5" siclog@ //The SS transmits an RRCResume message for the UE //@sic R5-237357 sic@ if(pc_reducedCP_Latency){ //@siclog "Step 14a5Aa1 - 14a5Aa2" siclog@ //The SS starts transmitting a periodic DCI from (10* 2 2^numerology +1+TDL) slots after step 14a5 to schedule PUSCH. //Check: Does the UE transmit an RRCResumeComplete message within (6*2^numerology+1+TDL+K2) slots after successful completion of step 14a5? f_NR5GC_RRC_ProcedureDelayWithRACH( nr_Cell1, cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_Now, cs_38508_RRCResume(tsc_NR_RRC_TI_Def, v_EncodedSCG_Config, v_RadioBearerConfigResume)),//@sic R5s220683 sic@ car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCResumeComplete), 6, "Step 14a5Aa2" ); } else { //@siclog "Step 14a5Ab1 - 14a5Ab2" siclog@ //The SS starts transmitting a periodic DCI from n slots after step 14a5 to schedule PUSCH. //Check: Does the UE transmit an RRCResumeComplete message within (10* 2 2^numerology +1+TDL+K2)slots after successful completion of step 14a5? f_NR5GC_RRC_ProcedureDelayWithRACH( nr_Cell1, cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_Now, cs_38508_RRCResume(tsc_NR_RRC_TI_Def, v_EncodedSCG_Config, v_RadioBearerConfigResume)),//@sic R5s220683 sic@ car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCResumeComplete), 10, "Step 14a5Ab2" ); } v_NR_RachProcedureConfig := f_NR_InitialiseRachProcedure(nr_Cell1); f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedureConfig); //@siclog "Step 14a6-14a7" siclog@ //Void } //@siclog "Step 14-21" siclog@ //Void //@siclog "Step 22" siclog@ //The SS re-adjusts the SS levels according to row "T1" in table 8.1.5.8.1.3.2-1/2 in order that the radio link quality of NR Cell 1 is degraded, and NR Cell 2 is suitable for camping f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 23" siclog@ //Wait 1s to let T310 expire t_Wait.start (1.0);//@sic R5s220683 sic@ //@siclog "Step 24" siclog@ //The UE transmits an RRCReestablishmentRequest message on Cell 2 v_RNTI_Value := f_NR_CellInfo_GetRNTI(nr_Cell1); v_PhysCellId := f_NR_CellInfo_GetPhysicalCellId(nr_Cell1); f_NR_CellInfo_SetRNTI(nr_Cell2, tsc_C_RNTI_Value4); f_NR_SS_C_RNTI_Config(nr_Cell2, tsc_C_RNTI_Value4, -, tsc_NoCnfReq, tsc_FollowOnFlag); //@sic R5s240284 sic@ v_RachProcedureConfigReestablishment.RachProcedureList[0].ContentionResolution.Msg4_Based.SearchSpaceAndDci.DciInfo.Format.Format_1_0.PdschHarqTimingIndicator.Value := f_NR_DciFormat_1_0_RRCDelayPdschHarqTimingIndicator(v_SCS); f_NR_SS_CommonCellConfig(nr_Cell2, cads_NR_RachProcedure_DcchDtchToCSS_Config_REQ(nr_Cell2, cs_TimingInfo_Now, tsc_CnfReq, v_RachProcedureConfigReestablishment, //@sic R5-225418, R5s230476 R5s230884 sic@ cs_NR_SearchSpaceDlDciAssignment_CSSType1(v_NR_PhysicalParameters), cs_NR_SearchSpaceUlDciAssignmentCSS(v_NR_PhysicalParameters, cs_NR_DciFormat_0_0_Params))); v_ShortMAC_I := f_NR_Calculate_ShortMAC(nr_Cell1, nr_Cell2); //@sic R5s220683 sic@ alt{ []SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell2, cr_38508_RRCReestablishmentRequest(cr_NR_ReestabUE_Identity (v_RNTI_Value, v_PhysCellId, v_ShortMAC_I), otherFailure))) { t_Wait.stop; } []t_Wait.timeout { SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell2, cr_38508_RRCReestablishmentRequest(cr_NR_ReestabUE_Identity (v_RNTI_Value, v_PhysCellId, v_ShortMAC_I), otherFailure))); } } //@siclog "Step 25 - 26 " siclog@ //The SS transmits an RRCReestablishment message on slot #i in the radio frame,where: i = 1 for micro=0,1 i = 21 for micro=3 //The SS starts transmitting a periodic DCI from (10*2^numerology+1+TDL) slots after step 25 to schedule PUSCH. IF UE transmits a Random Access Preamble, then SS sends RAR. (Note 1, Note 2) //Check: Does the UE transmit an RRCReestablishmentComplete message within (10*2^numerology+1+TDL+K2) slots after successful completion of step 25? (Note 1, Note 2, Note 3) //@sic R5-217815 R5-217830 sic@ f_NR_SS_AS_ActivateSecurity_Current(nr_Cell2, 0, -, true); f_NR5GC_RRC_ProcedureDelayWithRACH(nr_Cell2, cas_NR_SRB1_RrcPdu_REQ(nr_Cell2, cs_TimingInfo_Now, cs_38508_RRCReestablishment_Def(tsc_NR_RRC_TI_Def, 0)), car_NR_SRB1_RrcPdu_IND(nr_Cell2, cr_38508_RRCReestablishmentComplete), 10, "Step 26"); //@siclog "Step 26A - 26B" siclog@ //The SS transmits an RRCReconfiguration message to resume existing radio bearer. //The UE transmit an RRCReconfigurationComplete message. f_NR5GC_RRCReestablishment_Steps8To13(nr_Cell1, nr_Cell2); } //end of f_TC_8_1_5_8_1_NR5GC_TestBody

After Change

function f_TC_8_1_5_8_1_NR5GC_TestBody() runs on NR5GC_PTC { var RNTI_Value v_RNTI_Value; var PhysCellId v_PhysCellId; var ShortMAC_I v_ShortMAC_I; var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell2); //@sic R5s240284 sic@ var NR_SecurityParams_Type v_SecurityParams; var octetstring v_EncodeCapRequestNR; var template (value) DRB_CountMSB_InfoList v_CountList; var template (value) RadioBearerConfig v_RadioBearerConfig; var template (value) RadioBearerConfig v_RadioBearerConfigResume;//@sic R5s220683 sic@ var template (value) CellGroupConfig v_CellGroupConfig; var NR_RadioBearerList_Type v_SS_Config; var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext; var template (value) NR_RRC_MSG_Request_Type v_RrcMsg; var NR_SRB_COMMON_IND v_ReceivedNR_SRB_COMMON_IND; var template (omit) octetstring v_EncodedSCG_Config := omit; var NR_DRBInfoList_Type v_ExistingDRBs; var template (value) NR_RachProcedureConfig_Type v_NR_RachProcedureConfig := f_NR_InitialiseRachProcedure(nr_Cell1); //@sic R5s240284 sic@ var integer v_Max_nPRB := f_NR_CellInfo_GetCarrierBandwidth_DL(nr_Cell1); var NR_ResourceAllocation_Type v_NR_ResourceAllocation := {v_Max_nPRB, 28}; var template DRB_CountInfoList v_CountList_UE; var template (value) NG_NAS_MSG_RequestList_Type v_ServerAcceptMsg; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT0; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1; //@sic R5s220683 sic@ var template (omit) DRB_ToAddModList v_DRB_ToAddModList := omit; var integer i; var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(nr_Cell1); //@sic R5s240284 sic@ var template (value) NR_RachProcedureConfig_Type v_RachProcedureConfigReestablishment; //@sic R5s240284 sic@ timer t_Wait; v_CellPowerList_AtT0 :={ cs_NR_CellPower(nr_Cell2, -96, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR2)//@sic R5-237355 sic@ }; v_CellPowerList_AtT1:={ cs_NR_CellPower(nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE),//@sic R5-237355 sic@ cs_NR_CellPower(nr_Cell2, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2)//@sic R5-237355 sic@ }; f_NR_SetCellPowerList(v_CellPowerList_AtT0); //@sic R5s240284 sic@ v_NR_RachProcedureConfig.RachProcedureList[0].ContentionResolution.Msg4_Based.SearchSpaceAndDci.DciInfo.Format.Format_1_0.PdschHarqTimingIndicator.Value := f_NR_DciFormat_1_0_RRCDelayPdschHarqTimingIndicator(v_SCS); f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedureConfig); //@siclog "Step 1" siclog@ //The SS transmits a Paging message including a matched ng-5G-S-TMSI. f_NR_Paging_Def(nr_Cell1, RRC_IDLE); //@siclog "Step 2" siclog@ //The UE transmits an RRCSetupRequest message. f_NR_RRC_SetupRequest_Def(nr_Cell1); //@siclog "Steps 3 - 4" siclog@ //The SS transmits an RRCSetup message. //The SS starts transmitting a periodic DCI from (10*2^numerology+1+TDL) slots after step 3 to schedule PUSCH. //Check: Does the UE transmit an RRCSetupComplete message within (10*2^numerology+1+TDL+K2) slots15 (FDD)/18 (TDD) subframes after successful completion of step 3? v_ReceivedNR_SRB_COMMON_IND := f_NR5GC_RRC_ProcedureDelay(nr_Cell1, cas_NR_SRB0_RrcPdu_REQ(nr_Cell1,cs_TimingInfo_Now,f_NR_38508_RRCSetup(nr_Cell1, tsc_NR_RRC_TI_Def)), car_NR_SRB1_RrcNasPdu_IND(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?,- , *), cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected)), 10, "Step 4" , -, -, true,-,true); //@sic R5s240284 sic@ //@siclog "Step 5 - 6" siclog@ //The SS transmits a SecurityModeCommand message. //The SS starts transmitting a periodic DCI from (5*2^numerology+1+TDL) slots after step 5 to schedule PUSCH. //Check: Does the UE transmit a SecurityModeComplete message within (5*2^numerology+1+TDL+K2) 10 (FDD)/13 (TDD) subframes slots after successful completion of step 5? v_SecurityParams := f_NR_Security_Get(); v_SecurityParams := f_NR_Authentication_InitAS (v_SecurityParams, v_ReceivedNR_SRB_COMMON_IND.Signalling.Nas[0].SecurityProtection.NasCount); v_SecurityParams.AS_Ciphering.ActTimeList := f_NR_RRC_CipherActTime_GetInitial(nr_Cell1); f_NR_SS_RRC_EnableIntProt_CiphULandDL(nr_Cell1, v_SecurityParams.AS_Integrity, v_SecurityParams.AS_Ciphering); f_NR5GC_RRC_ProcedureDelay(nr_Cell1, cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_RRCSecurityModeCommand(tsc_NR_RRC_TI_Def, cs_38508_SecurityAlgorithmConfig(v_SecurityParams.AS_Ciphering.Algorithm, v_SecurityParams.AS_Integrity.Algorithm))), car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCSecurityModeComplete(tsc_NR_RRC_TI_Def)), 5, "Step 6" ); f_NR_Security_Set(v_SecurityParams); //@siclog "Step 7 - 8" siclog@ //The SS transmits a UECapabilityEnquiry message. //The SS starts transmitting a periodic DCI with the maximum grant allowed by the carrier bandwidthwith MCS=24 from (80*2^numerology+1+TDL) slots after step 7 to schedule PUSCH. //Check: Does the UE transmit a UECapabilityInformation message within (80*2^numerology+1+TDL+K2) slots after successful completion of step 7? v_EncodeCapRequestNR := bit2oct(encvalue( cs_38508_UECapabilityRequestFilterNR({ cs_NR_BandInformationNR(f_NR_CellInfo_GetBandIndicator(nr_Cell1)) }) )); f_NR5GC_RRC_ProcedureDelay(nr_Cell1, cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_UECapabilityEnquiry(tsc_NR_RRC_TI_Def, nr, v_EncodeCapRequestNR)), car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_UECapabilityInformationAny(tsc_NR_RRC_TI_Def)), 80, "Step 8", cs_NR_FreqDomainSchedulExplicit(-, v_NR_ResourceAllocation.Lrbs), //DCI with the maximum grant allowed by the carrier bandwidth @sic R5-217815 sic@ -,-,24);//@sic R5-243506 sic@ //@siclog "Step 9 - 10" siclog@ //The SS transmits an RRCReconfiguration message to establish SRB2 and DRB #n for the UE f_NR_BuildRRCReconfigExistingDRBs (v_RadioBearerConfig, v_CellGroupConfig, v_SS_Config, nr_Cell1); v_SecurityParams := f_NR_Security_Get(); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); //Reconfigure DRB in SS - existing DRBs are not reconfigured during release f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Config, cs_TimingInfo_Now); // Configure PDCP security v_SecurityParams.AS_Ciphering.ActTimeList := f_NR_RRC_CipherActTime_GetCurrent(nr_Cell1, false); f_NR_SS_RRC_EnableIntProt_CiphULandDL(nr_Cell1, v_SecurityParams.AS_Integrity, v_SecurityParams.AS_Ciphering); v_ServerAcceptMsg := {cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_ServiceAcceptMsg(v_ReceivedNR_SRB_COMMON_IND.Signalling.Nas[0].Pdu.Msg.service_Request.pduSessionStatus))}; v_RrcMsg := cs_NR_RRC_MSG_Request_DCCH(cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, v_RadioBearerConfig, -, -, v_V1530Ext)); f_NR5GC_RRC_ProcedureDelay(nr_Cell1, cas_NR_RrcNasPduList_REQ(nr_Cell1, tsc_NR_RbId_SRB1, cs_TimingInfo_Now, v_RrcMsg, v_ServerAcceptMsg, -, -, cs_RlcBearerRouting_NR(nr_Cell1)), car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete), 10, "Step 10" ); //@siclog "Step 11 - 12" siclog@ //The SS transmits a CounterCheck message. //The SS starts transmitting a periodic DCI from (5*2^numerology+1+TDL) slots after step 11 to schedule PUSCH. //Check: Does the UE transmit a CounterCheckResponse message within (5*2^numerology+1+TDL+K2)10 (FDD)/13 (TDD) subframes slots after successful completion of step 11? v_ExistingDRBs := f_NR5GC_MobileInfo_GetDRBInfoList(); v_CountList := {cs_38508_DRB_CountMSB_Info(v_ExistingDRBs[0].DRB_Id, 1, 1)}; v_CountList_UE := {cr_DRB_CountInfo(v_ExistingDRBs[0].DRB_Id)}; f_NR5GC_RRC_ProcedureDelay(nr_Cell1, cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_CounterCheck(tsc_NR_RRC_TI_Def, v_CountList)), car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_CounterCheckResponse(tsc_NR_RRC_TI_Def, v_CountList_UE)), 5, "Step 12" ); //@siclog "Step 13" siclog@ //Void //@siclog "Step 14a1-14a7" siclog@ if ( pc_inactiveState ){ //@siclog "Step 14a1" siclog@ //The SS transmits an RRCRelease message with suspendConfig. f_NR_RRC_InactiveDef(nr_Cell1); //@siclog "Step 14a2" siclog@ //Wait 1s to let UE enter RRC_INACTIVE state. f_Delay(1.0); //@siclog "Step 14a3" siclog@ //The SS transmits a Paging message including a matched ng-5G-S-TMSI. f_NR_Paging_Def(nr_Cell1, RRC_INACTIVE); v_CellGroupConfig := f_NR_GetSCGConfigResume(nr_Cell1); v_EncodedSCG_Config := bit2oct(encvalue(v_CellGroupConfig)); //@sic R5s220683 sic@ for (i:= 0; i < lengthof(v_ExistingDRBs); i :=i+1) { v_DRB_ToAddModList[i] := cs_NR_DRB_ToAddMod(omit, v_ExistingDRBs[i].DRB_Id, true_, -, omit); } v_RadioBearerConfigResume := cs_38508_RadioBearerConfigDef( {cs_NR_SRB_ToAddMod_ReestablishPDCP(tsc_SRB2)}, v_DRB_ToAddModList); //@siclog "Step 14a4" siclog@ //The UE transmits an RRCResumeRequest message SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, mt_Access))); f_NR_BuildRRCReconfigExistingDRBs (v_RadioBearerConfig, v_CellGroupConfig, v_SS_Config, nr_Cell1); f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Config); //Activate security on newly configured DRB f_NR_SS_AS_ActivateSecurity_NoRefresh(nr_Cell1, cs_TimingInfo_Now); //@siclog "Step 14a5" siclog@ //The SS transmits an RRCResume message for the UE //@sic R5-237357 sic@ if(pc_reducedCP_Latency){ //@siclog "Step 14a5Aa1 - 14a5Aa2" siclog@ //The SS starts transmitting a periodic DCI from (10* 2 2^numerology +1+TDL) slots after step 14a5 to schedule PUSCH. //Check: Does the UE transmit an RRCResumeComplete message within (6*2^numerology+1+TDL+K2) slots after successful completion of step 14a5? f_NR5GC_RRC_ProcedureDelayWithRACH( nr_Cell1, cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_Now, cs_38508_RRCResume(tsc_NR_RRC_TI_Def, v_EncodedSCG_Config, v_RadioBearerConfigResume)),//@sic R5s220683 sic@ car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCResumeComplete), 6, "Step 14a5Aa2",true ); } else { //@siclog "Step 14a5Ab1 - 14a5Ab2" siclog@ //The SS starts transmitting a periodic DCI from n slots after step 14a5 to schedule PUSCH. //Check: Does the UE transmit an RRCResumeComplete message within (10* 2 2^numerology +1+TDL+K2)slots after successful completion of step 14a5? f_NR5GC_RRC_ProcedureDelayWithRACH( nr_Cell1, cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_Now, cs_38508_RRCResume(tsc_NR_RRC_TI_Def, v_EncodedSCG_Config, v_RadioBearerConfigResume)),//@sic R5s220683 sic@ car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCResumeComplete), 10, "Step 14a5Ab2",true ); } v_NR_RachProcedureConfig := f_NR_InitialiseRachProcedure(nr_Cell1); f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedureConfig); //@siclog "Step 14a6-14a7" siclog@ //Void } //@siclog "Step 14-21" siclog@ //Void //@siclog "Step 22" siclog@ //The SS re-adjusts the SS levels according to row "T1" in table 8.1.5.8.1.3.2-1/2 in order that the radio link quality of NR Cell 1 is degraded, and NR Cell 2 is suitable for camping f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 23" siclog@ //Wait 1s to let T310 expire t_Wait.start (1.0);//@sic R5s220683 sic@ //@siclog "Step 24" siclog@ //The UE transmits an RRCReestablishmentRequest message on Cell 2 v_RNTI_Value := f_NR_CellInfo_GetRNTI(nr_Cell1); v_PhysCellId := f_NR_CellInfo_GetPhysicalCellId(nr_Cell1); f_NR_CellInfo_SetRNTI(nr_Cell2, tsc_C_RNTI_Value4); f_NR_SS_C_RNTI_Config(nr_Cell2, tsc_C_RNTI_Value4, -, tsc_NoCnfReq, tsc_FollowOnFlag); v_RachProcedureConfigReestablishment := cs_NR_RachProcedureConfig_RRCReestablishment(v_NR_PhysicalParameters, f_NR_CellInfo_GetRNTI(nr_Cell2), 256); //@sic R5s240284 sic@ v_RachProcedureConfigReestablishment.RachProcedureList[0].ContentionResolution.Msg4_Based.SearchSpaceAndDci.DciInfo.Format.Format_1_0.PdschHarqTimingIndicator.Value := f_NR_DciFormat_1_0_RRCDelayPdschHarqTimingIndicator(v_SCS); f_NR_SS_CommonCellConfig(nr_Cell2, cads_NR_RachProcedure_DcchDtchToCSS_Config_REQ(nr_Cell2, cs_TimingInfo_Now, tsc_CnfReq, v_RachProcedureConfigReestablishment, //@sic R5-225418, R5s230476 R5s230884 sic@ cs_NR_SearchSpaceDlDciAssignment_CSSType1(v_NR_PhysicalParameters), cs_NR_SearchSpaceUlDciAssignmentCSS(v_NR_PhysicalParameters, cs_NR_DciFormat_0_0_Params))); v_ShortMAC_I := f_NR_Calculate_ShortMAC(nr_Cell1, nr_Cell2); //@sic R5s220683 sic@ alt{ []SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell2, cr_38508_RRCReestablishmentRequest(cr_NR_ReestabUE_Identity (v_RNTI_Value, v_PhysCellId, v_ShortMAC_I), otherFailure))) { t_Wait.stop; } []t_Wait.timeout { SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell2, cr_38508_RRCReestablishmentRequest(cr_NR_ReestabUE_Identity (v_RNTI_Value, v_PhysCellId, v_ShortMAC_I), otherFailure))); } } //@siclog "Step 25 - 26 " siclog@ //The SS transmits an RRCReestablishment message on slot #i in the radio frame,where: i = 1 for micro=0,1 i = 21 for micro=3 //The SS starts transmitting a periodic DCI from (10*2^numerology+1+TDL) slots after step 25 to schedule PUSCH. IF UE transmits a Random Access Preamble, then SS sends RAR. (Note 1, Note 2) //Check: Does the UE transmit an RRCReestablishmentComplete message within (10*2^numerology+1+TDL+K2) slots after successful completion of step 25? (Note 1, Note 2, Note 3) //@sic R5-217815 R5-217830 sic@ f_NR_SS_AS_ActivateSecurity_Current(nr_Cell2, 0, -, true); f_NR5GC_RRC_ProcedureDelayWithRACH(nr_Cell2, cas_NR_SRB1_RrcPdu_REQ(nr_Cell2, cs_TimingInfo_Now, cs_38508_RRCReestablishment_Def(tsc_NR_RRC_TI_Def, 0)), car_NR_SRB1_RrcPdu_IND(nr_Cell2, cr_38508_RRCReestablishmentComplete), 10, "Step 26",true); //@siclog "Step 26A - 26B" siclog@ //The SS transmits an RRCReconfiguration message to resume existing radio bearer. //The UE transmit an RRCReconfigurationComplete message. f_NR5GC_RRCReestablishment_Steps8To13(nr_Cell1, nr_Cell2); } //end of f_TC_8_1_5_8_1_NR5GC_TestBody

## 2.2 Change 2

| Function name | f_NR5GC_RRC_ProcedureDelay() |
| --- | --- |
| Reason for change | DL scheduling is not as per TDD_DL_UL config which is impacted because of modified K1 for Step3,Step14a5,Step25. And TDL(V_Delta1/ V_Delta2 ) is also not set according to test procedure Note1 defination. |
| Summary of change | Corrected DL scheduling Subframe and set appropriate TDL |
| TTCN module | NR5GC_RRCSteps.ttcn |
| MCC160 Comment |  |

Before Change

function f_NR5GC_RRC_ProcedureDelay(NR_CellId_Type p_NR_CellId, template (value) NR_SRB_COMMON_REQ p_SRB_COMMON_REQ, template (present) NR_SRB_COMMON_IND p_SRB_COMMON_IND, integer p_Nms, charstring p_TestcaseStep, template (value) NR_FreqDomainSchedulExplicit_Type p_FreqDomainSchedul := cs_NR_FreqDomainSchedulExplicitUL_Def, boolean p_NRDC_PduSession := false, boolean p_InitialAccess := false, template (value) NR_ImcsValue_Type p_ImcsValue := tsc_NR_DefaultGrant_Imcs ) runs on NR5GC_PTC return NR_SRB_COMMON_IND { //@sic R5s230158: added p_NRDC_PduSession for TC_8_2_6_2_2 R5s240284: added p_InitialAccess R5s240229: added p_ImcsValue for TC8_1_5_8_1 sic@ var integer v_T1Slots; var integer v_T2Slots; var integer v_T3Slots; var integer v_T4Slots; var SubFrameTiming_Type v_T1; // Send DL Msg var SubFrameTiming_Type v_T2; // HARQ ACK receive var SubFrameTiming_Type v_T3; // Start UL Grant start var SubFrameTiming_Type v_T4; // UL RRC PDU received var SubFrameTiming_Type v_Tnow; var integer v_SlotsNumberPerSubframe := f_NR_CellInfo_GetSlotsNumberPerSubframe(p_NR_CellId); var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(p_NR_CellId); var integer v_Nslots := p_Nms*v_SlotsNumberPerSubframe + 1; //@sic R5s210842 change 2.1 sic@ var integer v_Delta1; var integer v_Delta2; var integer v_GrantPeriodicity; var integer v_DL_RetransmissionCnt := 0; var NR_SYSTEM_IND v_SYS_IND; var boolean v_T2Flag := false; var boolean v_T4Flag := false; var integer v_K1; var integer v_K2 := 4; var NR_SRB_COMMON_IND v_ReceivedNR_SRB_COMMON_IND; var integer v_MaxNumberSlots := 1024 * 10 * v_SlotsNumberPerSubframe; // Maximum number per SFN complete cycle var template (value) NR_SRB_COMMON_REQ v_SRB_COMMON_REQ := p_SRB_COMMON_REQ; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Internet_DNN); //@sic R5s230158 sic@ timer t_Watchdog; //@sic R5s210842 sic@ if (f_TimingInfo_IsNow(v_SRB_COMMON_REQ.Common.TimingInfo)) { v_T1 := f_NR_GetNextSendOccasion_DL(p_NR_CellId); v_SRB_COMMON_REQ.Common.TimingInfo.SubFrame := v_T1; } else { v_T1 := valueof(v_SRB_COMMON_REQ.Common.TimingInfo.SubFrame); } v_T1Slots := f_NR_ConvertTiming_To_TimingInSlots(v_T1, v_SlotsNumberPerSubframe); // Initialise GrantPeriodicity, Delta1, K1 if (f_NR_CellInfo_GetIsFDD(p_NR_CellId) and not(f_NR_CellInfo_GetHalfDuplexFDD(p_NR_CellId))){ //@sic R5-232191 sic@ v_GrantPeriodicity := 1; // Grant repeated in every slot v_Delta1 := 0; v_K1 := 7; } else { // TDD select(v_SCS) { case(kHz15) { v_GrantPeriodicity := 5; //Grant repeated every 5 slots v_Delta1 := (5 - (v_T1Slots + v_Nslots)) mod 5; v_K1 := 7; } case(kHz30) { v_GrantPeriodicity := 10; //Grant repeated every 10 slots v_Delta1 := (4 - (v_T1Slots + v_Nslots) mod 10) mod 10; v_K1 := 5; } case(kHz120) { v_GrantPeriodicity := 5; //Grant repeated every 5 slots v_Delta1 := (5- (v_T1Slots + v_Nslots) mod 5) mod 5; v_K1 := 5; } case else { FatalError(__FILE__, __LINE__, "Invalid SCS"); } } } if (p_InitialAccess) { //For RRCSetup DCI 1_0 is used - Set K1 as per Table 8.1.5.8.1.3.3-11 v_K1 := bit2int(f_NR_DciFormat_1_0_RRCDelayPdschHarqTimingIndicator(v_SCS)) + 1; } //Calculate T3 v_T3Slots := v_T1Slots + v_Nslots + v_Delta1; //@sic R5s210842 change 2 sic@ v_T3 := f_NR_ConvertTimingInSlots_To_Timing(v_T3Slots, v_SCS, v_SlotsNumberPerSubframe); //T1: send DL Msg SRB.send(v_SRB_COMMON_REQ); //Enable HARQ ack/nack reporting f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_UL_HARQ(enable), cs_TimingInfo_NR(v_T1)); //@sic R5s210842 change 2.4 sic@ //T3 configure SS with periodic UL Grant at T3 //@sic R5s240229 sic@ f_NR_ULGrantConfiguration_Start(p_NR_CellId, cs_TimingInfo_NR(v_T3), p_ImcsValue, p_FreqDomainSchedul, -, cs_UL_GrantConfig_ContPeriodic(v_GrantPeriodicity)); // Start WatchDog timer v_Tnow := f_NR_GetCurrentTiming(p_NR_CellId); t_Watchdog.start (int2float(f_SubFrameTiming_Duration(v_Tnow, v_T1) + p_Nms + 50) / 1000.0); /* 50ms added to be at the safe side */ alt { /* HARQ NACK */ []SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(p_NR_CellId, nack)) { v_DL_RetransmissionCnt := v_DL_RetransmissionCnt + 1; if (v_DL_RetransmissionCnt > 4) { t_Watchdog.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "DL Msg retransmissions > 4"); } else { repeat; } } /* HARQ ACK */ []SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(p_NR_CellId, ack))-> value v_SYS_IND { v_T2 := v_SYS_IND.Common.TimingInfo.SubFrame; f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_UL_HARQ(disable), -, tsc_NoCnfReq); /* disable indication of HARQ ack/nack; @sic R5s220683 sic@ */ v_T2Flag := true; repeat; } /* HARQ error indication */ [] SYSIND.receive (car_NR_SYSTEM_HARQ_ERROR(p_NR_CellId, cr_NR_HarqErrorUL_Any)) { f_SetVerdictInconc(__FILE__, __LINE__, p_TestcaseStep & "HARQ error for UL transmission in RRC Delay test"); t_Watchdog.stop; } [] SRB.receive(p_SRB_COMMON_IND) -> value v_ReceivedNR_SRB_COMMON_IND { v_T4 := v_ReceivedNR_SRB_COMMON_IND.Common.TimingInfo.SubFrame; v_T4Flag := true; t_Watchdog.stop; } [] t_Watchdog.timeout { /* timeout => after T5 */ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "t_Watchdog timeout"); } } //@sic R5s230158 sic@ if (p_NRDC_PduSession) { SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered,tsc_SHT_IntegrityProtected)))) -> value v_ReceivedNR_SRB_COMMON_IND; if (not f_Check_NG_PDUSessionModificationComplete (v_ReceivedNR_SRB_COMMON_IND.Signalling.Nas[0].Pdu, v_GSM_MobilityInfo.SessionId)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Modification Complete failed"); } } //Stop UL Grant f_NR_ULGrantConfiguration_Stop(p_NR_CellId); if (not v_T2Flag) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "UL ACK not received"); } if (not v_T4Flag) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "UL Msg not received"); } //Check the Delay v_T2Slots := (((v_T2.SFN.Number * 10) + v_T2.Subframe.Number) * v_SlotsNumberPerSubframe) + f_NR_SubFrameTiming_GetSlotNumber(v_T2) + v_MaxNumberSlots; v_T4Slots := (((v_T4.SFN.Number * 10) + v_T4.Subframe.Number) * v_SlotsNumberPerSubframe) + f_NR_SubFrameTiming_GetSlotNumber(v_T4) + v_MaxNumberSlots; //Initialise Delta2 if (f_NR_CellInfo_GetIsFDD(p_NR_CellId)){ v_Delta2 := 0; } else { //TDD select(v_SCS) { case(kHz15) { v_Delta2 := (5 - (v_T2Slots - v_K1 + v_Nslots)) mod 5; } case(kHz30) { v_Delta2 := (4 - (v_T2Slots - v_K1 + v_Nslots) mod 10) mod 10; } case(kHz120) { v_Delta2 := (5- (v_T2Slots - v_K1 + v_Nslots) mod 5) mod 5; } case else { FatalError(__FILE__, __LINE__, "Invalid SCS"); } } } if ( ((v_T4Slots - v_K2) - (v_T2Slots - v_K1))mod v_MaxNumberSlots <= (v_Nslots + v_Delta2)) { f_NR_PreliminaryPass(__FILE__, __LINE__, p_TestcaseStep); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep); } return v_ReceivedNR_SRB_COMMON_IND; }

After Change

function f_NR5GC_RRC_ProcedureDelay(NR_CellId_Type p_NR_CellId, template (value) NR_SRB_COMMON_REQ p_SRB_COMMON_REQ, template (present) NR_SRB_COMMON_IND p_SRB_COMMON_IND, integer p_Nms, charstring p_TestcaseStep, template (value) NR_FreqDomainSchedulExplicit_Type p_FreqDomainSchedul := cs_NR_FreqDomainSchedulExplicitUL_Def, boolean p_NRDC_PduSession := false, boolean p_InitialAccess := false, template (value) NR_ImcsValue_Type p_ImcsValue := tsc_NR_DefaultGrant_Imcs, boolean p_harq_timing_indicator := false ) runs on NR5GC_PTC return NR_SRB_COMMON_IND { //@sic R5s230158: added p_NRDC_PduSession for TC_8_2_6_2_2 R5s240284: added p_InitialAccess R5s240229: added p_ImcsValue for TC8_1_5_8_1 sic@ var integer v_T1Slots; var integer v_T2Slots; var integer v_T3Slots; var integer v_T4Slots; var SubFrameTiming_Type v_T1; // Send DL Msg var SubFrameTiming_Type v_T2; // HARQ ACK receive var SubFrameTiming_Type v_T3; // Start UL Grant start var SubFrameTiming_Type v_T4; // UL RRC PDU received var SubFrameTiming_Type v_Tnow; var integer v_SlotsNumberPerSubframe := f_NR_CellInfo_GetSlotsNumberPerSubframe(p_NR_CellId); var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(p_NR_CellId); var integer v_Nslots := p_Nms*v_SlotsNumberPerSubframe + 1; //@sic R5s210842 change 2.1 sic@ var integer v_Delta1; var integer v_Delta2; var integer v_GrantPeriodicity; var integer v_DL_RetransmissionCnt := 0; var NR_SYSTEM_IND v_SYS_IND; var boolean v_T2Flag := false; var boolean v_T4Flag := false; var integer v_K1; var integer v_K2 := 4; var NR_SRB_COMMON_IND v_ReceivedNR_SRB_COMMON_IND; var integer v_MaxNumberSlots := 1024 * 10 * v_SlotsNumberPerSubframe; // Maximum number per SFN complete cycle var template (value) NR_SRB_COMMON_REQ v_SRB_COMMON_REQ := p_SRB_COMMON_REQ; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Internet_DNN); //@sic R5s230158 sic@ timer t_Watchdog; //@sic R5s210842 sic@ if (f_TimingInfo_IsNow(v_SRB_COMMON_REQ.Common.TimingInfo)) { v_T1 := f_NR_GetNextSendOccasion_DL(p_NR_CellId); if(p_harq_timing_indicator == true) {v_T1.Subframe.Number := 0;} v_SRB_COMMON_REQ.Common.TimingInfo.SubFrame := v_T1; } else { v_T1 := valueof(v_SRB_COMMON_REQ.Common.TimingInfo.SubFrame); } v_T1Slots := f_NR_ConvertTiming_To_TimingInSlots(v_T1, v_SlotsNumberPerSubframe); // Initialise GrantPeriodicity, Delta1, K1 if (f_NR_CellInfo_GetIsFDD(p_NR_CellId) and not(f_NR_CellInfo_GetHalfDuplexFDD(p_NR_CellId))){ //@sic R5-232191 sic@ v_GrantPeriodicity := 1; // Grant repeated in every slot v_Delta1 := 0; v_K1 := 7; } else { // TDD select(v_SCS) { case(kHz15) { v_GrantPeriodicity := 5; //Grant repeated every 5 slots v_Delta1 := (5 - (v_T1Slots + v_Nslots)) mod 5; v_K1 := 7; } case(kHz30) { v_GrantPeriodicity := 10; //Grant repeated every 10 slots v_Delta1 := (4 - (v_T1Slots + v_Nslots) mod 10) mod 10; //tdl=9 for k1=5 if(p_harq_timing_indicator == true) {v_Delta1:=13;} // tdl=13 for k1=7 v_K1 := 5; } case(kHz120) { v_GrantPeriodicity := 5; //Grant repeated every 5 slots v_Delta1 := (5- (v_T1Slots + v_Nslots) mod 5) mod 5; v_K1 := 5; } case else { FatalError(__FILE__, __LINE__, "Invalid SCS"); } } } if (p_InitialAccess) { //For RRCSetup DCI 1_0 is used - Set K1 as per Table 8.1.5.8.1.3.3-11 v_K1 := bit2int(f_NR_DciFormat_1_0_RRCDelayPdschHarqTimingIndicator(v_SCS)) + 1; } //Calculate T3 v_T3Slots := v_T1Slots + v_Nslots + v_Delta1; //@sic R5s210842 change 2 sic@ v_T3 := f_NR_ConvertTimingInSlots_To_Timing(v_T3Slots, v_SCS, v_SlotsNumberPerSubframe); //T1: send DL Msg SRB.send(v_SRB_COMMON_REQ); //Enable HARQ ack/nack reporting f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_UL_HARQ(enable), cs_TimingInfo_NR(v_T1)); //@sic R5s210842 change 2.4 sic@ //T3 configure SS with periodic UL Grant at T3 //@sic R5s240229 sic@ f_NR_ULGrantConfiguration_Start(p_NR_CellId, cs_TimingInfo_NR(v_T3), p_ImcsValue, p_FreqDomainSchedul, -, cs_UL_GrantConfig_ContPeriodic(v_GrantPeriodicity)); // Start WatchDog timer v_Tnow := f_NR_GetCurrentTiming(p_NR_CellId); t_Watchdog.start (int2float(f_SubFrameTiming_Duration(v_Tnow, v_T1) + p_Nms + 50) / 1000.0); /* 50ms added to be at the safe side */ alt { /* HARQ NACK */ []SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(p_NR_CellId, nack)) { v_DL_RetransmissionCnt := v_DL_RetransmissionCnt + 1; if (v_DL_RetransmissionCnt > 4) { t_Watchdog.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "DL Msg retransmissions > 4"); } else { repeat; } } /* HARQ ACK */ []SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(p_NR_CellId, ack))-> value v_SYS_IND { v_T2 := v_SYS_IND.Common.TimingInfo.SubFrame; f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_UL_HARQ(disable), -, tsc_NoCnfReq); /* disable indication of HARQ ack/nack; @sic R5s220683 sic@ */ v_T2Flag := true; repeat; } /* HARQ error indication */ [] SYSIND.receive (car_NR_SYSTEM_HARQ_ERROR(p_NR_CellId, cr_NR_HarqErrorUL_Any)) { f_SetVerdictInconc(__FILE__, __LINE__, p_TestcaseStep & "HARQ error for UL transmission in RRC Delay test"); t_Watchdog.stop; } [] SRB.receive(p_SRB_COMMON_IND) -> value v_ReceivedNR_SRB_COMMON_IND { v_T4 := v_ReceivedNR_SRB_COMMON_IND.Common.TimingInfo.SubFrame; v_T4Flag := true; t_Watchdog.stop; } [] t_Watchdog.timeout { /* timeout => after T5 */ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "t_Watchdog timeout"); } } //@sic R5s230158 sic@ if (p_NRDC_PduSession) { SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered,tsc_SHT_IntegrityProtected)))) -> value v_ReceivedNR_SRB_COMMON_IND; if (not f_Check_NG_PDUSessionModificationComplete (v_ReceivedNR_SRB_COMMON_IND.Signalling.Nas[0].Pdu, v_GSM_MobilityInfo.SessionId)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Modification Complete failed"); } } //Stop UL Grant f_NR_ULGrantConfiguration_Stop(p_NR_CellId); if (not v_T2Flag) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "UL ACK not received"); } if (not v_T4Flag) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "UL Msg not received"); } //Check the Delay v_T2Slots := (((v_T2.SFN.Number * 10) + v_T2.Subframe.Number) * v_SlotsNumberPerSubframe) + f_NR_SubFrameTiming_GetSlotNumber(v_T2) + v_MaxNumberSlots; v_T4Slots := (((v_T4.SFN.Number * 10) + v_T4.Subframe.Number) * v_SlotsNumberPerSubframe) + f_NR_SubFrameTiming_GetSlotNumber(v_T4) + v_MaxNumberSlots; //Initialise Delta2 if (f_NR_CellInfo_GetIsFDD(p_NR_CellId)){ v_Delta2 := 0; } else { //TDD select(v_SCS) { case(kHz15) { v_Delta2 := (5 - (v_T2Slots - v_K1 + v_Nslots)) mod 5; } case(kHz30) { v_Delta2 := (4 - (v_T2Slots - v_K1 + v_Nslots) mod 10) mod 10; if(p_harq_timing_indicator == true) {v_Delta2:=13;} } case(kHz120) { v_Delta2 := (5- (v_T2Slots - v_K1 + v_Nslots) mod 5) mod 5; } case else { FatalError(__FILE__, __LINE__, "Invalid SCS"); } } } if ( ((v_T4Slots - v_K2) - (v_T2Slots - v_K1))mod v_MaxNumberSlots <= (v_Nslots + v_Delta2)) { f_NR_PreliminaryPass(__FILE__, __LINE__, p_TestcaseStep); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep); } return v_ReceivedNR_SRB_COMMON_IND; }

## 2.3 Change 3

| Function name | f_NR5GC_RRC_ProcedureDelayWithRACH() |
| --- | --- |
| Reason for change | TDL is not set according to Test procedure Note1. In current TTCN implementation twice v_T1Slots calculation present but according to Timing calculation as per 38.523-3 clause 7.3.7.3. we should give prefrence first v_T1Slots at which time actual RRCReestablishment sent to UE. |
| Summary of change | Added correct TDL for TDD, and commented 2nd v_T1Slots |
| TTCN module | NR5GC_RRCSteps.ttcn |
| MCC160 Comment |  |

Before Change

function f_NR5GC_RRC_ProcedureDelayWithRACH(NR_CellId_Type p_NR_CellId, template (value) NR_SRB_COMMON_REQ p_SRB_COMMON_REQ, template (present) NR_SRB_COMMON_IND p_SRB_COMMON_IND, integer p_Nms, charstring p_TestcaseStep ) runs on NR_BASE_PTC return NR_SRB_COMMON_IND { //@sic R5-237419 renamed function to f_NR5GC_RRC_ProcedureDelayWithRACH sic@ var integer v_T1Slots; var integer v_T2Slots; var integer v_T3Slots; var integer v_T4Slots; var SubFrameTiming_Type v_T1; // Send DL Msg var SubFrameTiming_Type v_T2; // HARQ ACK receive var SubFrameTiming_Type v_T3; // Start UL Grant start var SubFrameTiming_Type v_T4; // UL RRC PDU received var SubFrameTiming_Type v_Tnow; var integer v_SlotsNumberPerSubframe := f_NR_CellInfo_GetSlotsNumberPerSubframe(p_NR_CellId); var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(p_NR_CellId); var integer v_Nslots := p_Nms*v_SlotsNumberPerSubframe + 1; //@sic R5s210842 change 2.1 sic@ var integer v_Delta1; var integer v_Delta2; var integer v_GrantPeriodicity; var integer v_DL_RetransmissionCnt := 0; var NR_SYSTEM_IND v_SYS_IND; var boolean v_T2Flag := false; var boolean v_T4Flag := false; var integer v_K1; var integer v_K2 := 4; var NR_SRB_COMMON_IND v_ReceivedNR_SRB_COMMON_IND; var integer v_MaxNumberSlots := 1024 * 10 * v_SlotsNumberPerSubframe; // Maximum number per SFN complete cycle var boolean v_RACH_ProcedureStarted := false; var integer v_DeltaTDLmax := 0; var template (value) NR_SRB_COMMON_REQ v_SRB_COMMON_REQ := p_SRB_COMMON_REQ; timer t_Watchdog; // The SS transmits an RRCReestablishment/RRCResume message on slot #i in the radio frame,where: // i = 1 for micro=0,1 // i = 21 for micro=3 //@sic R5s240042 sic@ if (f_TimingInfo_IsNow(v_SRB_COMMON_REQ.Common.TimingInfo)) { v_T1 := f_NR_GetNextSendOccasion_DL(p_NR_CellId); v_T1.SFN.Number := (v_T1.SFN.Number + 1) mod 1024; } else { v_T1 := valueof(v_SRB_COMMON_REQ.Common.TimingInfo.SubFrame); } select (v_SCS) { case (kHz15) { v_T1.Subframe.Number := 1; v_T1.Slot.SlotOffset.Numerology0 := true; if (f_NR_CellInfo_GetIsFDD(p_NR_CellId)) { if (p_Nms == 6) { //pc_reducedCP_Latency @sic R5-237357 sic@ v_T1.Subframe.Number := 1; } else { v_T1.Subframe.Number := 5; } } } case (kHz30) { v_T1.Subframe.Number := 0; v_T1.Slot.SlotOffset.Numerology1 := 1; } case (kHz120) { v_T1.Subframe.Number := 2; v_T1.Slot.SlotOffset.Numerology3 := 5; if (p_Nms == 6) { //pc_reducedCP_Latency @sic R5-237357 sic@ v_T1.Subframe.Number := 5; } } case else { FatalError(__FILE__, __LINE__, "Invalid SCS");} } v_SRB_COMMON_REQ.Common.TimingInfo.SubFrame := v_T1; //@sic R5s240284 R5-242531 sic@ v_K1 := bit2int(f_NR_DciFormat_1_0_RRCDelayPdschHarqTimingIndicator(v_SCS)) + 1; v_T1Slots := f_NR_ConvertTiming_To_TimingInSlots(v_T1, v_SlotsNumberPerSubframe); // Initialise GrantPeriodicity, Delta1, K1 if (f_NR_CellInfo_GetIsFDD(p_NR_CellId)){ v_GrantPeriodicity := 1; // Grant repeated in every slot v_Delta1 := 0; v_DeltaTDLmax := 7; if (f_NR_CellInfo_GetHalfDuplexFDD(p_NR_CellId)) { select (p_Nms) { //@sic R5-232191 R5-237429 R5s240042 sic@ case (10, 16) {v_DeltaTDLmax := 2;} case (6) {v_DeltaTDLmax := 6;} case else { FatalError(__FILE__, __LINE__, "Invalid Nsf"); } } } } else { // TDD select(v_SCS) { case(kHz15) { v_GrantPeriodicity := 5; //Grant repeated every 5 slots v_Delta1 := (5 - (v_T1Slots + v_Nslots)) mod 5; select (p_Nms) { //@sic R5-237429 R5s240042 sic@ case (10, 16) {v_DeltaTDLmax := 7;} case (6) {v_DeltaTDLmax := 11;} case else { FatalError(__FILE__, __LINE__, "Invalid Nsf"); } } } case(kHz30) { v_GrantPeriodicity := 10; //Grant repeated every 10 slots v_Delta1 := (4 - (v_T1Slots + v_Nslots) mod 10) mod 10; select (p_Nms) { //@sic R5-237429 R5s240042 sic@ case (10, 16) {v_DeltaTDLmax := 17;} case (6) {v_DeltaTDLmax := 25;} case else { FatalError(__FILE__, __LINE__, "Invalid Nsf"); } } } case(kHz120) { v_GrantPeriodicity := 5; //Grant repeated every 5 slots v_Delta1 := (5- (v_T1Slots + v_Nslots) mod 5) mod 5; select (p_Nms) { //@sic R5-237429 R5s240042 sic@ case (10, 16) {v_DeltaTDLmax := 32;} case (6) {v_DeltaTDLmax := 24;} case else { FatalError(__FILE__, __LINE__, "Invalid Nsf"); } } } case else { FatalError(__FILE__, __LINE__, "Invalid SCS"); } } } //Calculate T3 - UL grant when CSS is used v_T3Slots := v_T1Slots + v_Nslots + v_Delta1; v_T3 := f_NR_ConvertTimingInSlots_To_Timing(v_T3Slots, v_SCS, v_SlotsNumberPerSubframe); //T1: send DL Msg SRB.send(v_SRB_COMMON_REQ); //Enable HARQ ack/nack reporting f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_UL_HARQ(enable), cs_TimingInfo_NR(v_T1)); //@sic R5s210842 change 2.4 sic@ //Enable RACH preamble reporting f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_RachPreamble(enable)); //T3 configure SS with periodic UL Grant at T3 f_NR_ULGrantConfiguration_Start(p_NR_CellId, cs_TimingInfo_NR(v_T3), -, -, -, cs_UL_GrantConfig_ContPeriodic(v_GrantPeriodicity)); // Start WatchDog timer v_Tnow := f_NR_GetCurrentTiming(p_NR_CellId); t_Watchdog.start (int2float(f_SubFrameTiming_Duration(v_Tnow, v_T1) + p_Nms + 50) / 1000.0); /* 50ms added to be at the safe side */ alt { /* HARQ NACK */ []SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(p_NR_CellId, nack)) { v_DL_RetransmissionCnt := v_DL_RetransmissionCnt + 1; if (v_DL_RetransmissionCnt > 4) { t_Watchdog.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "DL Msg retransmissions > 4"); } else { repeat; } } /* HARQ ACK */ []SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(p_NR_CellId, ack))-> value v_SYS_IND { v_T2 := v_SYS_IND.Common.TimingInfo.SubFrame; f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_UL_HARQ(disable), -, tsc_NoCnfReq); /* disable indication of HARQ ack/nack; @sic R5s220683 sic@ */ v_T2Flag := true; repeat; } /* HARQ error indication */ [] SYSIND.receive (car_NR_SYSTEM_HARQ_ERROR(p_NR_CellId, cr_NR_HarqErrorUL_Any)) { f_SetVerdictInconc(__FILE__, __LINE__, p_TestcaseStep & "HARQ error for UL transmission in RRC Delay test"); t_Watchdog.stop; } //@sic R5s221179 sic@ /* UE initiates RACH procedure */ [] SYSIND.receive(car_NR_PRACH_Preamble_IND(?, cr_TimingInfo_Any, ?)) { f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_RachPreamble(disable), -, tsc_NoCnfReq); // @sic R5s220683 sic@ v_RACH_ProcedureStarted := true; repeat; //@sic R5s220813 sic@ } [] SRB.receive(p_SRB_COMMON_IND) -> value v_ReceivedNR_SRB_COMMON_IND { v_T4 := v_ReceivedNR_SRB_COMMON_IND.Common.TimingInfo.SubFrame; v_T4Flag := true; t_Watchdog.stop; } [] t_Watchdog.timeout { /* timeout => after T5 */ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "t_Watchdog timeout"); } } //Stop UL Grant f_NR_ULGrantConfiguration_Stop(p_NR_CellId); if (not v_T2Flag) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "UL ACK not received"); } if (not v_T4Flag) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "UL Msg not received"); } v_T4Slots := (((v_T4.SFN.Number * 10) + v_T4.Subframe.Number) * v_SlotsNumberPerSubframe) + f_NR_SubFrameTiming_GetSlotNumber(v_T4) + v_MaxNumberSlots; if (not v_RACH_ProcedureStarted) { // No RACH procedure was initiated by UE // Timing calculation as per 38.523-3 clause 7.3.7.2 //Check the Delay v_T2Slots := (((v_T2.SFN.Number * 10) + v_T2.Subframe.Number) * v_SlotsNumberPerSubframe) + f_NR_SubFrameTiming_GetSlotNumber(v_T2) + v_MaxNumberSlots; v_T4Slots := (((v_T4.SFN.Number * 10) + v_T4.Subframe.Number) * v_SlotsNumberPerSubframe) + f_NR_SubFrameTiming_GetSlotNumber(v_T4) + v_MaxNumberSlots; //Initialise Delta2 if (f_NR_CellInfo_GetIsFDD(p_NR_CellId)){ v_Delta2 := 0; } else { //TDD select(v_SCS) { case(kHz15) { v_Delta2 := (5 - (v_T2Slots - v_K1 + v_Nslots)) mod 5; } case(kHz30) { v_Delta2 := (4 - (v_T2Slots - v_K1 + v_Nslots) mod 10) mod 10; } case(kHz120) { v_Delta2 := (5- (v_T2Slots - v_K1 + v_Nslots) mod 5) mod 5; } case else { FatalError(__FILE__, __LINE__, "Invalid SCS"); } } } if ( ((v_T4Slots - v_K2) - (v_T2Slots - v_K1))mod v_MaxNumberSlots <= (v_Nslots + v_Delta2)) { f_NR_PreliminaryPass(__FILE__, __LINE__, p_TestcaseStep); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep); } } else { // RACH procedure was initiated by UE // Timing calculation as per 38.523-3 clause 7.3.7.3 v_T1Slots := (((v_T1.SFN.Number * 10) + v_T1.Subframe.Number) * v_SlotsNumberPerSubframe) + f_NR_SubFrameTiming_GetSlotNumber(v_T1) + v_MaxNumberSlots; //@sic R5s220683 sic@ if ( (v_T4Slots - v_T1Slots) <= ( v_Nslots + v_DeltaTDLmax) ) { //@sic R5s220683 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, p_TestcaseStep); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep); } } return v_ReceivedNR_SRB_COMMON_IND; }

After Change

function f_NR5GC_RRC_ProcedureDelayWithRACH(NR_CellId_Type p_NR_CellId, template (value) NR_SRB_COMMON_REQ p_SRB_COMMON_REQ, template (present) NR_SRB_COMMON_IND p_SRB_COMMON_IND, integer p_Nms, charstring p_TestcaseStep, boolean p_harq_timing_indicator := false ) runs on NR_BASE_PTC return NR_SRB_COMMON_IND { //@sic R5-237419 renamed function to f_NR5GC_RRC_ProcedureDelayWithRACH sic@ var integer v_T1Slots; var integer v_T2Slots; var integer v_T3Slots; var integer v_T4Slots; var SubFrameTiming_Type v_T1; // Send DL Msg var SubFrameTiming_Type v_T2; // HARQ ACK receive var SubFrameTiming_Type v_T3; // Start UL Grant start var SubFrameTiming_Type v_T4; // UL RRC PDU received var SubFrameTiming_Type v_Tnow; var integer v_SlotsNumberPerSubframe := f_NR_CellInfo_GetSlotsNumberPerSubframe(p_NR_CellId); var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(p_NR_CellId); var integer v_Nslots := p_Nms*v_SlotsNumberPerSubframe + 1; //@sic R5s210842 change 2.1 sic@ var integer v_Delta1; var integer v_Delta2; var integer v_GrantPeriodicity; var integer v_DL_RetransmissionCnt := 0; var NR_SYSTEM_IND v_SYS_IND; var boolean v_T2Flag := false; var boolean v_T4Flag := false; var integer v_K1; var integer v_K2 := 4; var NR_SRB_COMMON_IND v_ReceivedNR_SRB_COMMON_IND; var integer v_MaxNumberSlots := 1024 * 10 * v_SlotsNumberPerSubframe; // Maximum number per SFN complete cycle var boolean v_RACH_ProcedureStarted := false; var integer v_DeltaTDLmax := 0; var template (value) NR_SRB_COMMON_REQ v_SRB_COMMON_REQ := p_SRB_COMMON_REQ; timer t_Watchdog; // The SS transmits an RRCReestablishment/RRCResume message on slot #i in the radio frame,where: // i = 1 for micro=0,1 // i = 21 for micro=3 //@sic R5s240042 sic@ if (f_TimingInfo_IsNow(v_SRB_COMMON_REQ.Common.TimingInfo)) { v_T1 := f_NR_GetNextSendOccasion_DL(p_NR_CellId); v_T1.SFN.Number := (v_T1.SFN.Number + 1) mod 1024; } else { v_T1 := valueof(v_SRB_COMMON_REQ.Common.TimingInfo.SubFrame); } select (v_SCS) { case (kHz15) { v_T1.Subframe.Number := 1; v_T1.Slot.SlotOffset.Numerology0 := true; if (f_NR_CellInfo_GetIsFDD(p_NR_CellId)) { if (p_Nms == 6) { //pc_reducedCP_Latency @sic R5-237357 sic@ v_T1.Subframe.Number := 1; } else { v_T1.Subframe.Number := 5; } } } case (kHz30) { v_T1.Subframe.Number := 0; v_T1.Slot.SlotOffset.Numerology1 := 1; } case (kHz120) { v_T1.Subframe.Number := 2; v_T1.Slot.SlotOffset.Numerology3 := 5; if (p_Nms == 6) { //pc_reducedCP_Latency @sic R5-237357 sic@ v_T1.Subframe.Number := 5; } } case else { FatalError(__FILE__, __LINE__, "Invalid SCS");} } v_SRB_COMMON_REQ.Common.TimingInfo.SubFrame := v_T1; //@sic R5s240284 R5-242531 sic@ v_K1 := bit2int(f_NR_DciFormat_1_0_RRCDelayPdschHarqTimingIndicator(v_SCS)) + 1; v_T1Slots := f_NR_ConvertTiming_To_TimingInSlots(v_T1, v_SlotsNumberPerSubframe); // Initialise GrantPeriodicity, Delta1, K1 if (f_NR_CellInfo_GetIsFDD(p_NR_CellId)){ v_GrantPeriodicity := 1; // Grant repeated in every slot v_Delta1 := 0; v_DeltaTDLmax := 7; if (f_NR_CellInfo_GetHalfDuplexFDD(p_NR_CellId)) { select (p_Nms) { //@sic R5-232191 R5-237429 R5s240042 sic@ case (10, 16) {v_DeltaTDLmax := 2;} case (6) {v_DeltaTDLmax := 6;} case else { FatalError(__FILE__, __LINE__, "Invalid Nsf"); } } } } else { // TDD select(v_SCS) { case(kHz15) { v_GrantPeriodicity := 5; //Grant repeated every 5 slots v_Delta1 := (5 - (v_T1Slots + v_Nslots)) mod 5; select (p_Nms) { //@sic R5-237429 R5s240042 sic@ case (10, 16) {v_DeltaTDLmax := 7;} case (6) {v_DeltaTDLmax := 11;} case else { FatalError(__FILE__, __LINE__, "Invalid Nsf"); } } } case(kHz30) { v_GrantPeriodicity := 10; //Grant repeated every 10 slots v_Delta1 := (4 - (v_T1Slots + v_Nslots) mod 10) mod 10; if(p_harq_timing_indicator == true) {v_Delta1:=13;} select (p_Nms) { //@sic R5-237429 R5s240042 sic@ case (10, 16) {v_DeltaTDLmax := 17;} case (6) {v_DeltaTDLmax := 25;} case else { FatalError(__FILE__, __LINE__, "Invalid Nsf"); } } } case(kHz120) { v_GrantPeriodicity := 5; //Grant repeated every 5 slots v_Delta1 := (5- (v_T1Slots + v_Nslots) mod 5) mod 5; select (p_Nms) { //@sic R5-237429 R5s240042 sic@ case (10, 16) {v_DeltaTDLmax := 32;} case (6) {v_DeltaTDLmax := 24;} case else { FatalError(__FILE__, __LINE__, "Invalid Nsf"); } } } case else { FatalError(__FILE__, __LINE__, "Invalid SCS"); } } } //Calculate T3 - UL grant when CSS is used v_T3Slots := v_T1Slots + v_Nslots + v_Delta1; v_T3 := f_NR_ConvertTimingInSlots_To_Timing(v_T3Slots, v_SCS, v_SlotsNumberPerSubframe); //T1: send DL Msg SRB.send(v_SRB_COMMON_REQ); //Enable HARQ ack/nack reporting f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_UL_HARQ(enable), cs_TimingInfo_NR(v_T1)); //@sic R5s210842 change 2.4 sic@ //Enable RACH preamble reporting f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_RachPreamble(enable)); //T3 configure SS with periodic UL Grant at T3 f_NR_ULGrantConfiguration_Start(p_NR_CellId, cs_TimingInfo_NR(v_T3), -, -, -, cs_UL_GrantConfig_ContPeriodic(v_GrantPeriodicity)); // Start WatchDog timer v_Tnow := f_NR_GetCurrentTiming(p_NR_CellId); t_Watchdog.start (int2float(f_SubFrameTiming_Duration(v_Tnow, v_T1) + p_Nms + 50) / 1000.0); /* 50ms added to be at the safe side */ alt { /* HARQ NACK */ []SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(p_NR_CellId, nack)) { v_DL_RetransmissionCnt := v_DL_RetransmissionCnt + 1; if (v_DL_RetransmissionCnt > 4) { t_Watchdog.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "DL Msg retransmissions > 4"); } else { repeat; } } /* HARQ ACK */ []SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(p_NR_CellId, ack))-> value v_SYS_IND { v_T2 := v_SYS_IND.Common.TimingInfo.SubFrame; f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_UL_HARQ(disable), -, tsc_NoCnfReq); /* disable indication of HARQ ack/nack; @sic R5s220683 sic@ */ v_T2Flag := true; repeat; } /* HARQ error indication */ [] SYSIND.receive (car_NR_SYSTEM_HARQ_ERROR(p_NR_CellId, cr_NR_HarqErrorUL_Any)) { f_SetVerdictInconc(__FILE__, __LINE__, p_TestcaseStep & "HARQ error for UL transmission in RRC Delay test"); t_Watchdog.stop; } //@sic R5s221179 sic@ /* UE initiates RACH procedure */ [] SYSIND.receive(car_NR_PRACH_Preamble_IND(?, cr_TimingInfo_Any, ?)) { f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_RachPreamble(disable), -, tsc_NoCnfReq); // @sic R5s220683 sic@ v_RACH_ProcedureStarted := true; repeat; //@sic R5s220813 sic@ } [] SRB.receive(p_SRB_COMMON_IND) -> value v_ReceivedNR_SRB_COMMON_IND { v_T4 := v_ReceivedNR_SRB_COMMON_IND.Common.TimingInfo.SubFrame; v_T4Flag := true; t_Watchdog.stop; } [] t_Watchdog.timeout { /* timeout => after T5 */ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "t_Watchdog timeout"); } } //Stop UL Grant f_NR_ULGrantConfiguration_Stop(p_NR_CellId); if (not v_T2Flag) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "UL ACK not received"); } if (not v_T4Flag) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "UL Msg not received"); } v_T4Slots := (((v_T4.SFN.Number * 10) + v_T4.Subframe.Number) * v_SlotsNumberPerSubframe) + f_NR_SubFrameTiming_GetSlotNumber(v_T4) + v_MaxNumberSlots; if (not v_RACH_ProcedureStarted) { // No RACH procedure was initiated by UE // Timing calculation as per 38.523-3 clause 7.3.7.2 //Check the Delay v_T2Slots := (((v_T2.SFN.Number * 10) + v_T2.Subframe.Number) * v_SlotsNumberPerSubframe) + f_NR_SubFrameTiming_GetSlotNumber(v_T2) + v_MaxNumberSlots; v_T4Slots := (((v_T4.SFN.Number * 10) + v_T4.Subframe.Number) * v_SlotsNumberPerSubframe) + f_NR_SubFrameTiming_GetSlotNumber(v_T4) + v_MaxNumberSlots; //Initialise Delta2 if (f_NR_CellInfo_GetIsFDD(p_NR_CellId)){ v_Delta2 := 0; } else { //TDD select(v_SCS) { case(kHz15) { v_Delta2 := (5 - (v_T2Slots - v_K1 + v_Nslots)) mod 5; } case(kHz30) { v_Delta2 := (4 - (v_T2Slots - v_K1 + v_Nslots) mod 10) mod 10; if(p_harq_timing_indicator == true) {v_Delta2:=13;} } case(kHz120) { v_Delta2 := (5- (v_T2Slots - v_K1 + v_Nslots) mod 5) mod 5; } case else { FatalError(__FILE__, __LINE__, "Invalid SCS"); } } } if ( ((v_T4Slots - v_K2) - (v_T2Slots - v_K1))mod v_MaxNumberSlots <= (v_Nslots + v_Delta2)) { f_NR_PreliminaryPass(__FILE__, __LINE__, p_TestcaseStep); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep); } } else { // RACH procedure was initiated by UE // Timing calculation as per 38.523-3 clause 7.3.7.3 //v_T1Slots := (((v_T1.SFN.Number * 10) + v_T1.Subframe.Number) * v_SlotsNumberPerSubframe) + f_NR_SubFrameTiming_GetSlotNumber(v_T1) + v_MaxNumberSlots; //@sic R5s220683 sic@ if ( (v_T4Slots - v_T1Slots) <= ( v_Nslots + v_DeltaTDLmax) ) { //@sic R5s220683 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, p_TestcaseStep); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep); } } return v_ReceivedNR_SRB_COMMON_IND; }