3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240612

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0954 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_GERAN_508CheckCampOnNewGeranCell |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2024-09-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current TTCN implementation, the function call f_IP_Handling_StopPDN() is used to perform the routing table reset to handle the RAU procedure. However this procedure takes longer as it involves resetting the IPv4 and IPv6 TCP connections for PDN_1 and before this can finish, UE can trigger “RACH” on Geran Cell e.g. at Step 5b4 for test case 19.3.2 after the disconnection of call. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The function call f_IP_Handling_StopPDN() is replaced with f_IP_Routing_ChangeToRawmode(IP). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 19.3.2, 19.3.2b, 19.3.2c |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required.	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk24 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required.

## Change 1

| Function name | function f_GERAN_508CheckCampOnNewGeranCell |
| --- | --- |
| Reason for change | In the current TTCN implementation, the function call f_IP_Handling_StopPDN() is used to perform the routing table reset to handle the RAU procedure. However this procedure takes longer as it involves resetting the IPv4 and IPv6 TCP connections for PDN_1 and before this can finish, UE can trigger “RACH” on Geran Cell e.g. at Step 5b4 for test case 19.3.2 after the disconnection of call. |
| Summary of change | The function call f_IP_Handling_StopPDN() is replaced with f_IP_Routing_ChangeToRawmode(IP). |
| TTCN module | GERAN_CommonFunctions.ttcn |
| MCC160 Comment |  |

Before Change:

function f_GERAN_508CheckCampOnNewGeranCell(GERAN_CellId_Type p_CellId, boolean p_WaitForXID := true, integer p_NSAPI := 0, // @sic R5s130883 sic@ boolean p_SGSNChange := false) runs on GERAN_PTC { f_IP_Handling_StopPDN(IP, cs_IP_StopPDN(PDN_1)); // @sic R5s230518 sic@ f_GERAN_RAU(p_CellId, '0'B, (tsc_I_UpdateTypeRAOnly, tsc_I_UpdateTypeCombined, tsc_I_UpdateTypeCombinedWithIMSI), true, p_WaitForXID, p_NSAPI, p_SGSNChange); // @sic R5s110087, R5s110720, R5s120967, R5s140158, R5s140876, R5-153114 sic@ f_GERAN_PreliminaryPass(__FILE__, __LINE__, ""); }

After Change:

function f_GERAN_508CheckCampOnNewGeranCell(GERAN_CellId_Type p_CellId, boolean p_WaitForXID := true, integer p_NSAPI := 0, // @sic R5s130883 sic@ boolean p_SGSNChange := false) runs on GERAN_PTC { f_IP_Routing_ChangeToRawmode(IP); f_GERAN_RAU(p_CellId, '0'B, (tsc_I_UpdateTypeRAOnly, tsc_I_UpdateTypeCombined, tsc_I_UpdateTypeCombinedWithIMSI), true, p_WaitForXID, p_NSAPI, p_SGSNChange); // @sic R5s110087, R5s110720, R5s120967, R5s140158, R5s140876, R5-153114 sic@ f_GERAN_PreliminaryPass(__FILE__, __LINE__, ""); }