3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240615

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0956 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to f_IMS_MOCallSetup_ReceiveUpdateSend200OK function |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-09-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test cases (see Clauses affected) using a tel uri with px_IMS_CalleeContactUri will fail the test case. Reason for this comes from function f_IMS_MOCallSetup_NR5GC_WithPreconditions_Step4_7 and input parameter passed to set the expected Request Line for UPDATE message. In the case of setting a tel uri, when following line is executed v_ContactUrlStr := f_SIP_SipUrl2Str(v_ContactUrl); The result will be a string with subscriber (number only). Later in function, f_IMS_MOCallSetup_ReceiveUpdateSend200OK, calling f_SIP_ParseURI which will be used for reception template to set the expected request line of UPDATE, will expect subscriber but urlparameters will be expected as omit, while UE will include them in UPDATE message, causing test case to fail. Proposal in this CR, is relax the check for url parameters from omit to “*”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replaced f_SIP_ParseURI with f_SIP_BuildSipUri_RX. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Test cases using TS 34.229-5 A.4.1a, A.25, A.19 and A.15.1a Annexes in their test case steps. In addition of 7.20, 7.24a and 7.24b. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Online, , 8th Dec 2023 - 31st Dec 2024	1

Table of Contents	2

Overview	2

1	Corrections required:	2

1.1	Change to function f_IMS_MOCallSetup_ReceiveUpdateSend200OK	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2024-06_D24wk37 related to the title of this CR.

Contact: 	Juan Garcia Martin

Juan.GarciaMartin@rohde-schwarz.com

- Corrections required:

  - Change to function f_IMS_MOCallSetup_ReceiveUpdateSend200OK

| Function name | f_IMS_MOCallSetup_ReceiveUpdateSend200OK |
| --- | --- |
| Reason for change | In the case of setting a tel uri, when following line is executed v_ContactUrlStr := f_SIP_SipUrl2Str(v_ContactUrl); The result will be a string with subscriber (number only). Later in function, f_IMS_MOCallSetup_ReceiveUpdateSend200OK, calling f_SIP_ParseURI which will be used for reception template to set the expected request line of UPDATE, will expect a subscriber but urlparamters will be expected as omit, while UE will include them in UPDATE, causing test case to fail. Proposal in this CR, is relax the check for url parameters from omit to “*”. |
| Summary of change | Replaced f_SIP_ParseURI with f_SIP_BuildSipUri_RX. |
| TTCN module | IMS_Procedures_CallControl4G5G |
| MCC160 Comment |  |

Before Change:

<< SKIPPED CODE >> function f_IMS_MOCallSetup_ReceiveUpdateSend200OK(INVITE_Request p_InviteRequest, charstring p_ContactUri, template (present) SDP_Message p_ExpectedSdpOffer, boolean p_GeolocationInfoAvailable := false, IMS_CallType_Type p_CallType := NORMAL_CALL, IMS_PreconditionIndication_Type p_PreconditionIndicationForUPDATE := PreconditionRequired) runs on IMS_PTC return SDP_Message { /* p_PreconditionIndicationForUPDATE: specifies whether there is a 'precondition' option tag in the Supported header or in the Require header or not included at all in the UPDATE */ var SipUrl v_ContactUrl := f_SIP_ParseURI(p_ContactUri); /* Contact address of the SS as assigned to the UE e.g. in the 183 Session Progress * NOTE 1: in general the SipUrl is exactly the same as sent to the UE in the 183 Session Progress (f_MessageHeader_GetContactSipUrl) * => the template does not have any matching pattern * NOTE 2: in general px_IMS_CalleeContactUri is used in the 183 Session Progress */ << SKIPPED CODE >>

After Change:

<< SKIPPED CODE >> function f_IMS_MOCallSetup_ReceiveUpdateSend200OK(INVITE_Request p_InviteRequest, charstring p_ContactUri, template (present) SDP_Message p_ExpectedSdpOffer, boolean p_GeolocationInfoAvailable := false, IMS_CallType_Type p_CallType := NORMAL_CALL, IMS_PreconditionIndication_Type p_PreconditionIndicationForUPDATE := PreconditionRequired) runs on IMS_PTC return SDP_Message { /* p_PreconditionIndicationForUPDATE: specifies whether there is a 'precondition' option tag in the Supported header or in the Require header or not included at all in the UPDATE */ var template SipUrl v_ContactUrl := f_SIP_BuildSipUri_RX(p_ContactUri);/* Contact address of the SS as assigned to the UE e.g. in the 183 Session Progress * NOTE 1: in general the SipUrl is exactly the same as sent to the UE in the 183 Session Progress (f_MessageHeader_GetContactSipUrl) * => the template does not have any matching pattern * NOTE 2: in general px_IMS_CalleeContactUri is used in the 183 Session Progress */ << SKIPPED CODE >>