3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240616

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3553 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC testcase 8.1.2.1.5.4 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, LTE_NR_DC_CA_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-10-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the function f_TC_8_1_2_1_5_x_NR5GC_TestBody - spCellConfigDedicated is not filled when configuring spCellConfig ; this leads to UE not having Scheduling Request related configurations which are used in the later part in the case , for data loop back Mac Bearer Routing is not included when sending the reconfiguration message also not considered when receiving the reconfiguration complete at steps 4-5 |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the function f_TC_8_1_2_1_5_x_NR5GC_TestBody - spCellConfigDedicated filled when configuring spCellConfig - Mac Bearer Routing is included for reconfiguration and reconfiguration complete at steps 4-5 . To incorporate mac bearer routing into the reconfiguration complete message, it is proposed to replace the common function f_NR5GC_508RRC_IntraNR_HO_IntraCell_Step9_10 with associated individual function calls. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testcase will not be added to the ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.1.5.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

1.1	Verification Test Summary	4

2	Corrections required	4

2.1	Correction to the Function f_TC_8_1_2_1_5_x_NR5GC_TestBody ()	4

3	Branches executed	10

4	Execution Log Files	10

4.1	MediaTek MT6983	10

5	References	10

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 8.1.2.1.5.4 which is part of the NR5GC testsuite in iwd-TTCN3-B2024-06_D24wk37 TTCN-3

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case: 	8.1.2.1.5.4.NR5GC

ATS Version:	iwd-TTCN3-B2024-06_D24wk37

System Simulator used:	R&S® 5G Protocol Conformance Test platform

UEs used:	MediaTek MT6983

Verification Status:	PASS

# Corrections required

## Correction to the Function f_TC_8_1_2_1_5_x_NR5GC_TestBody ()

| Function name | f_TC_8_1_2_1_5_x_NR5GC_TestBody |
| --- | --- |
| Reason for change | In the function f_TC_8_1_2_1_5_x_NR5GC_TestBody - spCellConfigDedicated is not filled when configuring spCellConfig ; this leads to UE not having Scheduling Request related configurations which are used in the later part in the case , for data loop back - Mac Bearer Routing is not included when sending the reconfiguration message also not considered when receiving the reconfiguration complete at steps 4-5 |
| Summary of change | In the function f_TC_8_1_2_1_5_x_NR5GC_TestBody - spCellConfigDedicated filled when configuring spCellConfig - Mac Bearer Routing is included for reconfiguration and reconfiguration complete at steps 4-5 To incorporate mac bearer routing into the reconfiguration complete message, it is proposed to replace the common function f_NR5GC_508RRC_IntraNR_HO_IntraCell_Step9_10 with associated individual function calls. |
| TTCN module | NR5GC\8_1_2\RRC_Reconfiguration_CA_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_8_1_2_1_5_x_NR5GC_TestBody(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell, template (omit) SCellConfig.sCellState_r16 p_SCellState := omit ) runs on NR5GC_PTC { //@sic R5-227562: added p_SCellState sic@ var template (value) CellGroupConfig v_SCGConfig; var SubFrameTiming_Type v_Timing; var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext; var boolean v_ActivateSCell := true; //@sic R5-227562 sic@ var template (value) ServingCellConfigCommon v_ServingCellConfigCommon := f_NR_GetServingCellConfigCommon(p_PCell); var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_PCell); //@siclog Steps 1-2 siclog@ //The SS transmits an RRCReconfigurationmessage containing an sCellToAddModList with SCell NR Cell 3 addition //Check: Does the UE transmits an RRCReconfigurationComplete message? f_NR5GC_AddMod_1SCell( p_PCell, p_SCell,-,-,-,-, p_SCellState); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); //@siclog "Step 3" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB1 and Cell 3? if (isvalue(p_SCellState)) { //@sic R5-227562 sic@ if (valueof(p_SCellState) == activated) { v_ActivateSCell := false; //SCell shall be activated without MAC CE indication } } f_NR5GC_CheckDataPath_CA(p_PCell, p_SCell, "Step 3", v_ActivateSCell ); //@siclog "Steps 4-5" siclog@ //The SS transmits an RRCReconfigurationmessage containing an sCellToAddModList with SCell NR Cell 3 modification. //Check: Does the UE transmit an RRCReconfigurationComplete message? v_SCGConfig := f_NR_GetSCGConfig_SCellAdd(p_SCell, tsc_NR_SCellIndex_1, -, -, -, -, p_SCellState); v_SCGConfig.sCellToAddModList[0].sCellConfigCommon := omit; v_SCGConfig.sCellToAddModList[0].sCellConfigDedicated.pdsch_ServingCellConfig.setup.nrofHARQ_ProcessesForPDSCH := n16; //@sic R5-212046 sic@ //Set firstActiveDownlinkBWP_Id as per Table 8.1.2.1.5.1.1.3.3-4: ServingCellConfig //@sic R5-212046 sic@ v_SCGConfig.sCellToAddModList[0].sCellConfigDedicated.firstActiveDownlinkBWP_Id := omit; v_SCGConfig.sCellToAddModList[0].sCellConfigDedicated.uplinkConfig := omit; if (ispresent(p_SCellState)) { //Add reconfigurationWithSync for Tc 8.1.2.1.5.4/5/6 //@sic R5-237353 sic@ v_SCGConfig.spCellConfig := cs_NR_SpCellConfigDef(omit,cs_NR_ReconfigurationWithSyncDef(v_ServingCellConfigCommon, tsc_C_RNTI_Value3, -, omit), omit, omit); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_SCGConfig))); //Partial HO Intra-Cell procedure with no RLC/PDCO re-establishment v_Timing := f_NR_GetNextSendOccasion_DL(p_PCell, tsc_NR_DelayBeforeIntraCellHO); SRB.send(cas_NR_SRB1_RrcPdu_REQ(p_PCell, cs_TimingInfo_NR(v_Timing), cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, -, -, -, v_V1530Ext))); // At T + 5ms: Configure RACH procedure C-RNTI based v_Timing := f_SubFrameTiming_AddMilliSeconds(v_Timing, 5); f_NR_SS_C_RNTI_Config(p_PCell, tsc_C_RNTI_Value3, cs_TimingInfo_NR(v_Timing), tsc_NoCnfReq, tsc_FollowOnFlag); f_NR_SS_RachProcedureConfigHO(p_PCell, cs_NR_RachProcedureConfig_CRNTI_HO(v_NR_PhysicalParameters, tsc_C_RNTI_Value3), cs_TimingInfo_NR(v_Timing)); f_NR_CellInfo_SetRNTI(p_PCell, tsc_C_RNTI_Value3); //Reconfigure SCell in SS f_NR_SS_CommonCellConfig(p_SCell, cads_NR_PDSCH_Config_REQ(p_SCell, cs_TimingInfo_NR(v_Timing), cs_NR_PDSCH_CellLevelConfigDef(v_SCGConfig.sCellToAddModList[0].sCellConfigDedicated.pdsch_ServingCellConfig.setup))); f_NR5GC_508RRC_IntraNR_HO_IntraCell_Step9_10(p_PCell); } else { v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_SCGConfig))); v_Timing := f_NR_GetNextSendOccasion_DL(p_PCell); //Reconfigure SCell in SS f_NR_SS_CommonCellConfig(p_SCell, cads_NR_PDSCH_Config_REQ(p_SCell, cs_TimingInfo_NR(v_Timing), cs_NR_PDSCH_CellLevelConfigDef(v_SCGConfig.sCellToAddModList[0].sCellConfigDedicated.pdsch_ServingCellConfig.setup))); //Send RRCReconfiguration to UE and wait for the complete f_NR_SendRRCReconfiguration(p_PCell, tsc_NR_RbId_SRB1, -, v_V1530Ext, -, -,cs_TimingInfo_NR(v_Timing), -, -, crs_MacBearerRouting(p_PCell)); } f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); //@siclog "Step 6" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB1 and Cell 3? f_NR5GC_CheckDataPath_CA(p_PCell, p_SCell, "Step 6", false); //@siclog "Steps 7-8" siclog@ //The SS transmits an RRCReconfigurationmessage containing an sCellToReleaseList with SCell NR Cell 3. //Check: Does the UE transmit an RRCReconfigurationComplete message? f_NR5GC_Rel_1SCell(p_PCell, p_SCell); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8"); }

After Change:

function f_TC_8_1_2_1_5_x_NR5GC_TestBody(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell, template (omit) SCellConfig.sCellState_r16 p_SCellState := omit ) runs on NR5GC_PTC { //@sic R5-227562: added p_SCellState sic@ var template (value) CellGroupConfig v_SCGConfig; var SubFrameTiming_Type v_Timing; var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext; var boolean v_ActivateSCell := true; //@sic R5-227562 sic@ var template (value) ServingCellConfigCommon v_ServingCellConfigCommon := f_NR_GetServingCellConfigCommon(p_PCell); var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_PCell); var template (value) ServingCellConfig v_ServingCellConfig:= f_NR_GetServingCellConfig(p_PCell); //WA#WI=1328238 //@siclog Steps 1-2 siclog@ //The SS transmits an RRCReconfigurationmessage containing an sCellToAddModList with SCell NR Cell 3 addition //Check: Does the UE transmits an RRCReconfigurationComplete message? f_NR5GC_AddMod_1SCell( p_PCell, p_SCell,-,-,-,-, p_SCellState); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); //@siclog "Step 3" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB1 and Cell 3? if (isvalue(p_SCellState)) { //@sic R5-227562 sic@ if (valueof(p_SCellState) == activated) { v_ActivateSCell := false; //SCell shall be activated without MAC CE indication } } f_NR5GC_CheckDataPath_CA(p_PCell, p_SCell, "Step 3", v_ActivateSCell ); //@siclog "Steps 4-5" siclog@ //The SS transmits an RRCReconfigurationmessage containing an sCellToAddModList with SCell NR Cell 3 modification. //Check: Does the UE transmit an RRCReconfigurationComplete message? v_SCGConfig := f_NR_GetSCGConfig_SCellAdd(p_SCell, tsc_NR_SCellIndex_1, -, -, -, -, p_SCellState); v_SCGConfig.sCellToAddModList[0].sCellConfigCommon := omit; v_SCGConfig.sCellToAddModList[0].sCellConfigDedicated.pdsch_ServingCellConfig.setup.nrofHARQ_ProcessesForPDSCH := n16; //@sic R5-212046 sic@ //Set firstActiveDownlinkBWP_Id as per Table 8.1.2.1.5.1.1.3.3-4: ServingCellConfig //@sic R5-212046 sic@ v_SCGConfig.sCellToAddModList[0].sCellConfigDedicated.firstActiveDownlinkBWP_Id := omit; v_SCGConfig.sCellToAddModList[0].sCellConfigDedicated.uplinkConfig := omit; if (ispresent(p_SCellState)) { //Add reconfigurationWithSync for Tc 8.1.2.1.5.4/5/6 //@sic R5-237353 sic@ v_SCGConfig.spCellConfig := cs_NR_SpCellConfigDef(omit,cs_NR_ReconfigurationWithSyncDef(v_ServingCellConfigCommon, tsc_C_RNTI_Value3, -, omit), omit, v_ServingCellConfig); //WA#WI=1328238 v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_SCGConfig))); //Partial HO Intra-Cell procedure with no RLC/PDCO re-establishment v_Timing := f_NR_GetNextSendOccasion_DL(p_PCell, tsc_NR_DelayBeforeIntraCellHO); // Mac Bearer Routing is included //WA#WI=1328238 SRB.send(cas_NR_SRB1_RrcPdu_REQ(p_PCell, cs_TimingInfo_NR(v_Timing), cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, -, -, -, v_V1530Ext),crs_MacBearerRouting(p_PCell))); // At T + 5ms: Configure RACH procedure C-RNTI based v_Timing := f_SubFrameTiming_AddMilliSeconds(v_Timing, 5); f_NR_SS_C_RNTI_Config(p_PCell, tsc_C_RNTI_Value3, cs_TimingInfo_NR(v_Timing), tsc_NoCnfReq, tsc_FollowOnFlag); f_NR_SS_RachProcedureConfigHO(p_PCell, cs_NR_RachProcedureConfig_CRNTI_HO(v_NR_PhysicalParameters, tsc_C_RNTI_Value3), cs_TimingInfo_NR(v_Timing)); f_NR_CellInfo_SetRNTI(p_PCell, tsc_C_RNTI_Value3); //Reconfigure SCell in SS f_NR_SS_CommonCellConfig(p_SCell, cads_NR_PDSCH_Config_REQ(p_SCell, cs_TimingInfo_NR(v_Timing), cs_NR_PDSCH_CellLevelConfigDef(v_SCGConfig.sCellToAddModList[0].sCellConfigDedicated.pdsch_ServingCellConfig.setup))); //f_NR5GC_508RRC_IntraNR_HO_IntraCell_Step9_10(p_PCell); //WA#WI=1328238 // Calling each function calls separately as we need to include crs_MacBearerRouting in the reconfiguration complete SRB.receive(car_NR_SRB1_RrcPdu_IND(p_PCell, cr_38508_RRCReconfigurationComplete,-,crs_MacBearerRouting(p_PCell))); //WA#WI=1328238 //Step 9 (after step 8) Re-configure RACH procedure as for initial access, enable TA transmissions. //NOTE 4: The FollowOnFlag is set to true in the ASP reconfiguring RACH. f_NR_SS_RachProcedureConfig_Def(p_PCell, -, tsc_NoCnfReq, tsc_FollowOnFlag); //@sic R5-206369 sic@ //WA#WI=1328238 // enable TA transmissions f_NR_ULGrantConfiguration_Common(p_PCell, -, -, cs_NR_UplinkTimeAlignment_Start, cs_UL_GrantConfig_OnSR); //WA#WI=1328238 } else { v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_SCGConfig))); v_Timing := f_NR_GetNextSendOccasion_DL(p_PCell); //Reconfigure SCell in SS f_NR_SS_CommonCellConfig(p_SCell, cads_NR_PDSCH_Config_REQ(p_SCell, cs_TimingInfo_NR(v_Timing), cs_NR_PDSCH_CellLevelConfigDef(v_SCGConfig.sCellToAddModList[0].sCellConfigDedicated.pdsch_ServingCellConfig.setup))); //Send RRCReconfiguration to UE and wait for the complete f_NR_SendRRCReconfiguration(p_PCell, tsc_NR_RbId_SRB1, -, v_V1530Ext, -, -,cs_TimingInfo_NR(v_Timing), -, -, crs_MacBearerRouting(p_PCell)); } f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); //@siclog "Step 6" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB1 and Cell 3? f_NR5GC_CheckDataPath_CA(p_PCell, p_SCell, "Step 6", false); //@siclog "Steps 7-8" siclog@ //The SS transmits an RRCReconfigurationmessage containing an sCellToReleaseList with SCell NR Cell 3. //Check: Does the UE transmit an RRCReconfigurationComplete message? f_NR5GC_Rel_1SCell(p_PCell, p_SCell); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8"); }

# Branches executed

## This NR TC was executed in NR5GC CA FR1 Intra-band Contiguous CA Band CA_n78C (MT6983) with NEA1 and NIA1 ciphering and integrity algorithms.

# Execution Log Files

## MediaTek MT6983

The MediaTek MT6983 passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:
TC_8_1_2_1_5_4_MTK.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s240617: Supporting information for addition of NR5GC testcase 8.1.2.1.5.4 in FR1 |
| --- | --- |