3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240622

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3555 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC test case 8.1.2.1.5.6 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, LTE_NR_DC_CA_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-10-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the function f_TC_8_1_2_1_5_x_NR5GC_TestBody - spCellConfigDedicated is not filled when configuring spCellConfig ; this leads to UE not having Scheduling Request related configurations which are used in the later part in the case , for data loop back Mac Bearer Routing is not included when sending the reconfiguration message also not considered when receiving the reconfiguration complete at steps 4-5 |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the function f_TC_8_1_2_1_5_x_NR5GC_TestBody - spCellConfigDedicated filled when configuring spCellConfig - Mac Bearer Routing is included for reconfiguration and reconfiguration complete at steps 4-5 . To incorporate mac bearer routing into the reconfiguration complete message, it is proposed to replace the common function f_NR5GC_508RRC_IntraNR_HO_IntraCell_Step9_10 with associated individual function calls. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testcase will not be added to the ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.1.5.6.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Correction to the Function f_TC_8_1_2_1_5_x_NR5GC_TestBody ()	3

3	Branches executed	9

4	Execution Log Files	9

4.1	MediaTek MT6983	9

5	References	9

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 8.1.2.1.5.6 which is part of the NR5GC test suite in iwd-TTCN3-B2024-06_D24wk37 TTCN-3

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case: 	8.1.2.1.5.6.NR5GC

ATS Version:	iwd-TTCN3-B2024-06_D24wk37

System Simulator used:	R&S® 5G Protocol Conformance Test platform

UEs used:	MediaTek MT6983

Verification Status:	PASS

# Corrections required

Same as R5s240616

# Branches executed

## This NR TC was executed in NR5GC CA FR1 Inter-band CA_n41A_n79A (MT6983) with NEA1 and NIA1 ciphering and integrity algorithms

# Execution Log Files

## MediaTek MT6983

The MediaTek MT6983 passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:
TC_8_1_2_1_5_6_MTK.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s240621: Supporting information for addition of NR5GC testcase 8.1.2.1.5.6 |
| --- | --- |