3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240632

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4861 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_EUTRA_CellInfo_Init |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2024-10-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current tTCN implementation of 6.1.2.15b , f_EUTRA_Init fl_EUTRA_Common_Init f_EUTRA_CellArray_Init f_EUTRA_CellInfo_Init f_EUTRA_InitFrequency_f1Tof4 fl_EUTRA_InitFrequencyDef_f1Tof4 case (13) { // Band 13 @sic R5-101207 sic@ v_Freq.f1 := v_FreqLMH.FrequencyMid; v_Freq.f2 := cs_DummyCarrierFreqEUTRA; v_Freq.f3 := cs_DummyCarrierFreqEUTRA; } Is assigned. This means that ul_CarrierFreq_r9 is set as OMIT. Later in the function f_EUTRA_Init fl_EUTRA_Common_Init f_EUTRA_CellArray_Init f_EUTRA_CellInfo_Init f_EUTRA_InitFrequency_f1Tof4 following is assigned : case (eutra_Cell3, eutra_Cell12, eutra_Cell23, eutra_Cell28, eutra_Cell29) { // @sic R5-121888 sic@ v_UL_DL_Earfcn := v_Freq.f2; // f2 } This causes testcase failure when testcase 6.1.2.15b is run with Band 13 and px_SinglePLMN_Tested = MultiPLMNinPrimaryBand. With this setting, when the function f_EUTRA_CellConfig_Def (eutra_Cell3); is called, it throws a runtime error : f_EUTRA_CellConfig_Def (eutra_Cell3); f_EUTRA_SS_ConfigureActiveCell v_UplinkStaticCellInfoUL:= cs_UplinkStaticCellInfo( // @sic R5-134786 sic@ v_CellInfo.Frequency.UL_DL_Earfcn.ul_CarrierFreq_r9 , //@sic R5s160858 sic@ v_CellInfo.Frequency.UL_ChBandwidth); Also, according to 36.508 Table 6.3.2.1-1a, for 10MHz bandwidth for Band 13 this should be set to either f1 or f5. This needs to be corrrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the TTCN implementation of function f_EUTRA_CellInfo_Init to handle the above issue. Please see detailed change below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.15b |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2024-06_D24wk37 related to the title of this CR.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | f_EUTRA_CellInfo_Init |
| --- | --- |
| Reason for change | In the current tTCN implementation of 6.1.2.15b , f_EUTRA_Init fl_EUTRA_Common_Init f_EUTRA_CellArray_Init f_EUTRA_CellInfo_Init f_EUTRA_InitFrequency_f1Tof4 fl_EUTRA_InitFrequencyDef_f1Tof4 case (13) { // Band 13 @sic R5-101207 sic@ v_Freq.f1 := v_FreqLMH.FrequencyMid; v_Freq.f2 := cs_DummyCarrierFreqEUTRA; v_Freq.f3 := cs_DummyCarrierFreqEUTRA; } Is assigned. This means that ul_CarrierFreq_r9 is set as OMIT. Later in the function f_EUTRA_Init fl_EUTRA_Common_Init f_EUTRA_CellArray_Init f_EUTRA_CellInfo_Init f_EUTRA_InitFrequency_f1Tof4 following is assigned : case (eutra_Cell3, eutra_Cell12, eutra_Cell23, eutra_Cell28, eutra_Cell29) { // @sic R5-121888 sic@ v_UL_DL_Earfcn := v_Freq.f2; // f2 } This causes testcase failure when testcase 6.1.2.15b is run with Band 13 and px_SinglePLMN_Tested = MultiPLMNinPrimaryBand. With this setting, when the function f_EUTRA_CellConfig_Def (eutra_Cell3); is called, it throws a runtime error : f_EUTRA_CellConfig_Def (eutra_Cell3); f_EUTRA_SS_ConfigureActiveCell v_UplinkStaticCellInfoUL:= cs_UplinkStaticCellInfo( // @sic R5-134786 sic@ v_CellInfo.Frequency.UL_DL_Earfcn.ul_CarrierFreq_r9 , //@sic R5s160858 sic@ v_CellInfo.Frequency.UL_ChBandwidth); Also, according to 36.508 Table 6.3.2.1-1a, for 10MHz bandwidth for Band 13 this should be set to either f1 or f5. This needs to be corrrected. |
| Summary of change | Updated the TTCN implementation of function f_EUTRA_CellInfo_Init to handle the above issue. Please see detailed change below. |
| TTCN module | Eutra_CellInfoInit.ttcn |
| MCC160 Comment |  |

Before Change:

function f_EUTRA_CellInfo_Init(EUTRA_CellId_Type p_CellId, integer p_FrequencyBand, Combination_Type p_SysinfoCombination, Dl_Bandwidth_Type p_DL_Bandwidth, Ul_Bandwidth_Type p_UL_Bandwidth) return EUTRA_CellInfo_Type { /* @sic R5-096641 sic@ */ //@sic R5-163085 p_FrequencyBand type changed to integer sic@ var CellIdentities_Type v_CellIds := fl_EUTRA_InitCellIdentities(p_CellId); var EUTRA_FDD_TDD_CellInfo_Type v_FDD_TDD_Info := f_EUTRA_InitFDD_TDD_FromBand(p_FrequencyBand); var EUTRA_FDD_TDD_Mode_Type v_FDD_TDD := v_FDD_TDD_Info.FDD_TDD; var CellTimingInfo_Type v_Celltiming := fl_EUTRA_InitialiseCellTiming(p_CellId, v_FDD_TDD, LTE_AllCellsOnSamePLMN); //the parameter "LTE_AllCellsOnSamePLMN" is dummy for RRC cells, it is used only for NAS test cases var Combination_Type v_SysinfoCombination := fl_EUTRA_CheckSysinfoCombination(p_CellId, p_SysinfoCombination); var RootSequenceIndex_Type v_RootSequenceIndex_Type := f_EUTRA_InitialisePRACH_RootSequenceIndex(p_CellId, v_FDD_TDD, LTE_AllCellsOnSamePLMN); var RACH_TimingAdvance_Type v_RAR_TA := fl_EUTRA_InitialiseRAR_TA(v_FDD_TDD, v_Celltiming); // @sic R5-121777 sic@ var CellCarrierFreqEUTRA_Type v_UL_DL_Earfcn; var NAS_Parameter_Type v_NAS_Param; var template (value) BcchInfo_Type v_BCCH_Info; var template (value) EUTRA_CellInfo_Type v_CellInfo; var Frequency_fList_Type v_Freq; var ChannelBandwidthDependency_Type v_ChannelBandwidthDependency := f_EUTRA_BandDependentParam(p_DL_Bandwidth, p_UL_Bandwidth); //@sic R5s090180: return CFI value sic@ var SysinfoType_Type v_SysinfoType := sysinfoOnly; //@sic R5-167569 sic@ var template (omit) Bcch_BRInfo_Type v_BCCH_BR_Info := omit; //@sic R5-167569 sic@ // Initialise Frequencies f1/f2/f3/f4 v_Freq := f_EUTRA_InitFrequency_f1Tof4(p_FrequencyBand, p_DL_Bandwidth, p_UL_Bandwidth); // Initialised Frequency per cell. See 36.508 Table 6.2.3.1-1 // Cell 1/2/4/11 -> Frequency f1 // Cell 3/12 -> Frequency f2 // Cell 6/13 -> Frequency f3 // Cell 14 -> Frequency f4 select (p_CellId) { case (eutra_Cell1, eutra_Cell2, eutra_Cell4, eutra_Cell11, eutra_Cell10, eutra_Cell30, eutra_Cell31) { v_UL_DL_Earfcn := v_Freq.f1; //f1 or f5 } case (eutra_Cell3, eutra_Cell12, eutra_Cell23, eutra_Cell28, eutra_Cell29) { // @sic R5-121888 sic@ v_UL_DL_Earfcn := v_Freq.f2; // f2 } case (eutra_Cell6, eutra_Cell13) { v_UL_DL_Earfcn := v_Freq.f3; // f3 } case (eutra_Cell14) { v_UL_DL_Earfcn := v_Freq.f4; // f4 } case else { FatalError(__FILE__, __LINE__, "invalid cell Idendtity"); } } // Initialise NAS parameters v_NAS_Param := fl_EUTRA_InitNAS_forRRC_Cells(p_CellId, p_FrequencyBand, p_SysinfoCombination); //@sic R5s130498 sic@ // Initialise sysinfoType v_SysinfoType := f_EUTRA_InitSysinfoType(); // Initialise SYSINFO v_BCCH_Info := f_EUTRA_InitSystemInformation(p_CellId, v_CellIds.CellIdentity, v_SysinfoCombination, p_FrequencyBand, p_DL_Bandwidth, p_UL_Bandwidth, cs_PlmnIdList_1Entry(v_NAS_Param.Guti_Parameters.PLMN_Identity), v_NAS_Param.TrackingAreaCode, v_FDD_TDD_Info, v_RootSequenceIndex_Type, v_SysinfoType, v_UL_DL_Earfcn); if (v_SysinfoType != sysinfoOnly) { v_BCCH_BR_Info := f_EUTRA_InitSystemInformation_BR(v_CellIds.CellIdentity, v_SysinfoCombination, p_FrequencyBand, p_DL_Bandwidth, p_UL_Bandwidth, cs_PlmnIdList_1Entry(v_NAS_Param.Guti_Parameters.PLMN_Identity), v_NAS_Param.TrackingAreaCode, v_FDD_TDD_Info); } v_CellInfo := cs_EUTRA_CellInfoDef(v_CellIds, v_SysinfoCombination, p_FrequencyBand, v_UL_DL_Earfcn, p_DL_Bandwidth, p_UL_Bandwidth, v_NAS_Param, v_BCCH_Info, v_Celltiming, v_FDD_TDD_Info, v_ChannelBandwidthDependency.Cfi, v_RAR_TA, v_ChannelBandwidthDependency.sr_PUCCH_ResourceIndex, // @sic R5s090312 sic@ v_SysinfoType, v_BCCH_BR_Info); // @sic R5s090180 sic@ return valueof(v_CellInfo); // Note: valueof cannot be avoided here }

After Change:

function f_EUTRA_CellInfo_Init(EUTRA_CellId_Type p_CellId, integer p_FrequencyBand, Combination_Type p_SysinfoCombination, Dl_Bandwidth_Type p_DL_Bandwidth, Ul_Bandwidth_Type p_UL_Bandwidth) return EUTRA_CellInfo_Type { /* @sic R5-096641 sic@ */ //@sic R5-163085 p_FrequencyBand type changed to integer sic@ var CellIdentities_Type v_CellIds := fl_EUTRA_InitCellIdentities(p_CellId); var EUTRA_FDD_TDD_CellInfo_Type v_FDD_TDD_Info := f_EUTRA_InitFDD_TDD_FromBand(p_FrequencyBand); var EUTRA_FDD_TDD_Mode_Type v_FDD_TDD := v_FDD_TDD_Info.FDD_TDD; var CellTimingInfo_Type v_Celltiming := fl_EUTRA_InitialiseCellTiming(p_CellId, v_FDD_TDD, LTE_AllCellsOnSamePLMN); //the parameter "LTE_AllCellsOnSamePLMN" is dummy for RRC cells, it is used only for NAS test cases var Combination_Type v_SysinfoCombination := fl_EUTRA_CheckSysinfoCombination(p_CellId, p_SysinfoCombination); var RootSequenceIndex_Type v_RootSequenceIndex_Type := f_EUTRA_InitialisePRACH_RootSequenceIndex(p_CellId, v_FDD_TDD, LTE_AllCellsOnSamePLMN); var RACH_TimingAdvance_Type v_RAR_TA := fl_EUTRA_InitialiseRAR_TA(v_FDD_TDD, v_Celltiming); // @sic R5-121777 sic@ var CellCarrierFreqEUTRA_Type v_UL_DL_Earfcn; var NAS_Parameter_Type v_NAS_Param; var template (value) BcchInfo_Type v_BCCH_Info; var template (value) EUTRA_CellInfo_Type v_CellInfo; var Frequency_fList_Type v_Freq; var ChannelBandwidthDependency_Type v_ChannelBandwidthDependency := f_EUTRA_BandDependentParam(p_DL_Bandwidth, p_UL_Bandwidth); //@sic R5s090180: return CFI value sic@ var SysinfoType_Type v_SysinfoType := sysinfoOnly; //@sic R5-167569 sic@ var template (omit) Bcch_BRInfo_Type v_BCCH_BR_Info := omit; //@sic R5-167569 sic@ // Initialise Frequencies f1/f2/f3/f4 v_Freq := f_EUTRA_InitFrequency_f1Tof4(p_FrequencyBand, p_DL_Bandwidth, p_UL_Bandwidth); // Initialised Frequency per cell. See 36.508 Table 6.2.3.1-1 // Cell 1/2/4/11 -> Frequency f1 // Cell 3/12 -> Frequency f2 // Cell 6/13 -> Frequency f3 // Cell 14 -> Frequency f4 select (p_CellId) { case (eutra_Cell1, eutra_Cell2, eutra_Cell4, eutra_Cell11, eutra_Cell10, eutra_Cell30, eutra_Cell31) { v_UL_DL_Earfcn := v_Freq.f1; //f1 or f5 } case (eutra_Cell3) { //WA#WI1341544 WA#6_1_2_15b if (p_UL_Bandwidth == n50) { log("Patch 1 for 6.1.2.15b"); v_UL_DL_Earfcn := v_Freq.f1; // For 10 MHz bandwidth (n50), As per 36.508 Table 6.2.3.1-1a f1 and f5 are initialized. }else { v_UL_DL_Earfcn := v_Freq.f2; // f2 } } case (//WA#WI1341544 WA#6_1_2_15b MOVED eutra_Cell3 out, eutra_Cell12, eutra_Cell23, eutra_Cell28, eutra_Cell29) { // @sic R5-121888 sic@ v_UL_DL_Earfcn := v_Freq.f2; // f2 } case (eutra_Cell6, eutra_Cell13) { v_UL_DL_Earfcn := v_Freq.f3; // f3 } case (eutra_Cell14) { v_UL_DL_Earfcn := v_Freq.f4; // f4 } case else { FatalError(__FILE__, __LINE__, "invalid cell Idendtity"); } } // Initialise NAS parameters v_NAS_Param := fl_EUTRA_InitNAS_forRRC_Cells(p_CellId, p_FrequencyBand, p_SysinfoCombination); //@sic R5s130498 sic@ // Initialise sysinfoType v_SysinfoType := f_EUTRA_InitSysinfoType(); // Initialise SYSINFO v_BCCH_Info := f_EUTRA_InitSystemInformation(p_CellId, v_CellIds.CellIdentity, v_SysinfoCombination, p_FrequencyBand, p_DL_Bandwidth, p_UL_Bandwidth, cs_PlmnIdList_1Entry(v_NAS_Param.Guti_Parameters.PLMN_Identity), v_NAS_Param.TrackingAreaCode, v_FDD_TDD_Info, v_RootSequenceIndex_Type, v_SysinfoType, v_UL_DL_Earfcn); if (v_SysinfoType != sysinfoOnly) { v_BCCH_BR_Info := f_EUTRA_InitSystemInformation_BR(v_CellIds.CellIdentity, v_SysinfoCombination, p_FrequencyBand, p_DL_Bandwidth, p_UL_Bandwidth, cs_PlmnIdList_1Entry(v_NAS_Param.Guti_Parameters.PLMN_Identity), v_NAS_Param.TrackingAreaCode, v_FDD_TDD_Info); } v_CellInfo := cs_EUTRA_CellInfoDef(v_CellIds, v_SysinfoCombination, p_FrequencyBand, v_UL_DL_Earfcn, p_DL_Bandwidth, p_UL_Bandwidth, v_NAS_Param, v_BCCH_Info, v_Celltiming, v_FDD_TDD_Info, v_ChannelBandwidthDependency.Cfi, v_RAR_TA, v_ChannelBandwidthDependency.sr_PUCCH_ResourceIndex, // @sic R5s090312 sic@ v_SysinfoType, v_BCCH_BR_Info); // @sic R5s090180 sic@ return valueof(v_CellInfo); // Note: valueof cannot be avoided here }