3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240640

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-5 | CR | 0121 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of f_MCX_JSON_CreateTokens |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2024-10-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to TS 36.579-1 Table 5.5.4.10.4-1 depending on conditions MCPTT, MCVIDEO and MCDATA the Access token MCX claims "mcptt_id", "mcvideo_id" or "mcdata_id". Nevertheless, in f_MCX_JSON_CreateTokens the claim is hard coded to "mcptt_id". |  |  |  |  |  |  |  |  |
| Summary of change: |  | f_MCX_JSON_CreateTokens corrected to consider the MC service type. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect TTCN implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All 36.579-5 TTCN-3 ATSs |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	TTCN Changes	3

# Overview

This document lists all the changes needed for baseline moving based on iwd-TTCN3-B2024-06_D24wk37.

Contact:	Oliviers Genoud
	olivier.genoud@etsi.org

# TTCN Changes

| Reason for change | According to 36.579-1 Table 5.5.4.10.4-1 depending on conditions MCPTT, MCVIDEO and MCDATA the Access token MCX claims "mcptt_id", "mcvideo_id" or "mcdata_id". Nevertheless in f_MCX_JSON_CreateTokens the claim is hard-coded to "mcptt_id" |
| --- | --- |
| Summary of change | f_MCX_JSON_CreateTokens corrected to consider the MC service type |
| TTCN modules | MCX_JSON_Objects, MCX_HTTP_CommonFunctions |

function fl_MCX_JSON_MCService(MCX_ServiceType_Type p_ServiceType) return template (value) JSON.ObjectMember { var charstring v_MCServiceID := f_MCX_MCService_ID_User_A(p_ServiceType); var charstring v_ServiceName; select (p_ServiceType) { case (MCPTT) { v_ServiceName := "mcptt_id"; } case (MCVideo) { v_ServiceName := "mcvideo_id"; } case (MCData) { v_ServiceName := "mcdata_id"; } case else { FatalError(__FILE__, __LINE__, "Invalid service type"); } } return cs_JSON_ObjectMember_String(v_ServiceName, v_MCServiceID); }

function f_MCX_JSON_CreateTokens( MCX_ServiceType_Type p_ServiceType, charstring p_Client_Id, charstring p_Issuer_Id, CharStringList_Type p_ScopeList, octetstring p_RsaPrivateKey) return MCX_Tokens_Type { var integer v_SystemTime := f_UnixEpoch_TimestampSeconds(); var integer v_ExpirationTime := v_SystemTime + 7199; var template (value) JSON.ObjectMember v_MCService := fl_MCX_JSON_MCService(p_ServiceType); var template (value) JSON.Object.memberList v_TokenHeader := { cs_JSON_ObjectMember_String("kid", "jws-rsa"), cs_JSON_ObjectMember_String("alg", "RS256") }; var template (value) JSON.Object.memberList v_IdTokenPayload := { v_MCService, cs_JSON_ObjectMember_String("sub", "1234567890"), // arbitrarily selected string cs_JSON_ObjectMember_String("aud", p_Client_Id), cs_JSON_ObjectMember_String("iss", p_Issuer_Id), cs_JSON_ObjectMember_Integer("exp", v_ExpirationTime), cs_JSON_ObjectMember_Integer("iat", v_SystemTime) }; var template (value) JSON.Object.memberList v_AccessTokenPayload := { v_MCService, cs_JSON_ObjectMember_StrArray("scope", p_ScopeList), cs_JSON_ObjectMember_Integer("exp", v_ExpirationTime), cs_JSON_ObjectMember_String("client_id", p_Client_Id) }; var charstring v_IdToken := f_JWS_EncodeToken(v_TokenHeader, v_IdTokenPayload, RS256, p_RsaPrivateKey); var charstring v_AccessToken := f_JWS_EncodeToken(v_TokenHeader, v_AccessTokenPayload, RS256, p_RsaPrivateKey); var charstring v_RefreshToken := "Y7NSzUJuS0Jp7G4SKpBKSOJVHIZxFbxqsqCIZhOEk9"; return {v_IdToken, v_AccessToken, v_RefreshToken}; }

function f_MCX_HTTP_UserAuthentication_Step9_10(MCX_OpenIdConnect_ParameterList_Type p_AuthenticationRequest, MCX_ServiceType_Type p_ServiceType) runs on MCX_HTTP_PTC return MCX_SecurityParameters_Type { var MCX_SecurityParameters_Type v_SecurityParameters := f_MCX_SecurityParameters_Init(tsc_MCX_KMS_Hostname); /* tsc_MCX_KMS_Hostname is used as KMS Identifier for Generation of MIKEY-SAKKE UID according to 33.180 F.2.1.1 */ var IP_Connection_Type v_HttpRoutingInfo; var HTTP_DATA_ASP v_HTTP_DATA_ASP; var MCX_OpenIdConnect_ParameterList_Type v_OpenIdConnectRequest; var charstring v_CodeChallenge; var charstring v_CodeVerifier; var charstring v_HttpBody; var MCX_Tokens_Type v_Tokens; var CharStringList_Type v_ScopeList; var MCX_OpenIdConnect_ValueList_Type v_ScopeValueList := f_MCX_OpenIdConnect_ScopeList(p_ServiceType); var integer v_StrLen; var integer i; // step 9: HTTP POST - 36.579-1 Table 5.5.4.3-1 with condition TOKEN v_HTTP_DATA_ASP := f_MCX_HTTP_ReceiveRequest(-, cr_MCX_HttpRequest_POST(tsc_MCX_IdMS_token_UriPath, -, -, cr_ContentTypeUrlEncoded)); v_HttpRoutingInfo := v_HTTP_DATA_ASP.routingInfo; v_OpenIdConnectRequest := f_MCX_OpenID_DecodeAndCheckMessageBody(v_HTTP_DATA_ASP.http.request.messageBody.text, cr_MCX_OpenIdConnect_TokenRequest(tsc_MCX_OpenIdConnect_Code, px_MCX_OAuth_ClientId_A, px_MCX_OAuth_RedirectURI_A)); v_CodeChallenge := f_MCX_OpenID_GetParameter(p_AuthenticationRequest, "code_challenge"); v_CodeVerifier := f_MCX_OpenID_GetParameter(v_OpenIdConnectRequest, "code_verifier"); v_StrLen := lengthof(v_CodeVerifier); if ((v_StrLen < 43) or (v_StrLen > 128)) { /* RFC 7636: code_verifier: high-entropy cryptographic random STRING with a minimum length of 43 characters and a maximum length of 128 characters */ f_MCX_HTTP_SetVerdictFailOrInconc(__FILE__, __LINE__, "code_verifier with invalid length"); } if (v_CodeChallenge != f_Base64urlEncode(oct2bit(fx_SHA_2(SHA_256, char2oct(v_CodeVerifier))))) { // According to 33.179 Table B.3.1.1-1 the code_challenge is "The base64url-encoded SHA-256 challenge derived from the code verifier that is sent in the authorization request" f_MCX_HTTP_SetVerdictFailOrInconc(__FILE__, __LINE__, "code_challenge does not match base64url-encoded SHA-256 hallenge derived from the code verifier"); } // step 10: HTTP 200 (OK) - Table 5.5.4.6-1 with condition TOKEN for (i := 0; i < lengthof(v_ScopeValueList); i := i + 1) { v_ScopeList[i] := v_ScopeValueList[i] }; // convert set-of to record-of v_Tokens := f_MCX_JSON_CreateTokens(p_ServiceType, px_MCX_OAuth_ClientId_A, tsc_MCX_IdMS_token_UriPath, v_ScopeList, px_MCX_TokenRsaPrivateKey); v_HttpBody := f_JSON_EncodeObject(cs_MCX_OpenIdConnect_TokenResponse(v_Tokens.AccessToken, v_Tokens.RefreshToken, v_Tokens.IdToken)); f_MCX_HTTP_Send200OK(v_HttpRoutingInfo, cs_ContentType("application/json", {cs_GenericParam("charset", "UTF-8")}), v_HttpBody); v_SecurityParameters := f_MCX_SecurityParameters_SetTokens(v_SecurityParameters, v_Tokens); return v_SecurityParameters; }