3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240694

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3594 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for Test case 6.3.1.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-12-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | A few UE may trigger deregistrationrequest _MO immediately after or parallelly with emrgency call release. So that UE may fail the case. To handle this scenario default variable for deregistrationrequest_mo should be activated before function f_NR5GC_EmergencyCallRelease. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Default variable for deregistrationrequest_mo moved before f_NR5GC_EmergencyCallRelease at step23 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.7 NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D24wk37 related to the title of this CR.

Contact:	Kalahasti, Narendra

Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | function fl_TC_6_3_1_7_NR5GC_TestBody |
| --- | --- |
| Reason for change | - A few UE may trigger deregistrationrequest _MO immediately after or parallelly with emrgency call release. So that UE may fail the case. To handle this scenario default variable for deregistrationrequest_mo should be activated before function f_NR5GC_EmergencyCallRelease. |
| Summary of change | - Default variable for deregistrationrequest_mo moved before f_NR5GC_EmergencyCallRelease at step23 |
| TTCN module | SOR_NR5GC |
| MCC160 Comment |  |

Before Change

function fl_TC_6_3_1_7_TestBody() runs on NR5GC_PTC { ------skip code----- //@siclog "Step 23" siclog@//@sic R5-216165 sic@ //Generic procedure of "Test Procedure for IMS MO Emergency call release" described in TS 38.508-1 [4] Table 4.9.12A.2.2-1 are performed. f_NR5GC_EmergencyCallRelease(nr_Cell11);//@sic R5s211479 sic@ // The above function won't finish until IMS messages have been sent and PDU session has been released @sic R5s220982 sic@ //@siclog "Step 24" siclog@//@sic R5-216165 sic@ //The SS releases the RRC connection. v_MyDefaultVar := activate(a_NR5GC_Deregistration_REQ(nr_Cell11)); //@sic R5-231404 sic@//moved from here f_NR_RRCRelease(nr_Cell11); deactivate(v_MyDefaultVar); //@sic R5-231404 sic@ }//End of fl_TC_6_3_1_7_TestBody }//end of f_TC_8_1_5_6_1_NR5GC_TestBody

After Change

function fl_TC_6_3_1_7_TestBody() runs on NR5GC_PTC { ------skip code----- //@siclog "Step 23" siclog@//@sic R5-216165 sic@ //Generic procedure of "Test Procedure for IMS MO Emergency call release" described in TS 38.508-1 [4] Table 4.9.12A.2.2-1 are performed. v_MyDefaultVar := activate(a_NR5GC_Deregistration_REQ(nr_Cell11)); f_NR5GC_EmergencyCallRelease(nr_Cell11);//@sic R5s211479 sic@ // The above function won't finish until IMS messages have been sent and PDU session has been released @sic R5s220982 sic@ //@siclog "Step 24" siclog@//@sic R5-216165 sic@ //The SS releases the RRC connection. // v_MyDefaultVar := activate(a_NR5GC_Deregistration_REQ(nr_Cell11)); //@sic R5-231404 sic@ f_NR_RRCRelease(nr_Cell11); deactivate(v_MyDefaultVar); //@sic R5-231404 sic@ }//End of fl_TC_6_3_1_7_TestBody }//end of f_TC_8_1_5_6_1_NR5GC_TestBody