3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240695

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3595 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC Idle mode test case 6.1.2.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-12-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The wait time of 34 sec at Step 2 is derived from TS 38.133, Table 4.2.2.4-1 which does not consider t-ReselectionNR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added t-ReselectionNR to the wait time at step 2 which makes it 41 sec. This change is agreed at RAN5#105 meeting via R5-247542. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required.	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2024-06_D24wk37 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required.

## Change 1

| Function name | function fl_TC_6_1_2_4_TestBody |
| --- | --- |
| Reason for change | The wait time of 34 sec at Step 2 is derived from TS 38.133, Table 4.2.2.4-1 which does not consider t-ReselectionNR. |
| Summary of change | Added t-ReselectionNR to the wait time at step 2 which makes it 41 sec. This change is agreed at RAN5#105 meeting via R5-247542. |
| TTCN module | Idle_CellReSelection_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_6_1_2_4_NR5GC_TestBody() runs on NR5GC_PTC { // @siclog "Step 1" siclog@ // The SS changes NR Cell 10 SS/PBCH EPRE level according to the row "T1" in table 6.1.2.4.3.2-1/2. f_NR_SetCellPower (nr_Cell10, -78, -82); // @siclog "Step 2" siclog@ // Wait for 34 seconds to allow UE to recognise the change. f_Delay(34.0); // @sic R5-20xxxx sic@ // @siclog "Step 3" siclog@ // Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.4.2.2-1 indicate that the UE is camped on NR Cell 10? f_NR5GC_CheckRRCIdle(nr_Cell10); }

After Change:

function f_TC_6_1_2_4_NR5GC_TestBody() runs on NR5GC_PTC { // @siclog "Step 1" siclog@ // The SS changes NR Cell 10 SS/PBCH EPRE level according to the row "T1" in table 6.1.2.4.3.2-1/2. f_NR_SetCellPower (nr_Cell10, -78, -82); // @siclog "Step 2" siclog@ // Wait for 41 seconds to allow UE to recognise the change. f_Delay(41.0); // @sic R5-24xxxx sic@ // @siclog "Step 3" siclog@ // Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.4.2.2-1 indicate that the UE is camped on NR Cell 10? f_NR5GC_CheckRRCIdle(nr_Cell10); }