3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240703

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3600 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to f_NR5GC_RRCReestablishment functions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-12-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - In the the 24wk50 implementation, the function f_NR5GC_RRCReestablishment_Steps0To1 configures the target cell with a new RNTI value. - However, when the source and target cells are identical, this configuration can lead to issues during the calculation of the Short MAC-I and the subsequent reception of the reestablishment request. - With this change , reestablishment request is expected to be received by TTCN, with the updated RNTI and a Short MAC-I calculated based on this new RNTI value ( this leads to the mismatch between the received and expected reestablishment request message content ) |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Before invoking the function f_NR5GC_RRCReestablishment_Steps0To1, the RNTI value of the source cell and the Short MAC calculation are obtained. - These values, specifically the Short MAC-I and the RNTI, are then provided as parameters to the function f_NR5GC_RRCReestablishment_Step2, which is used for processing the reestablishment request. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE shall fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.1.5.NR5GC, 8.1.5.6.1.NR5GC, 8.1.4.1.9.x.NR5GC, 8.1.5.9.1.NR5GC, 8.1.6.1.3.4.NR5GC, 8.1.6.1.3.6.NR5GC and 8.1.6.1.3.7.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

2.2	Change 2	5

2.3	Change 3	7

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D24wk50 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | f_NR5GC_RRCReestablishment_Steps0To2 |
| --- | --- |
| Reason for change | - In the the 24wk50 implementation, the function f_NR5GC_RRCReestablishment_Steps0To1 configures the target cell with a new RNTI value. - However, when the source and target cells are identical, this configuration can lead to issues during the calculation of the Short MAC-I and the subsequent reception of the reestablishment request. - This is because the reestablishment request is expected to be received by TTCN, with the updated RNTI and a Short MAC-I calculated based on this new RNTI value. |
| Summary of change | - Before invoking the function f_NR5GC_RRCReestablishment_Steps0To1, the RNTI value of the source cell and the Short MAC calculation are obtained. - These values, specifically the Short MAC-I and the RNTI, are then provided as parameters to the function f_NR5GC_RRCReestablishment_Step2, which is used for processing the reestablishment request. |
| TTCN module | NR5GC_RRCSteps.ttcn |

Before Change:

function f_NR5GC_RRCReestablishment_Steps0To2(NR_CellId_Type p_NR_SourceCellId, NR_CellId_Type p_NR_TargetCellId, template(present) ReestablishmentCause p_ReestablishmentCause := handoverFailure, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value4, template (omit) charstring p_VerdictMsg := omit, template (omit) MacBearerRouting_Type p_MacBearerRoutingCA := omit, template (omit) NR_CellPowerList_Type p_CellPowerListBeforeStep2 := omit ) runs on NR_BASE_PTC { //@sic R5-211391 R5-213516: added p_MacBearerRoutingCA sic@ //@sic R5-221467 renamed function from f_NR5GC_RRCReestablishment_Steps1To2 to f_NR5GC_RRCReestablishment_Steps0To2 sic@ //@sic R5s230515: added p_CellPowerListBeforeStep2 sic@ //@sic R5s240656: split f_NR5GC_RRCReestablishment_Steps0To2 in 2 functions sic@ f_NR5GC_RRCReestablishment_Steps0To1( p_NR_SourceCellId, p_NR_TargetCellId, p_RNTI_Value); f_NR5GC_RRCReestablishment_Step2( p_NR_SourceCellId, p_NR_TargetCellId, p_ReestablishmentCause, p_VerdictMsg, p_MacBearerRoutingCA, p_CellPowerListBeforeStep2 ); }

After Change:

function f_NR5GC_RRCReestablishment_Steps0To2(NR_CellId_Type p_NR_SourceCellId, NR_CellId_Type p_NR_TargetCellId, template(present) ReestablishmentCause p_ReestablishmentCause := handoverFailure, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value4, template (omit) charstring p_VerdictMsg := omit, template (omit) MacBearerRouting_Type p_MacBearerRoutingCA := omit, template (omit) NR_CellPowerList_Type p_CellPowerListBeforeStep2 := omit ) runs on NR_BASE_PTC { //@sic R5-211391 R5-213516: added p_MacBearerRoutingCA sic@ //@sic R5-221467 renamed function from f_NR5GC_RRCReestablishment_Steps1To2 to f_NR5GC_RRCReestablishment_Steps0To2 sic@ //@sic R5s230515: added p_CellPowerListBeforeStep2 sic@ //@sic R5s240656: split f_NR5GC_RRCReestablishment_Steps0To2 in 2 functions sic@ var RNTI_Value v_RNTI_Value := f_NR_CellInfo_GetRNTI(p_NR_SourceCellId); //C-RNTI used by UE in RRCReestablismentRequest var ShortMAC_I v_ShortMAC_I:= f_NR_Calculate_ShortMAC(p_NR_SourceCellId, p_NR_TargetCellId); //WA#WI=1367312 f_NR5GC_RRCReestablishment_Steps0To1( p_NR_SourceCellId, p_NR_TargetCellId, p_RNTI_Value); f_NR5GC_RRCReestablishment_Step2( p_NR_SourceCellId, p_NR_TargetCellId, p_ReestablishmentCause, p_VerdictMsg, p_MacBearerRoutingCA, p_CellPowerListBeforeStep2, v_RNTI_Value, v_ShortMAC_I); //WA#WI=1367312 }

## Change 2

| Function name | f_NR5GC_RRCReestablishment_Step2 |
| --- | --- |
| Reason for change | - In the the 24wk50 implementation, the function f_NR5GC_RRCReestablishment_Steps0To1 configures the target cell with a new RNTI value. - However, when the source and target cells are identical, this configuration can lead to issues during the calculation of the Short MAC-I and the subsequent reception of the reestablishment request. This is because the reestablishment request is expected to be received by TTCN, with the updated RNTI and a Short MAC-I calculated based on this new RNTI value. |
| Summary of change | - Before invoking the function f_NR5GC_RRCReestablishment_Steps0To1, the RNTI value of the source cell and the Short MAC calculation are obtained. - These values, specifically the Short MAC-I and the RNTI, are then provided as parameters to the function f_NR5GC_RRCReestablishment_Step2, which is used for processing the reestablishment request. |
| TTCN module | NR5GC_RRCSteps.ttcn |

Before Change:

function f_NR5GC_RRCReestablishment_Step2(NR_CellId_Type p_NR_SourceCellId, NR_CellId_Type p_NR_TargetCellId, template(present) ReestablishmentCause p_ReestablishmentCause := handoverFailure, template (omit) charstring p_VerdictMsg := omit, template (omit) MacBearerRouting_Type p_MacBearerRoutingCA := omit, template (omit) NR_CellPowerList_Type p_CellPowerListBeforeStep2 := omit ) runs on NR_BASE_PTC { //@sic R5s240656 sic@ var RNTI_Value v_RNTI_Value := f_NR_CellInfo_GetRNTI(p_NR_SourceCellId); //C-RNTI used by UE in RRCReestablismentRequest var PhysCellId v_PhysCellId := f_NR_CellInfo_GetPhysicalCellId(p_NR_SourceCellId); var ShortMAC_I v_ShortMAC_I:= f_NR_Calculate_ShortMAC(p_NR_SourceCellId, p_NR_TargetCellId); //@sic R5s211169 change 1.1 sic@ //@sic R5s230515 sic@ if ( isvalue(p_CellPowerListBeforeStep2)) { f_NR_SetCellPowerList (p_CellPowerListBeforeStep2); //compiler warning may be ignored due to check } // 2. Target Cell: Receive RRCReestablismentRequest. SRB.receive(car_NR_SRB0_RrcPdu_IND(p_NR_TargetCellId, cr_38508_RRCReestablishmentRequest(cr_NR_ReestabUE_Identity (v_RNTI_Value, v_PhysCellId, v_ShortMAC_I), p_ReestablishmentCause), p_MacBearerRoutingCA)); if (isvalue(p_VerdictMsg)) { f_NR_PreliminaryPass(__FILE__, __LINE__, valueof(p_VerdictMsg)); } }

After Change:

function f_NR5GC_RRCReestablishment_Step2(NR_CellId_Type p_NR_SourceCellId, NR_CellId_Type p_NR_TargetCellId, template(present) ReestablishmentCause p_ReestablishmentCause := handoverFailure, template (omit) charstring p_VerdictMsg := omit, template (omit) MacBearerRouting_Type p_MacBearerRoutingCA := omit, template (omit) NR_CellPowerList_Type p_CellPowerListBeforeStep2 := omit, //WA#WI=1367312 RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value1, //WA#WI=1367312 ShortMAC_I p_ShortMAC_I //WA#WI=1367312 ) runs on NR_BASE_PTC { //@sic R5s240656 sic@ //var RNTI_Value v_RNTI_Value := f_NR_CellInfo_GetRNTI(p_NR_SourceCellId); //C-RNTI used by UE in RRCReestablismentRequest //WA#WI=1367312 var PhysCellId v_PhysCellId := f_NR_CellInfo_GetPhysicalCellId(p_NR_SourceCellId); //var ShortMAC_I v_ShortMAC_I:= f_NR_Calculate_ShortMAC(p_NR_SourceCellId, p_NR_TargetCellId); //@sic R5s211169 change 1.1 sic@ //WA#WI=1367312 //@sic R5s230515 sic@ if ( isvalue(p_CellPowerListBeforeStep2)) { f_NR_SetCellPowerList (p_CellPowerListBeforeStep2); //compiler warning may be ignored due to check } // 2. Target Cell: Receive RRCReestablismentRequest. SRB.receive(car_NR_SRB0_RrcPdu_IND(p_NR_TargetCellId, cr_38508_RRCReestablishmentRequest(cr_NR_ReestabUE_Identity (p_RNTI_Value, v_PhysCellId, p_ShortMAC_I), p_ReestablishmentCause), p_MacBearerRoutingCA)); //WA#WI=1367312 if (isvalue(p_VerdictMsg)) { f_NR_PreliminaryPass(__FILE__, __LINE__, valueof(p_VerdictMsg)); } }

## Change 3

| Function name | f_TC_8_1_5_6_1_NR5GC_TestBody |
| --- | --- |
| Reason for change | - In the the 24wk50 implementation, the function f_NR5GC_RRCReestablishment_Steps0To1 configures the target cell with a new RNTI value. - However, when the source and target cells are identical, this configuration can lead to issues during the calculation of the Short MAC-I and the subsequent reception of the reestablishment request. - This is because the reestablishment request is expected to be received by TTCN, with the updated RNTI and a Short MAC-I calculated based on this new RNTI value. |
| Summary of change | - Before invoking the function f_NR5GC_RRCReestablishment_Steps0To1, the RNTI value of the source cell and the Short MAC calculation are obtained. - These values, specifically the Short MAC-I and the RNTI, are then provided as parameters to the function f_NR5GC_RRCReestablishment_Step2, which is used for processing the reestablishment request. |
| TTCN module | RRC_Others_NR5GC.ttcn |

Before Change:

function f_TC_8_1_5_6_1_NR5GC_TestBody() runs on NR5GC_PTC { << SKIPPED CODE >> //@siclog "Steps 3-7" siclog@ //Check: Does the UE send RRCReestablishmentRequest message on NR Cell 2? //The SS transmits RRCReestablishmentmessage. //The UE transmits RRCReestablishmentCompletemessage. //RRCReconfiguration procedure to resume existing radio bearer. //@sic R5s201244, R5-206339, R5-206369, R5s240656 sic@ f_NR5GC_RRCReestablishment_Step2(nr_Cell1, nr_Cell2, otherFailure, "Step 3"); f_NR5GC_RRCReestablishment_Steps3To7(nr_Cell2,0); f_NR5GC_RRCReestablishment_Steps8To10(nr_Cell1, nr_Cell2,-,-, true); f_NR5GC_RRCReestablishment_Steps11To13(nr_Cell1, nr_Cell2); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7"); << SKIPPED CODE >> }

After Change:

function f_TC_8_1_5_6_1_NR5GC_TestBody() runs on NR5GC_PTC { << SKIPPED CODE >> //@siclog "Steps 3-7" siclog@ //Check: Does the UE send RRCReestablishmentRequest message on NR Cell 2? //The SS transmits RRCReestablishmentmessage. //The UE transmits RRCReestablishmentCompletemessage. //RRCReconfiguration procedure to resume existing radio bearer. //@sic R5s201244, R5-206339, R5-206369, R5s240656 sic@ v_RNTI_Value := f_NR_CellInfo_GetRNTI(nr_Cell1); //C-RNTI used by UE in RRCReestablismentRequest //WA#WI=1367312 v_ShortMAC_I := f_NR_Calculate_ShortMAC(nr_Cell1, nr_Cell2); //WA#WI=1367312 f_NR5GC_RRCReestablishment_Step2(nr_Cell1, nr_Cell2, otherFailure, "Step 3",-,-,v_RNTI_Value,v_ShortMAC_I); //WA#WI=1367312 f_NR5GC_RRCReestablishment_Steps3To7(nr_Cell2,0); f_NR5GC_RRCReestablishment_Steps8To10(nr_Cell1, nr_Cell2,-,-, true); f_NR5GC_RRCReestablishment_Steps11To13(nr_Cell1, nr_Cell2); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7"); << SKIPPED CODE >> }