3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240709

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3604 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC eCall over IMS testcase 11.5.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2024-12-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add NR5GC test case 11.5.1 to the IWD_24wk37 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 11.5.1 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.5.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... TS 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | A prose CR will be submitted for changes done to steps 16-26. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Correction to the Function f_IMS_NR5GC_MTSpeechCall	3

2.2	Correction to the Function f_NR5GC_MTSpeechCall	5

2.3	Correction to the Function fl_TC_11_5_1_NR5GC_EUTRA_TestBody	7

2.4	Correction to the Function fl_TC_11_5_1_Body	8

2.5	Correction to the Function f_EUTRA38_MultiPDN_EstablishAdditionalPDNs	17

3	Branches executed	17

4	Execution Log Files	17

4.1	Qualcomm Snapdragon Auto 5G Modem-RF Gen 2	17

5	References	17

- Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 11.5.1.NR5GC which is part of the NR5GC test suite in iwd-TTCN3-B2024-06_D24wk37 TTCN-3.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Juan Garcia Martin

juan.garciamartin@rohde-schwarz.com

  - Verification Test Summary

Test Case: 	11.5.1.NR5GC

ATS Version:	iwd-TTCN3-B2024-06_D24wk37 TTCN-3

System Simulator used:	R&S® 5G Protocol Conformance Test platform

UE used:	Snapdragon Auto 5G Modem-RF Gen 2, Qualcomm Technologies

Verification Status:	PASS

- Corrections required

  - Correction to the Function f_IMS_NR5GC_MTSpeechCall

| Function name | f_IMS_NR5GC_MTSpeechCall |
| --- | --- |
| Reason for change | In the current TTCN Implementation of the function, coordination messages are not aligned. Test case will be stuck if pc_Preconditions is false as a coordination message will be not sent IMS component to confirm that PDU session has been modified. Note: this funcion is also called in 11.5.2.NR5GC and 8.1.3.1.24.NR5GC. |
| Summary of change | Expected coordination message with specific contents in case of no preconditions. |
| TTCN module | IMS_NR5GC_Functions.ttcn |
| MCC160 Comment |  |

Before Change:

<< SKIPPED CODE >> function f_IMS_NR5GC_MTSpeechCall() runs on IMS_PTC { var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var IMS_ResponseWithSdp_Type v_ResponseWithSdp; f_IMS_IPCAN_WaitForTrigger(IPCAN, "Start MT Speech IMS"); // @sic R5-225362 sic@ v_InviteRequestWithSdp := f_IMS_MTCallSetup_NR5GC_A_5_X_Step1(pc_Preconditions, -, EVS_B0_A1); v_ResponseWithSdp := f_IMS_MTCallSetup_NR5GC_A_5_X_Step2_5(v_InviteRequestWithSdp, pc_Preconditions, -, EVS_B0_A1); f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("Add Voice Bearer")); // @sic R5-225362 sic@ if (pc_Preconditions) { f_IMS_MTCallSetup_NR5GC_A_5_1x_Step6_7(v_InviteRequestWithSdp, v_ResponseWithSdp); } f_IMS_MTCallSetup_Receive180Ringing(v_InviteRequestWithSdp, -, pc_Preconditions); f_IMS_MTCallSetup_AcceptAndAcknowledge(v_InviteRequestWithSdp); // Call established f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("Voice Call Established")); // @sic R5-225362 sic@ } << SKIPPED CODE >>

After Change:

<< SKIPPED CODE >> function f_IMS_NR5GC_MTSpeechCall() runs on IMS_PTC { var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var IMS_ResponseWithSdp_Type v_ResponseWithSdp; f_IMS_IPCAN_WaitForTrigger(IPCAN, "Start MT Speech IMS"); // @sic R5-225362 sic@ v_InviteRequestWithSdp := f_IMS_MTCallSetup_NR5GC_A_5_X_Step1(pc_Preconditions, -, EVS_B0_A1); v_ResponseWithSdp := f_IMS_MTCallSetup_NR5GC_A_5_X_Step2_5(v_InviteRequestWithSdp, pc_Preconditions, -, EVS_B0_A1); f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("Add Voice Bearer")); // @sic R5-225362 sic@ if (pc_Preconditions) { f_IMS_MTCallSetup_NR5GC_A_5_1x_Step6_7(v_InviteRequestWithSdp, v_ResponseWithSdp); } f_IMS_IPCAN_WaitForTrigger(IPCAN, "Voice Bearer Added"); a_IMS_MTCallSetup_Receive180Ringing(v_InviteRequestWithSdp); f_IMS_MTCallSetup_AcceptAndAcknowledge(v_InviteRequestWithSdp); // Call established f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("Voice Call Established")); // @sic R5-225362 sic@ } << SKIPPED CODE >>

  - Correction to the Function f_NR5GC_MTSpeechCall

| Function name | f_NR5GC_MTSpeechCall |
| --- | --- |
| Reason for change | - In the current TTCN Implementation of the function, coordination messages are not aligned. Test case will be stuck if pc_Preconditions is false as a coordination message will be not sent IMS component to confirm that PDU session has been modified. - Logging is not precise as it declares call establishment before it actually happens. Note: this funcion is also called in 11.5.2.NR5GC and 8.1.3.1.24.NR5GC |
| Summary of change | - Aligned coordination messages to be run with and without preconditions. - Moved logging line. |
| TTCN module | NR5GC_IMSProcedures.ttcn |
| MCC160 Comment |  |

Before Change:

<< SKIPPED CODE >> function f_NR5GC_MTSpeechCall(NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC { var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo; // Step 1 f_NR_Paging_Def(p_NR_CellId, RRC_IDLE); // Steps 2-4 RRC connection Setup v_GSM_MobilityInfo := f_NR5GC_SpeechCall_Steps2_8(p_NR_CellId, mt_Access, '0010'B); // @sic R5s210037 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Voice Call Establishment: Step 8"); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IMS_IPCAN_Trigger("Start MT Speech IMS")); // @sic R5-225362 sic@ // Steps 10-12 f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1], "Add Voice Bearer"); // @sic R5-225362 sic@ f_NR5GC_ModifyPDUSession_AddVoice(p_NR_CellId, valueof(v_GSM_MobilityInfo)); // @sic R5-221423. R5-224689 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Voice Call Established"); // Call now established f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1], "Voice Call Established"); // @sic R5-225362 sic@ } << SKIPPED CODE >>

After Change:

<< SKIPPED CODE >> function f_NR5GC_MTSpeechCall(NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC { var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo; // Step 1 f_NR_Paging_Def(p_NR_CellId, RRC_IDLE); // Steps 2-4 RRC connection Setup v_GSM_MobilityInfo := f_NR5GC_SpeechCall_Steps2_8(p_NR_CellId, mt_Access, '0010'B); // @sic R5s210037 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Voice Call Establishment: Step 8"); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IMS_IPCAN_Trigger("Start MT Speech IMS")); // @sic R5-225362 sic@ // Steps 10-12 f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1], "Add Voice Bearer"); // @sic R5-225362 sic@ f_NR5GC_ModifyPDUSession_AddVoice(p_NR_CellId, valueof(v_GSM_MobilityInfo)); // @sic R5-221423. R5-224689 sic@ //f_NR_PreliminaryPass(__FILE__, __LINE__, "Voice Call Established"); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IMS_IPCAN_Trigger("Voice Bearer Added")); // Call now established f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1], "Voice Call Established"); // @sic R5-225362 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Voice Call Established"); } << SKIPPED CODE >>

  - Correction to the Function fl_TC_11_5_1_NR5GC_EUTRA_TestBody

| Function name | fl_TC_11_5_1_NR5GC_EUTRA_TestBody |
| --- | --- |
| Reason for change | - TAU from should be of emergency type. - Power need to transition to state NonSuitableOff when current set status is NonSuitable. - There is already a security context, this needs to be taken into account and pass it as an input parameter. |
| Summary of change | - Corrected input parameters at step 52. - Corrected power settings at step 53 - Corrected input parameters at step 54 to indicate an already mapped security context. |
| TTCN module | ECall_IRAT_NR5GC_EUTRA |
| MCC160 Comment |  |

Before Change:

<< SKIPPED CODE >> function fl_TC_11_5_1_NR5GC_EUTRA_TestBody()runs on EUTRA_Multi5GS_PTC { // @siclog "Step 1" siclog@ // The SS configures: // - E-UTRA Cell 1 as "Non-suitable "off" cell". f_EUTRA38_SetCellPower(eutra_Cell1, tsc_NonSuitableOffCellRS_EPRE, tsc_NonSuitableOffCellRS_EPRE); f_EUTRA_NR_SendCoOrd(NR, cms_EUTRA_NR_Trigger("Cell Power")); f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Cell Power"); // @siclog "Step 51" siclog@ // The SS configures: // - E-UTRA Cell 1 as "Serving cell". f_EUTRA38_SetCellPower(eutra_Cell1, tsc_ServingCellRS_EPRE, tsc_ServingCellRS_EPRE_FR2); f_EUTRA_NR_SendCoOrd(NR, cms_EUTRA_NR_Trigger("Cell Power")); // @siclog "Step 52" siclog@ // Check: Does the UE perform on the E-UTRA Cell 1 the TAU procedure for Inter-system change from N1 mode to S1 mode as described in TS 38.508-1 [4], Table 4.9.7.2.2-1? f_EUTRA38_TrackingAreaUpdateFromN1(eutra_Cell1, tsc_RRC_TI_Def); // @siclog "Step 53" siclog@ // The SS configures: // - E-UTRA Cell 1 as "Non-suitable "off" cell". f_EUTRA38_SetCellPower(eutra_Cell1, tsc_NonSuitableCellRS_EPRE, tsc_NonSuitableCellRS_EPRE); f_EUTRA_NR_SendCoOrd(NR, cms_EUTRA_NR_Trigger("Cell Power")); // @siclog "Step 54" siclog@ // Check: Does the UE perform on the NR Cell 1 the Test procedure for UE Tracking area updating for Inter-system change from S1 mode to N1 mode as described in TS 38.508-1 [4], Table 4.9.9.2.2-1? f_EUTRA38_NR5GC_S1ToN1_IdleMode(eutra_Cell1); } // End of fl_TC_11_5_1_NR5GC_EUTRA_TestBody() << SKIPPED CODE >>

After Change:

<< SKIPPED CODE >> function fl_TC_11_5_1_NR5GC_EUTRA_TestBody()runs on EUTRA_Multi5GS_PTC { // @siclog "Step 1" siclog@ // The SS configures: // - E-UTRA Cell 1 as "Non-suitable "off" cell". f_EUTRA38_SetCellPower(eutra_Cell1, tsc_NonSuitableOffCellRS_EPRE, tsc_NonSuitableOffCellRS_EPRE); f_EUTRA_NR_SendCoOrd(NR, cms_EUTRA_NR_Trigger("Cell Power")); f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Cell Power"); // @siclog "Step 51" siclog@ // The SS configures: // - E-UTRA Cell 1 as "Serving cell". f_EUTRA38_SetCellPower(eutra_Cell1, tsc_ServingCellRS_EPRE, tsc_ServingCellRS_EPRE_FR2); f_EUTRA_NR_SendCoOrd(NR, cms_EUTRA_NR_Trigger("Cell Power")); // @siclog "Step 52" siclog@ // Check: Does the UE perform on the E-UTRA Cell 1 the TAU procedure for Inter-system change from N1 mode to S1 mode as described in TS 38.508-1 [4], Table 4.9.7.2.2-1? f_EUTRA38_TrackingAreaUpdateFromN1(eutra_Cell1, tsc_RRC_TI_Def, omit, -, -, emergency); // @siclog "Step 53" siclog@ // The SS configures: // - E-UTRA Cell 1 as "Non-suitable "off" cell". f_EUTRA38_SetCellPower(eutra_Cell1, tsc_NonSuitableOffCellRS_EPRE, tsc_NonSuitableOffCellRS_EPRE); f_EUTRA_NR_SendCoOrd(NR, cms_EUTRA_NR_Trigger("Cell Power")); // @siclog "Step 54" siclog@ // Check: Does the UE perform on the NR Cell 1 the Test procedure for UE Tracking area updating for Inter-system change from S1 mode to N1 mode as described in TS 38.508-1 [4], Table 4.9.9.2.2-1? f_EUTRA38_NR5GC_S1ToN1_IdleMode(eutra_Cell1, true); } // End of fl_TC_11_5_1_NR5GC_EUTRA_TestBody() << SKIPPED CODE >>

  - Correction to the Function fl_TC_11_5_1_Body

| Function name | fl_TC_11_5_1_Body |
| --- | --- |
| Reason for change | - Initial registration procedure, security is not yet established. - Current implementation will handle first any number of PDUs declared with correspondant pics and later will handle emergency PDU. This may work for eCall Capable UEs but in case of eCall Only UEs emergency and other PDUs needs to be handled in parallel. - At step 27-28, SS needs to wait until IMS component confirmation on emergency call release to complete releasing the call. - Coordination message at steps 44-45 to release MT call is missing. - Messages are to be expected in SRB1. - NR Cell number correction at step 51. - At step 55, UE should be in idle state, then previous to reregistration, UE needs to establish a RRC connection. |
| Summary of change | - Passed Initial_NoSecurity input parameter. - Reworked PDU stablishment to handle configured PDUs in parallel of emergency PDU. - Modified input parameter to wait for coordination message confirming IMS release call. - Added sending of coordination message to release second call. - Changed SRB input parameter. - Corrected cell number. - Reworked reregistration steps. Expecting a RRC connectionr request previous to reregistration message. |
| TTCN module | ECall_IRAT_NR5GC_NR.ttcn |
| MCC160 Comment |  |

Before Change:

<< SKIPPED CODE >> function fl_TC_11_5_1_Body()runs on NR5GC_PTC { var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var NR_SRB_COMMON_IND v_ReceivedAsp; var float v_WaitMin := f_NR_SetTimerToleranceMin (nr_Cell1, nasTimer, 120.0); // step 3 timer t_T3444_Min := f_NR_SetTimerToleranceMin(nr_Cell1, nasTimer, 43200.0); // 12 hours timer t_T3512_Min := f_NR_SetTimerToleranceMin(nr_Cell1, nasTimer, 25200.0); // 7 hours //@siclog "Step 1" siclog@ //The SS configures: // - NR Cell 1 as "Serving cell" f_NR_SetCellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Cell Power"); // @siclog "Step 2" siclog@ // The UE is Switched ON f_UT_SwitchOnUE(UT); // @siclog "Step 3" siclog@ // Check: Does the UE send an RRCSetupRequest on NR Cell 1 within 120 seconds? f_NR_CheckNoRRCSetupReq (nr_Cell1, v_WaitMin); // @siclog "Step 4" siclog@ // A manual eCall is initiated. (Note 1) // Note 1: The request to originate a manual eCall may be performed by MMI or AT command. f_UT_InitiateECall(UT, MANUAL); // @siclog "Step 5" siclog@ // Check: Does the UE send REGISTRATION REQUEST message with 5GS registration type set to 'initial registration'? v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_NoSecurity)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } else{ f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 5 PASS"); } // @siclog "Steps 6-14" siclog@ // Steps 5-13 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, nr_Cell1, v_ReceivedMsg, f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell1)), TESTMode_OFF, Initial_Secure); // @siclog "Step 15" siclog@ // The SS transmits a REGISTRATION ACCEPT message and assigns value of 7 hours for the timer T3512. v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_NoSecurity, v_GMM_MobilityInfo, f_NR5GC_GetMobileIdGUTI (nr_Cell1), cs_NG_TAIListNonConsecutive(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell1)), {bit2oct(f_NR_CellInfo_GetTAC(nr_Cell1))}), -, -, -, -, -,-, -, -, -, -, -, -, cs_GPRS_Timer2_3_IEI ('5E'O, '001'B, '00111'B)); //T3512 value = 7 hours SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); // @siclog "Steps 16-20a1" siclog@ // Steps 15-19a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. f_NR5GC_RRC_Idle_Steps15_20(nr_Cell1, TESTMode_OFF, noRrcConnectionRelease); // @siclog "Step 21" siclog@ // Check: The UE transmits an ULInformationTransfer message with Request type set to "initial emergency request" and a PDU SESSION ESTABLISHMENT REQUEST message? // @siclog "Steps 22-26" siclog@ // Steps 22-26 of Table 4.9.29.2.2-1 of the test procedure in TS 38.508-1 [4] are performed. f_NR5GC_eCallSteps21_26(nr_Cell1); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 21 PASS"); // @siclog "Steps 27-28" siclog@ // Release eCall over IMS using the generic procedure described in TS 38.508-1 [4] subclause 4.9.12B. // Void f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); //Trigger eCall release over IMS using the generic procedure described in TS 34.229-5 [41] subclause A.8 f_NR5GC_EmergencyCallReleaseMT(nr_Cell1, false); //IMS messages have completed in IMS PTC, now to handle NAS and RRC actions. // @siclog "Step 29" siclog@ // The SS releases the RRC connection. (Note 2) // Note 2: Timer T3444 of 12hours starts. f_NR_RRCRelease(nr_Cell1); t_T3444_Min.start; // @siclog "Step 30" siclog@ // The SS waits 1 minute. f_Delay(60.0); // @siclog "Steps 31-43" siclog@ // Steps 1 to 13 of the generic test procedure for IMS MT speech call establishment in 5GC specified in TS 38.508-1 [4] subclause 4.9.16 are performed. f_NR5GC_MTSpeechCall(nr_Cell1); // @siclog "Steps 44-45" siclog@ // Generic test procedure for IMS MT call release in 5GS described in TS 38.508-1 [4] subclause 4.9.18 takes place. // Void f_NR5GC_CallRelease(nr_Cell1, IPCAN_SpeechCall, false); //MT release required // @siclog "Step 46" siclog@ // The SS releases the RRC connection. // Note 3: Timer T3512 of 7hours starts. f_NR_RRCRelease(nr_Cell1); t_T3512_Min.start; // @siclog "Step 47" siclog@ // Check: Does the UE transmit a REGISTRATION REQUEST message at the expiry of T3512? (Note 3) t_T3512_Min.timeout; v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, ?, tsc_SHT_IntegrityProtected); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Periodic)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message failed"); } else{ f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 47 PASS"); } // @siclog "Step 48" siclog@ // The SS transmits a REGISTRATION ACCEPT message including T3512 value IE. v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_NoSecurity, v_GMM_MobilityInfo, f_NR5GC_GetMobileIdGUTI (nr_Cell1), cs_NG_TAIListNonConsecutive(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell1)), {bit2oct(f_NR_CellInfo_GetTAC(nr_Cell1))}), -, -, -, -, -,-, -, -, -, -, -, -, cs_GPRS_Timer2_3_IEI ('5E'O, '001'B, '00111'B)); //T3512 value = 7 hours SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); // @siclog "Step 49" siclog@ // The UE transmits a REGISTRATION COMPLETE message f_NR5GC_Registration_Complete(nr_Cell1, tsc_NR_RbId_SRB2); // @siclog "Step 50" siclog@ // The SS releases the RRC connection. f_NR_RRCRelease(nr_Cell1); f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger("Cell Power")); //Set EUTRA Cell power f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Cell Power"); // @siclog "Step 51" siclog@ // The SS configures: // - NR Cell 1 as " Non-suitable "off" cell" f_NR_SetCellPower (nr_Cell11, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); // @siclog "Step 52" siclog@ // Check: Does the UE perform on the E-UTRA Cell 1 the TAU procedure for Inter-system change from N1 mode to S1 mode as described in TS 38.508-1 [4], Table 4.9.7.2.2-1? f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Cell Power"); //@siclog "Step 53" siclog@ //The SS configures: // - NR Cell 1 as "Serving cell" f_NR_SetCellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); // @siclog "Step 54" siclog@ // Check: Does the UE perform on the NR Cell 1 the Test procedure for UE Tracking area updating for Inter-system change from S1 mode to N1 mode as described in TS 38.508-1 [4], Table 4.9.9.2.2-1? f_NR5GC_IRAT_S1ToN1_IdleMode(nr_Cell1, -, -); // @siclog "Step 55" siclog@ // Check: Does the UE transmit a DEREGISTRATION REQUEST message at expiry of T3444? t_T3444_Min.timeout; SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB2, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not f_Check_NG_DeregistrationReqMOMsg (v_ReceivedAsp.Signalling.Nas[0].Pdu, crs_DeregisterType('0'B, '0'B, -))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Deregistration Request Message Failed"); } else{ f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 55 PASS"); } } //End of fl_TC_11_5_1_Body() << SKIPPED CODE >>

After Change:

<< SKIPPED CODE >> function fl_TC_11_5_1_Body()runs on NR5GC_PTC { var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var NR_SRB_COMMON_IND v_ReceivedAsp; var float v_WaitMin := f_NR_SetTimerToleranceMin (nr_Cell1, nasTimer, 120.0); // step 3 timer t_T3444_Min := f_NR_SetTimerToleranceMin(nr_Cell1, nasTimer, 43200.0); // 12 hours timer t_T3512_Min := f_NR_SetTimerToleranceMin(nr_Cell1, nasTimer, 25200.0); // 7 hours //@siclog "Step 1" siclog@ //The SS configures: // - NR Cell 1 as "Serving cell" f_NR_SetCellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Cell Power"); // @siclog "Step 2" siclog@ // The UE is Switched ON f_UT_SwitchOnUE(UT); // @siclog "Step 3" siclog@ // Check: Does the UE send an RRCSetupRequest on NR Cell 1 within 120 seconds? f_NR_CheckNoRRCSetupReq (nr_Cell1, v_WaitMin); // @siclog "Step 4" siclog@ // A manual eCall is initiated. (Note 1) // Note 1: The request to originate a manual eCall may be performed by MMI or AT command. f_UT_InitiateECall(UT, MANUAL); // @siclog "Step 5" siclog@ // Check: Does the UE send REGISTRATION REQUEST message with 5GS registration type set to 'initial registration'? v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_NoSecurity)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } else{ f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 5 PASS"); } // @siclog "Steps 6-14" siclog@ // Steps 5-13 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, nr_Cell1, v_ReceivedMsg, f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell1)), TESTMode_OFF, Initial_NoSecurity); // @siclog "Step 15" siclog@ // The SS transmits a REGISTRATION ACCEPT message and assigns value of 7 hours for the timer T3512. v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_NoSecurity, v_GMM_MobilityInfo, f_NR5GC_GetMobileIdGUTI (nr_Cell1), cs_NG_TAIListNonConsecutive(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell1)), {bit2oct(f_NR_CellInfo_GetTAC(nr_Cell1))}), -, -, -, -, -,-, -, -, -, -, -, -, cs_GPRS_Timer2_3_IEI ('5E'O, '001'B, '00111'B)); //T3512 value = 7 hours SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); // @siclog "Steps 16-20a1" siclog@ // The SS transmits a REGISTRATION COMPLETE message f_NR5GC_Registration_Complete(nr_Cell1); // @siclog "Step 21" siclog@ // Check: The UE transmits an ULInformationTransfer message with Request type set to "initial emergency request" and a PDU SESSION ESTABLISHMENT REQUEST message? f_NR5GC_PDUSessionEstablishment (nr_Cell1, pc_noOf_PDUsSameConnection+1, tsc_NR_RbId_SRB1, -, // @sic R5s210134 sic@ -, -, crs_SSC_Mode(-, '001'B), -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, (cr_NG_Request_Type('001'B), cr_NG_Request_Type('011'B)), -, -, -, -, -, -, -, -, -, -, false); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 21 PASS"); // @siclog "Steps 22-26" siclog@ // Steps 24-26 of Table 4.9.29.2.2-1 of the test procedure in TS 38.508-1 [4] are performed. f_NR5GC_eCallSteps24_26(nr_Cell1); // @siclog "Steps 27-28" siclog@ // Release eCall over IMS using the generic procedure described in TS 38.508-1 [4] subclause 4.9.12B. // Void f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); //Trigger eCall release over IMS using the generic procedure described in TS 34.229-5 [41] subclause A.8 f_NR5GC_EmergencyCallReleaseMT(nr_Cell1); //IMS messages have completed in IMS PTC, now to handle NAS and RRC actions. // @siclog "Step 29" siclog@ // The SS releases the RRC connection. (Note 2) // Note 2: Timer T3444 of 12hours starts. f_NR_RRCRelease(nr_Cell1); t_T3444_Min.start; // @siclog "Step 30" siclog@ // The SS waits 1 minute. f_Delay(60.0); // @siclog "Steps 31-43" siclog@ // Steps 1 to 13 of the generic test procedure for IMS MT speech call establishment in 5GC specified in TS 38.508-1 [4] subclause 4.9.16 are performed. f_NR5GC_MTSpeechCall(nr_Cell1); // @siclog "Steps 44-45" siclog@ // Generic test procedure for IMS MT call release in 5GS described in TS 38.508-1 [4] subclause 4.9.18 takes place. // Void f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); f_NR5GC_CallRelease(nr_Cell1, IPCAN_SpeechCall, false); //MT release required // @siclog "Step 46" siclog@ // The SS releases the RRC connection. // Note 3: Timer T3512 of 7hours starts. f_NR_RRCRelease(nr_Cell1); t_T3512_Min.start; // @siclog "Step 47" siclog@ // Check: Does the UE transmit a REGISTRATION REQUEST message at the expiry of T3512? (Note 3) t_T3512_Min.timeout; v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, ?, tsc_SHT_IntegrityProtected); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Periodic)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message failed"); } else{ f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 47 PASS"); } // @siclog "Step 48" siclog@ // The SS transmits a REGISTRATION ACCEPT message including T3512 value IE. v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_NoSecurity, v_GMM_MobilityInfo, f_NR5GC_GetMobileIdGUTI (nr_Cell1), cs_NG_TAIListNonConsecutive(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell1)), {bit2oct(f_NR_CellInfo_GetTAC(nr_Cell1))}), -, -, -, -, -,-, -, -, -, -, -, -, cs_GPRS_Timer2_3_IEI ('5E'O, '001'B, '00111'B)); //T3512 value = 7 hours SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); // @siclog "Step 49" siclog@ // The UE transmits a REGISTRATION COMPLETE message f_NR5GC_Registration_Complete(nr_Cell1, tsc_NR_RbId_SRB1); // @siclog "Step 50" siclog@ // The SS releases the RRC connection. f_NR_RRCRelease(nr_Cell1); f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger("Cell Power")); //Set EUTRA Cell power f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Cell Power"); // @siclog "Step 51" siclog@ // The SS configures: // - NR Cell 1 as " Non-suitable "off" cell" f_NR_SetCellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); // @siclog "Step 52" siclog@ // Check: Does the UE perform on the E-UTRA Cell 1 the TAU procedure for Inter-system change from N1 mode to S1 mode as described in TS 38.508-1 [4], Table 4.9.7.2.2-1? f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Cell Power"); //@siclog "Step 53" siclog@ //The SS configures: // - NR Cell 1 as "Serving cell" f_NR_SetCellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); // @siclog "Step 54" siclog@ // Check: Does the UE perform on the NR Cell 1 the Test procedure for UE Tracking area updating for Inter-system change from S1 mode to N1 mode as described in TS 38.508-1 [4], Table 4.9.9.2.2-1? f_NR5GC_IRAT_S1ToN1_IdleMode(nr_Cell1, -, -); // @siclog "Step 55" siclog@ // Check: Does the UE transmit a DEREGISTRATION REQUEST message at expiry of T3444? t_T3444_Min.timeout; v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, ?, tsc_SHT_IntegrityProtected); if (not f_Check_NG_DeregistrationReqMOMsg (v_ReceivedMsg.Pdu, crs_DeregisterType('0'B, '0'B))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Deregistration Request Message Failed"); } else{ f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 55 PASS"); } } //End of fl_TC_11_5_1_Body() << SKIPPED CODE >>

  - Correction to the Function f_EUTRA38_MultiPDN_EstablishAdditionalPDNs

| Function name | f_EUTRA38_MultiPDN_EstablishAdditionalPDNs |
| --- | --- |
| Reason for change | Current implementation during the TAU procedure for Inter-system change from N1 mode to S1 mode might fail as no apn is present. According to TS 38.508 4.5A.2B.2.2-2 this should be allowed. |
| Summary of change | Added ifpresent to default input parameter to match in case that UE does not include any value. |
| TTCN module | EUTRA38_NR5GC_InitialRegistration.ttcn |
| MCC160 Comment |  |

Before Change:

<< SKIPPED CODE >> function f_EUTRA38_MultiPDN_EstablishAdditionalPDNs(EUTRA_CellId_Type p_CellId, integer p_ExpectedNoOfNewPDNs, boolean p_FailWhenK_LessN := true, integer p_SRB := tsc_SRB2, // @sic R5s210341 sic@ template (omit) NasCount_Type p_NasCountUL := omit, // Only needed when called after TAU B3_Type p_RequestType := '001'B, template AccessPointName p_AccessPointName := cr_AccessPointName_Any, // this is due to hardcoding of PDU session id in 11.1.2 boolean p_N1disabled := false, template (omit) EUTRA_NR_PduSessionInfoList_Type p_PduInfoFromNR := omit ) runs on EUTRA_Multi5GS_PTC return integer << SKIPPED CODE >>

After Change:

<< SKIPPED CODE >> function f_EUTRA38_MultiPDN_EstablishAdditionalPDNs(EUTRA_CellId_Type p_CellId, integer p_ExpectedNoOfNewPDNs, boolean p_FailWhenK_LessN := true, integer p_SRB := tsc_SRB2, // @sic R5s210341 sic@ template (omit) NasCount_Type p_NasCountUL := omit, // Only needed when called after TAU B3_Type p_RequestType := '001'B, template AccessPointName p_AccessPointName := cr_AccessPointName_Any ifpresent, // this is due to hardcoding of PDU session id in 11.1.2 boolean p_N1disabled := false, template (omit) EUTRA_NR_PduSessionInfoList_Type p_PduInfoFromNR := omit ) runs on EUTRA_Multi5GS_PTC return integer << SKIPPED CODE >>

- Branches executed

## This NR5GC TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “n41” and LTE Band “FDD3” using NIA2 Integrity and NEA2 ciphering algorithms.

- Execution Log Files

  - Qualcomm Snapdragon Auto 5G Modem-RF Gen 2

The Snapdragon Auto 5G Modem-RF Gen 2, Qualcomm Technologies passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:
tc_11_5_1_QC.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

- References

| [1] | R5s240710: Supporting information for Addition of NR5GC testcase 11.5.1 in FR1 |
| --- | --- |