3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250027

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0962 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_IMS_BuildSDP_AudioVideo_RX |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2025-01-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | With implementation of R5-247609 prose CR, requeriments on b-, c- or a-lines on the video stream were loosen, current implemenation on ttcn3 code to reflect that changes, might contain syntax errors provoking runtime errors. In function f_IMS_BuildSDP_AudioVideo_RX for test cases were video stream is removed, instead of having requirements on attributes used in input parameter, it is set as AnyOrOmit. template (present) SDP_media_desc cr_SDP_Media_Desc(template (present) SDP_media_field p_SDP_Media_Field, template SDP_connection p_SDP_Connection, template SDP_bandwidth_list p_SDP_Bandwidth_List, template SDP_attribute_list p_SDP_Attribute_List) := /* @status APPROVED (IMS, IMS_IRAT, LTE, LTE_A_IRAT, LTE_A_PRO, LTE_A_R12, LTE_IRAT, NR5GC, NR5GC_IRAT) */ cr_SDP_Media_Desc_Common(p_SDP_Media_Field, -, { p_SDP_Connection, * } ifpresent, p_SDP_Bandwidth_List, p_SDP_Attribute_List); With p_SDP_Connection := *, reception template will be as follows, { *, * } ifpresent, provoking a runtime error. In order to allow desired implementation, a different template needs to be used to allow to set SDP_Connection_List as “*” as a whole. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Used template that allows a SDP_Connection_List to being set as ”*” while removing video stream. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 17.1, 17.2, 7.20.NR5GC and 7.21.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Change to f_IMS_BuildSDP_AudioVideo_RX	3

# Overview

This document lists all the essential changes needed to correct problems in the ATS iwd-TTCN3-B2024-06_D24wk50 related to the title of this CR.

Contact: 	Juan Garcia Martin

Juan.GarciaMartin@rohde-schwarz.com

# Corrections required

## Change to f_IMS_BuildSDP_AudioVideo_RX

| Function name | f_IMS_BuildSDP_AudioVideo_RX |
| --- | --- |
| Reason for change | In function f_IMS_BuildSDP_AudioVideo_RX for test cases were video stream is removed, instead of having requirements on attributes used in input parameter, it is set as AnyOrOmit. With p_SDP_Connection := *, reception template will be as follows, { *, * }, provoking a runtime error. |
| Summary of change | Used template that allows a SDP_Connection_List to being set as ”*” while removing video stream. |
| TTCN module | IMS_SDP_MessagesVideo.ttcn |
| MCC160 Comment |  |

Before Change:

function f_IMS_BuildSDP_AudioVideo_RX(template (present) SDP_time p_TimeDescription := cr_SDP_Time_0_0, template (present) SDP_media_field p_SDP_Media_Field_Audio, template (present) SDP_attribute_list p_SDP_MediaAttributes_Audio, template (present) SDP_attribute_list p_SDP_PreconditionAttributes_Audio, template (present) SDP_media_field p_SDP_Media_Field_Video, template (present) SDP_attribute_list p_SDP_MediaAttributes_Video, template (present) SDP_attribute_list p_SDP_PreconditionAttributes_Video) runs on IMS_PTC return template (present) SDP_Message { var charstring v_AddrType := f_IMS_PTC_NW_Address_GetTypeStr(); var template (present) charstring v_IPAddrOrigin := ?; var template (present) charstring v_IPAddrAudio := ?; var template (present) charstring v_IPAddrVideo := ?; var template SDP_connection v_SDP_Connection_Audio := cr_SDP_Connection(v_AddrType, v_IPAddrAudio); var template SDP_connection v_SDP_Connection_Video := cr_SDP_Connection(v_AddrType, v_IPAddrVideo); var template (present) integer v_BandwidthAudioRR := ?; var template (present) SDP_attribute_list v_MandatoryAttributes_Audio := f_SDP_Attributes_Concat_RX(p_SDP_MediaAttributes_Audio, p_SDP_PreconditionAttributes_Audio); var template (present) SDP_attribute_list v_MandatoryAttributes_Video := f_SDP_Attributes_Concat_RX(p_SDP_MediaAttributes_Video, p_SDP_PreconditionAttributes_Video); var template SDP_attribute_list v_Attribute_List_Audio; var template SDP_attribute_list v_Attribute_List_Video; var template (present) SDP_media_desc v_Media_Desc_Audio; var template (present) SDP_media_desc v_Media_Desc_Video; var template SDP_bandwidth_list v_BandwidthList_Audio; var template SDP_bandwidth_list v_BandwidthList_Video; var template (present) SDP_media_desc_list v_Media_Desc_List; if (f_IMS_PTC_RanTypeIsNR5GC()) { // @sic R5-211433 sic@ v_BandwidthAudioRR := (1..infinity); // not specified in 34.229-1 C.25 } v_BandwidthList_Audio := cr_SDP_Bandwidth_List_Media(-, -, v_BandwidthAudioRR); v_BandwidthList_Video := cr_SDP_Bandwidth_List_Media; v_Attribute_List_Audio := f_SDP_Attributes_Superset(v_MandatoryAttributes_Audio); // @sic R5-221481, R5-221482, R5s220396: f_SDP_Attributes_Superset sic@ v_Attribute_List_Video := f_SDP_Attributes_Superset(v_MandatoryAttributes_Video); // @sic R5-221481, R5-221482, R5s220396: f_SDP_Attributes_Superset sic@ v_Media_Desc_Audio := cr_SDP_Media_Desc(p_SDP_Media_Field_Audio, v_SDP_Connection_Audio, v_BandwidthList_Audio, v_Attribute_List_Audio); if (f_SDP_SDP_Media_Field_IsForMediaRelease(p_SDP_Media_Field_Video)) { // @sic R5-247609: There are no requirements on b-, c- or a-lines on the video stream sic@ v_BandwidthList_Video := *; if (not f_IMS_PTC_RanTypeIsWLAN()) { // In G.17.1 INVITE (Step 7) still specifies the c= line and attributes v_SDP_Connection_Video := *; v_Attribute_List_Video := *; } } v_Media_Desc_Video := cr_SDP_Media_Desc(p_SDP_Media_Field_Video, v_SDP_Connection_Video, v_BandwidthList_Video, v_Attribute_List_Video); v_Media_Desc_List := { v_Media_Desc_Audio, v_Media_Desc_Video }; return cr_SDP_Message_Common(-, v_AddrType, v_IPAddrOrigin, ?, -, p_TimeDescription, v_Media_Desc_List); }

After Change:

function f_IMS_BuildSDP_AudioVideo_RX(template (present) SDP_time p_TimeDescription := cr_SDP_Time_0_0, template (present) SDP_media_field p_SDP_Media_Field_Audio, template (present) SDP_attribute_list p_SDP_MediaAttributes_Audio, template (present) SDP_attribute_list p_SDP_PreconditionAttributes_Audio, template (present) SDP_media_field p_SDP_Media_Field_Video, template (present) SDP_attribute_list p_SDP_MediaAttributes_Video, template (present) SDP_attribute_list p_SDP_PreconditionAttributes_Video) runs on IMS_PTC return template (present) SDP_Message { var charstring v_AddrType := f_IMS_PTC_NW_Address_GetTypeStr(); var template (present) charstring v_IPAddrOrigin := ?; var template (present) charstring v_IPAddrAudio := ?; var template (present) charstring v_IPAddrVideo := ?; var template SDP_connection v_SDP_Connection_Audio := cr_SDP_Connection(v_AddrType, v_IPAddrAudio); var template SDP_connection v_SDP_Connection_Video := cr_SDP_Connection(v_AddrType, v_IPAddrVideo); var template SDP_connection_list v_SDP_Connection_Video_List := { v_SDP_Connection_Video, * } ifpresent; var template (present) integer v_BandwidthAudioRR := ?; var template (present) SDP_attribute_list v_MandatoryAttributes_Audio := f_SDP_Attributes_Concat_RX(p_SDP_MediaAttributes_Audio, p_SDP_PreconditionAttributes_Audio); var template (present) SDP_attribute_list v_MandatoryAttributes_Video := f_SDP_Attributes_Concat_RX(p_SDP_MediaAttributes_Video, p_SDP_PreconditionAttributes_Video); var template SDP_attribute_list v_Attribute_List_Audio; var template SDP_attribute_list v_Attribute_List_Video; var template (present) SDP_media_desc v_Media_Desc_Audio; var template (present) SDP_media_desc v_Media_Desc_Video; var template SDP_bandwidth_list v_BandwidthList_Audio; var template SDP_bandwidth_list v_BandwidthList_Video; var template (present) SDP_media_desc_list v_Media_Desc_List; if (f_IMS_PTC_RanTypeIsNR5GC()) { // @sic R5-211433 sic@ v_BandwidthAudioRR := (1..infinity); // not specified in 34.229-1 C.25 } v_BandwidthList_Audio := cr_SDP_Bandwidth_List_Media(-, -, v_BandwidthAudioRR); v_BandwidthList_Video := cr_SDP_Bandwidth_List_Media; v_Attribute_List_Audio := f_SDP_Attributes_Superset(v_MandatoryAttributes_Audio); // @sic R5-221481, R5-221482, R5s220396: f_SDP_Attributes_Superset sic@ v_Attribute_List_Video := f_SDP_Attributes_Superset(v_MandatoryAttributes_Video); // @sic R5-221481, R5-221482, R5s220396: f_SDP_Attributes_Superset sic@ v_Media_Desc_Audio := cr_SDP_Media_Desc(p_SDP_Media_Field_Audio, v_SDP_Connection_Audio, v_BandwidthList_Audio, v_Attribute_List_Audio); if (f_SDP_SDP_Media_Field_IsForMediaRelease(p_SDP_Media_Field_Video)) { // @sic R5-247609: There are no requirements on b-, c- or a-lines on the video stream sic@ v_BandwidthList_Video := *; if (not f_IMS_PTC_RanTypeIsWLAN()) { // In G.17.1 INVITE (Step 7) still specifies the c= line and attributes v_SDP_Connection_Video := *; v_SDP_Connection_Video_List := *; v_Attribute_List_Video := *; } } v_Media_Desc_Video := cr_SDP_Media_Desc_Common(p_SDP_Media_Field_Video, -, v_SDP_Connection_Video_List, v_BandwidthList_Video, v_Attribute_List_Video); v_Media_Desc_List := { v_Media_Desc_Audio, v_Media_Desc_Video }; return cr_SDP_Message_Common(-, v_AddrType, v_IPAddrOrigin, ?, -, p_TimeDescription, v_Media_Desc_List); }