3GPP TSG-RAN WG5 Meeting #76 	R5-174042r1

# Berlin, Germany, 21-25 August 2017

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| P-CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-2 | PCR | NA | rev | - | Current version: | 0.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MCPTT Testing: Additions for test case 5.1 in 36.579-2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BlackBerry |  |  |  |  |  |  |  |  |  |
| Source to TSG: (Not Applicable) | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCPTT-ConTest |  |  |  |  |  |  | Date: |  | 2017-08-10 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Default message contents and some message steps are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add some default message contents and message steps. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case not progressing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Changes from previous version: ‘Content-Type’ corrected in tables 5.1.3.3-1 and 5.1.3.3-2. - Duplicated table removed from table 5.1.3.3-1. - Table numbers for specific message contents corrected (to use consecutive numbering). - Table 5.1.3.3-5 added for specific message content for SIP SUBSCRIBE. - Notes in the conditions tables had their style corrected to TAH and put in a same table row. - Row in step sequence for previously deleted step C1 was completely deleted (to avoid leaving empty row). - Reference to E-UTRA registration was removed from the Preamble. Two initial steps and exception were added in test sequence table. - Table 5.1.3.3-1 now added as new table (to avoid having an empty column after change bars approval). |  |  |  |  |  |  |  |  |

<Unmodified text skipped >

## 5.1	Configuration / Authentication / User Authorisation / UE Configuration / User Profile

5.1.1	Test Purpose (TP)

(1)

with { UE (MCPTT Client) attached to EPS services }

ensure that {

when { the MCPTT User activates an MCPTT application and requests MCPTT initialisation }

then { UE (MCPTT Client) performs MCPTT User Authentication, key management, configuration management, group management and service authentication }

}

5.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TR 24.980 clauses 4.2.1 and 4.3.1, TS 24.482 clause 6.2.1 and Annex A.2.1.2, TS 24.484 clauses 4.2.1, 4.2.2, 6.2.2, 6.3.1.1, 6.3.2.1, 6.3.2.2, 6.3.13.2.1 and 6.3.13.2.2, TS 24.481 clauses 6.2.2.2, 6.2.3, 6.3.3.2.1, 6.3.3.2.2 and 6.3.13.2.1, TS 24.379 clauses 7.2.1. 7.2.1A, 7.2.2 and 7.2.3, TS 33.179 clauses 5.6.1, 7.2.3 and Annex D. Unless otherwise stated these are Rel-13 requirements.

[TR 24.980, clause 4.2.1]

The MCPTT UE follows the SIP registration procedures defined in 3GPP TS 24.229 [4]. In addition, when the conditions for performing IMS registration in bullets 2, 3, 4, 5 and 6 in subclause L.3.1.2 of 3GPP TS 24.229 [4] evaluate to true, the MCPTT UE registers with the IMS.

[TR 24.980, clause 4.3.1]

The MCPTT UE follows the procedures defined in 3GPP TS 24.229 [4] and 3GPP TS 33.203 [7] for authentication with IMS Authentication and Key Agreement (IMS-AKA), Sec-Agree and IPSec. The MCPTT UE supports integrity protection.

[TS 24.482, clause 6.2.1]

Upon an indication from the MCPTT client to initiate MCPTT user authentication, the IdM client shall perform the user authentication procedure according to 3GPP TS 33.179 [2] with the following clarifications:

1)	shall establish a TLS tunnel to the authorisation endpoint of the IdM server as specified in 3GPP TS 33.179 [2] using the configured URL of the authorisation endpoint of the IdM server as specified in the "/<x>/OnNetwork/AppServerInfo/IDMSAuthEndpoint" leaf node defined in 3GPP TS 24.383 [11] and the clarifications in annex A;

2)	shall generate an OIDC Authentication Request message as specified in the OpenID Connect 1.0 [6] and IETF RFC 6749 [5] with the following clarifications:

a)	shall generate an HTTP GET request method according to IETF RFC 2616 [4];

b)	shall include the configured parameter IdM client id as the client_id parameter specified in 3GPP TS 33.179 [2] in the query component of the authorization endpoint’s URI using the "application/x-www-form-urlencoded" format as specified in W3C.REC-html401-19991224 [7]; and

NOTE 1:	The configuration of client_id is specified in 3GPP TS 24.383 [11].

c)	shall include the remaining required parameters as specified in 3GPP TS 33.179 [2] in the query component of the authorization endpoint’s URI using the "application/x-www-form-urlencoded" format as specified in W3C.REC-html401-19991224 [7]; and

3)	shall send the HTTP GET request method towards the IdM server.

NOTE 2:	The OpenID Connect 1.0 [6] specification allows for an alternative mechanism for sending the OIDC Authentication request message using an HTTP POST request method which can be used in place of steps 1, 2, and 3 above.

Upon receipt of an HTTP 200 (OK) response from the IdM server, the IdM client:

1)	shall prompt the MCPTT user for their username and password;

NOTE 3:	Other types of authentication are supported and are not defined by the OIDC specifications. 3GPP TS 33.179 [2] has defined username and password as a mandatory authentication method to be supported; hence a procedure to realize that method is included here.

2)	shall generate an HTTP POST request method containing the MCPTT user's username and password; and

3)	shall send the HTTP POST request method towards the IdM server.

Upon receipt of an OIDC Authentication Response message, the IdM client:

1)	shall establish a TLS tunnel to the token endpoint of the IdM server as specified in 3GPP TS 33.179 [2] using the configured URL of the token endpoint of the IdM server as specified in the "/<x>/OnNetwork/AppServerInfo/IDMSTokenEndpoint" leaf node defined in 3GPP TS 24.383 [11] and the clarifications in annex A;

2)	shall generate an OIDC Token Request message as specified in OpenID Connect 1.0 [6] and IETF RFC 6749 [5] with the following clarifications:

a)	shall generate an HTTP POST request method according to IETF RFC 2616 [4]; and

b)	shall include the grant_type parameter set to a value of "authorization_code" and the other required parameters in the entity body of the HTTP POST request method using the using the "application/x-www-form-urlencoded" format as specified in 3GPP TS 33.179 [2]; and

3)	shall send the HTTP POST request method towards the IdM server.

Upon receipt of an OIDC Token Response message, the IdM client:

1)	shall validate the id_token, access_token and refresh token in the received OIDC Token Response message as specified in the OpenID Connect 1.0 [6] specification; and

2)	shall provide the id_token and access_token in the received OIDC Token Response message to the MCPTT client.

NOTE 4:	The method in which the IdM client provides the id_token and access_token to the MCPTT client is implementation specific.

[TS 24.482, Annex A.2.1.2]

The HTTP client in the UE shall establish a TCP connection towards the home HTTP proxy FQDN and the home HTTP proxy port, unless the specific TCP connection is to be used for the IdM client to IdM server procedures described in subclause 6.2 and subclause 6.3 in the present document, in which case the HTTP client shall establish a TCP connection towards the IdM server.

The HTTP client in the UE shall establish a TLS tunnel via the TCP connection as specified in 3GPP TS 33.179 [2]. When establishing the TLS tunnel, the HTTP client in the UE shall act as a TLS client and the UE shall perform the TLS tunnel authentication using the TLS authentication method indicated by the TLS tunnel authentication method parameter according to 3GPP TS 33.179 [2]. The UE shall use the configured TLS tunnel authentication X.509 certificate and the configured TLS tunnel authentication pre-shared key when applicable for the used TLS authentication method. In order to prevent man-in-the-middle attacks, the HTTP client in the UE shall check the home HTTP proxy FQDN against the server's identity as presented in the received server's certificate message if the TCP connection terminates on the HTTP proxy. The HTTP client in the UE shall not check the portion of dereferenced HTTP URL against the server's identity as presented in the received server's certificate message if the TCP connection terminates on the HTTP proxy, but shall do so if the TCP connection terminates on the IdM server.

NOTE:	The TLS tunnel can be terminated in the HTTP proxy (rather than in the HTTP server providing the dereferenced HTTP URL).

The HTTP client in the UE shall send and receive all HTTP messages via the TLS tunnel.

If the HTTP client in the UE has an access token of the "bearer" token type as specified in IETF RFC 6750 [14], the HTTP client in the UE shall include an Authorization header field with the "Bearer" authentication scheme as specified in IETF RFC 6750 [14] in HTTP requests.

[TS 33.179 Annex D]

All KMS communications are made via HTTPS. The MCPTT key management client is provisioned via XML content in the KMS's response. The XML content is designed to be extendable to allow KMS/client providers to add further information in the XML. Where the interface is extended, a different XML namespace should be used (so that may be ignored by non-compatible clients).

It is assumed that transmissions between the KMS and the key management client are secure and that the KMS has authenticated the identity of the key management client.

Additionally, to allow the transmission of key material securely between a secure element within the KMS and a secure element within the key management client, a security extension is defined which allows messages to be signed and key material to be encrypted using a shared Transport Key (TrK).

[TS 33.179 clause 5.6.1]

For key management authorization, the KM client in the UE presents an access token to the KMS over HTTP. The KMS validates the access token and if successful, provides user specific key material back to the UE KM client based on the MCPTT ID of the user. This includes identity based key information used for media and signalling protection.

The KMS client in the UE presents an access token to the KMS over HTTP. The KMS authorizes the user for key management services and replies to the client with identity specific key information.

[TS 33.179 clause 7.2.3]

Case that HTTP proxy is used between the KMC and KMS

0)	The key management client establishes a connection to the MCPTT KMS. As with other elements in the Common Services Core, the connection routed via, and secured by, the HTTP Proxy. The message flow below is within this secure connection.

NOTE:	Additionally, the connection between the MCPTT KMS and the HTTP Proxy is secured according to clause 8.

1)	The key management client makes a request for user key material from the MCPTT KMS. The request contains details of the identity (e.g. the MCPTT ID) requested for key management, and the time for which the key material is required.

2)	The KMS provides a response containing key material. The response includes the type of key material, the period of use for the material and any domain-specific parameters required for its use.  For public safety use, the key material itself shall be wrapped using a 256-bit transport key (TrK). The TrK is distributed via an out-of-band mechanism along with a 32-bit identifier, TrK-ID.

Case that HTTP proxy is not used between the KMC and KMS

0)	The key management client establishes a direct HTTPS connection to the MCPTT KMS. The following message flow is within this secure connection.

1)	The key management client makes a request for user key material from the MCPTT KMS. The request contains details of the identity requested for key management, and the time at which the key material is required.

2)	The KMS provides a response containing key material. The response includes the type of key material, the period of use for the material and any domain-specific parameters required for its use. Optionally, the key material itself may also be wrapped using a 256-bit transport key (TrK), distributed via an out-of-band mechanism along with a 32-bit identifier (TrK-ID).

[TS 24.484, clause 4.2.1]

In order to obtain access to the MCPTT service the MCPTT UE needs to obtain configuration data either online via the network or offline using some external device (e.g. a laptop). As part of the bootstrap process the MCPTT UE needs to discover either:

1.	the online configuration management server in the network that configures the MPCTT UE initial configuration MO and the default MCPTT user profile configuration MO, then the MCPTT UE:

a)	using the URI of the configuration management server obtained from the MPCTT UE initial configuration MO, obtains:

-	the MCPTT UE configuration document;

-	the MCPTT user profile configuration document; and

-	the MCPTT service configuration document; and

b)	using the URI of the group management server obtained from the MPCTT UE initial configuration MO obtain the MCPTT group document; or

[TS 24.484, clause 4.2.2]

The MCPTT UE contacts the identity management server using the HTTPS URI stored in the MCPTT UE initial configuration MO and performs MCPTT User authentication as specified in 3GPP TS 24.382 [6].

The MCPTT UE, using the MCPTT ID obtained during MCPTT user authentication, subscribes to the MCPTT UE configuration document, the MCPTT user profile configuration document and the MCPTT service configuration document using the procedure for subscribing to multiple documents simultaneously using the subscription proxy function specified in subclause 6.3.13.2.2(i.e., the CMS acts as a Subscription Proxy) and subscribes to the MCPTT group document using the procedure specified in 3GPP TS 24.381 [5]. If these documents have been updated since the current version stored in the MCPTT UE, then the MCPTT UE will receive a SIP NOTIFY request with an XCAP Diff document (see IETF RFC 5875 [11]), in which case the CMC updates its local document copies. Retrieval by the MCPTT UE using the notified HTTPS URI of the MCPTT group document is performed as specified in 3GPP TS 24.381 [5].

[TS 24.484, clause 6.2.2]

The CMC shall send the HTTP request over TLS connection as specified for the HTTP client in the UE in annex A of 3GPP TS 24.382 [6].

[TS 24.484, clause 6.3.1.1]

A CMC shall support subclause 6.1.1 "Document Management" of OMA OMA-TS-XDM_Core-V2_1 [2] and subclause 6.3.13.2.2 for subscribing to configuration management documents.

[TS 24.484, clause 6.3.3.2.1]

In order to retrieve a configuration management document, a GC shall send an HTTP GET request with the Request URI that references the document to be updated to the network according to procedures specified in IETF RFC 4825 [14] "Retrieve a Document".

[TS 24.484, clause 6.3.3.2.2]

In order to retrieve a configuration management document, a CMC shall perform the procedures in subclause 6.3.3.2.1 specified for GC. The CMC shall set the Request-URI of the HTTP GET request to the "CMSXCAPRootURI" configured as per 3GPP TS 24.383 [4] and include the "auid" as per the appropriate application usage in clause 7.

Subclause 7.5 specifies which configuration management documents can be retrieved from the CMS over the CSC-4 reference point.

[TS 24.484, clause 6.3.13.2.1]

This procedure enables the CMC to subscribe to notification of changes of one or more configuration management documents defined in clause 7.

This procedure enables the MCPTT server to subscribe to notification of changes of the MCPTT service configuration document.

[TS 24.484, clause 6.3.13.2.2]

In order to subscribe to Configuration management document, a CMC shall send an initial SIP SUBSCRIBE request to the network according to the UE originating procedures specified in 3GPP TS 24.229 [22] and IETF RFC 5875 [11]. In the initial SIP SUBSCRIBE request, the CMC:

a)	…

b)	if subscription to multiple documents simultaneously using the subscription proxy function is used:

1)	shall include an application/resource-lists+xml MIME body. In the application/resource-lists+xml MIME body, the CMC shall include one <entry> element for each document or element to be subscribed to, such that the "uri" attribute of the <entry> element contains a relative path reference:

A)	with the base URI being equal to the "CMSXCAPRootURI" configured in the CMC as per 3GPP TS 24.383 [4]; and

B)	with the "auid" parameter set to the appropriate application usage identifying a configuration management document as described in clause 7;

2)	shall set the Request-URI to the configured public service identity for performing subscription proxy function of the CMS;

c)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcptt-access-token> element set to the value of the access token received during authentication procedure as described in 3GPP TS 24.382 [6];

d)	if identity hiding is required:

1)	shall perform the confidentiality protection procedures and integrity protection procedures defined in 3GPP TS 24.379 [9] for MCPTT client on the application/vnd.3gpp.mcptt-info+xml MIME body and on the application/resource-lists+xml MIME body; and

2)	shall include an application/mikey MIME body with the CSK as specified in 3GPP TS 24.379 [9];

e)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [22]), in a P-Preferred-Service header field according to IETF RFC 6050 [23]; and

f)	shall include the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" in the Contact header field.

Upon receiving a SIP NOTIFY request associated with a subscription created as result of the sent initial SIP SUBSCRIBE request:

1)	if identity hiding is required, the CMC shall perform the confidentiality protection procedures and integrity protection procedures defined in 3GPP TS 24.379 [9] for MCPTT client; and

2)	shall handle the SIP NOTIFY request according to IETF RFC 5875 [11].

[TS 24.481, clauses 6.2.2.2]In order to address an existing group document defining a group ID known by GC, the GC shall set the Request-URI of an HTTP request to a XCAP URI identifying a group document addressed by a group ID as described in subclause 7.2.10.2, where the group ID is set to the group ID known by GC and where the XCAP root URI is the XCAP root URI configured in the UE.

[TS 24.481, clauses 6.2.3]The GMC shall send the HTTP request over a TLS connection as specified for the HTTP client in the UE in annex A of 3GPP TS 24.382 [10].

The GMC shall perform the procedures in subclause 6.2.2 specified for GC.

[TS 24.481, clauses 6.3.3.2.1]

In order to retrieve a group document, a GC shall send an HTTP GET request with the Request URI that references the document to be retrieved to the network according to procedures specified in IETF RFC 4825 [22] "Fetch a Document".

[TS 24.481, clauses 6.3.3.2.2]

In order to retrieve a group document, a GMC shall perform the procedures in subclause 6.3.3.2.1 specified for GC.

[TS 24.481, clauses 6.3.13.2.1]

In order to subscribe to notification of changes of:

a)	one or more MCPTT group documents of MCPTT groups identified by MCPTT group IDs;

…

a GMC shall send an initial SIP SUBSCRIBE request to the network according to the UE originating procedures specified in 3GPP TS 24.229 [12] and IETF RFC 5875 [13]. In the initial SIP SUBSCRIBE request, the GMC:

a)	shall include an application/resource-lists+xml MIME body. In the application/resource-lists+xml MIME body, the GMC shall include one <entry> element for each document or element to be subscribed to, such that the "uri" attribute of the <entry> element:

1)	contains a relative path reference:

A)	with the base URI being equal to the XCAP root URI configured in the GMC; and

B)	identifying a group document addressed by a group ID as described in subclause 7.2.10.2 where the group ID is set to the MCPTT group ID; or

2)	contains a relative path reference:

A)	…

B)	…; and

C)	…;

b)	shall set the Request-URI to the configured public service identity for performing subscription proxy function of the GMS;

c)	shall include an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcptt-access-token> element set to the value of the access token received during authentication procedure as described in 3GPP TS 24.382 [49];

d)	if identity hiding is required:

1)	shall perform the confidentiality protection procedures and integrity protection procedures defined in 3GPP TS 24.379 [5] for MCPTT client on the application/vnd.3gpp.mcptt-info+xml MIME body and on the application/resource-lists+xml MIME body; and

2)	shall include an application/mikey MIME body with the CSK as specified in 3GPP TS 24.379 [5];

e)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [12]), in a P-Preferred-Service header field according to IETF RFC 6050 [14]; and

f)	shall include the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" in the Contact header field.

Upon receiving a SIP NOTIFY request associated with a subscription created as result of the sent initial SIP SUBSCRIBE request:

1)	if identity hiding is required, the GMC shall perform the confidentiality protection procedures and integrity protection procedures defined in 3GPP TS 24.379 [5] for MCPTT client; and

2)	shall handle the SIP NOTIFY request according to IETF RFC 5875 [13].

[TS 24.379, clause 7.2.1]

When the MCPTT client performs SIP registration the MCPTT client shall perform the registration procedures as specified in 3GPP TS 24.229 [4].

The MCPTT client shall include the following media feature tags in the Contact header field of the SIP REGISTER request:

1)	the g.3gpp.mcptt media feature tag; and

2)	the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt".

…

If the MCPTT client, upon performing SIP registration:

1)	has successfully finished the user authentication procedure as described in 3GPP TS 24.382 [49];

2)	has available an access-token;

3)	based on implementation decides to use SIP REGISTER for service authorization;

4)	confidentiality protection is disabled as specified in subclause 6.6.2.3.1; and

5)	integrity protection is disabled as specified in subclause 6.6.3.3.1;

then the MCPTT client shall include an application/vnd.3gpp.mcptt-info+xml MIME body as defined in Annex F.1 with the <mcptt-access-token> element set to the value of the access token received during the user authentication procedures, in the SIP REGISTER request.

NOTE 2:	the access-token contains the MCPTT ID of the user.

If the MCPTT client, upon performing SIP registration:

1)	has successfully finished the user authentication procedure as described in 3GPP TS 24.382 [49];

2)	has an available access-token;

3)	based on implementation decides to use SIP REGISTER for service authorization; and

4)	either confidentiality protection is enabled as specified in subclause 6.6.2.3.1 or integrity protection is enabled as specified in subclause 6.6.3.3.1;

then the MCPTT client:

1)	…;

2)	if confidentiality protection is enabled as specified in subclause 6.6.2.3.1, shall encrypt the received access-token using the client server key (CSK) and shall include in the body of the SIP REGISTER request, an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcptt-access-token> element set to the encrypted access-token, as specified in subclause 6.6.2.3.3;

3)	if confidentiality protection is disabled as specified in subclause 6.6.2.3.1, shall include an application/vnd.3gpp.mcptt-info+xml MIME body as defined in Annex F.1 with the <mcptt-access-token> element set to the value of the access token received during the user authentication procedures; and

4)	if integrity protection is enabled as specified in subclause 6.6.3.3.1, shall use the CSK to integrity protect the application/vnd.3gpp.mcptt-info+xml MIME body by following the procedures in subclause 6.6.3.3.3.

[TS 24.379, clause 7.2.1A]

This procedure is only referenced from other procedures.

When populating the SIP PUBLISH request, the MCPTT client shall:

1)	shall set the Request-URI to the public service identity identifying the participating MCPTT function serving the MCPTT user;

2)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Preferred-Service header field according to IETF RFC 6050 [9];

3)	shall set the Event header field to the "poc-settings" value; and

4)	shall set the Expires header field according to IETF RFC 3903 [37], to 4294967295, if the MCPTT user is not removing the MCPTT service settings, otherwise to remove the MCPTT service settings the MCPTT client shall set the Expires header field to zero.

[TS 24.379, clause 7.2.2]

If based on implementation the MCPTT client decides to use SIP PUBLISH for MCPTT server settings to also perform service authorization and

1)	has successfully finished the user authentication procedure as described in 3GPP TS 24.382 [49]; and

2)	has available an access-token;

then the MCPTT client:

1)	shall perform the procedures in subclause 7.2.1A;

2)	if confidentiality protection is disabled as specified in subclause 6.6.2.3.1 and integrity protection is disabled, shall include in the body of the SIP PUBLISH request, an application/vnd.3gpp.mcptt-info+xml MIME body as specified in Annex F.1 with the <mcptt-access-token> element set to the value of the access token received during the user authentication procedures;

3)	if either confidentiality protection is enabled as specified in subclause 6.6.2.3.1 or integrity protection is enabled as specified in subclause 6.6.3.3.1 shall include an application/mikey MIME body with the CSK as MIKEY-SAKKE I_MESSAGE as specified in 3GPP TS 33.179 [46] in the body of the SIP PUBLISH request;

4)	if confidentiality protection is enabled as specified in subclause 6.6.2.3.1, shall include in the body of the SIP PUBLISH request an application/vnd.3gpp.mcptt-info+xml MIME body with:

a)	the <mcptt-access-token> element set to the received access-token encrypted using the client server key (CSK), as specified in subclause 6.6.2.3.3; and

b)	the <mcptt-client-id> element set to the encrypted MCPTT client ID of the originating MCPTT client, as specified in subclause 6.6.2.3.3;

5)	if confidentiality protection is disabled as specified in subclause 6.6.2.3.1, shall include in the body of the SIP PUBLISH request, an application/vnd.3gpp.mcptt-info+xml MIME body as specified in Annex F.1 with:

a)	the <mcptt-access-token> element set to the value of the access token received during the user authentication procedures in the body of the SIP PUBLISH request; and

b)	the <mcptt-client-id> element set to the value of the MCPTT client ID of the originating MCPTT client;

6)	shall include an application/poc-settings+xml MIME body containing the Answer-Mode Indication setting in the <am-settings> element of the poc-settings event package set to the current answer mode setting ("auto-answer" or "manual-answer") of the MCPTT client according to IETF RFC 4354 [55]; and

7)	if integrity protection is enabled as specified in subclause 6.6.3.3.1, shall use the CSK to integrity protect the application/vnd.3gpp.mcptt-info+xml MIME body and application/poc-settings+xml MIME body by following the procedures in subclause 6.6.3.3.3.

The MCPTT client shall send the SIP PUBLISH request according to 3GPP TS 24.229 [4].

[TS 24.379, clause 7.2.3]

To set, update, remove or refresh the MCPTT service settings, the MCPTT client shall generate a SIP PUBLISH request according 3GPP TS 24.229 [4], IETF RFC 3903 [37] and IETF RFC 4354 [55]. In the SIP PUBLISH request, the MCPTT client:

1)	shall perform the procedures in subclause 7.2.1A;

2)	if confidentiality protection is enabled as specified in subclause 6.6.2.3.1, shall include in the body of the SIP PUBLISH request, an application/vnd.3gpp.mcptt-info+xml MIME body with:

a)	the <mcptt-request-uri> element set to the targeted MCPTT ID encrypted using the client server key (CSK), as specified in subclause 6.6.2.3.3; and

b)	the <mcptt-client-id> element set to the encrypted MCPTT client ID of the originating MCPTT client, as specified in subclause 6.6.2.3.3;

3)	if confidentiality protection is disabled as specified in subclause 6.6.2.3.1, shall include an application/vnd.3gpp.mcptt-info+xml MIME body as specified in Annex F.1 with:

a)	the <mcptt-request-uri> set to the cleartext targeted MCPTT ID; and

b)	the <mcptt-client-id> element set to the value of the MCPTT client ID of the originating MCPTT client;

4)	shall include an application/poc-settings+xml MIME body containing the Answer-Mode Indication setting in the <am-settings> element of the poc-settings event package set to the current answer mode setting ("auto-answer" or "manual-answer") of the MCPTT client according to IETF RFC 4354 [55]; and

5)	if integrity protection is enabled as specified in subclause 6.6.3.3.1, shall use the CSK to integrity protect the application/vnd.3gpp.mcptt-info+xml MIME body and application/poc-settings+xml MIME body by following the procedures in subclause 6.6.3.3.3.

The MCPTT client shall send the SIP PUBLISH request according to 3GPP TS 24.229 [4].

On receiving the SIP 200 (OK) response to the SIP PUBLISH request the MCPTT client may indicate to the MCPTT User the successful communication of the MCPTT service settings to the MCPTT server.

5.1.3	Test description

5.1.3.1	Pre-test conditions

System Simulator

-	SS (MCPTT server).

UE

-	The MCPTT client is attached to EPS services.

Table 5.1.3.1-1: UE configuration (MO parameters)

| MO parameter | Value |
| --- | --- |
| /<x>/OnNetwork/GMSURI | sip:gmsuri1.example.net |
| /<x>/OnNetwork/GroupCreationXUI | TBD |
| /<x>/OnNetwork/GMSXCAPRootURI | http:GMSXCAPRootURI.example.net |
| /<x>/OnNetwork/CMSXCAPRootURI | http:CMSXCAPRootURI.example.net |
| /<x>/OnNetwork/Timers/T100 | 2 |
| /<x>/OnNetwork/Timers/T101 | 2 |
| /<x>/OnNetwork/Timers/T103 | 5 |
| /<x>/OnNetwork/Timers/T104 | 2 |
| /<x>/OnNetwork/Timers/T132 | 3 |
| /<x>/OnNetwork/HPLMN/PLMN | TBD |
| /<x>/OnNetwork/HPLMN/Service/MCPTTToConRef/<x>/ConRef | TBD |
| /<x>/OnNetwork/HPLMN/Service/MCCommonCoreToConRef/<x>/ConRef | TBD |
| /<x>/OnNetwork/HPLMN/Service/MCIDMToConRef/<x>/ConRef | TBD |
| /<x>/OnNetwork/AppServerInfo/IDMSAuthEndpoint | http:IDMSAuthEndpoint.example.net |
| /<x>/OnNetwork/AppServerInfo/IDMSTokenEndpoint | http:IDMSTokenEndpoint.example.net |
| /<x>/OnNetwork/AppServerInfo/GMS | sip:gms1.example.net |
| /<x>/OnNetwork/AppServerInfo/CMS | sip:cms1.example.net |
| /<x>/OnNetwork/AppServerInfo/KMS | http:kms.example.net |

Preamble

-	The UE is switched-off.

5.1.3.2	Test procedure sequence

Table 5.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| O1 | UE (MCPTT Client) is switched-on. NOTE: This is expected to be done via a suitable implementation dependent MMI and does not trigger message. | - | - | - | - |
| - | EXCEPTION: The E-UTRA/EPC actions which are related to the MCPTT call establishment are described in TS 56.579-1 [2], subclause 5.4.3 'Generic Test Procedure for MCPTT CO communication in E-UTRA'. The test sequence below shows only the MCPTT relevant messages exchanged. | - | - | - | - |
| O2 | Wait for 10 seconds. | - | - | - | - |
| A1 | UE (MCPTT Client) establishes a secure TLS tunnel with the SS [IdMS Auth endpoint]. Editor’s Note: The exact message order or potential parallel for key management, registration and the retrieval of the configuration documents (groups of steps A, B, C, D, E, F, G) is tbd. There are also different options for including the token. Hence exact step numbering is TBD. | - | - | - | - |
| A2 | UE sends an OpenID Connect Authentication Request to the SS. | --> | OIDC Authentication Request | - | - |
| A3 | The SS replies to the OIDC Authentication Request message with an HTTP 20 (OK) Editor’s Note: How to configure the parameters for TLS for the HTTP connections may need to be clarified. | <-- | HTTP 200 (OK) | - | - |
| A4 | User authentication is performed. NOTE: This is expected to be done via a suitable implementation dependent MMI and does not trigger message. | - | - | - | - |
| A5 | The UE sends an HTTP POST Request message to the SS containing user name and password over the TLS connection established to the IdMS Auth endpoint. | --> | HTTP POST Request | - | - |
| A6 | The SS sends an OpenID Connect Authentication Response to the UE containing an authorization code. | <-- | OIDC Authentication Response [HTTP 302 (Found)] | - | - |
| A7 | UE (MCPTT Client) establishes a secure TLS tunnel with the SS [IdMS Token endpoint]. Editor’s Note: Exact message(s) is/are TBD. | - | - | - | - |
| A8 | Check: Does the UE send an OpenID Connect Token Request to the SS, passing the authorization code over the TLS connection established to the IdMS Token endpoint. | --> | OIDC Token Request | 1 | P |
| A9 | The SS sends an OpenID Connect Token Response to the UE containing an id_token and an access token which identify the user of the MCPTT service. NOTE: The access_token is used by the UE to communicate the identity of the MCPTT user to the MCPTT server. | <-- | OIDC Token Response [HTTP 200 (OK)] | - | - |
| B1 | UE establishes a secure HTTP tunnel using HTTPS between the UE (HTTP client) and the SS. NOTE: This secure tunnel is also used for HTTP requests in steps (KMS) D and E. Editor’s note: It is FFS whether the communication of key management server needs to take place before the SIP Registration in order to obtain the keys for the confidentiality and integrity protection. |  |  |  |  |
| B2 | The key management client makes a request for user key material from the MCPTT KMS. The request contains details of the identity (e.g. the MCPTT ID) requested for key management, and the time for which the key material is required |  | TBD |  |  |
| B3 | The KMS provides a response containing key material. The response includes the type of key material, the period of use for the material and any domain-specific parameters required for its use. For public safety use, the key material itself shall be wrapped using a 256-bit transport key (TrK). The TrK is distributed via an out-of-band mechanism along with a 32-bit identifier, TrK-ID |  | TBD |  |  |
| C1 | UE sends a SIP Register message to the SS. Editor’s Note: Need to determine whether the integrity protection and the confidentiality protection are enabled, and whether the UE knows this when it first IMS registers (IMS Registration is required in order to subscribe to obtain the configuration management documents that indicate whether these protections are enabled). | --> | SIP REGISTER | - | - |
| C2 | The SS sends a SIP Auth_Challenge message to UE that includes parameters IMPI, RAND and AUTN. | <-- | SIP Auth_Challenge | - | - |
| C3 | UE verifies Authentication and generates Keys and RES. Check: Does the UE send SIP REGISTER message that includes parameters IMPI, RES and Access Token ? | --> | SIP REGISTER | 1 | P |
| C4 | The SS sends SIP Auth_OK message NOTE: 3rd party registration mechanism is used by the SS to obtain the Access token if included. The SS then validates the access token and binds the IMPU and MCPTT ID. | <-- | SIP Auth OK | - | - |
| C5 | US Subscribes to the Registration Event package | --> | SIP SUBSCRIBE |  |  |
| C6 | S-CSCF sends a 200 OK | <-- | SIP 200 OK |  |  |
| C7 | S-CSCF sends a NOTIFY request containing the registration evet package | <-- | SIP NOTIFY |  |  |
| C8 | UE sends a 200 OK | --> | SIP 200 OK |  |  |
| D1 | Check: Does the UE present an access token to the SS over HTTP? | --> | HTTP GET | 1 | P |
| D2 | The SS authorizes the user for key management services and replies to the UE with identity specific key information. | <-- | HTTP 200 (OK) | - | - |
| E1 | The UE (Configuration Management Client) sends a SIP SUBSCRIBE to the SS containing the access token and a resource list mine body containing a list of the following documents: MCPTT UE Configuration document, MCPTT User Profile Configuration Document, and the MCPTT Service configuration document. The base URI of each list entry is set to the CMS XCAP-ROOT-URI, and the AUID is set to the application usage of the specific document. | --> | SIP SUBSCRIBE | - | - |
| E2 | The SS sends a SIP 200 (OK) message to the UE. | <-- | SIP 200 (OK) | - | - |
| E3 | The SS authorizes the user and sends a SIP NOTIFY message to the UE that contains the XCAP-URI of the documents. | <-- | SIP NOTIFY | - | - |
| E4 | The UE sends a SIP 200 (OK) message to the SS. | --> | SIP 200 (OK) | - | - |
| E5 | The UE sends an HTTP GET Request message to the SS that contains the access token and the XCAP-URI of the MCPTT UE Configuration document. | --> | HTTP GET | - | - |
| E6 | The SS authorizes the user and sends the HTTP 200 (OK) message including the MCPTT UE Configuration Document. | <-- | HTTP 200 (OK) | - | - |
| E7 | The UE sends an HTTP GET Request message to the SS that contains the access token and the XCAP-URI of the MCPTT User Profile Configuration Document. | --> | HTTP GET | - | - |
| E8 | The SS authorizes the user and sends the HTTP 200 (OK) message including the MCPTT User Profile Configuration Document. | <-- | HTTP 200 (OK) | - | - |
| E9 | Check: Does the UE sends an HTTP GET Request message to the SS that contains the access token and the XCAP-URI of the MCPTT Service Configuration Document? | --> | HTTP GET | 1 | P |
| E10 | The SS authorizes the user and sends the HTTP 200 (OK) message including the MCPTT Service Configuration Document. | <-- | HTTP 200 (OK) | - | - |
| F1 | The UE (Group Management Client) sends a SIP SUBSCRIBE to the SS, containing the access token and a resource list mine body and a list of the Groups to be obtained. The base URI of each list entry is set to the GMS XCAP-ROOT-URI, and the MCPTT group ID identifies a group document. | --> | SIP SUBSCRIBE | - | - |
| F2 | The SS sends a SIP 200 (OK) message to the UE. | <-- | SIP 200 (OK) | - | - |
| F3 | The SS authorizes the user and sends a SIP NOTIFY message to the UE that contains the XCAP-URI of the Group documents. | <-- | SIP NOTIFY | - | - |
| F4 | The UE sends a SIP 200 (OK) message to the UE. | --> | SIP 200 (OK) | - | - |
| F5 | Check: Does the UE sends an HTTP GET Request message to the SS that contains the access token and the XCAP-URI of the Group document ‘MCPTT UE Configuration document’? | --> | HTTP GET | 1 | P |
| F6 | The SS authorizes the user and sends the HTTP 200 (OK) message including the Group Document ‘MCPTT UE Configuration document’. | <-- | HTTP 200 (OK) | - | - |
| F7 | Check: Does the UE sends an HTTP GET Request message to the SS that contains the access token and the XCAP-URI of the Group document ‘MCPTT User Profile Configuration Document’? | --> | HTTP GET | 1 | P |
| F8 | The SS authorizes the user and sends the HTTP 200 (OK) message including the Group Document ‘MCPTT User Profile Configuration Document’. | <-- | HTTP 200 (OK) | - | - |
| F9 | Check: Does the UE sends an HTTP GET Request message to the SS that contains the access token and the XCAP-URI of the Group document ‘MCPTT Service configuration document’? | --> | HTTP GET | 1 | P |
| F10 | The SS authorizes the user and sends the HTTP 200 (OK) message including the Group Document ‘MCPTT Service configuration document’. | <-- | HTTP 200 (OK) | - | - |
| G1 | Check: Does the UE sends the SIP PUBLISH message to the SS including the IMPU and Answer-mode indication, and including the access token if not previously included in the SIP REGISTER request message? | --> | SIP PUBLISH | 1 | P |
| G2 | The SS, if an access token was included, validates the access token and binds the IMPU and MCPTT ID, stores the MCPTT service setting including the answer-mode indication and sends a SIP 200 (OK) message to the UE. | <-- | SIP 200 (OK) | - | - |

5.1.3.3	Specific message contents

Table 5.1.3.3-1: SIP REGISTER (Steps C1, Table 5.1.3.2-1)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| Request-Line |  | Reference: RFC 3261 [15] |  |
| Method | REGISTER |  |  |
| Request-URI | SIP URI formed from home domain name as stored in EFDOMAIN (when using ISIM) or SIP URI formed from home domain name derived from the IMSI (when no ISIM available on the UICC) |  |  |
| SIP-Version | SIP/2.0 |  |  |
| Route | not present | Reference: RFC 3261 [15] |  |
| Via |  | Reference: |  |
| sent-protocol | SIP/2.0/UDP (when using UDP) or SIP/2.0/TCP (when using TCP) | RFC 3261 [15] RFC 3581 [96] |  |
| sent-by | IP address or FQDN, port (optional) and not checked |  | A1, A3 |
| sent-by | IP address or FQDN and , when using UDP, protected server port of the UE |  | A2 |
| via-branch | value starting with ‘z9hG4bK’ |  |  |
| response-port | rport (when using UDP) |  | A1, A3 |
| From |  | Reference: RFC 3261 [15] |  |
| addr-spec | any IMPU within the set of IMPUs on ISIM (when using ISIM; NOTE 2) or public user identity derived from IMSI (when no ISIM available on the UICC) |  | A1 |
| addr-spec | same public user identity as in initial REGISTER |  | A2 |
| addr-spec | public user identity derived from IMSI |  | A3 |
| Tag | must be present, value not checked |  |  |
| To |  | Reference: RFC 3261 [15] |  |
| addr-spec | any IMPU within the set of IMPUs on ISIM (when using ISIM; NOTE 2) or public user identity derived from IMSI (when no ISIM available on the UICC) |  | A1 |
| addr-spec | same public user identity as in initial REGISTER |  | A2 |
| addr-spec | public user identity derived from IMSI |  | A3 |
| Tag | not present |  |  |
| Contact |  | Reference: RFC 3261 [15] RFC 5627 [61] |  |
| addr-spec | SIP URI with IP address or FQDN and indicating either an unprotected port selected by the UE or no port at all. |  | A1, A3 |
| addr-spec | SIP URI with IP address or FQDN and protected server port of UE. |  | A2 |
| c-p-instance | +sip.instance="<urn:gsma:imei: (gsma-specifier-defined-substring)>” where gsma-specifier-defined-substring shall be the IMEI code of the UE, coded as specified in RFC 7254 [122], without optional parameters | Reference: RFC 7254 [122] | A4 |
| ..MCPTT feature tag and service identifier | +3gpp.mcptt g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt". | Reference: TS 24.379 |  |
| expires | 600000 (if present) |  |  |
| Expires | present if no expires parameter in Contact header | Reference: RFC 3261 [15] |  |
| delta-seconds | 600000 |  |  |
| Require |  | Reference: RFC 3261 [15] RFC 3329 [21] | A1, A2 |
| option-tag | sec-agree |  |  |
| Proxy-Require |  | Reference: RFC 3261 [15] RFC 3329 [21] | A1, A2 |
| option-tag | sec-agree |  |  |
| Supported |  | Reference: Reference: RFC 3261 [15] TS 24.229 [10] |  |
| option-tag | Gruu |  | A4 |
| option-tag | Path |  |  |
| CSeq |  | Reference: RFC 3261 [15] |  |
| Value | must be present, value not checked |  | A1, A3 |
| Value | must be incremented from the previous REGISTER |  | A2 |
| Method | REGISTER |  |  |
| Call-ID |  | Reference: RFC 3261 [15] |  |
| Called | value not checked |  |  |
| Security-Client |  | Reference: RFC 3329 [21] RFC 4835 [124] | A1, A2 |
| mechanism-name | ipsec-3gpp |  |  |
| Algorithm | hmac-sha-1-96 |  |  |
| Protocol | esp (if present) |  |  |
| Mode | trans (if present) |  |  |
| encrypt-algorithm | des-ede3-cbc or aes-cbc or null |  |  |
| spi-c | SPI number of the inbound SA at the protected client port |  |  |
| spi-s | SPI number of the inbound SA at the protected server port |  |  |
| port-c | protected client port |  |  |
| port-s | protected server port |  |  |
| Security-Verify |  | Reference: RFC 3329 [21] | A2 |
| sec-mechanism | same value as Security Server header sent by SS |  | A2 |
| Security-Verify | not present |  |  |
| Authorization | Digest | Reference: RFC 2617 [16] RFC 3310 [17] | A1 |
| Username | private user identity as stored in EFIMPI (when using ISIM) or private user identity derived from IMSI (when no ISIM available on the UICC) |  | A1 |
| Realm | home domain name as stored in EFDOMAIN (when using ISIM) or home domain name derived from the IMSI (when no ISIM available on the UICC) |  | A1 |
| Nonce | set to an empty value |  | A1 |
| digest-uri | SIP URI formed from home domain name as stored in EFDOMAIN (when using ISIM) or formed from home domain name derived from the IMSI (when no ISIM available on the UICC) |  | A1 |
| Response | set to an empty value |  | A1 |
| Authorization | Digest | Reference: RFC 2617 [16] RFC 3310 [17] | A2 |
| Username | private user identity as stored in EFIMPI (when using ISIM) or private user identity derived from IMSI (when no ISIM available on the UICC) |  | A2 |
| Realm | same value as received in the realm directive in the WWW Authenticate header sent by SS |  | A2 |
| Nonce | same value as in WWW-Authenticate header sent by SS |  | A2 |
| Opaque | same value as sent by the server in “401 Unauthorized for REGISTER” |  | A2 |
| digest-uri | SIP URI formed from home domain name as stored in EFDOMAIN (when using ISIM) or formed from home domain name derived from the IMSI (when no ISIM available on the UICC) |  | A2 |
| qop-value | auth |  | A2 |
| cnonce-value | value assigned by UE affecting the response calculation |  | A2 |
| nonce-count | counter to indicate how many times UE has sent the same value of nonce within successive REGISTERs, initial value shall be 1 |  | A2 |
| Response | response calculated by UE |  | A2 |
| Algorithm | AKAv1-MD5 |  | A2 |
| Max-Forwards |  | Reference: RFC 3261 [15] |  |
| Value | non-zero value |  |  |
| P-Access-Network-Info |  | Reference: RFC 7315 [132] | A2 |
| access-net-spec | access network technology and, if applicable, the cell ID |  | A2 |
| Content-Type Value | multipart/mime application/mikey application/vnd.3gpp.mcptt-info+xml | Reference: RFC 3261 [15] |  |
| Content-Length | header shall be present if the Authorization Token is present (there is a message-body) | Reference: RFC 3261 [15] |  |
| Value | length of request body, if such is present |  |  |

| Condition | Explanation |
| --- | --- |
| A1 | Initial unprotected REGISTER (IMS security, A.6a/2 3GPP TS 34.229-2 [5]) |
| A2 | Subsequent REGISTER sent over security associations (IMS security, A.6a/2 3GPP TS 34.229-2 [5]) |
| A3 | REGISTER for the case UE supports GIBA (A.6a/1 3GPP TS 34.229-2 [5]) |
| A4 | obtaining and using GRUUs in the Session Initiation Protocol (SIP) (A.4/53 3GPP TS 34.229-2 [5]). Mandatory from Rel-10 onwards. |
| NOTE 1: All choices for applicable conditions are described for each header. NOTE 2: Public user identity shall be the same for ‘From’ and ‘To’. |  |

Table 5.1.3.3-2: SIP PUBLISH (Step G1, Table 5.1.3.2-1)

| TBD |  |  |  |
| --- | --- | --- | --- |
| Derivation Path: TBD |  |  |  |
| Information Element | Value/remark | Comment | Condition |
| Request-Line |  | Reference: RFC 3903 [xx] TS 24.379 |  |
| Method | PUBLISH |  |  |
| Request-URI | SIP URI containing the public service identity identifying the participating MCPTT function serving the MCPTT user |  |  |
| SIP-Version | SIP/2.0 |  |  |
| Route | The service route as delivered in the 200 OK response to the REGISTER | Reference: RFC 3261 [15] |  |
| Via |  | RFC 3261 [15] RFC 3581 [96] |  |
| sent-protocol | SIP/2.0/UDP (when using UDP) or SIP/2.0/TCP (when using TCP) |  |  |
| sent-by | IP address or FQDN, port (optional) and not checked |  | A1, A3 |
| sent-by | IP address or FQDN and, when using UDP, protected server port of the UE |  | A2 |
| via-branch | value starting with ‘z9hG4bK’ |  |  |
| response-port | rport (when using UDP) |  | A1, A3 |
| From |  | Reference: RFC 3261 [15] |  |
| addr-spec | The Public User Identity from the P-Associated URI |  |  |
| addr-spec | as in initial REGISTER |  |  |
| tag | must be present, value not checked |  |  |
| To |  | Reference: RFC 3261 [15] |  |
| addr-spec | SIP URI containing the public service identity identifying the participating MCPTT function serving the MCPTT user |  |  |
| tag | not present |  |  |
| P-Preferred-Service |  | RFC 6050 [yy] TS 24.379 |  |
| Service-ID | "urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  |
| Event |  | Reference: RFC 6665 [zz] |  |
| event-type | "poc-settings" |  |  |
| Expires | present if no expires parameter in Contact header | RFC 3261 [15] RFC 3329 [21] |  |
| delta-seconds | 4294967295 |  |  |
| Proxy-Require |  | RFC 3261 [15] RFC 3329 [21] | A1, A2 |
| option-tag | sec-agree |  |  |
| Supported |  | RFC 3261 [15] TS 24.229 [10] |  |
| option-tag | Gruu |  | A4 |
| option-tag | Path |  |  |
| CSeq |  | RFC 3261 [15] |  |
| value | must be present, value not checked |  | A1, A3 |
| value | must be incremented from the previous REGISTER |  | A2 |
| method | REGISTER |  |  |
| Call-ID |  | RFC 3261 [15] |  |
| callid | value not checked |  |  |
| Security-Client |  | Reference: RFC 3329 [21] RFC 4835 [124] | A1, A2 |
| mechanism-name | ipsec-3gpp |  |  |
| algorithm | hmac-sha-1-96 |  |  |
| protocol | esp (if present) |  |  |
| mode | trans (if present) |  |  |
| encrypt-algorithm | des-ede3-cbc or aes-cbc or null |  |  |
| spi-c | SPI number of the inbound SA at the protected client port |  |  |
| spi-s | SPI number of the inbound SA at the protected server port |  |  |
| port-c | protected client port |  |  |
| port-s | protected server port |  |  |
| Security-Verify |  | RFC 3329 [21] | A2 |
| sec-mechanism | same value as Security Server header sent by SS |  | A2 |
| Security-Verify | not present |  | A1, A3 |
| Max-Forwards |  | RFC 3261 [15] |  |
| value | non-zero value |  |  |
| P-Access-Network-Info |  | Reference: RFC 7315 [132] | A2 |
| access-net-spec | access network technology and, if applicable, the cell ID |  | A2 |
| Content-Type value | multipart/mime application/mikey | Reference: RFC 3261 [15] |  |
| Content-Length |  | Reference: RFC 3261 [15] |  |
| value | length of request body |  |  |

| Condition | Explanation |
| --- | --- |
| A1 | Initial unprotected REGISTER (IMS security, A.6a/2 3GPP TS 34.229-2 [5]) |
| A2 | Subsequent REGISTER sent over security associations (IMS security, A.6a/2 3GPP TS 34.229-2 [5]) |
| A3 | REGISTER for the case UE supports GIBA (A.6a/1 3GPP TS 34.229-2 [5]) |
| A4 | obtaining and using GRUUs in the Session Initiation Protocol (SIP) (A.4/53 3GPP TS 34.229-2 [5]). Mandatory from Rel-10 onwards. |
| NOTE 1: All choices for applicable conditions are described for each header. NOTE 2: Public user identity shall be the same for ‘From’ and ‘To’. |  |

Table 5.1.3.3-3: SIP SUBSCRIBE (Steps C5, E1 and F1, Table 5.1.3.2-1)

| TBD |  |  |  |
| --- | --- | --- | --- |
| Derivation Path: TBD |  |  |  |
| Information Element | Value/remark | Comment | Condition |
| Request-Line |  | Reference: RFC 6665 [xx] TS 24.229 |  |
| Method | SUBSCRIBE |  |  |
| Request-URI | SIP URI |  |  |
| SIP-Version | SIP/2.0 |  |  |
| Route | The service route as delivered in the 200 OK response to the REGISTER | Reference: RFC 3261 [15] |  |
| Via |  | Ref. RFC 3261 RFC 3581 [96] |  |
| sent-protocol | SIP/2.0/UDP (when using UDP) or SIP/2.0/TCP (when using TCP) |  |  |
| sent-by | IP address or FQDN, port (optional) and not checked |  | A1, A3 |
| sent-by | IP address or FQDN and, when using UDP, protected server port of the UE |  | A2 |
| via-branch | value starting with ‘z9hG4bK’ |  |  |
| response-port | rport (when using UDP) |  | A1, A3 |
| From |  | Reference: RFC 3261 [15] |  |
| addr-spec | The Public User Identity from the P-Associated URI |  |  |
| addr-spec | as in initial REGISTER |  |  |
| tag | must be present, value not checked |  |  |
| To |  | Reference: RFC 3261 [15] |  |
| addr-spec | SIP URI containing t |  |  |
| tag | not present |  |  |
| Event |  | RFC 6665 [zz] |  |
| event-type | "reg-events" |  |  |
| Expires | present if no expires parameter in Contact header | Reference: RFC 3261 [15] RFC 3329 [21] |  |
| delta-seconds | 4294967295 |  |  |
| Proxy-Require |  | Reference: RFC 3261 [15] RFC 3329 [21] | A1, A2 |
| option-tag | sec-agree |  |  |
| Supported |  | Reference: RFC 3261 [15] TS 24.229 [10] |  |
| option-tag | Gruu |  | A4 |
| option-tag | Path |  |  |
| CSeq |  | Reference: RFC 3261 [15] |  |
| value | must be present, value not checked |  | A1, A3 |
| value | must be incremented from the previous REGISTER |  | A2 |
| method | REGISTER |  |  |
| Call-ID |  | Reference: RFC 3261 [15] |  |
| callid | value not checked |  |  |
| Security-Client |  | Reference: RFC 3329 [21] RFC 4835 [124] | A1, A2 |
| mechanism-name | ipsec-3gpp |  |  |
| algorithm | hmac-sha-1-96 |  |  |
| protocol | esp (if present) |  |  |
| mode | trans (if present) |  |  |
| encrypt-algorithm | des-ede3-cbc or aes-cbc or null |  |  |
| spi-c | SPI number of the inbound SA at the protected client port |  |  |
| spi-s | SPI number of the inbound SA at the protected server port |  |  |
| port-c | protected client port |  |  |
| port-s | protected server port |  |  |
| Security-Verify |  | Reference: RFC 3329 [21] | A2 |
| sec-mechanism | same value as Security Server header sent by SS |  | A2 |
| Security-Verify | not present |  | A1, A3 |
| Max-Forwards |  | Reference: RFC 3261 [15] |  |
| value | non-zero value |  |  |
| P-Access-Network-Info |  | Reference: RFC 7315 [132] | A2 |
| access-net-spec | access network technology and, if applicable, the cell ID |  | A2 |
| Content-Length |  | Reference: RFC 3261 [15] |  |
| value | 0 |  |  |
| Contact |  | Reference: RFC 3261 [15] RFC 5627 [61] |  |
| addr-spec | SIP URI with IP address or FQDN and indicating either an unprotected port selected by the UE or no port at all. |  | A1, A3 |
| addr-spec | SIP URI with IP address or FQDN and protected server port of UE. |  | A2 |

| Condition | Explanation |
| --- | --- |
| A1 | TBD |
| A2 | TBD |
| A3 | TBD |
| A4 | TBD |

Table 5.1.3.3-4: SIP NOTIFY (Steps C7, E3 and F3, Table 5.1.3.2-1)

| Header/param | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| Request-Line |  | RFC 3261 [21] |  |
| Method | NOTIFY |  |  |
| Request-URI | SIP URI containing the URI from the Contact header field in the SUBSCRIBE |  |  |
| SIP-Version | SIP/2.0 |  |  |
| Route | The Record-Route as delivered in the SIP SUBSCRIBE | RFC 3261 [15] |  |
| From |  | RFC 3261 [15] |  |
| addr-spec | The contents of the From Field from the SUBSCRIBE |  |  |
| tag | must be present |  |  |
| To |  | RFC 3261 [15] |  |
| addr-spec | The contents of the To Field from the SUBSCRIBE |  |  |
| tag | must be present |  |  |
| Event |  | RFC 6665 [zz] |  |
| event-type | "reg-events" |  |  |
| Expires | present if no expires parameter in Contact header | RFC 3261 [15] RFC 3329 [21] |  |
| delta-seconds | 4294967295 |  |  |
| Proxy-Require |  | RFC 3261 [15] RFC 3329 [21] | A1, A2 |
| option-tag | sec-agree |  |  |
| Supported |  | RFC 3261 [15] TS 24.229 [10] |  |
| option-tag | Gruu |  | A4 |
| option-tag | Path |  |  |
| CSeq |  | RFC 3261 [15] |  |
| value | must be present, value not checked |  | A1, A3 |
| value | must be incremented from the previous REGISTER |  | A2 |
| method | REGISTER |  |  |
| Call-ID |  | RFC 3261 [15] |  |
| callid | value not checked |  |  |
| Security-Client |  | RFC 3329 [21] RFC 4835 [124] | A1, A2 |
| mechanism-name | ipsec-3gpp |  |  |
| algorithm | hmac-sha-1-96 |  |  |
| protocol | esp (if present) |  |  |
| mode | trans (if present) |  |  |
| encrypt-algorithm | des-ede3-cbc or aes-cbc or null |  |  |
| spi-c | SPI number of the inbound SA at the protected client port |  |  |
| spi-s | SPI number of the inbound SA at the protected server port |  |  |
| port-c | protected client port |  |  |
| port-s | protected server port |  |  |
| Security-Verify |  | RFC 3329 [21] | A2 |
| sec-mechanism | same value as Security Server header sent by SS |  | A2 |
| Security-Verify | not present |  | A1, A3 |
| Max-Forwards |  | RFC 3261 [15] |  |
| value | non-zero value |  |  |
| P-Access-Network-Info |  | RFC 7315 [132] | A2 |
| access-net-spec | access network technology and, if applicable, the cell ID |  | A2 |
| Content-Type |  | RFC 3261 [15] |  |
| value |  |  |  |
| Content-Length |  | RFC 3261 [15] |  |
| value | length of request body |  |  |

| Condition | Explanation |
| --- | --- |
| A1 | TBD |
| A2 | TBD |
| A3 | TBD |
| A4 | TBD |

Table 5.1.3.3-5: SIP SUBSCRIBE (Step E1, Table 5.1.3.2-1)

| TBD |  |  |  |
| --- | --- | --- | --- |
| Derivation Path: TBD |  |  |  |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| Max-Forwards |  | 70 |  |  |  |  |  |
| P-Access-Network-Info | 3GPP-E-UTRAN-TDD |  |  |
| Route | <sip:pcscf1.home1.net:7531;lr> <sip:orig@scscf1.home1.net;lr> |  |  |
| P-Preferred-Identity | <sip:user1_public1@home1.net> |  |  |
| Privacy |  | None |  |  |  |  |  |
| From |  | <sip:user1_public1@home1.net> |  |  |  |  |  |
| tag |  | 31415 |  |  |  |  |  |
| To | <sip:cms1.example.nett> |  |  |
| Call-ID | b89rjhnedlrfjflslj40a222 |  |  |
| CSeq | 123 SUBSCRIBE |  |  |
| Require | sec-agree |  |  |
| Proxy-Require | sec-agree |  |  |
| Security-Verify | ipsec-3gpp |  |  |
| P-Preferred-Service:urn | 3gpp-service.ims.icsi.mcptt |  |  |
| Event | xcap-diff |  |  |
| Expires | 7200 |  |  |
| Accept | application/xcap-diff+xml |  |  |
| Contact | <sip:user1_public1@home1.net> |  |  |
| Content-Type | multipart/mixed |  |  |
| Content-Length |  | Length of request body |  |  |  |  |  |
| TBD |  |  |  |

Editor’s Note: Additional default messages are TBD.