3GPP TSG-RAN5 Meeting #79	R5-182493r3

# Busan, Korea (Republic Of), 21st May 2018 - 25th May 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1147 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update TFT configuration in Dedicated Bearer to Allow IP Ping Operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Japan Inc |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2018-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To allow IP ping operation in 5GS and check IP stack and Data flow functionality |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update to the TS 36.508 -ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST and Reference packet filters to include ICMP in TFT |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ICMP ping operation will not be possible for verification of IP stack and data flow. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.3 and 6.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact R5-182493R5-182493r1 – Added handling of IPv6 and Note 3 in Reference Packet Filter table#1and 2, removed the TFT condition from Packet Filer Table 3,4,5 R5-182493r2 Added QCI for Dedicated Bearer 1 for EN-DC case R5-182493r3 Removed the ACTIVATE DEDICATED EPS BEARER changes as they were redundant. Removed FFS regarding Pc_IP_Ping. Note3 is removed for checking IPv6 address assignement. |  |  |  |  |  |  |  |  |

### 6.6.2	Reference dedicated EPS bearer contexts

The following table defines Reference dedicated EPS bearer contexts. Dedicated EPS bearer context #1 is the default "dedicated EPS bearer context" which is used in the common procedures and test cases where no particular dedicated EPS bearer context is specified.

Table 6.6.2-1: Reference dedicated EPS bearer contexts

| Parameters | Reference dedicated EPS bearer context #1 | Reference dedicated EPS bearer context #2 | Reference dedicated EPS bearer context #3 | Reference dedicated EPS bearer context #4 | Reference dedicated EPS bearer context #5 |
| --- | --- | --- | --- | --- | --- |
| Linked EPS bearer identity | Reference default EPS bearer #1 | Reference default EPS bearer #1 | Reference default EPS bearer #2 | Reference default EPS bearer #2 | Reference default EPS bearer #3 |
| EPS QoS |  |  |  |  |  |
| QCI (Note 1) | 1 (GBR QCI), 8 (Note 9) (non-GBR QCI) | 5 (non-GBR QCI) | 2 (GBR QCI) | 1 (GBR QCI) | 65 (Note 8) (GBR QCI) |
| Maximum bit rate for uplink | 384 kbps | 384 kbps (Note 2) | 2104 (Note 7) | 384 kbps | 384 kbps |
| Maximum bit rate for downlink | 384 kbps | 384 kbps (Note 2) | 2104 (Note 7) | 384 kbps | 384 kbps |
| Guaranteed bit rate for uplink | 128 kbps | 128 kbps (Note 2) | 2104 (Note 7) | 128 kbps | 128 kbps |
| Guaranteed bit rate for downlink | 128 kbps | 128 kbps (Note 2) | 2104 (Note 7) | 128 kbps | 128 kbps |
| Maximum bit rate for uplink (extended) | 0 | 0 | 0 | 0 | 0 |
| Maximum bit rate for downlink (extended) | 0 | 0 | 0 | 0 | 0 |
| Guaranteed bit rate for uplink (extended) | 0 | 0 | 0 | 0 | 0 |
| Guaranteed bit rate for downlink (extended) | 0 | 0 | 0 | 0 | 0 |
| TFT |  |  |  |  |  |
| TFT operation code | "create new TFT" | "create new TFT" | "create new TFT" | "create new TFT" | "create new TFT" |
| E bit | 0 | 0 | 0 | 0 | 0 |
| Packet filters (Note 5) | 1, 2 | 3 | 5 | 4 | 1, 2 |
| Negotiated QoS |  |  |  |  |  |
| Traffic Class | conversational | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | conversational | conversational | conversational |
| Delivery Order | 'no' | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | ’no’ | 'no' | 'no' |
| Delivery of erroneous SDU | 'no' | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | ’no’ | 'no' | 'no' |
| Maximum SDU size | 150 | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | 1400 | 150 | 150 |
| Maximum bit rate for uplink | 384 kbps | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | 432 kbps | 384 kbps | 384 kbps |
| Maximum bit rate for downlink | 384 kbps | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | 432 kbps | 384 kbps | 384 kbps |
| Residual BER | 5*10-2 | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | 10-5 | 5*10-2 | 5*10-2 |
| SDU error ratio | 10-2 | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | 7*10-3 | 10-2 | 10-2 |
| Transfer delay | 80 ms | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | 130 ms | 80 ms | 80 ms |
| Traffic Handling priority | 0 | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | 0 | 0 | 0 |
| Guaranteed bit rate for uplink | 0 | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | 0 | 0 | 0 |
| Guaranteed bit rate for downlink | 0 | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | 0 | 0 | 0 |
| Signalling Indication | 0 | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | 0 | 0 | 0 |
| Source Statistics Descriptor | 0 | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | 0 | 1 | 0 |
| Maximum bit rate for downlink (extended) | 0 | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | 0 | 0 | 0 |
| Guaranteed bit rate for downlink (extended) | 0 | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | 0 | 0 | 0 |
| Maximum bit rate for uplink (extended) | 0 | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | 0 | 0 | 0 |
| Guaranteed bit rate for uplink (extended) | 0 | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | 0 | 0 | 0 |
| Negotiated LLC SAPI | 3 (Note 6) | See "PDP context3" (Note 6) | - | 3 (Note 6) | - |
| Radio priority | 1 (Note 6) | See "PDP context3" (Note 6) | - | 1 (Note 6) | - |
| Protocol configuration options | - | - | - | - | - |
| Note 1: For all non-GBR QCIs, the maximum and guaranteed bit rates shall be ignored. Note 2: According to TS 24.301, the UE ignores these parameters for a non-GBR QCI. Note 3: Parameters included for UEs capable of UTRAN according to TS 34.123-3 clause 8.10 (table 8.10.1 and 8.10.2). Note 5: This row refers to the reference packet filters defined in the tables below. For each reference dedicated EPS bearer context, a list of reference packet filter numbers is provided. Note 6: Parameter included for UEs capable of GERAN; when value is not provided then value from TS 51.010 subclause 40.5 is applied. Note 7: This value shall reflect the highest possible bandwidth requirement of the H.264 codec to EPS bearer mappings as specified in Appendix E of TS 26.114 [64]. Note 8: QCI 65 is used for MCPTT voice. Note 9: QCI 8 is used for EN-DC |  |  |  |  |  |

#### <Change2 Starts >

Table 6.6.2-2: Reference packet filter #1

| Derivation path: 24.008 table 10.5.162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Identifier | 0 0 0 1 0 0 0 0 | DL only filter,ID=0 |  |
| Evaluation precedence | (0 0 0 0 0 0 0 0) + EPS Bearer ID - 6 | 0 to 7 |  |
| Component type 1 ID | 0 0 0 1 0 0 0 0 | IPv4 remote address type | remoteIPv4 |
|  | 0 0 1 0 0 0 0 0 | IPv6 remote address type | remoteIPv6 |
| Component type 1 Value | remoteAddress 255.255.255.255 | See note 1 | remoteIPv4 |
|  | remoteAddress ff:ff:ff:ff:ff:ff:ff:ff: ff:ff:ff:ff:ff:ff:ff:ff | See note 1 | remoteIPv6 |
| Component type 2 ID | 0 1 0 1 0 0 0 0 | Single remote port type |  |
| Component type 2 Value | 31 160 + EPS Bearer ID - 6 |  |  |
| Component type 3 ID | 0 0 1 1 0 0 0 0 | Protocol identifier/Next header type |  |
| Component type 3 Value | 17 | UDP |  |
| Component type 3 Value | 1 | ICMP (See Note 2) | remote IPv4 |
| Component type 3 Value | 58 | ICMPv6 {see Note 2) | Remote IPv6 |
| Note 1: remoteAddress should be set to the address of an IP server able to send a flow of downlink IP/UDP packets to the UE. remoteIPv4 applies if the UE has acquired an IPv4 address only, remoteIPv6 applies if the UE has acquired an IPv6 address only, or both an IPv6 and an IPv4 address. Note 2: Component type 3 value of ICMP or ICMPv6 applies if TS 38.508-2 [5] pc_IP_Ping is true |  |  |  |

Table 6.6.2-3: Reference packet filter #2

| Derivation path: 24.008 table 10.5.162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Identifier | 0 0 1 0 0 0 0 1 | UL only filter, ID=1 |  |
| Evaluation precedence | (0 0 0 0 0 0 0 0) + EPS Bearer ID - 6 + 8 | 8 to 15 |  |
| Component type 1 ID | 0 0 0 1 0 0 0 0 | IPv4 remote address type | remoteIPv4 |
|  | 0 0 1 0 0 0 0 0 | IPv6 remote address type | remoteIPv6 |
| Component type 1 Value | remoteAddress 255.255.255.255 | See note 1 | remoteIPv4 |
|  | remoteAddress ff:ff:ff:ff:ff:ff:ff:ff: ff:ff:ff:ff:ff:ff:ff:ff | See note 1 | remoteIPv6 |
| Component type 2 ID | 0 1 0 1 0 0 0 0 | Single remote port type |  |
| Component type 2 Value | 61 000 + EPS Bearer ID – 6 |  |  |
| Component type 3 ID | 0 0 1 1 0 0 0 0 | Protocol identifier/Next header type |  |
| Component type 3 Value | 17 | UDP |  |
| Component type 3 Value | 1 | ICMP (See Note 2) | Remote IPv4 |
| Component type 3 Value | 58 | ICMPv6 {see Note 2) | Remote IPv6 |
| Note 1: remoteAddress should be set to the address of an IP server able to process a flow of uplink IP/UDP packets received from the UE. When configured together with packet filter #1, remoteAddress is the same as that for packet filter #1. remoteIPv4 applies if the UE has acquired an IPv4 address only, remoteIPv6 applies if the UE has acquired an IPv6 address only, or both an IPv6 and an IPv4 address. Note 2: Component type 3 value of ICMP or ICMPv6 applies if TS 38.508-2 [5] pc_IP_Ping is true |  |  |  |

Table 6.6.2-4: Reference packet filter #3

| Derivation path: 24.008 table 10.5.162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Identifier | 0 0 1 1 0 0 1 0 | Bidirectional filter, ID=2 |  |
| Evaluation precedence | 0 0 0 0 1 1 1 1 | Lowest priority |  |
| Component type 1 ID | 0 0 0 1 0 0 0 0 | IPv4 remote address type | remoteIPv4 |
|  | 0 0 1 0 0 0 0 0 | IPv6 remote address type | remoteIPv6 |
| Component type 1 Value | remoteAddress 255.255.255.255 | See note 1 | remoteIPv4 |
|  | remoteAddress ff:ff:ff:ff:ff:ff:ff:ff: ff:ff:ff:ff:ff:ff:ff:ff | See note 1 | remoteIPv6 |
| Note 1: remoteAddress should be set to the address of an IP server able to process a uplink IP packets from the UE and transmit downlink IP packets to the UE. remoteIPv4 applies if the UE has acquired an IPv4 address only, remoteIPv6 applies if the UE has acquired an IPv6 address only, or both an IPv6 and an IPv4 address. |  |  |  |

Table 6.6.2-5: Reference packet filter #4

| Derivation path: 24.008 table 10.5.162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Identifier | 0 0 1 1 0 0 1 1 | Bidirectional,ID=3 |  |
| Evaluation precedence | (0 0 0 0 0 0 0 0) + EPS Bearer ID – 6 | 0 to 7 |  |
| Component type 1 ID | 0 1 0 1 0 0 0 1 | Remote port range type |  |
| Component type 1 Value | media port | SS speech media port as used in the SDP negotiation (RTP remote port); see Note 1 |  |
|  | media port + 1 | RTCP remote port; see Note 1 |  |
| Component type 2 ID | 0 0 1 1 0 0 0 0 | Protocol identifier/Next header type |  |
| Component type 2 Value | 17 | UDP |  |
| Note 1: Acc. to TS 26.114 and RFC 4566 a "media port" can be understood as the transport port to which a media stream is sent. |  |  |  |

Table 6.6.2-6: Reference packet filter #5 (TS 36.508)

| Derivation path: 24.008 table 10.5.162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Identifier | 0 0 1 1 0 1 0 0 | Bidirectional,ID=4 |  |
| Evaluation precedence | (0 0 0 0 0 0 0 0) + EPS Bearer ID – 6 | 0 to 7 |  |
| Component type 1 ID | 0 1 0 1 0 0 0 1 | Remote port range type |  |
| Component type 1 Value | media port | SS video media port as used in the SDP negotiation (RTP remote port); see Note 1 |  |
|  | media port + 1 | RTCP remote port; see Note 1 |  |
| Component type 2 ID | 0 0 1 1 0 0 0 0 | Protocol identifier/Next header type |  |
| Component type 2 Value | 17 | UDP |  |
| Note 1: Acc. to TS 26.114 and RFC 4566 a "media port" can be understood as the transport port to which a media stream is sent. |  |  |  |

#### <Change2 Ends >