# Energy Consumption Analysis of Radio Interface Procedures for Positioning Enhancements

# introduction

New WI to improve the positioning measurements for IoT devices was approved in Rel-14. TA based multilateration method is described in [1] along with reference radio interface procedure and corresponding energy consumption analysis.

Enhancements to radio interface procedure for TA multilateration is proposed in [2] which reduces the required signalling and also MS transmissions with some alternative procedures. Multilateration based on MS OTD measurements along with serving cell TA measurements is proposed in [3].

This paper compares the energy consumption for the reference procedure [1] with that for the new procedures proposed for multilateration [2] and [3].

A combined solution of key aspects of [2] and [3] is proposed as candidate solution for EC-GSM-IoT positioning procedure.

# scenario and key assumptions for energy calculation

The energy calculation provided in [1] considers 3 neighbour cells for TA multilateration and estimates the energy for MS procedures in each of the cells.

If the UE measurement provides the 5 best cells for further consideration of multilateration as assumed in table 1 of [1], in many cases these 5 cells will belong to 2 neighbour base station sites and the serving base station site. The serving cell has the best signal strength compared to neighbour cells in most of the cases, hence when performing a trilateration based on these 3 sites, including the serving cell will normally be a reasonable choice. So the energy consumption analysis is not required to consider access to 3 neighbour cells – instead just 2 neighbour cell accesses will be sufficient.

For EC-GSM-IoT MS, it is possible that at least one of the neighbour cells which is a candidate for TA estimation is in extended coverage conditions. In this case the energy consumption analysis should consider a higher coverage class for at least one of the base stations.

The energy consumption comparison presented in the next section assumes

- Out of the candidate cells selected by SMLC for TA estimation, one of the cells will be serving cell itself. Hence the TA estimation procedure is required in only two neighbour cells (NCL-1 and NCL-2 in table 1).

- To consider a realistic case for the energy consumption, one of the neighbour cells (NCL-2) is assumed to be in extended coverage condition (CC2) for the MS in table 1.

- The network synchronisation time used in calculations is modified according to coverage class in which the MS is w.r.t. a base station.

- The voltage and current values used in the calculations are the same as specified in [1].

# energy consumption analysis of radio interface procedures

Table 1: Comparison of the energy consumption for a trilateration

Note : The energy consumption for measurement of 5 neighbour-cells remains same for all methods and it does not include additional time taken to measure the neighbour cell with CC2 condition for the MS.

In Table 1, the column for the reference procedure is based on the multilateration proposal in [1]. The column on the left hand side of the column for the reference procedure lists the numbers of bursts needed for the reference procedure, and on the right hand side, the numbers of bursts for the modified procedure proposed in [2] are listed. Two variants of the modified procedure are considered, first the TA measurements with the neighbour cells using dedicated channels and then the TA measurements with the neighbour cells using EC-CCCH. The column on the right hand side lists the energy consumption for the case that observed time differences reported by the MS can replace the TA measurements with neighbour cells.

The PRTI based procedure [2] improves the steps involved in Page response and triggering the MS to perform the measurement by the use of short identifier. Due to use of short identifier this step is simplified and does not require a TBF establishment. This enhancement is common for all new procedures.

Dedicated access procedure, as indicated, does not use the common channel access so these steps are avoided here. Common channel access procedure does not require TBF establishment so these steps are excluded here.

OTD based positioning procedure does not require any access to neighbour cells so complete steps related to neighbour cell access are excluded for this procedure.

The energy saving by the PRTI based procedure is estimated as 43% whereas, if the OTD based TA multilateration procedure can be applied, the energy saving even increases to around 74% compared to reference procedure. If the OTD based TA multilateration could be applied on average for every second positioning procedure, the average saving would be ~59 %.

# combined enhanced procedure for position estimation

The OTD based position estimation procedure requires the estimated real time differences between base stations to be updated regularly based on TA measurements from mobiles. So on regular basis TA measurements using the PRTI based procedure is required to keep the real time difference data updated in SMLC. Whenever all the necessary, valid real time difference information is available at SMLC, no TA measurements with neighbour cells are needed for position estimation.

Below message sequence diagram in Figure 1 shows the combined procedure. At the end, it indicates also the possibility that OTD values received with the neighbour cell measurement report are combined with one neighbour cell TA measurement to estimate the MS position.

Figure 1: Message sequence diagram for combined enhanced procedure.

# conclusion

As per the energy consumption analysis for the case that the MS is w.r.t. one of the neighbour cells in CC2, it is possible to improve the energy saving between 43% and 74% by modifying the radio interface procedures and also based on MS OTD measurements.

Even though the traffic model for positioning measurements is not available at this moment, the expected load will be higher than the network command traffic model specified in [4]. In such cases energy saving w.r.t. the reference procedure is essential to maintain longer battery life for IoT devices.

The sourcing company proposes to consider the new procedure sketched in this document for positioning estimation enhancements WI in Rel-14.

# references

[1] RP-161034, "Positioning enhancements for GERAN – introducing Timing Advance trilateration". Source Ericsson. 3GPP TSG RAN#72.

[2] R6-160034, "Radio Interface Enhancements for TA based multilateration". Source Nokia. 3GPP TSG RAN6#1.

[3] R6-160035, "Energy efficient hybrid TA/OTD multilateration for neighbour cells in extended coverage". Source Nokia. 3GPP TSG RAN6#1.

[4] 3GPP TR 45.820, Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things