3GPP TSG-RAN WG6 #1                                                                          R6-160054

Gothenburg, Sweden                                                                      Agenda item 6.1

August 22-26, 2016

Source: Nokia

# Radio Interface Enhancements

# for TA based multilateration

# (Update of RP-160034)

# introduction

New WI to enhance the positioning measurement for IoT devices is approved for Rel-14. One main method proposed for enhanced positioning accuracy in [1] uses timing advance based multilateration for estimating the positon of the device without any additional HW such as LMU.

As per initial evaluations indicated in [1], the energy consumption of this method is similar to transmission of 200 Byte MAR.

The traffic model for the position tracking is not known at this moment. Depending on the traffic model, the overall energy consumption at IoT device for position tracking varies. So one of the main objectives is to reduce the energy consumption of the procedure required for a single measurement as much as possible. This paper is update of R6-160034. The updates are highlighted in blue.

# RADIO INTERFACE procedures for TA based multilateration

The TA based multilateration procedure referred in [1] consists of below steps for MS in idle state.

- Page to MS and, after the page-response and the establishment of a TBF, the network triggers a neighbour cell measurement report from the MS.

- MS releases the TBF and measures the neighbour cells. On successful completion of the measurement, new UL TBF is established to report the neighbour cell measurements to base station.

- From the received measurement report, the BSS derives a candidate BTS list and provides it to SMLC. SMLC sends a cell list for TA measurement to the BSS.

- BSS sends cell reselection list to which MS needs to establish uplink TBF to send single block.

- MS synchronises and triggers random access followed by uplink TBF establishment for single block transmission to all the BTS in the cell reselection list given by base station.

Above steps require one TBF establishment for page-response and another one for sending measurement report, followed by one TBF establishment with each neighbour cell for transmission of uplink block to link the MS identity and random access.

These TBF establishments can be avoided if the radio-interface procedure uses alternative means for binding the access burst and the MS-identity. With this alternative, the energy consumption of the complete positioning procedure can be reduced significantly.

# ENERGY consumption analysis for TA based multlateration

As per [1] the energy consumption for various steps are indicated. Below table summarises the energy consumption from [1].

Table 1

| TA trilateration |  | Comment |
| --- | --- | --- |
| Message | Energy (in mJ) |  |
| Page (DL) | 0.11 |  |
| RACH (UL) | 2.34 | 1st TBF |
| FUA (DL) | 0.11 | 1st TBF |
| Page response (UL) | 9.37 | 1st TBF |
| PUAN (DL) | 0.23 | 1st TBF |
| Measurement Request (DL) | 0.23 | 1st TBF |
| Measure on 5 Neighbors+ reconfirm serving cell | 21.2 |  |
| RACH (UL) | 2.34 | 2nd TBF |
| Fixed UL assignment (DL) | 0.11 | 2nd TBF |
| Measurement Report (UL) | 9.37 | 2nd TBF |
| PUAN (DL) | 0.23 | 2nd TBF |
| Packet DL Assignment | 0.23 | 2nd TBF |
| Cell reselection list (DL) | 0.23 | 2nd TBF |
| PDAN (UL) | 9.37 | 2nd TBF |
| Sync to 3 cells | 7.42 |  |
| 3xRACH | 7.02 | 3rd-5th TBF |
| 3xFUA | 0.33 | 3rd-5th TBF |
| 3xControl Message UL (TLLI) | 28.1 | 3rd-5th TBF |
|  | 98 mJ | Total energy consumption |

The energy consumption for each of TBF establishments and data transfer from the above table is listed in the below table.

Table 2

| 1st TBF (For page response and Triggering measurement Report) | 12.28 mJ |
| --- | --- |
| 2nd TBF (For measurement report and Cell-Reselection-command) | 21.88 mJ |
| TBFs for TA Estimation | 3*11.82 mJ =35.45 mJ |

Out of total energy consumption of 98mJ for the complete procedure, 5 TBF establishments consume approximately 70mJ. If these TBF establishments or associated control channel signalling can be minimised along with use of associated control channels instead of data blocks, the energy consumptions related to these TBFs will reduce by significant amount.

# enhanced radio interface procedure for TA based Multilateration

All the steps of the reference procedures requires TBF establishment for single block transmission which is needed to bind the MS identity to the procedure steps. If short identifier can be used for binding the various steps of the procedure, overall energy consumption can be reduced by considerable amount.

Section 4.1 proposes modifications to the first step of procedure where paging followed by measurement is triggered towards MS. Section 4.2 and 4.2 proposes changes to TA estimation procedure using short identifier.

## Enhancements for triggering the positioning measurements for device in idle mode

As per the reference procedure, the positioning measurements towards idle mode MS starts with paging the MS. MS responds with page response over single block TBF, and neighbour cell measurement is triggered over this TBF.

This part of the procedure can be optimised as follows:

- New paging message for positioning is sent towards MS which also contains a new "position reference temporary identifier" (PRTI) which can be used by MS and the network to temporarily identify the MS in the multiple UL and DL steps of the positioning procedure.

- MS responds to the page by sending the PRTI in the (EC-)RACH channel. Base station acknowledges the (EC-)RACH and also triggers the neighbour cell measurement towards UE.

- With above changes, the first step of positioning measurement can be completed without TBF establishment.

The message sequence for TA estimation by neighbour cells using dedicated EC-PACCH/T is illustrated in the below figure 2.

Figure 1: Message sequence for triggering positioning measurements for device in idle mode.

If MS does not receive Acknowledgement to its page response, MS may retry the RACH attempt in the serving cell again as similar to other RACH mechanisms.

## Access burst transmission on a dedicated channel for TA estimation

The TA estimation procedure which requires MS to access and establish TBF in two or more cells can be optimised if BSS can reserve dedicated resources for access burst transmission.

The key aspects of TA estimation using access bursts on dedicated channels are given below.

When BSS triggers the TA estimation from cell reselection lists, BSS can reserve a dedicated EC-PACCH block within PDCH for transmission of the access burst in the neighbour cells instead of directing the MS to random access on a common channel.

The PACCH block for transmission of dedicated access burst in candidate cells needs to be scheduled in such a way that it allows MS to synchronise to the base station and send the access burst.

This mechanism reduces the MS transmission on common channel and also risk of collision for TA estimation.

With access burst transmission on a dedicated channel, inclusion of MS identity is not required to bind the access burst against the MS identity. However in this option network can provide short identifier which should be sent over the EC-PACCH block. Transmission of known data-bits on the EC-PACCH blocks will enable better channel estimation on the access burst.

Use of dedicated EC-PACCH approach for extended coverage will require 4 consecutive PDCHs. This will require 4 consecutive PDCH slots on the BCCH carrier itself for this approach. As BCCH carrier will have only limited time-slots for its operation, the dedicated EC-PACCH solution should fit within one time-slot. For this purpose, new EC-PACCH logical channel mapping with single timeslot is required.

New "EC-PACCH/T" channel is defined for this solution where the blind physical layer transmissions of higher coverage classes are mapped across TDMA frames with single time-slot itself.

Table 3

|  | Number of blind physical layer transmissions |
| --- | --- |
| CC1 | 1(Single burst within Radio block) |
| CC2 | 4 (One radio block) |
| CC3 | 16(4 radio blocks) |
| CC4 | 48 (12 radio blocks) |

When dedicated EC-PACCH/T is assigned for CC1 user, the EC-PACCH block and the burst number within the block is given to the MS for sending access burst on the dedicated channel.

Number of blind physical layer transmissions of EC-PACCH/T is similar to EC-RACH but on dedicated PDCH without collision.

The message sequence for TA estimation by neighbour cells using dedicated EC-PACCH/T is illustrated in the below figure 2.

Figure 2: Message sequence for TA estimation with access burst transmission on a dedicated channel.

When base station sends the cell reselection list via downlink (EC-)PDTCH, this block needs to acknowledged via PDAN message in uplink EC-PACCH before starting the cell reselection. This PDAN message transmission also can be replaced by single access burst transmission instead of 4 burst (EC-)PACCH transmission.

In case if the base station is not receiving the access burst on expected location on the dedicated channel, PDAN message including further location for access burst transmission can be sent.

With this modified procedure, the mobile station's Tx energy consumption for TA estimation is reduced to transmission of access bursts to the candidate cells. Considering the same energy as that of EC-RACH consumed for these access bursts, the Tx energy consumption for TA estimation can be calculated as

Tx energy consumption(TA Estimation-Dedicated-AB) = 3*2.34 =7.02 mJ.

## Connectionless Access burst transmission for TA Estimation

The reservation of EC-PACCH/T resources in neighbour cells may be difficult for neighbour cells belonging to a different BSC than the mobile station's serving cell. In this case, the access burst may be sent on the (EC-)CCCH/U instead of EC-PACCH/T.

Instead of MS establishing single block TBF with a neighbour cell for sending the MS-Identity to bind the TA estimated from random access (as proposed in [1]), the random access channel itself can carry a shorter identifier which can be mapped to the MS identifier at base station.

For this purpose, BSS assigns short identifier PRTI (Positioning-Reference-Temp-identifier) when it triggers the Random access transmission from MS according to cell reselection list. If the procedure is triggered towards idle mode MS the PRTI will be assigned as part of  paging procedure itself as described in section 4.1.

MS sends the PRTI as payload bits instead of regular 11 Bit RACH payload contents. One of existing bits can be used to indicate the payload type.

The BSS, on reception of the access burst, estimates the TA and binds the estimated TA values against the PRTI contained in the access burst.

The message sequence for TA estimation from selected cells using access burst transmission on EC-RACH is illustrated in the below figure 3.

Figure 3: Message sequence for TA estimation with connectionless access burst transmission.

In this option, MS only sends (EC-)RACH access bursts for TA estimation instead of TBF establishment followed by single block transmission. Contention resolution for the RACH access is also achieved using transmission of PRTI in (EC-)AGCH instead of 3 bit channel reference.

With this option, the energy consumption for TA estimation is estimated as below.

Energy-consumption (TA-Estimation-connectionless) = 3*(2.34+0.11) mJ =7.35 mJ

# further details of design aspects

Transmission of PRTI value of 10 bits over (EC-)RACH is possible with one spare-bit of RACH payload indicating that the message carries 10 bit PRTI value.

BSS can direct the MS to either use dedicated resources for the access bursts or connectionless access burst mechanism on the CCCH for TA estimation depending on the resource availability for dedicated access burst transmission.

As BSS also can provide PRTI value for access burst transmission on dedicated resources, in case if MS cannot transmit on the dedicated bursts (e.g. because the synchronization takes too long), it can switch to RACH transmission using the same access burst.

The PRTI value used in the approach need not be unique within BSS. It needs to be unique within the serving cell and preferably also within the involved neighbour cells where the TA Estimation procedure is ongoing. With this reuse of PRTI inside BSS, it is possible to address all parallel positioning transactions with 10 bit PRTI.

When the positioning procedure is triggered for MS in idle mode, the locally available PRTI value can be used if the first page is sent to specific cell-id or group. When further pages need to be sent across the complete BSS, normal paging procedure can be used. If BSS can reserve some PRTI for BSS wide paging, they can be used for these paging also to optimise the procedure.

This new procedure needs to be supported in all Rel-14 base stations. BSS can decide to use the enhanced approach only if all the candidate base stations for TA multilateration are capable of this procedure. Otherwise BSS should follow existing procedure.

## Proposal for RACH Payload bits for multiplexing positioning and channel access

As per Rel-13 specification TS 44.018 and TS 44.060, the EC Packet Channel Request message has one additional spare bit for future use. This can be used to differentiate RACH and EC-RACH access attempts for other purposes.

As per the latest findings indicated in [3], the code point used for PEO One Phase Access Request is still being used for short access in field deployments. So [3] and [4] propose to reuse the synchronization sequences TS3 and TS4 on RACH, which have been defined for an EC-GSM-IoT mobile for EC-RACH CC1 access on TN0 or TN1, and use the spare bit to differentiate the access for a mobile in PEO or in EC operation. This proposal has the drawback that the EC-RACH payload cannot be used for Rel-14 extensions, such as for positioning related access and hence will require to design further TSC(s) for such purposes.

The sourcing company proposes an alternative mechanism, where it is possible to allow for signalling a channel access request in PEO or EC operation and also for signalling a positioning related access in the same EC-RACH payload.

As per this proposal one code point from the EC-NumberOfBlocks information element in EC Packet Channel Request will be used to indicate that the access request is meant for PEO. Moreover, the spare bit can be used as an access discriminator to differentiate a channel access request for a PEO/EC-GSM-IOT mobile from a positioning related access attempt as described in this contribution. The modifications required for the message content are proposed below.

1) For a mobile station in EC operation performing channel access, the EC Packet Channel Request is structured as follows:

< EC Packet channel request message content > ::=

< EC-NumberOfBlocks : bit (3)  >

< EC Priority : bit (1) >

< RandomBits : bit (3) >

< Signal Strength : bit (3) >

< Access Discriminator: bit (1) >;

EC-NumberOfBlocks (3 bit field) This field indicates the type of payload a mobile station has to send as follows: bit 3 2 1 0 0 0 page response 0 0 1 RLC/MAC control message 0 1 0 PEO channel access 0 1 1 cell update or uplink data transfer requiring 1 MCS-1 coded RLC data block 1 0 0 uplink data transfer – 1 MCS-1 coded RLC data blocks required 1 0 1 uplink data transfer – 2 MCS-1 coded RLC data blocks required . . . 1 1 1 uplink data transfer – 5 or more MCS-1 coded RLC data blocks required

Access Discriminator (1 bit field) This field indicates whether the access is meant for positioning or channel request: 0 channel request 1 positioning access

All other information elements are kept identical. The access discriminator bit is set to 0 and the valid code points in the EC-NumberOfBlocks are all except ‘010’.

2) For a mobile station in PEO performing channel access sending the PEO One Phase Access Request on RACH using TS3 or TS4, the message content is as follows:

< EGPRS Packet channel request message content > ::=

< PEO One Phase Access Request :	010	< Access Cause : bit (2)  >

< PEO Priority : bit (1) >

< EGPRS capability : bit (1) >

< RandomBits : bit (3) > >

< Access Discriminator : bit (1) == L > > ;

The access discriminator bit is set to 0.

3) For a Rel-14 mobile station sending a Positioning Access attempt on RACH (TN0) as per this proposal the message content is defined as follows:

< Positioning Access Attempt message content > ::=

< MS Type : bit(1) >

< Positioning Reference : bit (9) >

< Access Discriminator : bit (1) =1 > ;

The MS Type field indicates whether the access attempt is from MS in EC operation or not. This information will be required by the base station to decide whether FUA or normal TBF needs to be established in case TBF establishment is required in response to the positioning access attempt. The coverage class information is not required on RACH since the MS in a higher coverage class will not access on RACH.

4) For a Rel-14 mobile station sending a Positioning Access attempt on EC-RACH (TN1) as per this proposal the message content is defined as follows:

< Positioning Access Attempt message content > ::=

< DLHigherCCInd : bit(1) >

< Positioning Reference : bit (9) >

< Access Discriminator : bit (1) =1 > ;

The DLHigherCCInd field indicates that the MS has a preference for selecting a higher coverage class for reception of the acknowledgement on EC-AGCH. The entire DL CC related information is not expected to be required since the base station uses also UL CC detection and related measurements for pathloss estimation for determination of the coverage class selection for EC-AGCH. Alternative solutions for conveying reduced DL CC information lesser than 2 bits can also be considered.

Proposal:  One code point of EC-NumberOfBlocks can be used to differentiate EC or PEO access request so that the spare bit can be used as access discriminator to differentiate further use of access burst from channel request.

# conclusion

Energy efficient radio interface procedure along with new logical channels is proposed for Rel-14 TA multilateration based positioning mechanism.

Comparison of energy consumption of reference procedure for TA multilateration specified in [1] and enhanced procedures are captured in [2].

Sourcing company proposes to consider this enhanced radio interface procedure positioning enhancement for the WI in Rel-14.

Sourcing company can provide further details on link level performance of new channels and CRs for the impacted specifications in this regard.

It is proposed to agree on the above made proposal in section 5.

# REFERENCEs

[1] RP-161034, "Positioning enhancements for GERAN – introducing Timing Advance trilateration", source Ericsson LM, 3GPP TSG RAN#72

[2] R6-160036, "Energy consumption analysis of radio interface procedures for positioning enhancements", source Nokia Networks, 3GPP TSG RAN6#1

[3] RP-160024, “CR 44.018-1024 Clarification of PEO codepoint usage, source Ericsson LM, 3GPP TSG RAN6#1

[4] RP-160025, “CR 44.060-1629 Clarification of PEO codepoint usage, source Ericsson Inc., 3GPP TSG RAN6#1