3GPP TSG RAN WG6 #3                                                                       R6-170119

# Athens, Greece, February 13-17, 2017	                Revision of R6-170069

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 45.003 | CR | 0145 | rev | 1 | Current version: | 14.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of new uplink coverage class CC5 for EC-PDTCH and EC-PACCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CIoT_EC_GSM_radio_enh-Core |  |  |  |  |  |  | Date: |  | 2017-02-07 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | New uplink coverage class CC5 for improved uplink MCL is introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Channel coding for EC-PACCH and EC-PDTCH MCS1’ for CC5 on uplink is defined. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New uplink coverage class CC5 will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 51b.4, 5.2b |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 43.064 CR 0111 TS 45.001 CR 0097 TS 45.002 CR 0206 TS 45.005 CR 0600 TS 45.008 CR 0649 TS 44.018 CR 1061 TS 44.060 CR 1639 TS 24.008 (draft CR) |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS 51.010 CR XXXX, TS 51.021 CR XXXX |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st Modification

## 5.1b	Extended Coverage Packet data traffic channel (EC-PDTCH)

### 5.1b.1	General

Twelve coding schemes are specified for the Extended Coverage Packet data traffic channels. The coding schemes MCS-1 to MCS-9 are identical to those defined for PDTCH in subclause 5.1. The coding schemes MCS-1/16, MCS-1/8 and MCS-1/4 are based on MCS-1 defined in subclause 5.1.

The following additions apply:

-	Blind physical layer transmissions are employed.

-	For downlink, multiple sequences of USF bits are delivered to the encoder. Each precoded USF sequence is mapped onto one instance of the repeated block.

The following restrictions apply:

-	Reduced Transmission Time Interval (RTTI) shall not be used on EC-PDTCH.

-	A PAN field shall not be included in the message delivered to the encoder.

-	For data coding, a reduced set of puncturing schemes is used on an EC-PDTCH, compared to a PDTCH. Only the puncturing schemes listed in table 0 shall be used.

Table 0: Allowed puncturing schemes on EC-PDTCH

| MCS | Allowed puncturing schemes in subclause 5.1 |
| --- | --- |
| MCS-1/16 MCS-1/8 MCS-1/4 MCS-1 MCS-2 | P1 |
| MCS-3 MCS-4 | P1 or P2 |
| Other MCSs | No restriction |

### 5.1b.2	Downlink MCS-1/M

#### 5.1b.2.1	Block constitution

The message delivered to the encoder has a fixed size of 206 information bits {a(0),a(1),...,a(205)}. In addition, M sequences of USF bits {u(m,0),u(m,1),u(m,2)}, m=0,…,M-1, are delivered to the encoder, where M=16, 8 and 4 for MCS-1/16, MCS-1/8 and MCS-1/4, respectively.

#### 5.1b.2.2	Encoding and blind physical layer transmission

For each of the M blind physical layer transmissions, the block is encoded as specified for MCS-1 in subclause 5.1.5.1. The message {d(0),d(1),...,d(208)} delivered to the encoder (see subclause 5.1.5.1.1) for blind physical layer transmission k is defined as

d(k)=u(m,k)		for 		k=0,1,2;

d(k)=a(k-3)		for 		k=3,…,208.

Denote the resulting bursts from encoding of blind physical layer transmissions m as R(m,B,j) = e(B,j) for m=0,…,M-1, B=0,…,3 and  j=0,…,115, where e(B,j) is defined in subclause 5.1.5.1.6.1.

#### 5.1b.2.3	Mapping onto PDCHs

Burst R(m,B,j), j=0,…,115, is mapped onto burst B’ of PDCH number pn, where

- for EC channels with 4 PDCHs assigned

B’ = B + 4(m div 4);

pn = (m mod 4).

- for EC channels with 2 PDCHs assigned

B’ = B + 4(m div 2);

pn = (m mod 2).

NOTE 1:	The burst number B’ denotes the relative transmission order of the bursts on a PDCH. The mapping to absolute TDMA frame number is specified in 3GPP TS 45.002 [8] and 3GPP TS 44.060 [6].

NOTE 2:	pn denotes the relative PDCH number within the set of PDCHs on which the EC-PDTCH is mapped, where PDCHs are numbered from lower to higher timeslot number. The absolute timeslot number depends on the assignment, see 3GPP TS 44.060 [6].

### 5.1b.3	Uplink MCS-1/M

#### 5.1b.3.1	Block constitution

The message delivered to the encoder has a fixed size of 209 information bits {a(0),a(1),...,a(208)}.

#### 5.1b.3.2	Encoding and blind physical layer transmission

For each of the M blind physical layer transmissions the block is encoded as specified for MCS-1 in subclause 5.1.5.2. The message {d(0),d(1),...,d(208)} delivered to the encoder (see subclause 5.1.5.1.1) for blind physical layer transmission m is defined as

d(k)=a(k)	for k=0,…,208.

Denote the resulting bursts from encoding of blind physical layer transmission m as R(m,B,j) = e(B,j)  for m=0,…,M-1, B=0,…,3 and  j=0,…,115, where e(B,j) is defined in subclause 5.1.5.1.6.1.

#### 5.1b.3.3	Mapping onto PDCHs

The mapping is done as specified for MCS-1/M DL in subclause 5.1b.2.3.

### 5.1b.4	Uplink MCS-1’/48

#### 5.1b.4.1	Block constitution

The message delivered to the encoder has a fixed size of 194 information bits {d(0),d(1),...,d(193)}. It is delivered on a burst mode.

##### 5.1b.4.1.1	Header coding

a)	Parity bits:

Eight header parity bits p(0),p(1),...,p(7) are defined in such a way that in GF(2) the binary polynomial:

d(0)D23 +...+ d(15)D8 + p(0)D7 +...+ p(7), when divided by:

D8 + D6 + D3 + 1, yields a remainder equal to:

D7 + D6 + D5 + D4 + D3 + D2 + D+1.

b)	Tail biting:

The six last header parity bits are added before information and parity bits, the result being a block of 30 bits {u”(-6),…,u”(0),u”(1),...,u”(23)} with six negative indexes:

u”(k-6)	= p(k+2)		for k = 0,1,...,5

u”(k)		= d(k)			for k = 0,1,...,15

u”(k)		= p(k-16)		for k = 16,17,...,23

c)	Convolutional encoder

This block of 30 bits {u”(-6),…,u”(0),u”(1),...,u”(23)} is encoded with the 1/3 rate convolutional mother code defined by the polynomials:

G4 = 1 + D2 + D3 + D5 + D6

G7 = 1 + D + D2 + D3 + D6

G5 = 1 + D + D4 + D6

This results in a block of 72 coded bits: {C(0),C(1),...,C(71)} defined by:

C(3k)     = u”(k) + u”(k-2) + u”(k-3) + u”(k-5) + u”(k-6)

C(3k+1) = u”(k) + u”(k-1) + u”(k-2) + u”(k-3) + u”(k-6)

C(3k+2) = u”(k) + u”(k-1) + u”(k-4) + u”(k-6)     		for k = 0,1,...,23

The code is punctured in such a way that the following coded bits:

C(2+12j), C(5+12j), C(8+12j), C(11+12j)  for j = 0,1,...,5} are not transmitted

The result is a block of 48 coded bits, {hc(0),hc(1),...,hc(47)}.

##### 5.1b.4.1.2	Data coding

a)	Parity bits:

Twelve data parity bits p(0),p(1),...,p(11) are defined in such a way that in GF(2) the binary polynomial:

d(16)D189 +...+ d(193)D12 + p(0)D11 +...+ p(11), when divided by:

D12 + D11 + D10 + D8 + D5 + D4 + 1, yields a remainder equal to:

D11 + D10 + D9 + D8 + D7 + D6 + D5 + D4 + D3 + D2 + D + 1.

b)	Tail bits:

Six tail bits equal to 0 are added to the information and parity bits, the result being a block of 196 bits 
{u(0),u(1),...,u(195)}:

u(k)	= d(k+16)		for k = 0,1,...,177

u(k)	= p(k-178)	for k = 178,179,...,189

u(k)	= 0				for k = 190,191,…,195 (tail bits)

c)	Convolutional encoder

This block of 196 bits {u(0),u(1),...,u(195)} is encoded with the 1/3 rate convolutional mother code defined by the polynomials:

G4 = 1 + D2 + D3 + D5 + D6

G7 = 1 + D + D2 + D3 + D6

G5 = 1 + D + D4 + D6

This results in a block of 588 coded bits: {C(0),C(1),...,C(587)} defined by:

C(3k)     = u(k) + u(k-2) + u(k-3) + u(k-5) + u(k-6)

C(3k+1) = u(k) + u(k-1) + u(k-2) + u(k-3) + u(k-6)

C(3k+2) = u(k) + u(k-1) + u(k-4) + u(k-6)     for k = 0,1,...,195; u(k) = 0 for k < 0

The code is punctured using always P1 as defined below.

| P1 | {C(2+21j), C(5+21j), C(8+21j), C(10+21j), C(11+21j), C(14+21j), C(17+21j), C(20+21j) for j = 0,1,...,27} are not transmitted except {C(k) for k = 73,136,199,262,325,388,451,514} which are transmitted |
| --- | --- |

The result is a block of 372 coded bits, {dc(0),dc(1),...,dc(371)}.

##### 5.1b.4.1.3	Interleaving

The header and data are put together as one entity as described by the following rule:

c(k)	= hc(k)		for k = 0,1,...,47

c(k)	= dc(k-48)	)		for k = 48,49,...,419

c’(n,k) = c(n,k)		for k = 0,1,...,24

c’(n,k) = c(n,k-1)		for k = 26,27,...,81

c’(n,k) = c(n,k-2)		for k = 83,84,...,138

c’(n,k) = c(n,k-3)		for k = 140,141,...,400

c’(n,k) = c(n,k-4)		for k = 402,403,...,423

c’(n,25) = q(10)		c’(n,82) = q(11)		c’(n,139) = q(12)		c’(n,401) = q(13)

c(n,k) are the coded bits and q(10),q(11),q(12),q(13) = 0,0,0,0 are four extra stealing flags

The resulting block is interleaved according to the following rule:

i(B,j) = c’(n,k)	for	k = 0,1,...,423

n = 0,1,...,N,N+1,...

B = B0 + 4n + (k mod 4)

j = 2((49k) mod 53) + ((k mod 8) div 4)

##### 5.1b.4.1.4	Mapping on a burst

The mapping is given by the rule:

e(B,j) = i(B,j)  and  e(B,63+j) = i(B,53+j)		for j = 0,1,...,52

and

e(B+m,53+i) = q(i) and  e(B+58+i) =q(i+5)   		for m = 0,1,2,3 and i = 0,1,2,...,4

where

q(0),q(1),..,q(9) = 0,0,0,0,0,0,0,0,0,0.

#### 5.1b.4.2	Blind physical layer transmission

The resulting bursts from encoding of blind physical layer transmission m are denoted as R(m,B,j) = e(B,j)  for m=0,…,47, B=0,…,3 and  j=0,…,115, where e(B,j) is defined in subclause 5.1b.4.1.4.

#### 5.1b.4.3	Mapping onto PDCHs

The mapping is done as specified for MCS-1/M DL in subclause 5.1b.2.3.

2nd Modification

## 5.2b	Extended coverage common control channel downlink (EC-CCCH/D/M) and Extended coverage packet associated control channel (EC-PACCH/M)

### 5.2b.1	Block constitution

The message delivered to the encoder has a fixed size of Kd information bits {d(0),d(1),...,d(Kd -1)}, where:

-	EC-CCCH/D: Kd=88

- 	EC-PACCH/D: Kd=80

-	EC-PACCH/U/M: Kd=64 for M=1,4,8,16

-    EC-PACCH/U/48 Kd=56 for M=48

In addition, for EC-PACCH/D, up to M sequences of USF bits {u(m,0),u(m,1),u(m,2)}, m=0,…,M-1, are delivered to the encoder, where M=16, 8, 4 or 1 depending on the number of blind physical layer transmissions.

NOTE:	USF bits need not be delivered for those blocks that are transmitted on timeslots no GPRS/EGPRS MS is monitoring for USF.

### 5.2b.2	Data coding

a)	Parity bits:

Eighteen data parity bits p(0),p(1),...,p(17) are defined in such a way that in GF(2) the binary polynomial:

d(0)D Kd +18-1 +...+ d(Kd-1)D18 + p(0)D17 +...+ p(17), when divided by:

D18 + D17 + D14 + D13 + D11 + D10 + D8 + D7 + D6 + D3 + D2 + 1, yields a remainder equal to:

D17 + D16 + D15 + D14 + D13 + D12 + D11 + D10 + D9 + D8 + D7 + D6 + D5 + D4 + D3 + D2 + D + 1.

The parity bits are added after the block of Kd bits, the result being a block of Kd+18 bits, {b(0),…,b(Kd+18-1)}, defined as:

b(k)	= d(k)		for k = 0,1,..., Kd-1

b(k)	= p(k-Kd)	for k = Kd,..., Kd+18-1

b) Tail-biting convolutional encoder

The six last bits are added before the block of Kd+18 bits, the result being a block of Kd+24 bits {c(-6),…,c(0),c(1),...,c(Kd+18-1)} with six negative indices:

c(k)		= b(Kd+18+k)		for k = -6,...,-1

c(k)		= b(k)				for k = 0,1,..., Kd+18-1

This block of Kd+24 bits is encoded with the 1/3 rate convolutional mother code defined by the polynomials:

G4 = 1 + D2 + D3 + D5 + D6

G7 = 1 + D + D2 + D3 + D6

G5 = 1 + D + D4 + D6

This results in a block of (Kd+18)*3 coded bits {C(0),...,C((Kd+18)*3-1)} defined by:

C(3k)   = c(k) + c(k-2) + c(k-3) + c(k-5) + c(k-6)

C(3k+1) = c(k) + c(k-1) + c(k-2) + c(k-3) + c(k-6)

C(3k+2) = c(k) + c(k-1) + c(k-4) + c(k-6)   for k = 0,1,..., Kd+18-1

The code is punctured in such a way that the following coded bits are not transmitted:

| EC-CCCH/D | C(floor(n*318/202)) for n=0,…,201 |
| --- | --- |
| EC-PACCH/U/(1,4,8,16) | C(floor(n*246/130)) for n=0,…,129 |
| EC-PACCH/D | C(floor(n*294/180)) for n=0,…,179 |
| EC-PACCH/U/48 | C(floor(n*222/116)) for n=0,…,115 |

The result is a block of Kpc coded bits {pc(0),pc(1),...,pc(Kpc-1)}, where Kpc=114 for EC-PACCH/D and Kpc=116 for EC-CCCH/D and EC-PACCH/U/1, EC-PACCH/U/4, EC-PACCH/U/8, EC-PACCH/U/16. Kpc=106 for EC-PACCH/U/48.

### 5.2b.3	USF coding (only for EC-PACCH/D)

For each of the up to M delivered USF sequences, the USF  bits are block coded as for Packet data block type 4 (CS-4) in subclause 5.1.4.2. The input d(0),d(1),d(2) in subclause 5.1.4.2 shall be {u(m,0),u(m,1),u(m,2)} for USF number m.

Denote the resulting twelve encoded bits of USF number m as {u’(m,0),u’(m,1),...,u’(m,11)}.

### 5.2b.4	Mapping on a Burst

#### 5.2b.4.1	EC-CCCH/D/M

The mapping is given by the rule:

e(B, j)		= pc(j)			for j = 0,1,...,115 and B=0,1

#### 5.2b.4.2	EC-PACCH/D/M

The mapping is given by the rule:

e(B,j) = pc(j)  and  e(B,59+j) = pc(57+j)		for j = 0,1,...,56 and B=0,1,2,3

and

e(B,57) = q(2B)  and  e(B,58) = q(2B+1)		for B = 0,1,2,3

where

q(0),q(1),...,q(7) = 0,0,0,1,0,1,1,0.

NOTE:	For a standard GPRS MS, bits q(0),...,q(7) indicates that the USF is coded as for CS-4.

#### 5.2b.4.3	EC-PACCH/U/M

The mapping is given by the following rule for M = 1,4,8 or 16:

e(B, j)		= pc(j)			for j = 0,1,...,115 and B=0,1,2,3

The mapping is given by the following rule for M =48

The mapping is given by the rule:

e(B,j) = pc(j)  and  e(B,63+j) = pc(57+j)		for j = 0,1,...,52 and B=0,1,2,3

and

e(B,53+i) = q(i)  for i = 0,1,2,3,4,5,6,7,8,9

where

q(0),q(1),...,q(9) = 0,0,0,0,0,0,0,0,0,0

### 5.2b.5	Blind physical layer transmission

Each burst is transmitted M times, where M is the number of blind physical layer transmissions. For EC-PACCH/D, M=1, 4, 8 or 16. For EC-PACCH/U, M=1, 4, 8, 16 or 48. For EC-CCCH/D, M=1, 8, 16 or 32.

Denote the resulting bursts of transmission m as R(m,B,j) = e(B,j)  for m=0,…,M-1, B=0,1 (EC-CCCH) or B=0,1,2,3 (EC-PACCH), and  j=0,…,115.

### 5.2b.6	Bit stealing for USF (only for EC-PACCH/D)

For those m for which a USF has been delivered to the encoder, replace bits in the repeated bursts with encoded USF bits as follows:

for m=0,…,M-1

R(m,0,0) = u’(m,0)

R(m,0,51) = u’(m,4)

R(m,0,102) = u’(m,8)

R(m,1,100) = u’(m,1)

R(m,1,35) = u’(m,5)

R(m,1,86) = u’(m,9)

R(m,2,84) = u’(m,2)

R(m,2,19) = u’(m,6)

R(m,2,70) = u’(m,10)

R(m,3,68) = u’(m,3)

R(m,3,3) = u’(m,7)

R(m,3,52) = u’(m,11)

### 5.2b.7	Mapping onto physical channels

#### 5.2b.7.1	EC-PACCH

The mapping onto PDCHs is done as specified for downlink MCS-1/M in subclause 5.1b.2.3 for M=1,4,8,16. The mapping onto PDCHs is done as specified for uplink MCS-1’/48 in subclause 5.1b.4.3.

#### 5.2b.7.2	EC-CCCH/D

Burst R(m,B,j), B=0,1, j=0…,115, is mapped onto burst B’ of the timeslot carrying the EC-CCCH/D, where

B’ = B + 2m

NOTE:	The burst number B’ denotes the relative transmission order of the bursts on the timeslot. The mapping to absolute TDMA frame number is specified in 3GPP TS 45.002 [8].

End of Modifications