3GPP TSG RAN WG6 #3 	R6-170136

# Athens, Greece, February 13-17, 2017	                  Revision of R6-170102

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 45.003 | CR | 0144 | rev | 2 | Current version: | 14.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Extended Access Burst for connectionless Multilateration Positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R6 |  |  |  |  |  |  |  |  |  |
| Work item code: | ePOS_GERAN-Core |  |  |  |  |  |  | Date: |  | 2017-02-03 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | The Extended Access Burst is used for Multilateration Timing Advance (MTA) for accessing neighbour cells that are not in extended coverage if indicated by the network in the Multilateration TA Request message and if the cell indicates support for MTA as depicted in R6-170047. The usage of Extended Access Burst within the Connectionless MTA procedure provides energy saving benefits compared to the Connection-oriented MTA procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In sub-clause 4.6 the new random access message format based on 30 information bits is added for the RACH channel. - A new title for sub-clause 4.6.1 for the existing RACH encoding for 8 info bits is added. - In sub-clause 4.6.2 the encoding for the new random access message format based on 30 information bits is depicted. - In sub-clause 5.3 the new message format for EC-RACH is added. - In sub-clause 5.3a.5 the conditions for the usage of the new message format are mentioned and a reference to the new encoding for RACH is made. - Changes in Revision 1 are marked in yellow. In particular the procedure is referred to as Multilateration Timing Advance using the Extended Access Burst method. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No encoding for Extended Access Burst is defined, hence missed energy saving for MS requested to perform MTA. Inconsistency to stage 2 content. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6, 4.6.2 (new), 5.3a, 5.3a.5 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 43.059 CR 0083 TS 45.001 CR 0096 TS 45.002 CR 0205 TS 45.005 CR 0598 TS 44.018 CR 1060 TS 44.031 CR 0229 TS 48.008 CR 0411 TS 49.031 CR 0068. |
| affected: |  | X |  | Test specifications |  |  | TS 51.010 CR xxxx TS 51.021 CR xxxx. |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st Modification

## 4.6	Random access channel (RACH)

Three coding schemes are specified for the burst carrying the random access uplink message:

- the access burst containing 8 information bits,

- the access burst containing 11 information bits and

- the access burst containing 30 information bits.

The encoding of the access burst containing 8 information bits is as defined in subclause 4.6.1.

The encoding of the access burst containing 11 information bits is as defined in sub-clause 5.3.2 for the packet random access channel (PRACH, CPRACH and MPRACH).

The encoding of the access burst containing 30 information bits is as defined in sub-clause 4.6.2. It is used for the Multilateration Timing Advance procedure when the Extended Access Burst method is used (see 3GPP TS 43.059).

### 4.6.1	RACH carrying 8 information bits

The encoding of the access burst containing 8 information bits is defined as follows. It contains 8 information bits 
d(0),d(1),...,d(7).

Six parity bits p(0),p(1),...,p(5) are defined in such a way that in GF(2) the binary polynomial:

d(0)D13 +...+ d(7)D6 + p(0)D5 +...+ p(5), when divided by D6 + D5 + D3 + D2 + D + 1 yields a remainder equal to D5 + D4 + D3 + D2 + D + 1.

The six bits of the BSIC, {B(0),B(1),...,B(5)}, of the BS to which the Random Access is intended, are added bitwise modulo 2 to the six parity bits, {p(0),p(1),...,p(5)}. This results in six colour bits, C(0) to C(5) defined as C(k) = b(k) + p(k) (k = 0 to 5) where:

b(0) = MSB of PLMN colour code

b(5) = LSB of BS colour code.

This defines {u(0),u(1),..., u(17)} by:

u(k)	= d(k)		for k = 0,1,...,7

u(k)	= C(k-8)	for k = 8,9,...,13

u(k)	= 0			for k = 14,15,16,17 (tail bits)

The bits {e(0),e(1),..., e(35)} are obtained by the same convolutional code of rate ½ as for TCH/FS, defined by the polynomials:

G0 = 1 + D3 + D4

G1 = 1 + D + D3 + D4

and with:

e(2k)		= u(k) + u(k-3) + u(k-4)

e(2k+1)	= u(k) + u(k-1) + u(k-3) + u(k-4)		for k = 0,1,...,17 ; u(k) = 0 for k < 0

### 4.6.2	(EC-)RACH carrying 30 information bits

In case of the Multilateration Timing Advance procedure when the Extended Access Burst method is used, indicated by the network, the mobile station first sends a random access message carrying 11 information bits, using the encoding for the access burst containing 11 bits, as specified in sub-clause 5.3.2, and after access grant by the network sends a subsequent random access message carrying 30 information bits as soon as possible after receiving the access grant while still using the principles for (EC-)RACH transmission opportunity selection described in 3GPP TS 44.018. The encoding of the 30 information bits is as given in the following sub-clauses.

#### 4.6.2.1	Block constitution

The burst carrying the extended random access uplink message for connectionless Multilateration Positioning contains 30 information bits d(0),d(1),...,d(29). The first 11 bits d(0),...,d(10) are encoded in the same way as specified for the access burst containing 11 information bits in sub-clause 5.3.2 resulting in a block of 36 coded bits {e(0), e(1),...,e(35)}. The encoding of the remaining 19 information bits d(11),...,d(29) is specified in the following sub-clauses.

#### 4.6.2.2	Parity bits

Six parity bits p(0),p(1),...,p(5) are defined in such a way that in GF(2) the binary polynomial:

d(11)D24 +...+ d(29)D6 + p(0)D5 +...+ p(5), when divided by D6 + D5 + D3 + D2 + D + 1 yields a remainder equal to D5 + D4 + D3 + D2 + D + 1.

#### 4.6.2.3	Tail bits

Four tail bits are added to information bits and parity bits defining the bit sequence{u(0),u(1),..., u(28)} as follows:

u(k-11) = d(k)		for k = 11,12,...,29

u(k+19) = p(k)		for k = 0,...,5

u(k+25) = 0			for k = 0,1,2,3 (tail bits)

#### 4.6.2.4	Convolutional code

The coded bits {c(0),c(1),..., c(57)} are obtained by the same convolutional code of rate ½ as for TCH/FS, defined by the polynomials:

G0 = 1 + D3 + D4

G1 = 1 + D + D3 + D4

and with:

c(2k) = u(k) + u(k-3) + u(k-4)

c(2k+1) = u(k) + u(k-1) + u(k-3) + u(k-4)		for k = 0,1,...,28 ; u(k) = 0 for k < 0

The code is punctured in such a way that the following coded bit:

c(57) is not transmitted.

This results in a block of 57 coded bits, {e(36),...,e(92)} with

e(k+36) = c(k)	for k = 0,1,...,56 ;

#### 4.6.2.5	Burst mapping

Both data blocks {e(0),...,e(35)} and {e(36),...,e(92)} are mapped onto the Extended Access burst as specified in 3GPP TS 45.002.

2nd Modification

## 5.3a	Extended coverage random access channel (EC-RACH/M)

Channel encoding for the random access message in EC operation containing 11 information bits is depicted in the following sub-clauses 5.3a.1 to 5.3a.4. Sub-clause 5.3a.5 deals with the channel encoding for the random access message in EC operation containing 30 information bits.

### 5.3a.1	Block constitution

The message delivered to the encoder has a fixed size of 11 information bits {d(0),d(1),...,d(10)}.

### 5.3a.2	Coding

The message is encoded as for the 11 information bit access defined in subclause 5.3.2, resulting in a block of 36 coded bits {e(0), e(1),...,e(35)}.

### 5.3a.3	Blind physical layer transmissions

The block is transmitted M times, where M=1, 4, 16 or 48 for 1 TS EC-RACH and M=4, 16 or 48 for 2 TS EC-RACH, resulting in the repeated bursts R(m,B,j)=e(j) for m=0,…,M-1, B=0 and j=0,…,35.

### 5.3a.4	Mapping onto physical channel(s)

#### 5.3a.4.1	1 TS EC-RACH

Burst R(m,B,j), B=0, j=0…,35, is mapped onto burst B’ of the timeslot carrying the EC-RACH, where

B’ = m

NOTE:	The burst number B’ denotes the relative transmission order of the bursts on the timeslot. The mapping to absolute TDMA frame number is specified in 3GPP TS 45.002 [8].

#### 5.3a.4.2	2 TS EC-RACH

Burst R(m,B,j), B=0, j=0…,35, is mapped onto burst B’ of timeslot number tn of the timeslots carrying the EC-RACH, where

B’ = m div 2

tn = m mod 2

NOTE 1:	The burst number B’ denotes the relative transmission order of the bursts on the timeslot. The mapping to absolute TDMA frame number is specified in 3GPP TS 45.002 [8].

NOTE 2:	tn denotes the relative timeslot number within the timeslot pair on which the EC-RACH is mapped. The absolute timeslot number depends on the configuration, see 3GPP TS 44.018 [4].

### 5.3a.5	EC-RACH carrying 30 information bits

In case of performing the Multilateration Timing Advance procedure using the Extended Access Burst method in        EC operation, i.e. the mobile station has selected Coverage Class 1 for the uplink and the downlink, the MS sends a random access message carrying 11 information bits using the encoding according to the above sub-clauses, and after access grant by the network sends a subsequent random access message carrying 30 information bits. The encoding of the 30 information bits follows the one for (EC-)RACH specified in subclause 4.6.2.

End of Modifications