3GPP TSG RAN WG6 #4 	R6-170210

# Hangzhou, P.R. China, May 15-19, 2017                                         Revision of R6-170119

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 45.003 | CR | 0145 | rev | 2 | Current version: | 14.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of uplink coverage class CC5 for UL MCL improvement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CIoT_EC_GSM_radio_enh-Core |  |  |  |  |  |  | Date: |  | 2017-05-05 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | New uplink coverage class CC5 for improved uplink MCL is introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In clause 2.1 the structure of the channel encoding for EC-RACH and EC-PDTCH for CC5 are added. In sub-clause 5.1b.1 a reference to the coding scheme for EC-PDTCH/MCS-1'/48 for CC5 in uplink is added. In sub-clause 5.1b.4 channel coding for EC-PDTCH/MCS-1’/48 for CC5 in uplink is defined. In sub-clause 5.2b channel coding for EC-PACCH for CC5 on uplink is defined. In sub-clause 5.3a.0 the reference to EC-RACH for CC5 is added. In sub-clause 5.3a.6 channel encoding and blind physical layer transmissions for EC-RACH for CC5 using the 2TS EC-RACH format are specified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New uplink coverage class CC5 will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2.1, 5.1b.1, 5.1b.4, 5.2b, 5.3a.0, 5.3a.6 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 43.064 CR 0111 TS 45.001 CR 0097 TS 45.002 CR 0206 TS 45.005 CR 0600 TS 45.008 CR 0649 TS 44.018 CR 1061 TS 44.060 CR 1639 TS 24.008 (draft CR) |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS 51.010 CR XXXX, TS 51.021 CR XXXX |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Changes to revision 1 are marked in yellow. |  |  |  |  |  |  |  |  |

1st Modification

## 2.1	General organization

Each channel has its own coding and interleaving scheme. However, the channel coding and interleaving is organized in such a way as to allow, as much as possible, a unified decoder structure.

Each channel uses the following sequence and order of operations:

-	the information bits are coded with a systematic block code, building words of information + parity bits;

-	these information + parity bits are encoded with a convolutional code or a turbo code, building the coded bits;

-	reordering and interleaving the coded bits, and adding a stealing flag, gives the interleaved bits.

All these operations are made block by block, the size of which depends on the channel. However, most of the channels use a block of either 456 coded bits or 1368 coded bits, corresponding to 456 coded symbols, which is interleaved and mapped onto bursts in a very similar way for all of them. This block of 456 coded symbols is the basic structure of the channel coding scheme. Figures 1a, 1b, 1c, 1d, 1ea, 1eb, 1f, 1g, 1h, 1i, 1j, 1k, 1l, 2 and 2aa give diagrams showing the general structure of the channel coding.

In the case of full rate speech TCH, a block of 456 coded bits carries the information of one speech frame. In case of control channels, it carries one message.

In the case of half rate speech TCH, the information of one speech frame is carried in a block of 228 coded bits.

In the case of the Enhanced full rate speech the information bits coming out of the source codec first go through a preliminary channel coding. Then the channel coding as described above takes place.

In the case of 8-PSK modulated speech TCH, the information of one speech frame is carried in a block of 1368 coded bits (456 coded symbols) for full rate channels or 684 coded bits (228 coded symbols) for half rate channels.

In the case of a packet switched channel the block of 456, 1096, 1384, 1848, 2200, 2312 or 2748 coded bits carries one RLC/MAC block.

In the case of E-TCH/F28.8 or E-TCH/F43.2, the block of 1368 coded bits (456 coded symbols) carries one radio interface data block. In the case of E-TCH/F32.0, the block of 1392 coded bits (464 coded symbols) carries one radio interface data block.

In the case of FACCH, a coded message block of 456 bits is divided into eight sub-blocks. The first four sub-blocks are sent by stealing the even numbered bits of four timeslots in consecutive frames used for the TCH. The other four sub-blocks are sent by stealing the odd numbered bits of the relevant timeslot in four consecutive used frames delayed 2 or 4 frames relative to the first frame. Along with each block of 456 coded bits there is, in addition, a stealing flag (8 bits), indicating whether the block belongs to the TCH or to the FACCH. In the case of SACCH, BCCH, CCCH or CTSCCH, this stealing flag is dummy. In the case of a packet switched channel, these bits are used to indicate the coding scheme used.

In the case of E-FACCH/F, a coded message block of 456 bits is divided into four sub-blocks. The four sub-blocks are sent by stealing all symbols of four timeslots in consecutive frames used for the E-TCH and using GMSK modulation. The indication of the E-FACCH/F is based on the identification of the modulation.  Along with each block of 456 coded bits there is, in addition, a stealing flag (8 bits), indicating whether the block belongs to the E-FACCH, FACCH or TCH.

Some cases do not fit in the general organization, and use short blocks of coded bits which are sent completely in one timeslot. They are the random access messages of:

-	the RACH;

-	or PRACH, CPRACH and MPRACH;

on uplink and the synchronization information broadcast on the SCH or CSCH on the downlink. In CTS, they are the access request message of the CTSARCH on uplink and the information broadcast on the CTSBCH-SB on downlink.

In the coding/multiplexing unit of FLO, error detection, forward error correction and rate matching is applied to each transport channel independently. However the transport channels share a common multiplexing, TFCI mapping, interleaving and burst mapping. All these operations are made every transmission time interval and the number of coded bits produced by the coding/multiplexing unit depends on the basic physical subchannel. In the case of full rate GMSK basic physical subchannel, blocks of 464 bits are produced. In the case of half rate GMSK basic physical subchannel, blocks of 232 bits are produced. In the case of full rate 8PSK basic physical subchannel, blocks of 1392 bits are produced. In the case of half rate 8PSK basic physical subchannel, blocks of 696 bits are produced.

For EC-GSM-IoT, the same nine modulation and coding schemes as for EGPRS are defined for the extended coverage packet data traffic channels (EC-PDTCH). In addition three modulation and coding schemes employing MCS-1 and blind physical layer transmissions are defined for EC-PDTCH. For extended coverage common control channels in downlink (EC-CCCH/D) and the extended coverage packet associated control channel (EC-PACCH/U and EC-PACCH/D), three coding schemes having the same structure, differing mainly in message size and puncturing, are defined. The encoded bits are mapped onto a single burst, which is transmitted using blind physical layer transmissions. The extended coverage broadcast control channel (EC-BCCH) uses the same coding scheme as the broadcast control channel (BCCH) with subsequent blind physical layer transmissions. The extended coverage synchronization channel (EC-SCH) uses the same coding scheme as the synchronization channel (SCH), with subsequent blind physical layer transmissions and cyclic bit shifting. The extended coverage random access channel (EC-RACH) uses the same coding scheme as the random access channel (RACH), with subsequent blind physical layer transmissions, with the exception of the EC-RACH used for coverage class CC5, that applies a modified coding scheme with subsequent blind physical layer transmissions. Figures 1m, 1n, 2ab, 2ac, 2ad, 2ae and 2af give diagrams showing the general structure of the channel coding for EC-channels.

[no further changes until Figure 1n]

Figure 1n: Channel Coding and Interleaving Organization for EC-GSM-IoT Uplink Packet Data Traffic Channels (MCS-1/M is used for CC1 to CC4, MCS-1'/48 for CC5.)

[no further changes until Figure 2af]

Figure 2af: Channel Coding and Interleaving Organization for EC-RACH (M=1, 4, 16, 48 is used for CC1, CC2, CC3, CC4 and M=132 for CC5).

In each box, the last line indicates the subclause defining the function. M denotes the number of blind physical layer transmissions.

2nd Modification

## 5.1b	Extended Coverage Packet data traffic channel (EC-PDTCH)

### 5.1b.1	General

Twelve coding schemes are specified for the Extended Coverage Packet data traffic channels. The coding schemes MCS-1 to MCS-9 are identical to those defined for PDTCH in subclause 5.1 and apply for downlink and uplink coverage class CC1. The coding schemes MCS-1/16, MCS-1/8 and MCS-1/4 for downlink and uplink coverage classes CC4, CC3 and CC2, respectively, are based on MCS-1 defined in subclause 5.1. The coding scheme MCS-1'/48 for uplink coverage class CC5 is specified in subclause 5.1b.4.

The following additions apply:

-	Blind physical layer transmissions are employed.

-	For downlink, multiple sequences of USF bits are delivered to the encoder. Each precoded USF sequence is mapped onto one instance of the repeated block.

The following restrictions apply:

-	Reduced Transmission Time Interval (RTTI) shall not be used on EC-PDTCH.

-	A PAN field shall not be included in the message delivered to the encoder.

-	For data coding, a reduced set of puncturing schemes is used on an EC-PDTCH, compared to a PDTCH. Only the puncturing schemes listed in table 0 shall be used.

Table 0: Allowed puncturing schemes on EC-PDTCH

| MCS | Allowed puncturing schemes in subclause 5.1 |
| --- | --- |
| MCS-1/16 MCS-1/8 MCS-1/4 MCS-1 MCS-2 | P1 |
| MCS-3 MCS-4 | P1 or P2 |
| Other MCSs | No restriction |

For information only

### 5.1b.2	Downlink MCS-1/M

#### 5.1b.2.1	Block constitution

The message delivered to the encoder has a fixed size of 206 information bits {a(0),a(1),...,a(205)}. In addition, M sequences of USF bits {u(m,0),u(m,1),u(m,2)}, m=0,…,M-1, are delivered to the encoder, where M=16, 8 and 4 for MCS-1/16, MCS-1/8 and MCS-1/4, respectively.

#### 5.1b.2.2	Encoding and blind physical layer transmission

For each of the M blind physical layer transmissions, the block is encoded as specified for MCS-1 in subclause 5.1.5.1. The message {d(0),d(1),...,d(208)} delivered to the encoder (see subclause 5.1.5.1.1) for blind physical layer transmission k is defined as

d(k)=u(m,k)		for 		k=0,1,2;

d(k)=a(k-3)		for 		k=3,…,208.

Denote the resulting bursts from encoding of blind physical layer transmissions m as R(m,B,j) = e(B,j) for m=0,…,M-1, B=0,…,3 and  j=0,…,115, where e(B,j) is defined in subclause 5.1.5.1.6.1.

#### 5.1b.2.3	Mapping onto PDCHs

Burst R(m,B,j), j=0,…,115, is mapped onto burst B’ of PDCH number pn, where

- for EC channels with 4 PDCHs assigned

B’ = B + 4(m div 4);

pn = (m mod 4).

- for EC channels with 2 PDCHs assigned

B’ = B + 4(m div 2);

pn = (m mod 2).

NOTE 1:	The burst number B’ denotes the relative transmission order of the bursts on a PDCH. The mapping to absolute TDMA frame number is specified in 3GPP TS 45.002 [8] and 3GPP TS 44.060 [6].

NOTE 2:	pn denotes the relative PDCH number within the set of PDCHs on which the EC-PDTCH is mapped, where PDCHs are numbered from lower to higher timeslot number. The absolute timeslot number depends on the assignment, see 3GPP TS 44.060 [6].

### 5.1b.3	Uplink MCS-1/M

#### 5.1b.3.1	Block constitution

The message delivered to the encoder has a fixed size of 209 information bits {a(0),a(1),...,a(208)}.

#### 5.1b.3.2	Encoding and blind physical layer transmission

For each of the M blind physical layer transmissions the block is encoded as specified for MCS-1 in subclause 5.1.5.2. The message {d(0),d(1),...,d(208)} delivered to the encoder (see subclause 5.1.5.1.1) for blind physical layer transmission m is defined as

d(k)=a(k)	for k=0,…,208.

Denote the resulting bursts from encoding of blind physical layer transmission m as R(m,B,j) = e(B,j)  for m=0,…,M-1, B=0,…,3 and  j=0,…,115, where e(B,j) is defined in subclause 5.1.5.1.6.1.

#### 5.1b.3.3	Mapping onto PDCHs

The mapping is done as specified for MCS-1/M DL in subclause 5.1b.2.3, except when 2 PDCHs are assigned. In this case, burst R(m,B,j), j=0,…,115, is mapped onto burst B’ of PDCH number pn, where:

B’ = B(M div 2) + (m div 2);

pn = (m mod 2).

3rd Modification

### 5.1b.4	Uplink MCS-1’/48

#### 5.1b.4.1	Block constitution

The message delivered to the encoder has a fixed size of 194 information bits {d(0),d(1),...,d(193)}. It is delivered on a burst mode.

#### 5.1b.4.2	Header coding

a)	Parity bits:

Eight header parity bits p(0),p(1),...,p(7) are defined in such a way that in GF(2) the binary polynomial:

d(0)D23 +...+ d(15)D8 + p(0)D7 +...+ p(7), when divided by:

D8 + D6 + D3 + 1, yields a remainder equal to:

D7 + D6 + D5 + D4 + D3 + D2 + D+1.

b)	Tail biting:

The six last header parity bits are added before information and parity bits, the result being a block of 30 bits {u”(-6),…,u”(0),u”(1),...,u”(23)} with six negative indexes:

u”(k-6)	= p(k+2)		for k = 0,1,...,5

u”(k)		= d(k)			for k = 0,1,...,15

u”(k)		= p(k-16)		for k = 16,17,...,23

c)	Convolutional encoder

This block of 30 bits {u”(-6),…,u”(0),u”(1),...,u”(23)} is encoded with the 1/3 rate convolutional mother code defined by the polynomials:

G4 = 1 + D2 + D3 + D5 + D6

G7 = 1 + D + D2 + D3 + D6

G5 = 1 + D + D4 + D6

This results in a block of 72 coded bits: {C(0),C(1),...,C(71)} defined by:

C(3k)     = u”(k) + u”(k-2) + u”(k-3) + u”(k-5) + u”(k-6)

C(3k+1) = u”(k) + u”(k-1) + u”(k-2) + u”(k-3) + u”(k-6)

C(3k+2) = u”(k) + u”(k-1) + u”(k-4) + u”(k-6)     		for k = 0,1,...,23

The code is punctured in such a way that the following coded bits:

C(2+12j), C(5+12j), C(8+12j), C(11+12j)  for j = 0,1,...,5} are not transmitted

The result is a block of 48 coded bits, {hc(0),hc(1),...,hc(47)}.

#### 5.1b.4.3	Data coding

a)	Parity bits:

Twelve data parity bits p(0),p(1),...,p(11) are defined in such a way that in GF(2) the binary polynomial:

d(16)D189 +...+ d(193)D12 + p(0)D11 +...+ p(11), when divided by:

D12 + D11 + D10 + D8 + D5 + D4 + 1, yields a remainder equal to:

D11 + D10 + D9 + D8 + D7 + D6 + D5 + D4 + D3 + D2 + D + 1.

b)	Tail bits:

Six tail bits equal to 0 are added to the information and parity bits, the result being a block of 196 bits 
{u(0),u(1),...,u(195)}:

u(k)	= d(k+16)		for k = 0,1,...,177

u(k)	= p(k-178)	for k = 178,179,...,189

u(k)	= 0				for k = 190,191,…,195 (tail bits)

c)	Convolutional encoder

This block of 196 bits {u(0),u(1),...,u(195)} is encoded with the 1/3 rate convolutional mother code defined by the polynomials:

G4 = 1 + D2 + D3 + D5 + D6

G7 = 1 + D + D2 + D3 + D6

G5 = 1 + D + D4 + D6

This results in a block of 588 coded bits: {C(0),C(1),...,C(587)} defined by:

C(3k)     = u(k) + u(k-2) + u(k-3) + u(k-5) + u(k-6)

C(3k+1) = u(k) + u(k-1) + u(k-2) + u(k-3) + u(k-6)

C(3k+2) = u(k) + u(k-1) + u(k-4) + u(k-6)     for k = 0,1,...,195; u(k) = 0 for k < 0

The code is punctured using always P1 as defined below.

| P1 | {C(2+21j), C(5+21j), C(8+21j), C(10+21j), C(11+21j), C(14+21j), C(17+21j), C(20+21j) for j = 0,1,...,27} are not transmitted except {C(k) for k = 73,136,199,262,325,388,451,514} which are transmitted |
| --- | --- |

The result is a block of 372 coded bits, {dc(0),dc(1),...,dc(371)}.

#### 5.1b.4.4	Interleaving

The header and data are put together as one entity as described by the following rule:

c(k)	= hc(k)		for k = 0,1,...,47

c(k)	= dc(k-48)	)		for k = 48,49,...,419

c’(n,k) = c(n,k)		for k = 0,1,...,24

c’(n,k) = c(n,k-1)		for k = 26,27,...,81

c’(n,k) = c(n,k-2)		for k = 83,84,...,138

c’(n,k) = c(n,k-3)		for k = 140,141,...,400

c’(n,k) = c(n,k-4)		for k = 402,403,...,423

c’(n,25) = q(10)		c’(n,82) = q(11)		c’(n,139) = q(12)		c’(n,401) = q(13)

c(n,k) are the coded bits and q(10),q(11),q(12),q(13) = 0,0,0,0 are four extra stealing flags

The resulting block of 424 bits is interleaved according to the following rule:

i(B,j) = c’(n,k)	for	k = 0,1,...,423

n = 0,1,...,N,N+1,...

B = B0 + 4n + (k mod 4)

j = 2((49k) mod 53) + ((k mod 8) div 4)

#### 5.1b.4.5	Mapping on a burst

The mapping is given by the rule:

e(B,j) = i(B,j)  and  e(B,63+j) = i(B,53+j)		for j = 0,1,...,52

and

e(B+m,53+i) = q(i) and  e(B+58+i) =q(i+5)   		for m = 0,1,2,3 and i = 0,1,2,...,4

where

q(0),q(1),..,q(9) = 0,0,0,0,0,0,0,0,0,0.

#### 5.1b.4.6	Blind physical layer transmission

The resulting bursts from encoding of blind physical layer transmission m are denoted as R(m,B,j) = e(B,j)  for m=0,…,47, B=0,…,3 and  j=0,…,115, where e(B,j) is defined in subclause 5.1b.4.5.

#### 5.1b.4.7	Mapping onto PDCHs

Burst R(m,B,j), j=0,…,115, is mapped onto burst B’ of PDCH number pn, where

- for EC channels with 4 PDCHs assigned

B’ = B + 4*(m div 4);

pn = (m mod 4).

- for EC channels with 2 PDCHs assigned

B’ = B(M div 2) + (m div 2);

pn = (m mod 2).

NOTE 1:	The burst number B’ denotes the relative transmission order of the bursts on a PDCH. The mapping to absolute TDMA frame number is specified in 3GPP TS 45.002 [8] and 3GPP TS 44.060 [6].

NOTE 2:	pn denotes the relative PDCH number within the set of PDCHs on which the EC-PDTCH is mapped, where PDCHs are numbered from lower to higher timeslot number. The absolute timeslot number depends on the assignment, see 3GPP TS 44.060 [6].

4th Modification

## 5.2b	Extended coverage common control channel downlink (EC-CCCH/D/M) and Extended coverage packet associated control channel (EC-PACCH/M)

### 5.2b.1	Block constitution

The message delivered to the encoder has a fixed size of Kd information bits {d(0),d(1),...,d(Kd -1)}, where:

-	EC-CCCH/D: Kd=88

- 	EC-PACCH/D: Kd=80

-	EC-PACCH/U/M: Kd=64 for M=1,4,8,16 and Kd=56 for M=48

In addition, for EC-PACCH/D, up to M sequences of USF bits {u(m,0),u(m,1),u(m,2)}, m=0,…,M-1, are delivered to the encoder, where M=16, 8, 4 or 1 depending on the number of blind physical layer transmissions.

NOTE:	USF bits need not be delivered for those blocks that are transmitted on timeslots no GPRS/EGPRS MS is monitoring for USF.

### 5.2b.2	Data coding

a)	Parity bits:

Eighteen data parity bits p(0),p(1),...,p(17) are defined in such a way that in GF(2) the binary polynomial:

d(0)D Kd +18-1 +...+ d(Kd-1)D18 + p(0)D17 +...+ p(17), when divided by:

D18 + D17 + D14 + D13 + D11 + D10 + D8 + D7 + D6 + D3 + D2 + 1, yields a remainder equal to:

D17 + D16 + D15 + D14 + D13 + D12 + D11 + D10 + D9 + D8 + D7 + D6 + D5 + D4 + D3 + D2 + D + 1.

The parity bits are added after the block of Kd bits, the result being a block of Kd+18 bits, {b(0),…,b(Kd+18-1)}, defined as:

b(k)	= d(k)		for k = 0,1,..., Kd-1

b(k)	= p(k-Kd)	for k = Kd,..., Kd+18-1

b) Tail-biting convolutional encoder

The six last bits are added before the block of Kd+18 bits, the result being a block of Kd+24 bits {c(-6),…,c(0),c(1),...,c(Kd+18-1)} with six negative indices:

c(k)		= b(Kd+18+k)		for k = -6,...,-1

c(k)		= b(k)				for k = 0,1,..., Kd+18-1

This block of Kd+24 bits is encoded with the 1/3 rate convolutional mother code defined by the polynomials:

G4 = 1 + D2 + D3 + D5 + D6

G7 = 1 + D + D2 + D3 + D6

G5 = 1 + D + D4 + D6

This results in a block of (Kd+18)*3 coded bits {C(0),...,C((Kd+18)*3-1)} defined by:

C(3k)   = c(k) + c(k-2) + c(k-3) + c(k-5) + c(k-6)

C(3k+1) = c(k) + c(k-1) + c(k-2) + c(k-3) + c(k-6)

C(3k+2) = c(k) + c(k-1) + c(k-4) + c(k-6)   for k = 0,1,..., Kd+18-1

The code is punctured in such a way that the following coded bits are not transmitted:

| EC-CCCH/D | C(floor(n*318/202)) for n=0,…,201 |
| --- | --- |
| EC-PACCH/U (M=1,4,8,16) | C(floor(n*246/130)) for n=0,…,129 |
| EC-PACCH/D | C(floor(n*294/180)) for n=0,…,179 |
| EC-PACCH/U (M=48) | C(floor(n*222/116)) for n=0,…,115 |

The result is a block of Kpc coded bits {pc(0),pc(1),...,pc(Kpc-1)}, where Kpc=114 for EC-PACCH/D and Kpc=116 for EC-CCCH/D and EC-PACCH/U/1, EC-PACCH/U/4, EC-PACCH/U/8, EC-PACCH/U/16 and Kpc=106 for EC-PACCH/U/48.

### 5.2b.3	USF coding (only for EC-PACCH/D)

For each of the up to M delivered USF sequences, the USF  bits are block coded as for Packet data block type 4 (CS-4) in subclause 5.1.4.2. The input d(0),d(1),d(2) in subclause 5.1.4.2 shall be {u(m,0),u(m,1),u(m,2)} for USF number m.

Denote the resulting twelve encoded bits of USF number m as {u’(m,0),u’(m,1),...,u’(m,11)}.

### 5.2b.4	Mapping on a Burst

#### 5.2b.4.1	EC-CCCH/D/M

The mapping is given by the rule:

e(B, j)		= pc(j)			for j = 0,1,...,115 and B=0,1

#### 5.2b.4.2	EC-PACCH/D/M

The mapping is given by the rule:

e(B,j) = pc(j)  and  e(B,59+j) = pc(57+j)		for j = 0,1,...,56 and B=0,1,2,3

and

e(B,57) = q(2B)  and  e(B,58) = q(2B+1)		for B = 0,1,2,3

where

q(0),q(1),...,q(7) = 0,0,0,1,0,1,1,0.

NOTE:	For a standard GPRS MS, bits q(0),...,q(7) indicates that the USF is coded as for CS-4.

#### 5.2b.4.3	EC-PACCH/U/M

The mapping is given by the following rule for M = 1,4,8 or 16:

e(B, j)		= pc(j)			for j = 0,1,...,115 and B=0,1,2,3

The mapping is given by the following rule for M = 48:

e(B,j) = pc(j)  and  e(B,63+j) = pc(53+j)		for j = 0,1,...,52 and B=0,1,2,3

and

e(B,53+i) = q(i)  for i = 0,1,…,9

where

q(0),q(1),...,q(9) = 0,0,0,0,0,0,0,0,0,0

### 5.2b.5	Blind physical layer transmission

Each burst is transmitted M times, where M is the number of blind physical layer transmissions. For EC-PACCH/D, M=1, 4, 8 or 16. For EC-PACCH/U, M=1, 4, 8, 16 or 48. For EC-CCCH/D, M=1, 8, 16 or 32.

Denote the resulting bursts of transmission m as R(m,B,j) = e(B,j)  for m=0,…,M-1, B=0,1 (EC-CCCH) or B=0,1,2,3 (EC-PACCH), and  j=0,…,115.

### 5.2b.6	Bit stealing for USF (only for EC-PACCH/D)

For those m for which a USF has been delivered to the encoder, replace bits in the repeated bursts with encoded USF bits as follows:

for m=0,…,M-1

R(m,0,0) = u’(m,0)

R(m,0,51) = u’(m,4)

R(m,0,102) = u’(m,8)

R(m,1,100) = u’(m,1)

R(m,1,35) = u’(m,5)

R(m,1,86) = u’(m,9)

R(m,2,84) = u’(m,2)

R(m,2,19) = u’(m,6)

R(m,2,70) = u’(m,10)

R(m,3,68) = u’(m,3)

R(m,3,3) = u’(m,7)

R(m,3,52) = u’(m,11)

### 5.2b.7	Mapping onto physical channels

#### 5.2b.7.1	EC-PACCH

The mapping onto PDCHs is done as specified for downlink MCS-1/M in subclause 5.1b.2.3 for M=1,4,8,16. The mapping onto PDCHs is done as specified for uplink MCS-1’/48 in subclause 5.1b.4.3.

#### 5.2b.7.2	EC-CCCH/D

Burst R(m,B,j), B=0,1, j=0…,115, is mapped onto burst B’ of the timeslot carrying the EC-CCCH/D, where

B’ = B + 2m

NOTE:	The burst number B’ denotes the relative transmission order of the bursts on the timeslot. The mapping to absolute TDMA frame number is specified in 3GPP TS 45.002 [8].

4th Modification

## 5.3a	Extended coverage random access channel (EC-RACH/M)

### 5.3a.0	General

Channel encoding for the random access message in EC operation containing 11 information bits is depicted in the following sub-clauses 5.3a.1 to 5.3a.4, except for EC-RACH for CC5 specified in clause 5.3a.6. Sub-clause 5.3a.5 deals with the channel encoding for the random access message in EC operation containing 30 information bits.

### 5.3a.1	Block constitution

The message delivered to the encoder has a fixed size of 11 information bits {d(0),d(1),...,d(10)}.

### 5.3a.2	Coding

The message is encoded as for the 11 information bit access defined in subclause 5.3.2, resulting in a block of 36 coded bits {e(0), e(1),...,e(35)}.

### 5.3a.3	Blind physical layer transmissions

The block is transmitted M times, where M=1, 4, 16 or 48 for 1 TS EC-RACH and M=4, 16 or 48 for 2 TS EC-RACH, resulting in the repeated bursts R(m,B,j)=e(j) for m=0,…,M-1, B=0 and j=0,…,35.

### 5.3a.4	Mapping onto physical channel(s)

#### 5.3a.4.1	1 TS EC-RACH

Burst R(m,B,j), B=0, j=0…,35, is mapped onto burst B’ of the timeslot carrying the EC-RACH, where

B’ = m

NOTE:	The burst number B’ denotes the relative transmission order of the bursts on the timeslot. The mapping to absolute TDMA frame number is specified in 3GPP TS 45.002 [8].

#### 5.3a.4.2	2 TS EC-RACH

Burst R(m,B,j), B=0, j=0…,35, is mapped onto burst B’ of timeslot number tn of the timeslots carrying the EC-RACH, where

B’ = m div 2

tn = m mod 2

NOTE 1:	The burst number B’ denotes the relative transmission order of the bursts on the timeslot. The mapping to absolute TDMA frame number is specified in 3GPP TS 45.002 [8].

NOTE 2:	tn denotes the relative timeslot number within the timeslot pair on which the EC-RACH is mapped. The absolute timeslot number depends on the configuration, see 3GPP TS 44.018 [4].

### 5.3a.5	EC-RACH carrying 30 information bits

In case of performing the Multilateration Timing Advance procedure using the Extended Access Burst method in        EC operation, i.e. the mobile station has selected Coverage Class 1 for the uplink and the downlink, the MS sends a random access message carrying 11 information bits using the encoding according to the above sub-clauses, and after access grant by the network sends a subsequent random access message carrying 30 information bits. The encoding of the 30 information bits follows the one for (EC-)RACH specified in subclause 4.6.2.

### 5.3a.6	Extended coverage random access channel (EC-RACH/132)

#### 5.3a.6.0	General

Channel encoding, blind physical layer transmissions and mapping onto physical channels for the random access channel EC-RACH/132 used for CC5 in EC operation is depicted in the following sub-clauses 5.3a.6.1 to 5.3a.6.6.

#### 5.3a.6.1	Block constitution

The message delivered to the encoder has a fixed size of 11 information bits {d(0),d(1),...,d(10)}.

#### 5.3a.6.2	Parity bits

Six parity bits p(0),p(1),...,p(5) are defined in such a way that in GF(2) the binary polynomial:

d(0)D16 +...+ d(10)D6 + p(0)D5 +...+ p(5), when divided by D6 + D5 + D3 + D2 + D + 1 yields a remainder equal to D5 + D4 + D3 + D2 + D + 1.

#### 5.3a.6.3	Addition of BSIC

The MS shall use the nine bit BSIC of the BTS to which the access burst is intended. The first six bits of the BSIC, {b(0),b(1),...,b(5)}, are added bitwise modulo 2 to the six parity bits while the last three bits of the BSIC {b(6),b(7),b(8)} are added bitwise modulo 2 to the three last information bits d(8), d(9) and d(10). The bitwise modulo 2 operation results in nine colour bits, C(0) to C(8), defined as C(k) = b(k) + p(k) (for k = 0 to 5) and C(k) = b(k) + d(k+2) (for k = 6 to 8) where:

b(0) = MSB of PLMN colour code

b(8) = LSB of Radio frequency colour code.

This defines {u(0),u(1),..., u(20)} by:

u(k) = d(k)		for k = 0,1,...,7

u(k) = C(k-2)		for k = 8,9,10

u(k) = C(k-11)	for k = 11,12,...,16

u(k) = 0			for k = 17,18,19,20 (tail bits)

#### 5.3a.6.4	Convolutional code

The coded bits {c(0),c(1),..., c(41)} are obtained by the same convolutional code of rate ½ as for TCH/FS, defined by the polynomials:

G0 = 1 + D3 + D4

G1 = 1 + D + D3 + D4

and with:

c(2k) = u(k) + u(k-3) + u(k-4)

c(2k+1) = u(k) + u(k-1) + u(k-3) + u(k-4)		for k = 0,1,...,20 ; u(k) = 0 for k < 0

The code is punctured in such a way that the following coded bits:

[c(0), c(3), c(7), c(12), c(14), c(17), c(24), c(27), c(31), c(34), c(39), c(41)] are not transmitted.

This results in a block of 30 coded bits, {e(0), e(1),...,e(29)}.

#### 5.3a.6.5	Blind physical layer transmissions

The block of encoded data is transmitted 264 times using the Segmented AB for the 2 TS EC-RACH format (see 3GPP TS 45.002[8]). The first burst with normal burst size carries 3 instances of the encoded data block and the second burst with access burst size carries one instance of the encoded data block. The bursts are transmitted in a timeslot pair which is repeated over 66 TDMA frames forming 132 bursts in total.

#### 5.3a.6.6	Mapping onto physical channel(s)

The data block {e(0),...,e(29)} is mapped onto the Segmented AB, repeated over 66 TDMA frames in total, as specified in 3GPP TS 45.002[8].

For tn = TN mod 2 = 0, the data block {e(0), e(1),...,e(29)} is mapped 3 times on the burst with normal burst size.

For tn = TN mod 2 = 1, the data block {e(0), e(1),...,e(29)} is mapped once on the burst with access burst size.

NOTE :	tn denotes the relative timeslot number within the timeslot pair on which the EC-RACH is mapped. The absolute timeslot number depends on the configuration, see 3GPP TS 44.018 [4].

End of Modifications