Abstract of the contribution: This WI summary contains the WI summary for Radio Interface Enhancements for EC-GSM-IoT for agreement at this meeting and for submission for information to RAN#76.

_________________________________________________________

3GPP TSG RAN Meeting #76					RP-16XXXX

West Palm Beach, Florida, USA, 5-8 June

Agenda Item:		<XX>

Source:		Nokia

Title:				Summary for WI Radio Interface Enhancements for EC-GSM-IoT

WI code(s):

leading WG:		RAN6

Release:		Rel-14

# introduction

EC-GSM-IoT is an evolution of EGPRS providing a streamlined protocol implementation, reducing MS complexity while supporting energy efficient operation with extended coverage compared to GPRS/EGPRS.

EC-GSM-IoT improves the coverage performance of CIoT devices by 20 dB compared to EGPRS devices along with long battery life time achieved by energy efficient methods over radio interface. The extended coverage is achieved by large number of blind physical layer transmissions along with modified channel coding schemes.

Base station supporting EC-GSM-IoT requires minimum 4 timeslot resources reserved for packet data operation to allow extended coverage operation in Rel-13. The coverage 3improvement for low power EC-GSM-IoT devices is limited to 10 dB in Rel-13.

As part of Rel-14 additional enhancements are introduced to allow EC Operations with reduced number of timeslot resources. New uplink coverage class CC5 is introduced to improve the MCL performance in uplink by 4 dB compared to Rel-13 systems.

# description

## EC operation with reduced number of PDCH resources

Additional coverage class mapping of blind physical layer transmissions introduced for higher coverage classes where only 2 Timeslot resources per TDMA frame are available for extended coverage operations. New coverage class mapping will have increased BTTI compared to existing coverage classes as blind physical layer transmissions needs to be extended over more TDMA frames.

The reference sensitivity performance of new coverage class mapping is comparable to Rel-13 systems. This feature allows network to deploy EC-GSM-IoT by allocating minimum number of time-slots for EC operation.

Figure 1 : EC Operation with 4 timeslots (Rel13)

Figure 2: EC Operation with 4 timeslots (Rel14)

## New uplink coverage class for improved uplink MCL Performance

With introduction of  new uplink coverage class CC5 ,the uplink MCL performance of EC-GSM-IoT MS will improve by additional 4 dB. This feature allows the low power EC-GSM-Devices (23 dBm) to operate in further extended coverage condition upto 4 dB c ompared to Rel-13 EC-GSM. The uplink MCL Performance of low power device will increase to 14 dB from 10 dB with this improvement.

On CC5 uplink traffic channels the coverage improvement is achieved by increasing the number of bits for channel estimation for traffic channels along with increased number of blind physical layer transmissions. For random access channel for CC5 operation class can use 2 extended access burst format spanning across 2 TS duration. One format uses extended synchronisation sequence whereas other format uses more number of blind physical layer transmissions within TDMA frame.

# references

[1] WID

[2] 3GPP TS 43.064

[3] 3GPP TS 45.001

[5] 3GPP TS 45.002

[6] 3GPP TS 45.003

[7] 3GPP TS 45.004

[8] 3GPP TS 45.005

[9] 3GPP TS 45.