3GPP TSG RAN WG6 #5 	R6-170382

# Berlin, Germany, August 21-25, 2017	                  Revision of R6-170xxx

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 43.064 | CR | 0115 | rev | - | Current version: | 14.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Restricted Use of Enhanced Coverage |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14 |  |  |  |  |  |  | Date: |  | 2017-08-11 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Restricted use of enhanced coverage was recently introduced in SA2 specification TS 23.060 by S2-166937 and in CT1 specification TS 24.008 by C1-170529 and C1-172085 as part of the CIoT_Ext work item in SA. However, for EC-GSM-IoT the impacts to existing radio link control procedures such as cell reselection, DL and UL CC selection, the enabling of paging, DL coverage class update and routing area update from introduction of this feature have not been considered so far. These aspects require changes to GERAN specifications as outlined in R6-170381, which also relate to the present specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In sub-clause 3.3.9.1 it is stated that the feature restricted use of enhanced coverage applies optionally for the network and a MS supporting EC-GSM-IoT. - In sub-clause 3.3.9.5 the impacts from restricted use of enhanced coverage on radio link control procedures such as cell reselection, DL and UL coverage class selection, and DL CC update as well as on RAU and paging procedures to inform the MS about the removal of the enhanced coverage restriction are described. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The restricted use of enhanced coverage is not correctly implemented on radio layer, hence it cannot be used in a Rel-14 network supporting EC-GSM-IoT. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3.9.1, 3.3.9.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 44.018 CR 1072 TS 45.008 CR 0654 TS 48.018 CR 0442 |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 51.010 CR XXXX |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | None. |  |  |  |  |  |  |  |  |

1st Modification

### 3.3.9	Extended Coverage GSM for Internet of Things (EC-GSM-IoT)

#### 3.3.9.1	General

EC-GSM-IoT is an evolution of EGPRS providing a streamlined protocol implementation, reducing MS complexity while supporting energy efficient operation with extended coverage compared to GPRS/EGPRS. EC-GSM-IoT also mandates the use of an improved security framework by both the network and the mobile station.

EC-GSM-IoT makes use of a 9 bit BSIC wherein the 6 bit BSIC is supplemented with a 3 bit Radio frequency Colour Code with the purpose to help distinguish between cells in tight frequency reuse networks.

EC-GSM-IoT makes use of Fixed Uplink Allocation for allocating uplink resources for EC-PDTCHs and hence does not support USF based uplink allocation.

No simultaneous uplink and downlink packet transfer is supported.

A MS that has enabled EC operation makes use of EC-channels (e.g. EC-PDTCH), except for the FCCH. An EC-GSM-IoT MS shall also support RACH and AGCH, in addition to EC-RACH and EC-AGCH. RACH and AGCH can be used by the MS when in GPRS/EGPRS coverage range (CC1, see subclause 3.3.9.2) if indicated by the network. EC-channels are used in idle mode as well as in packet transfer mode. The MS shall disable EC operation in case it enters a cell that does not support EC-GSM-IoT. The MS may also disable EC operation (see 3GPP TS 45.008) at any time, in which case it operates as if it was in a cell that does not support EC-GSM-IoT. The MS shall inform the network that EC operation has been disabled by e.g. a cell update. A MS that has disabled EC operation is no longer subject to the relaxed mobility related requirements (see sub-clause 3.3.9.3).

An EC-GSM-IoT mobile station need not comply with GPRS requirements, but shall comply with EGPRS requirements unless otherwise stated, see 3GPP TS 45.005 [14].

An EC-GSM-IoT mobile station need only support EC operation. The mobile station can optionally also support other PS services, such as GPRS, EGPRS and/or EGPRS2, or CS related services.

EC-GSM-IoT supports overload control by PLMN specific barring in the EC SI (see 3GPP TS 44.018 [6]) and optionally by using the Implicit Reject Status field within the EC-SCH (see 3GPP TS 44.018 [6]).

EC-GSM-IoT shall not be operated in reduced latency TBF mode, see subclause 3.3.5, nor shall it support the Fast Ack/Nack reporting procedure.

DTM is not supported in EC operation.

The EGPRS modulation and coding schemes for PDTCH are reused for EC-PDTCH. The EC-BCCH and EC-RACH CC1 to CC4 re-uses the same coding scheme as the BCCH and RACH respectively. Modified coding schemes, or specific coding schemes, are defined for EC-SCH, EC-PCH, EC-AGCH, EC-RACH CC5 and EC-PACCH, see 3GPP TS 45.003 [12].

The support of EC-GSM-IoT is optional for the mobile station and the network.

A MS that supports EC-GSM-IoT shall support Overlaid CDMA (see subclause 3.3.10).

The presence of EC-SCH indicates that a cell supports EC-CCCH and therefore allows an EC-GSM-IoT mobile station to camp on it.

A MS supporting EC-GSM-IoT as well as the network may support the restricted use of enhanced coverage (see subclause 3.3.9.5).

For Information Only

#### 3.3.9.2	Extended coverage

##### 3.3.9.2.1	General

EC-GSM-IoT MS is able to operate in extended coverage in both uplink and downlink, which is defined as improved MS and BTS sensitivity and interference performance. The feature has been designed to improve coverage by 20 dB and also the interference level by 20 dB compared to GPRS/EGPRS.

A predefined number of logical channel specific blind physical layer transmissions is used to support a certain level of extended coverage. For some logical channels, the number of blind physical layer transmissions can vary depending on the coverage extension required. Four different Coverage Classes are defined, each one approximated with a level of extended coverage compared to GPRS/EGPRS (see table 3.3.9.2-1), denoted as CC1, CC2, CC3 and CC4 respectively. In case of significant coverage extension, a fixed predefined number of blind physical layer transmissions is applied per logical channel. This number of blind physical layer transmissions, used by CC2, CC3 and CC4, may differ between logical channels for the same Coverage Class. Different Coverage Classes can be used on uplink and downlink. Logical channels supporting operation in extended coverage are referred to as EC-channels. Also the FCCH channel is considered to be operable in the extended coverage range aimed for by EC-GSM-IoT, and is hence used for synchronization purposes.

Blind physical layer transmissions should on the EC-PDTCH be used together with type II hybrid ARQ to achieve the approximate coverage level shown in Table 3.3.9.2-1.

Blind physical layer transmissions on the EC-PDTCH and EC-PACCH channels are mapped on to either 2 consecutive PDCH resources or 4 consecutive PDCH resources according to broadcast information in EC SI. The use of 2 PDCH resources is foreseen for resource constrained situations, e.g. the base station operates with limited resources due to a low number of TRX or with a limited set of PDCH resources in the PS domain.

Table 3.3.9.2-1. Approximate coverage level per Coverage Class compared to GPRS/EGPRS

| Coverage class | Approximate extended coverage level compared to GPRS/EGPRS [dB]1 |
| --- | --- |
| CC1 | 0-6 |
| CC2 | 6-12 |
| CC3 | 12-15 |
| CC4 | 15-20 |
| NOTE: The values apply for the same maximum output power (33dBm ) of an EC-GSM-IoT MS and a GPRS/EGPRS MS. |  |

The procedures for selection and communication of Coverage Class are described in 3GPP TS 45.008 [15] and 3GPP TS 44.018 [6]. In idle mode, the MS performs Coverage Class selection and communicates the Coverage Class to the network, while in Packet Transfer Mode, the network performs Coverage Class selection and communicates the Coverage Class to the MS. For the purpose of paging, the MS shall communicate changes in the Coverage Class under certain circumstances, see 45.008 [15].

It is mandatory for the EC-GSM-IoT capable MS to support all defined Coverage Classes for all EC-channels. The network shall support at least CC1 and CC4. Support for blind physical layer transmissions over 2 consecutive PDCH resources is mandatory for the EC-GSM-IoT capable MS and optional for a network that supports EC-GSM-IoT.

The set of coverage classes supported on EC-CCCH is broadcasted in System Information (EC SI).

##### 3.3.9.2.2	Extended coverage improvement for MS with low output power

An EC-GSM-IoT MS of power class 6 (see 3GPP TS 45.005 [14]) in addition to the Coverage Classes CC1 to CC4 shall support a Coverage Class CC5 for its uplink logical channels, which is suited to operate in further extended coverage beyond the coverage condition covered by CC4. The uplink logical channels in CC5 coverage condition use predefined number of blind physical layer transmissions, where a fixed predefined number of blind physical layer transmissions is applied per logical channel.

In addition to blind physical layer transmissions, dedicated uplink logical channels (EC-PDTCH, EC-PACCH) using CC5 use modified header encoding and burst interleaving schemes (see 3GPP TS 45.002 [11]) to achieve the approximate coverage level shown in Table 3.3.9.2-2. The EC-RACH logical channel uses the Extended Synchronization Access Burst format or Extended Dual slot Access burst format (see 3GPP TS 45.002 [11]) for CC5 operation to achieve the required coverage extension. The burst format of EC-RACH logical channel to be used for CC5 operation is broadcasted in System Information (EC SI).

Table 3.3.9.2-2: Approximate coverage level for Coverage Class 5 (Uplink only) compared to GPRS/EGPRS

| Coverage class | Approximate extended coverage level compared to GPRS/EGPRS [dB]1 |
| --- | --- |
| CC5 | [20-TBD] |
| NOTE1: The value applies for the same maximum output power (23dBm) of an EC-GSM-IoT MS and a GPRS/EGPRS MS. |  |

The support of CC5 is optional for the network and the support for CC5 on uplink logical channels (EC-RACH, EC-PDTCH/U and EC-PACCH/U) is broadcasted in System Information (EC SI).

#### 3.3.9.3	Energy efficient operation

In idle mode, energy efficient operation is enabled by the use of relaxed mobility related requirements (see subclause 3.3.8.1, and 3GPP TS 45.008 [15]) and optionally eDRX (see 3GPP TS 45.002 [11]) or Power Saving Mode (see 3GPP TS 23.682 [18]), PSM. A mobile station supporting EC-GSM-IoT may support eDRX and/or PSM, and shall support the use of relaxed mobility related requirements.

The paging monitoring procedure is also optimized compared to GPRS/EGPRS where

-	A two burst block is used on EC-PCH (compared to a four burst block in EGPRS/GPRS), with the possibility to decode it after only one burst has been received.

-	Early suspension of paging block reception is possible, by inclusion of the DL CC in downlink messages, as well as by using different training sequences depending on which CC the EC-CCCH/D block is intended for. For the former case, a MS that has selected a lower CC than what is indicated by DL CC can suspend the monitoring for the remainder of the EC-CCCH/D block. For the latter case, a MS that has selected CC2, CC3 or CC4 and that detects, through TSC detection, that CC1 is transmitted, can suspend reception for the remainder of the EC-CCCH/D block of the selected CC.

In packet transfer mode, energy efficient operation is enabled by e.g. the following functionality:

-	As for EC-PCH in idle mode, a two burst block is also used on EC-AGCH minimizing reception time when in good radio conditions;

-	Only one phase access is defined in EC operation;

-	Efficient contention resolution that reduces the transmission time from the MS, by the use of a reduced TLLI;

-	Early suspension of downlink reception possible, by inclusion of the DL CC in downlink messages;

-	EC-PACCH on the downlink is possible to decode after only one burst has been received;

-	Energy efficient initial resource allocation by inclusion of measured radio conditions in the channel request message;

-	No neighbor cells monitoring

#### 3.3.9.4	Improved security

EC-GSM-IoT mandates the use of an improved security framework by both the network and the mobile station, see 3GPP TS 43.020 [20]. In this context it should be noted that integrity protection has been limited to LLC unacknowledged operation [8].

2nd Modification

#### 3.3.9.5	Restricted Use of Enhanced Coverage

Use of enhanced coverage, requiring extensive network resources including radio resources, may be restricted by the network. An EC-GSM-IoT capable MS that supports restricted use of enhanced coverage, indicates its capability support during the Attach and Routing Area Update procedures (see 3GPP TS 23.060 [3] and 3GPP TS 24.008 [19]. The network supporting restricted use of enhanced coverage provides the mobile station with the restricted enhanced coverage status in its response.

In case the mobile station receives an indication that enhanced coverage is not restricted, it continues EC operation if camping on a EC-GSM-IoT supporting cell and (E)GPRS operation if camping on a non-EC-GSM-IoT supporting cell.

Otherwise, in case the mobile station receives an indication that enhanced coverage is restricted, it performs cell reselection with modified cell reselection parameters corresponding to the coverage range of CC1 according to 3GPP TS 45.008 [15]. Only in case the mobile station is not able to camp on any cell in EC operation, it performs cell reselection as in case of no enhanced coverage restriction in order to attempt for camping on an EC-GSM-IoT supporting cell. For the purpose of uplink or downlink data transfer, the mobile station is only allowed to use CC1 on uplink and downlink, else shall stop the EC packet access procedure. As an exception, the mobile station is allowed to perform system access using a higher coverage class than CC1 in order to perform the normal Routing Area Update procedure (see 3GPP TS 45.008[15]), i.e. applying the same behaviour as in case of no coverage restriction, to update the network on its current Routing Area due to the change of Routing Area of the serving cell. The Downlink Coverage Class update procedure is not executed in case of restricted use of enhanced coverage. The network may grant enhanced coverage again to the MS at a certain point in time. To this purpose the SGSN checks the status of Enhanced Coverage Restriction (RestrictEC indication in 3GPP TS 24.008 [46]) obtained from HSS/HLR or retrieves the status from the stored MS context and initiates the packet paging procedure at the BSS (see 3GPP TS 48.018 [X]), which sends a paging request to the mobile station camping either on a non-EC-GSM-IoT supporting cell or an EC-GSM-IoT supporting cell and thereby informs about the removed restriction for use of enhanced coverage (see 3GPP TS 44.018 [6]). This paging request is sent using the appropriate Downlink Coverage Class based on the previously sent information from the mobile station in order to ensure proper reception. The mobile station sends a paging response acknowledging reception of the paging request and in case of removed restriction for use of enhanced coverage continues operation in unrestricted enhanced coverage with modified cell reselection and DL/UL coverage class selection according to 3GPP TS 45.008 [15].

End of Modifications