Security Enhancement for MTA positioning procedure

# introduction

Security issues for the MTA positioning procedure for specific network deployments were discussed in RAN6#6 [1] and it was agreed to develop a solution to address the issue for Rel-15. Following is the conclusion of the RAN6#6 meeting regarding further development of a solution for mitigating such security issues [2]:

Additional enhancements to improve the security of the MTA signalling procedure for deployments, which do not have LLC security enabled, are required in Rel-15". Companies are encouraged to bring in solution proposals to RAN6#7 under TEI15.

High level solution aspects and impacts to Stage 2 and Stage 3 specifications for the security enhancement using the MTA Signature method are provided in this document.

# mta signature based solution

When LLC ciphering is not enabled at SGSN, SGSN can optionally request the MS to generate the MTA Signature at the end of multilateration procedure based on the parameters associated with the successfully completed radio access part of multilateration procedure with different cells.

The MTA positioning procedure modified for inclusion of MTA Signature for verification is illustrated in Figure 1, and described here below.

- SGSN includes additional flag to indicate the need for MTA Signature generation as part of Perform Location Request message to BSS. This flag is included if the SGSN does not support LLC security. SGSN includes this parameter if the MS capability indicates the support for MTA Signature generation

- SMLC decides the multilateration method to be used based on BSS and MS capability. If MTA based positioning method is selected, SMLC includes additional parameter in the RRLP message to indicate the MS to generate MTA Signature.

- MS on reception of the RRLP message with indication to generate MTA Signature completes the radio access part of multilateration with the neighbor cells based on the measurements. MS stores the Cell identity and MS Transmission offset value sent to each of the cell where MTA radio access procedure is successful.

- MS generates the MTA Signature when it receives the response for its random access attempt from BSS based on the Cell Identity+MS-Transmission-offset values sent to each of the cells. The MTA Signature is generated by using the MTA-Sequence as input and using the Integrity protection key negotiated at NAS level. The MTA Sequence is the concatenation of all the stored Cell-Identity and MS Transmission offset values.

- MS includes the MTA Signature as part of the RLC block as additional length indicator.

- BSS includes the MTA Signature value to SMLC when it sends the last timing advance value along with MTA Sequence used for MTA Signature generation to SMLC.

- SMLC includes the MTA sequence and MTA Signature in Perform Location Response message towards SGSN.

- SGSN generates MTA Signature based on received MTA sequence and compare against the received MTA Signature value. If it matches, SMLC forwards the final response to GMLC. Otherwise it sends an Error indication to GMLC.

Figure 1: Message flow for Security Enhancement for MTA positioning procedure.

# MTA Signature generation

The MTA Signature generation follows the integrity protection mechanism described in [3] for generation of MAC code based on message contents using the Integrity protection algorithm.

The MTA Signature is generated as per the following mechanism, shown in Figure 2, which is used to generate MAC/XMAC for LLC/GMM messages.

Figure 2: Generation of MAC/XMAC for LLC/GMM messages [3].

In the above diagram, all the parameters except MESSAGE are same as mentioned in H.5.3.1 of [3] corresponds to integrity protection at GMM layer. The MESSAGE is generated based on concatenation of <cell-identity +ms-transmission offset > parameters corresponds to the successful MTA access attempts.

# specification impacts

Implementation of security enhancements for MTA based positioning procedure as referred in section 2 requires following modifications to Stage 2 and Stage 3 specifications.

| Specification |  | Proposed Changes |
| --- | --- | --- |
| TS 43.059 | Stage 2 | Multilateration Timing advance procedure description needs to be modified to include additional security enhancements using MTA Signature. |
| TS 44.060 | Stage 3 | New Length Indicator to be included for MTA Signature. Length of MTA Signature =32 bits (4 octets) |
| TS 49.031 | Stage 3 | BSSMAP-LE CONNECTION ORIENTED INFORMATION message should include two additional information elements MTA-Sequence and MTA-Signature. MTA-Sequence is the MESSAGE part for which the integrity protection is required at SGSN. MTA-Signature is the MAC value received from MS. |
| TS 48.018 | Stage 3 | BSSGP-Perform Location Request message should include additional parameter for MTA Signature generation. BSSGP-Perform Location Response should contain two additional parameters MTA-Sequence and MTA-Signature. If SGSN receives these parameters, SGSN should perform Integrity protection on these values before refering to the position information given in the message. |
| TS 44.031 | Stage 3 | The MTA Procedure description needs to be updated to include additional security using MTA Signature. RRLP message should include new parameter for MTA Signature Required. |
| TS 24.008 | Stage 3 (CT1) | MS capability support for generation of MTA Signature needs to be introduced into MSRAC. |

# summary

The sourcing companies propose following agreement:

Implement the enhancement proposed for MTA based positioning procedure in this contribution in GERAN Rel-15 specifications.

The corresponding specification changes to TS 43.059, TS 44.060, TS 48.018 and TS 24.008 are provided in [4], [5], [6] and [7]. Specification changes to TS 44.031 and TS 49.031 are also required and are foreseen to be submitted to RAN6#8.

# References

[1]	R6-170497, “On Security Aspects of MTA Procedure”, Nokia, Nokia Shanghai Bell, RAN6#6

[2]    Meeting Report RAN6#6, Reno, USA.

[3]    3GPP TS 43.020, “Security related network functions”

[4] 	R6-180023, CR 43.059-0088, “Security Enhancement for MTA in network configuration without LLC security”, Nokia, Nokia Shanghai Bell, RAN6#7

[5]    R6-180024, CR 44.060-0444, “Security Enhancement for MTA in network configuration without LLC security”, Nokia, Nokia Shanghai Bell, RAN6#7

[6]    R6-180025, CR 48.018-1648, “Security Enhancement for MTA in network configuration without LLC security”, Nokia, Nokia Shanghai Bell, RAN6#7

[7]    R6-1800xx, CR 24.008-xxxx, “Introduction of MTA Signature capability support to MSRAC”, Nokia, Nokia Shanghai Bell, RAN6#7