3GPP TSG RAN WG6 #8 	R6-180084

# Busan, Korea, 21 – 25 May, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 45.005 | CR | 0614 | rev | - | Current version: | 13.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Annex S for 2 TS EC-RACH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CIoT_EC_GSM-Perf |  |  |  |  |  |  | Date: |  | 2018-05-15 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Annex S is not treating the case for 2 TS EC-RACH mapping, which is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Annex S, the description for verifying the normalized coherency error for 2 TS EC-RACH mapping for CC2 to CC4 is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Performance requirements for EC-GSM-IoT are not finalized. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex S |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 51.010-1 CR 5156 TS 51.010-2 CR 4402 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | None. |  |  |  |  |  |  |  |  |

1st Modification

Annex S (normative):
Normalized coherency error

The normalized RMS (∆rms) is defined as the ratio of the maximum RMS value of the error vector magnitude (∆x,y,rms) between any two consecutive bursts Ax and Ay, from a set of blind physical layer transmissions in the same TDMA frame, compared to the average RMS value over the set of blind physical layer transmissions (Arms). The normalized RMS is calculated over the useful part of the burst. The normalized coherency error shall be measured after a measurement receive filter at baseband. The specification is based on using the specified, windowed, raised-cosine filter with roll-off 0.25 and single side-band bandwidth of 90 kHz (see subclause 4.6.2).

Ax,rms=1Nn=0N-1Ax(n)2

∆x,yn= Ay(n)-Ax(n)

∆x,y,rms =1Nn=0N-1∆x,y(n)2

Arms= 1Mm=0M-1Am,rms

∆rms= max⁡(∆x,y,rms)Arms

where

-	n=0N-1  is the sum over the useful part of the burst where N is the number of samples (147 or 148 for EC-PDTCH using CC2 to CC4; and 88 for 2 TS EC-RACH mapping using CC2 to CC4) which shall be taken at the symbol rate

-	x and y are indices to any given pair of consecutive bursts out of the blind physical layer transmissions

- 	M is the number of blind physical layer transmissions per TDMA frame (i.e. M=4 for EC-PDTCH and M=2 for 2 TS EC-RACH mapping)

Before taking the logarithm, the measured average of ∆rms over at least 200 TDMA frames (Δrms) in case M=4 shall be computed. For M=4, the TDMA frames over which the average is computed shall be spread over at least 50 separate FUAs (Fixed Uplink Allocations, see 3GPP TS 44.060 [38]), each with a similar size of the allocation, between which EC-PACCH/D or EC-PACCH/D/2TS is transmitted to the MS.

For M=2, the measured average of ∆rms over at least 190 TDMA frames (Δrms) shall be computed. The TDMA frames over which the average is computed shall be spread over at least 6 EC Packet Channel Request messages (see 3GPP TS 44.018 [7]), triggered by a matching paging request. Since the network does not respond to the channel request, it enforces the MS to repeat its EC Packet Channel Request for configured number of maximum retransmissions.

NOTE: 	For CC4 using 2 TS EC-RACH mapping, 8 EC Packet Paging Requests, with configured EC_Max_Retrans = 7, occupy 192 TDMA frames. For CC2 and CC3 using 2 TS EC-RACH mapping, a higher number of EC Packet Channel Request messages need to be sent to fulfil the above requirement.

Then for M=4 or M=2, the measured average of ∆rms shall be transferred to dB scale according to

Δrms [dB]=20log10Δrms

and is referred to as Normalized Coherency Error.

End of Modification