3GPP TSG RAN WG6 meeting #11 							                                    R6-190019

# Electronic Agreement Meeting (7 January – 4 February, 2019)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 45.005 | CR | 0621 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Indication of performance requirements based on 30% BLER |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 21/01/2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | A note is mssing for few tables to indicate that specific performance requirements apply to channels for BLER = 30%. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A note indicating that requirements apply to channels for higher BLER (BLER = 30%) is removed from Table 2a and added to Table 2f, 2t, 2x. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Possible erroneous interpretation of the performance requirement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Table 2a, 2f, 2t, 2x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The correction is only applied to the active release (Rel-16). |  |  |  |  |  |  |  |  |

1st Modification

# Table 2a: Interference ratio at reference performance for GMSK modulated signals

| GSM 900, ER-GSM 900 and GSM 850 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Type of |  | Propagation conditions |  |  |  |  |
| channel |  | TU3 (no FH) | TU3 (ideal FH) | TU50 (no FH) | TU50 (ideal FH) | RA250 (no FH) |
| PDTCH/CS-1 | dB | 13 | 9 | 10 | 9 | 9 |
| PDTCH/CS-2 | dB | 15 | 13 | 14 | 13 | 13 |
| PDTCH/CS-3 | dB | 16 | 15 | 16 | 15 | 16 |
| PDTCH/CS-4 | dB | 21 | 23 | 24 | 24 | * |
| USF/CS-1 | dB | 19 | 10 | 12 | 10 | 10 |
| USF/CS-2 to 4 | dB | 18 | 9 | 10 | 9 | 8 |
| PRACH/11 bits1) | dB | 8 | 8 | 8 | 8 | 10 |
| PRACH/8 bits1) | dB | 8 | 8 | 8 | 8 | 9 |
| GSM 850, MXM 850, ER-GSM 900 and GSM 900 |  |  |  |  |  |  |
| Type of |  | Propagation conditions |  |  |  |  |
| channel |  | TU3 (no FH) | TU3 (ideal FH) | TU50 (no FH) | TU50 (ideal FH) | RA250 (no FH) |
| PDTCH/MCS-05) | dB | 10 | 8.5 | 9 | 8.5 | 9 |
| PDTCH/MCS-1 | dB | 13 | 9.5 | 10.5 | 9.5 | 10 |
| PDTCH/MCS-2 | dB | 15 | 12 | 12.5 | 12 | 12 |
| PDTCH/MCS-3 | dB | 16.5 | 16.5 | 17 | 17 | 19 |
| PDTCH/MCS-4 | dB | 19 | 21.5 | 22 | 22 | * |
| USF/MCS-1 to 4 | dB | 18 | 10 | 11 | 9.5 | 9.5 |
| PRACH/11 bits2), 3) | dB | 8 | 8 | 8 | 8 | 10 |
| PRACH/8 bits1) | dB | 8 | 8 | 8 | 8 | 9 |
| RACH/30 bits7) | dB | 11 | - | 11 | - | - |
| DCS 1 800 & PCS 1 900 |  |  |  |  |  |  |
| Type of |  | Propagation conditions |  |  |  |  |
| channel |  | TU1,5 (no FH) | TU1,5 (ideal FH) | TU50 (no FH) | TU50 (ideal FH) | RA130 (no FH) |
| PDTCH/CS-1 | dB | 13 | 9 | 9 | 9 | 9 |
| PDTCH/CS-2 | dB | 15 | 13 | 13 | 13 | 13 |
| PDTCH/CS-3 | dB | 16 | 15 | 16 | 16 | 16 |
| PDTCH/CS-4 | dB | 21 | 23 | 27 | 27 | * |
| USF/CS-1 | dB | 19 | 10 | 10 | 10 | 10 |
| USF/CS-2 to 4 | dB | 18 | 9 | 9 | 9 | 7 |
| PRACH/11 bits1) | dB | 9 | 9 | 9 | 9 | 10 |
| PRACH/8 bits1) | dB | 8 | 8 | 8 | 8 | 9 |
| DCS 1800, PCS 1 900 and MXM 1900 |  |  |  |  |  |  |
| Type of |  | Propagation conditions |  |  |  |  |
| channel |  | TU1,5 (no FH) | TU1,5 (ideal FH) | TU50 (no FH) | TU50 (ideal FH) | RA130 (no FH) |
| PDTCH/MCS-05) | dB | 10 | 8.5 | 9 | 8.5 | 9 |
| PDTCH/MCS-1 | dB | 13 | 9.5 | 10 | 9.5 | 10 |
| PDTCH/MCS-2 | dB | 15 | 12 | 12 | 12 | 12 |
| PDTCH/MCS-3 | dB | 16.5 | 16.5 | 17 | 18 | 19 |
| PDTCH/MCS-4 | dB | 19 | 21.5 | 23 | 23 | * |
| USF/MCS-1 to 4 | dB | 18 | 10 | 9.5 | 9.5 | 9.5 |
| PRACH/11 bits2), 3) | dB | 9 | 9 | 9 | 9 | 10 |
| PRACH/8 bits1) | dB | 8 | 8 | 8 | 8 | 9 |
| RACH/30 bits7) | dB | 11 | - | 11 | - | - |
| NOTE 1: The specification for PDTCH/CS-1 applies also for PACCH, PBCCH, PAGCH, PPCH, PTCCH/D. NOTE 2: Ideal FH case assumes perfect decorrelation between bursts. This case may only be tested if such a decorrelation is ensured in the test. For TU50 (ideal FH), sufficient decorrelation may be achieved with 4 frequencies spaced over 5 MHz. The TU3 (ideal FH) and TU1.5 (ideal FH), sufficient decorrelation cannot easily be achieved. These performance requirements are given for information purposes and need not be tested. NOTE 3: PDTCH/CS-4 and PDTCH/MCS-x cannot meet the reference performance for some propagation conditions (*). NOTE 4: Identification of the correct Training sequence is required. Cases identified by 1) include one training sequence and cases identified by 2) include 3 training sequences according to 3GPP TS 45.002. The specification identified by 3) also applies to CPRACH. NOTE 5: The specification of MCS-0 only applies in DL RTTI configuration. NOTE 6: The specification for PDTCH/CS-3 applies also when used for PACCH in DLMC configuration (see 3GPP TS 44.060). NOTE 7: The performance requirement applies for the random access message carrying 30 information bits (see 3GPP TS 45.003) sent using the Extended AB burst format (see 3GPP TS 45.002) as part of the Multilateration Timing Advance procedure using the Extended Access Burst method. |  |  |  |  |  |  |

2nd Modification

Table 2f: Adjacent channel interference ratio (for normal BTS) at reference performance for 8-PSK modulated signals (convolutional coding)

| GSM 900, ER-GSM 900, GSM 850 and MXM 850 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Type of |  | Propagation conditions |  |  |  |  |
| channel |  | TU3 (no FH) | TU3 (ideal FH) | TU50 (no FH) | TU50 (ideal FH) | RA250 (no FH) |
| PDTCH/MCS-5 | dB | 2.5 | -2 | -2 | -2 | 1 |
| PDTCH/MCS-6 | dB | 4.5 | 0.5 | 1 | 1 | 6.5 |
| PDTCH/MCS-7 | dB | 8 | 8 | 8.5 | 8.5 | 13.5** |
| PDTCH/MCS-8 | dB | 10.5 | 12 | 9** | 9.5** | * |
| PDTCH/MCS-9 | dB | 12 | 14 | 13.5** | 13.5** | * |
| DCS 1 800, PCS 1900 and MXM 1900 |  |  |  |  |  |  |
| Type of |  | Propagation conditions |  |  |  |  |
| channel |  | TU1,5 (no FH) | TU1,5 (ideal FH) | TU50 (no FH) | TU50 (ideal FH) | RA130 (no FH) |
| PDTCH/MCS-5 | dB | 2.5 | -2 | -2 | -1.5 | 1 |
| PDTCH/MCS-6 | dB | 4.5 | 0.5 | 1.5 | 1.5 | 6.5 |
| PDTCH/MCS-7 | dB | 8 | 8 | 10.5 | 11 | 13.5** |
| PDTCH/MCS-8 | dB | 10.5 | 12 | 10** | 9.5** | * |
| PDTCH/MCS-9 | dB | 12 | 14 | 16** | 16** | * |
| NOTE 1: Ideal FH case assumes perfect decorrelation between bursts. This case may only be tested if such a decorrelation is ensured in the test. For TU50 (ideal FH), sufficient decorrelation may be achieved with 4 frequencies spaced over 5 MHz. The TU3 (ideal FH) and TU1.5 (ideal FH), sufficient decorrelation cannot easily be achieved. These performance requirements are given for information purposes and need not be tested. NOTE 2: PDTCH for MCS-x can not meet the reference performance for some propagation conditions (*). NOTE 3: Performance is specified at 30% BLER for those cases identified with mark **. |  |  |  |  |  |  |

3rd Modification

Table 2t: Cochannel interference ratio (for normal BTS) at reference performance for QPSK, 16-QAM and 32-QAM modulated signals (Higher symbol rate and BTTI) (EGPRS2-B UL)

| GSM 900, GSM 850 and MXM 850 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type of |  | Propagation conditions |  |  |  |  |  |  |  |
| channel |  | TU3 (no FH) |  | TU3 (ideal FH) | TU50 (no FH) |  | TU50 (ideal FH) |  | RA250 (no FH) |
| PDTCH/UBS-5 | dB |  | -4,5 / [tbd] | (2) | -4,5 / [tbd] | -6,0 / [tbd] |  |  | -1,5 / [tbd] |
| PDTCH/UBS-6 | dB |  | -2,0 / [tbd] | (2) | -3,0 / [tbd] | -3,5 / [tbd] |  |  | 1,5 / [tbd] |
| PDTCH/UBS-7 | dB |  | 2,5 / [tbd] | (2) | 1,5 / [tbd] | 1,0 / [tbd] |  |  | 5,5 / [tbd] |
| PDTCH/UBS-8 | dB |  | 4,5 / [tbd] | (2) | 4,0 / [tbd] | 3,5 / [tbd] |  |  | 9,5 / [tbd] |
| PDTCH/UBS-9 | dB |  | 5,5 / [tbd] | (2) | 5,5 / [tbd] | 5,0 / [tbd] |  |  | 13,0 / [tbd] |
| PDTCH/UBS-10 | dB |  | 11,0 / [tbd] | (2) | 12,5 / [tbd] | 12,0 / [tbd] |  |  | 26,0 / [tbd] |
| PDTCH/UBS-11 | dB |  | 13,5 / [tbd] | (2) | 16,5 / [tbd] | 17,0 / [tbd] |  |  | 35,0 / [tbd] |
| PDTCH/UBS-12 | dB |  | 15,0 / [tbd] | (2) | 19,5 / [tbd] | 19,5 / [tbd] |  |  | 28,0** / [tbd] |
| DCS 1 800. PCS 1900 and MXM 1900 |  |  |  |  |  |  |  |  |  |
| Type of |  | Propagation conditions |  |  |  |  |  |  |  |
| channel |  | TU1,5 (no FH) |  | TU1,5 (ideal FH) | TU50 (no FH) |  | TU50 (ideal FH) |  | RA130 (no FH) |
| PDTCH/UBS-5 | dB |  | (2) | (2) | -4,5 / [tbd] | (2) |  | (2) |  |
| PDTCH/UBS-6 | dB |  | (2) | (2) | -2,5 / [tbd] | (2) |  | (2) |  |
| PDTCH/UBS-7 | dB |  | (2) | (2) | 2,5 / [tbd] | (2) |  | (2) |  |
| PDTCH/UBS-8 | dB |  | (2) | (2) | 5,0 / [tbd] | (2) |  | (2) |  |
| PDTCH/UBS-9 | dB |  | (2) | (2) | 6,5 / [tbd] | (2) |  | (2) |  |
| PDTCH/UBS-10 | dB |  | (2) | (2) | 14,5 / [tbd] | (2) |  | (2) |  |
| PDTCH/UBS-11 | dB |  | (2) | (2) | 22,5 / [tbd] | (2) |  | (2) |  |
| PDTCH/UBS-12 | dB |  | (2) | (2) | 28,5 / [tbd] | (2) |  | (2) |  |
| NOTE 1: Ideal FH case assumes perfect decorrelation between bursts. This case may only be tested if such a decorrelation is ensured in the test. For TU50 (ideal FH), sufficient decorrelation may be achieved with 4 frequencies spaced over 5 MHz. The TU3 (ideal FH) and TU1.5 (ideal FH), sufficient decorrelation cannot easily be achieved. These performance requirements are given for information purposes and need not be tested. NOTE 2: The requirements for the DCS 1800 & PCS 1900 TU1.5 (no FH) propagation condition are the same as for the GSM 850 & GSM 900 TU3 (no FH) propagation condition, the requirements for the GSM 850 & GSM 900 TU3 (ideal FH), DCS 1800 & PCS 1900 TU1.5 (ideal FH) and DCS 1800 & PCS 1900 TU50 (ideal FH) propagation conditions are the same as for the DCS 1800 & PCS 1900 TU50 (no FH) propagation condition, and the requirements for the DCS 1800 & PCS 1900 RA130 (no FH) propagation condition are the same as for the GSM 850 & GSM 900 RA250 (no FH) propagation condition. NOTE 3: RX diversity requirements are given by the left numbers. Requirements without RX diversity are given by the right numbers. NOTE 4: Performance is specified at 30% BLER for those cases identified with mark **. |  |  |  |  |  |  |  |  |  |

4th Modification

Table 2x: Adjacent channel interference ratio (for normal BTS) at reference performance for QPSK, 16-QAM and 32-QAM modulated signals (Higher symbol rate, BTTI and no PAN) 
using narrow pulse shaping filter (EGPRS2-B UL)

| GSM 900, GSM 850 and MXM 850 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type of |  |  |  |  | Propagation conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| channel |  |  |  |  | TU3 (no FH) |  |  | TU3 (ideal FH) |  |  | TU50 (no FH) |  |  | TU50 (ideal FH) |  |  | RA250 (no FH) |  |  |
| PDTCH/UBS-5 |  | dB |  |  | -14,5 / [tbd] |  |  | (2) |  |  | -14,0 / [tbd] |  |  | -14,0 / [tbd] |  |  | -2,5 / [tbd] |  |  |
| PDTCH/UBS-6 |  | dB |  |  | -12,0 / [tbd] |  |  | (2) |  |  | -11,0 / [tbd] |  |  | -11,5 / [tbd] |  |  | 1,5 / [tbd] |  |  |
| PDTCH/UBS-7 |  | dB |  |  | -1,5 / [tbd] |  |  | (2) |  |  | -2,5 / [tbd] |  |  | -3,0 / [tbd] |  |  | 7,0 / [tbd] |  |  |
| PDTCH/UBS-8 |  | dB |  |  | 1,0 / [tbd] |  |  | (2) |  |  | 0,5 / [tbd] |  |  | 0,5 / [tbd] |  |  | 11,0 / [tbd] |  |  |
| PDTCH/UBS-9 |  | dB |  |  | 2,5 / [tbd] |  |  | (2) |  |  | 2,0 / [tbd] |  |  | 2,0 / [tbd] |  |  | 12,5 / [tbd] |  |  |
| PDTCH/UBS-10 |  | dB |  |  | 7,0 / [tbd] |  |  | (2) |  |  | 9,0 / [tbd] |  |  | 9,0 / [tbd] |  |  | 18,0 / [tbd] |  |  |
| PDTCH/UBS-11 |  | dB |  |  | 8,5 / [tbd] |  |  | (2) |  |  | 13,0 / [tbd] |  |  | 13,5 / [tbd] |  |  | 18,5** / [tbd] |  |  |
| PDTCH/UBS-12 |  | dB |  |  | 9,0 / [tbd] |  |  | (2) |  |  | 14,5 / [tbd] |  |  | 15,0 / [tbd] |  |  | 22,5** / [tbd] |  |  |
| DCS 1 800, PCS 1900 and MXM 1900 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type of |  |  |  |  | Propagation conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| channel |  |  |  |  | TU1,5 (no FH) |  |  | TU1,5 (ideal FH) |  |  | TU50 (no FH) |  |  | TU50 (ideal FH) |  |  | RA130 (no FH) |  |  |
| PDTCH/UBS-5 |  |  | dB |  | (2) |  |  | (2) |  |  | -10,0 / [tbd] |  |  | (2) |  |  | (2) |  |  |
| PDTCH/UBS-6 |  |  | dB |  | (2) |  |  | (2) |  |  | -6,5 / [tbd] |  |  | (2) |  |  | (2) |  |  |
| PDTCH/UBS-7 |  |  | dB |  | (2) |  |  | (2) |  |  | 2,0 / [tbd] |  |  | (2) |  |  | (2) |  |  |
| PDTCH/UBS-8 |  |  | dB |  | (2) |  |  | (2) |  |  | 5,0 / [tbd] |  |  | (2) |  |  | (2) |  |  |
| PDTCH/UBS-9 |  |  | dB |  | (2) |  |  | (2) |  |  | 7,0 / [tbd] |  |  | (2) |  |  | (2) |  |  |
| PDTCH/UBS-10 |  |  | dB |  | (2) |  |  | (2) |  |  | 14,0 / [tbd] |  |  | (2) |  |  | (2) |  |  |
| PDTCH/UBS-11 |  |  | dB |  | (2) |  |  | (2) |  |  | 12,5** / [tbd] |  |  | (2) |  |  | (2) |  |  |
| PDTCH/UBS-12 |  |  | dB |  | (2) |  |  | (2) |  |  | 14,0** / [tbd] |  |  | (2) |  |  | (2) |  |  |
| NOTE 1: Ideal FH case assumes perfect decorrelation between bursts. This case may only be tested if such a decorrelation is ensured in the test. For TU50 (ideal FH), sufficient decorrelation may be achieved with 4 frequencies spaced over 5 MHz. The TU3 (ideal FH) and TU1.5 (ideal FH), sufficient decorrelation cannot easily be achieved. These performance requirements are given for information purposes and need not be tested. NOTE 2: The requirements for the DCS 1800 & PCS 1900 TU1.5 (no FH) propagation condition are the same as for the GSM 850 & GSM 900 TU3 (no FH) propagation condition, the requirements for the GSM 850 & GSM 900 TU3 (ideal FH), DCS 1800 & PCS 1900 TU1.5 (ideal FH) and DCS 1800 & PCS 1900 TU50 (ideal FH) propagation conditions are the same as for the DCS 1800 & PCS 1900 TU50 (no FH) propagation condition, and the requirements for the DCS 1800 & PCS 1900 RA130 (no FH) propagation condition are the same as for the GSM 850 & GSM 900 RA250 (no FH) propagation condition. NOTE 3: RX diversity requirements are given by the left numbers. Requirements without RX diversity are given by the right numbers. NOTE 4: Performance is specified at 30% BLER for those cases identified with mark **. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

End of Modifications