3GPP TSG RAN WG6 meeting #13 							                                      R6-190059

# Electronic Agreement Meeting (15 July – 8 August, 2019)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 45.008 | CR | 0676 | rev | - | Current version: | 14.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Default value for signal level filter period in EC operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CIoT_EC_GSM-Core |  |  |  |  |  | Date: |  |  | 02/08/2019 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The specification does not contain a default value for the signal level filter period TAVG_T used for Cn value computation. For (E)GPRS, the value is broadcasted on BCCH (in SI13), but for EC-GSM-IoT it is only sent in an RLC/MAC control message (EC Packet Power Control/Timing Advance) sent on EC-PACCH. In case the MS does not receive this message prior to EC TBF operation, it is unaware of the signal filter period TAVG_T. Thus, a default value is needed which the MS can use for signal level filtering in packet transfer mode in this case when the EC TBF operation starts. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In subclause 10.2.3.1.2.2, the default value for the signal level filter period TAVG_T in EC operation is specified. A default value of 1/3 is specified for EC TBF reception in CC1 and CC2, corresponding to a signal level filter period of 80 ms and a default value of 2/3 is specified for EC TBF reception in CC3 or CC4, corresponding to a signal filter period of 80 ms or 160 ms, since applied to 40 ms or 80 ms lasting radio blocks for CC3 or CC4, respectively. In subclause 10.4, EC-PACCH is added for signalling of TAVG_T. Also a comma is added at the end of the section. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification remains erroneous and incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.2.3.1.2.2, 10.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None. |  |  |  |  |  |  |  |  |

For Information only

##### 10.2.3.1.2	Packet transfer mode or MAC-Shared state

###### 10.2.3.1.2.1	MS that have not enabled EC operation

In packet transfer mode or MAC-Shared state, the MS shall use the same received signal level measurements as made for cell reselection on the BCCH carrier of the serving cell (see 10.1.1.2) or, for COMPACT, on the CPBCCH carrier of the serving cell (see 12.4.1.2). The measurements shall be filtered with a running average filter:

Cn = (1-b)  Cn-1 + b  SS n,

where

SSn is the received signal level of the measurement samples.

b is the forgetting factor:

b = 1 for n=1;

b = 1/(6*TAVG_T).

n is the iteration index. The filter shall be restarted with n=1 for the first sample at every cell change. When entering packet transfer mode from packet idle mode or MAC-Shared state from MAC-Idle state, the filter shall continue from the n and Cn values obtained during packet idle mode or MAC-Idle state. In case the MS and the network support enhanced DTM CS release procedure, when entering packet transfer mode from DTM, the filter shall continue from the n and Cn values obtained during DTM. When entering DTM from dedicated mode, the filter shall be restarted with n=1 for the first sample in case there is no valid C value during dedicated mode. In case the MS and the network support enhanced DTM CS establishment procedure, when entering DTM from packet transfer mode, the filter shall continue from the n and Cn values obtained during packet transfer mode.

If indicated by the parameter PC_MEAS_CHAN, the MS shall instead measure the received signal level of each radio block on one of the PDCH s in BTTI configuration or on one of the PDCH-pairs in RTTI configuration monitored by the MS for PACCH. If downlink power control is used, PC_MEAS_CHAN shall indicate measurements on the BCCH or CPBCCH, or the MS is not required to fulfil 45.005 requirements. The MS may discard new PC_MEAS_CHAN values received during packet transfer mode or MAC-Shared state. For each downlink radio block Cblock,n shall be derived according to formula (2) specified in 10.2.3.1.1 (if PBCCH does not exist, and for COMPACT, Pb = 0). Finally, the Cblock,n values are filtered with a running average filter:

Cn = (1-c)  Cn-1 + c  Cblock,n,

where c is the forgetting factor:

c = 1 for n=1;

c = 1/(12*TAVG_T).

n is the iteration index. The filter shall be restarted with n=1 for the first sample at every cell change. When entering packet transfer mode from packet idle mode or MAC-Shared state from MAC-Idle state, the filter shall continue from the n and Cn values obtained during packet idle mode or MAC-Idle state. In case the MS and the network support enhanced DTM CS release procedure, when entering packet transfer mode from DTM, the filter shall continue from the n and Cn values obtained during DTM. When entering DTM from dedicated mode, the filter shall be restarted with n=1 for the first sample in case there is no valid C value during dedicated mode. In case the MS and the network support enhanced DTM CS establishment procedure, when entering DTM from packet transfer mode, the filter shall continue from the n and Cn values obtained during packet transfer mode.

NOTE 1:	This method is suitable in the case where BCCH or CPBCCH is in another frequency band than the used PDCHs.

The current Cn value shall be used to update formula (1) in subclause 10.2.1 each time a new Cn value is obtained or whenever the MS applies new CH or  values.

For each correctly received block on one of the PDCHs monitored by the MS, the MS shall calculate the variance of the received signal level as:

BL_VARn = 1/(j-1)*SUM(SSk - SSblock,n)2, k = 1,...,4

where	SSk is the received signal level of burst k within the block.

SSblock,n is the mean of the received signal level of the j normal bursts that compose the radio block.

j is the number of bursts in the radio block = 4.

If frequency hopping is used and 1 burst of the block is received on the BCCH carrier, that burst shall be discarded from the calculation (j = 3). If 2 bursts are received on the BCCH carrier, the whole block shall be discarded. If 3 bursts are received on the BCCH carrier, the other burst shall be discarded (j = 3).

If more than one PDCH are monitored the MS shall for each block period try to find one correctly received block for the BL_VAR calculation. The block may be taken from any of the monitored PDCHs.

The reported value, SIGN_VAR, shall be the average of BL_VAR within the reporting period. The first reporting period starts with and includes the first assignment message for an uplink or downlink transfer. The reporting period ends, and the subsequent reporting period starts, no earlier than two blocks before the transmission of a quality report and no later than one block before the transmission of a quality report. In averaging, measurements made during previous reporting periods shall always be discarded.

SIGN_VAR shall be included in the channel quality report (see subclause 10.2.3.2.3). If the channel quality report is included in a PACKET RESOURCE REQUEST message, which is retransmitted due to lack of response (see 3GPP TS 44.060), the same or the most recent SIGN_VAR value shall be sent and no new reporting period shall be started. This will ensure that a valid SIGN_VAR value exists. The most recent SIGN_VAR value corresponds to the most recent average of BL_VAR computed since the beginning of the last reporting period.

TAVG_T and PC_MEAS_CHAN are broadcast on PBCCH or, if PBCCH does not exist, on BCCH or on CPBCCH or optionally sent to mobile station in an RLC/MAC control message (see 3GPP TS 44.060).

First modification

###### 10.2.3.1.2.2	MS that have enabled EC operation

A MS that has enabled EC operation shall for each correctly received EC-PDTCH block and EC-PACCH block addressed to it derive Cblock,n as the mean of the received signal level of either all bursts of the block if Coverage Class 1 is used, or of all blind physical layer transmissions of all bursts of the block if a Coverage Class higher than 1 is used. The measured received signal level shall exclude contributions from other sources such as interference and noise.

The Cblock,n values are filtered with a running average filter:

Cn = (1-c)  Cn-1 + c  Cblock,n,

where c is the forgetting factor defined as c = 1/(12*TAVG_T).

n is the iteration index.

When entering packet transfer mode from packet idle mode, the filter shall be initiated with n=1 and C0 = the most recent C value obtained during packet idle mode.

In addition, if the DL_CC_Selection parameter (see 3GPP TS 44.018 [17]) indicates that DL CC selection shall be based on SLA, the MS shall for each block used to derive Cblock,n as described above, also derive Iblock,n as the mean of the interference plus noise level of all bursts of the block, if Coverage Class 1 is used, or of all blind physical layer transmissions of all bursts of the block, if a Coverage Class higher than 1 is used.

The Iblock,n values are filtered with a running average filter:

In = (1-c)  In-1 + c  Iblock,n,

where c is the forgetting factor defined above.

When entering packet transfer mode from packet idle mode, the filter shall be initiated with n=1 and I0 = the most recent INA value (see subclause 6.9) obtained during packet idle mode.

TAVG_T is not broadcast on EC-BCCH. A default value of TAVG_T = [1/3] shall be used by the mobile station for EC TBF reception in CC1 or CC2 and a default value of TAVG_T = [1/6] for EC TBF reception in CC3 or CC4. TAVG_T may be sent to the mobile station in an RLC/MAC control message (see 3GPP TS 44.060 [19]) overwriting the default value and being valid until the mobile station receives a new value in the serving cell or until it reselects to another cell.

The current Cn value shall be used to update formula (1) in subclause 10.2.1 each time a new Cn value is obtained or whenever the MS applies new CH or  values.

If the DL_CC_Selection parameter indicates that DL CC selection shall be based on RLA_EC, the current Cn is also reported to the BSS as the C value, as described in subclause 10.2.3.2.3. If the DL_CC_Selection parameter indicates that DL CC selection shall be based on SLA, the current Cn and In are combined into the reported C value, as described in subclause 10.2.3.2.3.

Second modification

## 10.4	Control parameters

A non-exhaustive list of parameters employed to control the radio links for GPRS are shown in table 3.

Table 3: Radio sub-system link control parameters for GPRS
((s) and (n) denote serving cell and non-serving cell respectively)

| Parameter name | Description | Range | Bits | Channel |
| --- | --- | --- | --- | --- |
| BA(GPRS) | BCCH Allocation for GPRS re-selection Note: If PBCCH does not exist, BA(GPRS) = BA(BCCH) | - | - | PBCCH D/L |
| BSIC(s+n) | Base station Identification Code for carriers in BA(GPRS) and the serving BCCH carrier | 0-63 | 6 | PBCCH D/L (**) |
| MS_TXPWR_MAX_CCH | See table 1. | 0-31 | 5 | BCCH D/L |
| POWER OFFSET(s) | See table 1. | 0-3 | 2 | BCCH D/L |
| RXLEV_ACCESS_MIN | See table 1. | 0-63 | 6 | BCCH D/L |
| GPRS_MS_TXPWR_MAX_CCH(s+n) | The maximum TX power level an MS may use when accessing the system | 0-31 | 5 | PBCCH D/L |
| LB_MS_TXPWR_MAX_CCH(s) | The maximum allowed TX power level for GPRS access on the serving cell, on all other than DCS 1800 and PCS 1900 frequency bands. 0 = 43 dBm, 1 = 41 dBm, 2 = 39 dBm,...,18 = 7 dBm, 19 = 5 dBm, 20 = 5 dBm,...,31 = 5dBm. | 0-31 | 5 | BCCH D/L PBCCH D/L PCCCH D/L PACCH D/L |
| GPRS_RXLEV_ACCESS_MIN(s+n) | Minimum received signal level at the MS required for access to the system. | 0-63 | 6 | PBCCH D/L |
| GPRS_RESELECT_OFFSET (n) | Applies an offset and hysteresis to the C32 re-selection criterion. -52, -48,..., -12, -10,..., 12, 16, ...,48 dB | 0-31 | 5 | PBCCH D/L |
| PRIORITY_CLASS (s+n) | The HCS priority for the cells | 0-7 | 3 | PBCCH D/L |
| LSA ID (s+n) | The LSA identities for the cells |  |  | PBCCH D/L |
| HCS_THR(s+n) | HCS signal level threshold -110, -108,..., -50, infinity dB | 0-31 | 5 | PBCCH D/L |
| GPRS_TEMPORARY_OFFSET(n) | Applies a negative offset to C32 for the duration of PENALTY_TIME. 0, 10,..., 60 dB, infinity | 0-7 | 3 | PBCCH D/L |
| GPRS_PENALTY_TIME(n) | Gives the duration for which the temporary offset is applied. 10, 20,..., 320 seconds | 0-31 | 5 | PBCCH D/L |
| GPRS_CELL_RESELECT_ HYSTERESIS | Additional hysteresis applied in GMM Ready state or RRC-Cell_Shared state for cells in the same RA. 0, 2,..., 14 dB | 0-7 | 3 | PBCCH D/L |
| RA_RESELECT_HYSTERESIS | Additional hysteresis applied for cells in different RAs. 0, 2,..., 14 dB | 0-7 | 3 | PBCCH D/L |
| CELL_RESELECT_HYSTERESIS | Additional hysteresis applied for cells in different RAs if PCCCH does not exist. See table 1. | 0-7 | 3 | BCCH D/L |
| C32_QUAL | Flag indicating an exception rule for GPRS_RESELECT_OFFSET | 1/0 | 1 | PBCCH D/L |
| C31_HYST | Flag indicating if hysteresis shall be applied to C31. | 1/0 | 1 | PBCCH D/L |
| MULTIBAND_REPORTING | The number of carriers from each frequency band that shall be included in the list of 6 strongest cells or in the measurement report. | 0-3 | 2 | PBCCH D/L BCCH D/L |
|  | Power control parameter 0,0.1,...,1 | 0-10 | 4 | PBCCH D/L PACCH D/L (**) |
| Pb | Power reduction used by BTS on PBCCH blocks, relatively to the output power used on BCCH 0, -2,..., -30 dB | 0-15 | 4 | PBCCH D/L PACCH D/L |
| PC_MEAS_CHAN | Flag that indicates whether the downlink measurements for power control shall be made on BCCH or PDCH. | 0/1 | 1 | PBCCH D/L PACCH D/L (**) |
| TAVG_W | Signal level filter period for power control in packet idle mode or MAC-Idle state 2(k/2)/ 6 multiframes, k = 0,1,..., 25 | 0-25 | 5 | PBCCH D/L PACCH D/L (**) |
| TAVG_T | Signal level filter period for power control in packet transfer mode or MAC-Shared state 2(k/2)/ 6 multiframes, k = 0,1,..., 25 | 0-25 | 5 | PBCCH D/L PACCH D/L EC-PACCH D/L (**) |
| NAVG_I | Interference signal level filter constant for power control 2(k/2), k = 0,1,..., 15 | 0-15 | 4 | PBCCH D/L PACCH D/L (**) |
| (continued) |  |  |  |  |

Table 3 (concluded): Radio sub-system link control parameters for GPRS

| Parameter name |  |  |  |  | Description |  |  |  |  | Range |  |  |  |  | Bits |  |  |  | Channel |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BEP_PERIOD |  |  |  |  | Filter constant for EGPRS Channel quality measurements. See subclause 10.2.3.2.1 |  |  |  |  | 0-15 |  |  |  |  | 4 |  |  |  | PBCCH D/L (**) |  |  |  |  |
| BEP_PERIOD2 |  |  |  |  | Filter constant for EGPRS Channel quality measurements. See subclause 10.2.3.2.1 |  |  |  |  | 0-15 |  |  |  |  | 4 |  |  |  | PACCH D/L |  |  |  |  |
| NETWORK_CONTROL_ORDER |  |  |  |  | Controls cell re-selection and measurement reporting |  |  |  |  | 0-3 |  |  |  |  | 2 |  |  |  | PBCCH D/L PCCCH D/L PACCH D/L (**) |  |  |  |  |
| NC_FREQUENCY_LIST |  |  |  |  | Frequency list for cell re-selection measurement reporting |  |  |  |  | - |  |  |  |  | - |  |  |  | PCCCH D/L PACCH D/L |  |  |  |  |
| NC_REPORTING_PERIOD_I NC_REPORTING_PERIOD_T |  |  |  |  | Time period for measurement reporting 0.48, 0.96, 1.92, ..., 61.44 seconds |  |  |  |  | 0-7 |  |  |  |  | 3 |  |  |  | PBCCH D/L PCCCH D/L PACCH D/L (**) |  |  |  |  |
| NCC_PERMITTED |  |  |  |  | Bit map of NCC part of BSIC for which the MS shall report enhanced measurements. |  |  |  |  | - |  |  |  |  | 8 |  |  |  | PBCCH D/L PCCCH D/L PACCH D/L |  |  |  |  |
| SCALE |  |  |  |  | Indication of the offset, which applies for the reported RXLEV values. 0 = 0 dB, 1 = +10 dB |  |  |  |  | 0-1 |  |  |  |  | 1 |  |  |  | PACCH U/L |  |  |  |  |
| SCALE_ORD |  |  |  |  | Indication of the offset, which shall be used for the reported RXLEV values. 0 = +0 dB, 1 = + 10 dB, 2 = automatic Default value = 0 dB. |  |  |  |  | 0-2 |  |  |  |  | 2 |  |  |  | PBCCH D/L PCCCH D/L PACCH D/L (**) |  |  |  |  |
| CELL_BAR_ACCESS_2 |  |  |  |  | See table 3b |  |  |  |  | 0/1 |  |  |  |  | 1 |  |  |  | PBCCH D/L |  |  |  |  |
| Qsearch_P |  |  |  |  | Search for UTRAN FDD cells if signal level below threshold (0-7): - 98, - 94, … , - 74 dBm, (always) or above threshold (8-15): - 78, - 74, … , - 54 dBm, (never). Default value = (never). Search for UTRAN TDD cells if signal level below threshold (0-7): - 98, - 94, … , - 74 dBm, (always) or above threshold (8-15): - 90, - 86, … , -66 dBm, (never). |  |  |  |  | 0-15 |  |  |  |  | 4 |  |  |  | PBCCH D/L PCCCH D/L PACCH D/L (**) (See note 1) |  |  |  |  |
| FDD_GPRS_Qoffset |  |  |  |  | Applies an offset to RLA_P for UTRAN FDD cell re-selection, 0 = - (always select a cell if acceptable), 1 = -28 dB, 2 = -24 dB, … , 15 = 28 dB. Default value = 0 dBm. |  |  |  |  | 0-15 |  |  |  |  | 4 |  |  |  |  | PBCCH D/L |  |  |  |  |
| TDD_GPRS_Qoffset |  |  |  |  | An absolute threshold of RSCP for UTRAN TDD cell re-selection, 0 = -105dBm, 1 = -102dBm, 2=-99dBm, 3 = -96dBm, 4 = -93dBm, 5 = -90dBm, 6 = -87dBm, 7 = -84dBm, 8 = -81dBm, 9 = -78dBm, 10 = -75dBm, 11 = -72dBm, 12 = -69dBm, 13 = -66dBm, 14 = -63dBm, 15 = -60dBm. Default value = -90dBm. |  |  |  |  | 0-15 |  |  |  |  | 4 |  |  |  |  | PBCCH D/L |  |  |  |  |
| FDD_Qmin |  |  |  |  | A minimum threshold for Ec/No for UTRAN FDD cell re-selection, 0= -20dB, 1= -6dB, 2= -18dB, 3= -8dB, 4= -16dB, 5= -10dB, 6= -14dB, 7= -12dB. Default value= -12dB. |  |  |  |  | 0-7 |  |  |  |  | 3 |  |  |  |  | PBCCH D/L |  |  |  |  |
| FDD_Qmin_Offset |  |  |  |  | Applies an offset to FDD_Qmin value, 0 = 0 dB, 1 = 2 dB, 2 = 4 dB, 3 = 6 dB, 4 = 8 dB, 5 = 10 dB, 6 = 12 dB, 7 = 14 dB. Default value = 0 dB. |  |  |  |  | 0-7 |  |  |  |  | 3 |  |  |  |  | PBCCH D/L PCCCH D/L PACCH D/L (**) |  |  |  |  |
| FDD_RSCPmin |  |  |  |  | A minimum threshold of RSCP for UTRAN FDD cell re-selection, 0 = -114 dBm, 1 = -112 dBm, 2 = -110 dBm, 3 = -108 dBm, 4 = -106 dBm, 5 = -104 dBm, 6 = -102 dBm, 7 = -100 dBm, 8 = -98 dBm, 9 = -96 dBm, 10 = -94 dBm, 11 = -92 dBm, 12 = -90 dBm, 13 = -88 dBm, 14 = -86 dBm, 15 = -84 dBm. Default value = -102 dBm. |  |  |  |  | 0-15 |  |  |  |  | 4 |  |  |  |  | PBCCH D/L PCCCH D/L PACCH D/L (**) |  |  |  |  |
| XXX_MULTIRAT_REPORTING |  |  |  |  | The number of cells from the access technology/mode XXX (one or more) that shall be included in the measurement report. |  |  |  |  | 0-3 |  |  |  |  | 2 |  |  |  |  | PBCCH D/L PCCCH D/L PACCH D/L (**) |  |  |  |  |
| SERVING_BAND_REPORTING |  |  |  |  | The number of cells from the GSM serving frequency band that shall be included in the measurement report. Default value = 3 |  |  |  |  | 0-3 |  |  |  |  | 2 |  |  |  | PBCCH D/L PCCCH D/L PACCH D/L (**) |  |  |  |  |
| REPORT_TYPE |  |  |  |  | Indicates which report type the MS shall use, 0 = enhanced, 1 = normal. Default value = normal. |  |  |  |  | 0/1 |  |  |  |  | 1 |  |  |  | PBCCH D/L PCCCH D/L PACCH D/L (**) |  |  |  |  |
| REP_PRIORITY |  |  |  |  | Indicates the reporting priority per cell, 0 = normal, 1 = high Default value = normal |  |  |  |  | 0/1 |  |  |  |  | 1 |  |  |  | PBCCH D/L PCCCH D/L PACCH D/L (**) |  |  |  |  |
| REPORTING_RATE |  |  |  |  | Indicates the allowed reporting rate, 0 = normal, 1 = reduced |  |  |  |  | 0/1 |  |  |  |  | 1 |  |  |  | PBCCH D/L PCCCH D/L PACCH D/L (**) |  |  |  |  |
| INVALID_BSIC_REPORTING |  |  |  |  | Indicates if GSM cells with invalid BSIC and allowed NCC part may be reported, 0 = no, 1 = yes |  |  |  |  | 0/1 |  |  |  |  | 1 |  |  |  | PBCCH D/L PCCCH D/L PACCH D/L (**) |  |  |  |  |
| XXX_REPORTING_THRESHOLD |  |  |  |  | Apply priority reporting if the reported value is above threshold for GSM frequency band or access technology/mode XXX (one or more), 0, 6, … , -36, (never). Default value = always. |  |  |  |  | 0-7 |  |  |  |  | 3 |  |  |  | PBCCH D/L PCCCH D/L PACCH D/L (**) |  |  |  |  |
| FDD_REPORTING_THRESHOLD_2 |  |  |  |  | Reporting threshold for the CPICH parameter (Ec/No or RSCP) that is not reported according to FDD_REP_QUANT. Default value = 0 (disabled) |  |  |  |  | 0-63 |  |  |  |  | 6 |  |  |  | PBCCH D/L PCCCH D/L PACCH D/L (**) |  |  |  |  |
| XXX_REPORTING_OFFSET |  |  |  |  | Apply an offset to the reported value when prioritising the cells for reporting for GSM frequency band or access technology/mode XXX (one or more), 0, 6, … , 42. Default value = 0. |  |  |  |  | 0-7 |  |  |  |  | 3 |  |  |  | PBCCH D/L PCCCH D/L PACCH D/L (**) |  |  |  |  |
| FDD_REP_QUANT |  |  |  |  | Indicates the reporting quantity for UTRAN FDD cells, 0 = RSCP, 1 = Ec/No |  |  |  |  | 0/1 |  |  |  |  | 1 |  |  |  | PBCCH D/L PCCCH D/L PACCH D/L (**) |  |  |  |  |
| RTD |  |  |  |  | The real time difference to other GSM cells, modulo 51 TDMA frames, step: 1 or 1/64 TDMA frame |  |  |  |  | 0-50 or 0-3263 |  |  |  |  | 6 or 12 |  |  |  | PBCCH D/L (**) |  |  |  |  |
| 3G_BA_IND |  |  |  |  | Sequence number of 3G neighbour cell list and/or E-UTRAN neighbour cell list |  |  |  |  | 0/1 |  |  |  |  | 1 |  |  |  | PBCCH DL PCCCH D/L PACCH D/L (**) |  |  |  |  |
| THRESH_GSM_low |  |  |  |  | A threshold below which the MS is allowed to reselect to lower priority layers, 0 = 0 dB, 1 = 2 dB, 2 = 4 dB, …, 13 = 26 dB, 14 = 28 dB, 15 = (always). |  |  |  |  | 0-15 |  |  |  |  | 4 |  |  |  | PACCH D/L (**) |  |  |  |  |
| THRESH_priority_search |  |  |  |  | A threshold for the serving cell that controls measurement of inter-RAT cells or frequencies of lower priority when the priority-based cell reselection algorithm is used, 0 = -98 dBm, 1 = -95 dBm, 2 = -92 dBm, …, 13 = -59 dBm, 14 = -56 dBm, 15 = (always). |  |  |  |  | 0-15 |  |  |  |  | 4 |  |  |  | PACCH D/L (**) |  |  |  |  |
| GERAN_PRIORITY |  |  |  |  | Priority of GSM cells, 0 = lowest priority, …, 7 = highest priority. |  |  |  |  | 0-7 |  |  |  |  | 3 |  |  |  | PACCH D/L (**) |  |  |  |  |
| UTRAN_QRXLEVMIN |  |  |  |  | Minimum required RX level for cells on the target UTRAN frequency (dBm), 0 = -119 dBm, 1 = -117 dBm, 2 = -115 dBm, …, 30 = -59 dBm, 31 = -57 dBm. Default value = -119 dBm. |  |  |  |  | 0-31 |  |  |  |  | 5 |  |  |  | PACCH D/L (**) |  |  |  |  |
| THRESH_UTRAN_high, THRESH_UTRAN_low |  |  |  |  | Reselection thresholds towards UTRAN FDD or TDD cells, 0 = 0 dB, 1 = 2 dB, 2 = 4 dB, 3 = 6 dB, …, 30 = 60 dB, 31 = 62 dB. Default value of THRESH_UTRAN_low = value of THRESH_UTRAN_high. |  |  |  |  | 0-31 |  |  |  |  | 5 |  |  |  | PACCH D/L (**) |  |  |  |  |
| UTRAN_PRIORITY |  |  |  |  | Priority of a UTRAN frequency layer, 0 = lowest priority, …, 7 = highest priority. |  |  |  |  | 0-7 |  |  |  |  | 3 |  |  |  | PACCH D/L (**) |  |  |  |  |
| E-UTRAN_QRXLEVMIN |  |  |  |  | Minimum required RX level for cells on the target E-UTRAN frequency (dBm), 0 = -140 dBm, 1 = -138 dBm, 2 = -136 dBm, …, 30 = -80 dBm, 31 = -78 dBm. Default value = -140 dBm. |  |  |  |  | 0-31 |  |  |  |  | 5 |  |  |  | PACCH D/L (**) |  |  |  |  |
| THRESH_E-UTRAN_high, THRESH_E-UTRAN_low |  |  |  |  | Reselection thresholds towards E-UTRAN FDD or TDD cells, 0 = 0 dB, 1 = 2 dB, 2 = 4 dB, 3 = 6 dB, …, 30 = 60 dB, 31 = 62 dB. Default value of THRESH_E-UTRAN_low = value of THRESH_E-UTRAN_high. |  |  |  |  | 0-31 |  |  |  |  | 5 |  |  |  | PACCH D/L (**) |  |  |  |  |
| E-UTRAN_PRIORITY |  |  |  |  | Priority of a E-UTRAN frequency layer, 0 = lowest priority, …, 7 = highest priority. |  |  |  |  | 0-7 |  |  |  |  | 3 |  |  |  | PACCH D/L (**) |  |  |  |  |
| H_PRIO |  |  |  |  | Hysteresis used in the priority reselection algorithm, 0 = (rule disabled), 1 = 5 dB, 2 = 4 dB, 3 = 3 dB. |  |  |  |  | 0-3 |  |  |  |  | 2 |  |  |  | PACCH D/L (**) |  |  |  |  |
| T_reselection |  |  |  |  | Time hysteresis in the reselection algorithm, 0 = 5 sec, 1 = 10 sec, 2 = 15 sec, 3 = 20 sec. Default value = 0 (5 sec) |  |  |  |  | 0-3 |  |  |  |  | 2 |  |  |  | PACCH D/L (**) |  |  |  |  |
| E-UTRAN_REP_QUANT |  |  |  |  | Indicates the reporting quantity for E-UTRAN cells, 0 = RSRP, 1 = RSRQ |  |  |  |  | 0/1 |  |  |  |  | 1 |  |  |  | PACCH D/L (**) |  |  |  |  |
| E-UTRAN_FDD_REPORTING_THRESHOLD_2 |  |  |  |  | Reporting threshold for the parameter (RSRP or RSRQ) that is not reported according to E-UTRAN_REP_QUANT. Default value = 0 (disabled) |  |  |  |  | 0-63 |  |  |  |  | 6 |  |  |  | PACCH D/L (**) |  |  |  |  |
| E-UTRAN_TDD_REPORTING_THRESHOLD_2 |  |  |  |  | Reporting threshold for the parameter (RSRP or RSRQ) that is not reported according to E-UTRAN_REP_QUANT. Default value = 0 (disabled) |  |  |  |  | 0-63 |  |  |  |  | 6 |  |  |  | PACCH D/L (**) |  |  |  |  |
| E-UTRAN_FDD_MEASUREMENT_REPORT_OFFSET |  |  |  |  | Parameter used in the calculation of the value of the quantity (RSRP or RSRQ) that is reported according to E-UTRAN_REP_QUANT, when 3-bit reporting is used. For RSRP the mapping is as follows: 0 = -140 dBm, 1 = -139 dBm, 2 = -138 dBm, …, 62 = -78 dBm, 63 = -77 dBm. For RSRQ the mapping is as follows: 0 = -19.5 dB, 1 = -19 dB, 2 = -18.5 dB, …, 31 = -4 dB, 32 = -3.5 dB, 33 = -3 dB. Default value = 0. |  |  |  |  | 0-63 |  |  |  |  | 6 |  |  |  | PACCH D/L (**) |  |  |  |  |
| E-UTRAN_TDD_MEASUREMENT_REPORT_OFFSET |  |  |  |  | Parameter used in the calculation of the value of the quantity (RSRP or RSRQ) that is reported according to E-UTRAN_REP_QUANT, when 3-bit reporting is used. For RSRP the mapping is as follows: 0 = -140 dBm, 1 = -139 dBm, 2 = -138 dBm, …, 62 = -78 dBm, 63 = -77 dBm. For RSRQ the mapping is as follows: 0 = -19.5 dB, 1 = -19 dB, 2 = -18.5 dB, …, 31 = -4 dB, 32 = -3.5 dB, 33 = -3 dB. Default value = 0. |  |  |  |  | 0-63 |  |  |  |  | 6 |  |  |  | PACCH D/L (**) |  |  |  |  |
| REPORTING_GRANULARITY |  |  |  |  | Signals whether fine or coarse granularity is used in 3-bit reporting. For RSRP: 0 = 2 dB step size, 1 = 3 dB step size; for RSRQ: 0 = 1 dB step size, 1 = 2 dB step size). Default value = 0. |  |  |  |  | 0/1 |  |  |  |  | 1 |  |  |  | PACCH D/L (**) |  |  |  |
| Qsearch_P_E-UTRAN |  |  |  |  | Search for E-UTRAN cells if signal level below threshold (0-7): - 98, - 94, … , - 74 dBm, (always) or above threshold (8-15): - 78, - 74, … , - 54 dBm, (never). Default value = (never). |  |  |  |  | 0-15 |  |  |  |  | 4 |  |  |  |  | PACCH D/L (**) |  |  |  |
| Measurement_Control_E-UTRAN |  |  |  |  | Frequency-specific search enabled 0 = never search 1 = use Qsearch_P_E-UTRAN |  |  |  |  | 0/1 |  |  |  |  | 1 |  |  |  |  | PACCH D/L |  |  |  |
| Measurement_Control_UTRAN |  |  |  |  | Frequency-specific search enabled 0 = never search 1 = use Qsearch_P |  |  |  |  | 0/1 |  |  |  |  | 1 |  |  |  |  | PACCH D/L |  |  |  |
| E-UTRAN_Qmin |  |  |  |  | Minimum required quality for cells on the target E-UTRAN frequency (dB), 0= -34dB, 1= -32dB, 2= -30dB, …, 14= -6dB, 15= -4dB. Default value= -34dB. |  |  |  |  | 0-15 |  |  |  |  | 4 |  |  |  |  | PACCH D/L |  |  |  |
| E-UTRAN_RSRPmin |  |  |  |  | Minimum required RX level for cells on the target E-UTRAN frequency (dBm), 0 = -140 dBm, 1 = -138 dBm, 2 = -136 dBm, …, 30 = -80 dBm, 31 = -78 dBm. Default value = -140 dBm. |  |  |  |  | 0-31 |  |  |  |  | 5 |  |  |  |  | PACCH D/L |  |  |  |
| E-UTRAN_QQUALMIN |  |  |  |  | Minimum required quality for cells on the target E-UTRAN frequency (dB), 0= -34dB, 1= -32dB, 2= -30dB, …, 14= -6dB, 15= -4dB. Default value= -34dB. |  |  |  |  | 0-15 |  |  |  |  | 4 |  |  |  |  | PACCH D/L |  |  |  |
| THRESH_E-UTRAN_high_Q, THRESH_E-UTRAN_low_Q |  |  |  |  | Reselection thresholds towards E-UTRAN FDD or TDD cells, 0 = 0 dB, 1 = 1 dB, 2 = 2 dB, 3 = 3 dB, …, 30 = 30 dB, 31 = 31 dB. Default value of THRESH_E-UTRAN_low_Q = value of THRESH_E-UTRAN_high_Q. |  |  |  |  | 0-31 |  |  |  |  | 5 |  |  |  |  | PACCH D/L |  |  |  |
| UTRAN_CSG_FDD_REPORTING_THRESHOLD |  |  |  |  | Reporting threshold for the reported value (Ec/No) for UTRAN FDD CSG cells, 0, 6, … , 36, (never). Default value = same as the value of FDD_REPORTING_THRESHOLD if signalled and if FDD_REP_QUANT = 1; otherwise 4 (-12.5 dB ≤ CPICH Ec/Io < -12 dB). |  |  |  |  | 0-7 |  |  |  |  | 3 |  |  |  |  | PACCH D/L (**) |  |  |  |
| UTRAN_CSG_FDD_REPORTING_THRESHOLD_2 |  |  |  |  | Reporting threshold for the non-reported value (RSCP) for UTRAN FDD CSG cells, Default value = same as the value of FDD_REPORTING_THRESHOLD_2 if signalled and if FDD_REP_QUANT = 1; otherwise 6 (-110 dBm ≤ CPICH RSCP < -109 dBm). |  |  |  |  | 0-63 |  |  |  |  | 6 |  |  |  |  | PACCH D/L (**) |  |  |  |
| UTRAN_CSG_TDD_REPORTING_THRESHOLD |  |  |  |  | Reporting threshold for the reported value (RSCP) for UTRAN TDD CSG cells, 0, 6, … , 36, (never). Default value = TDD_REPORTING_THRESHOLD if signalled; otherwise 0. |  |  |  |  | 0-7 |  |  |  |  | 3 |  |  |  |  | PACCH D/L (**) |  |  |  |
| E-UTRAN_CSG_FDD_REPORTING_THRESHOLD |  |  |  |  | Reporting threshold for the reported value (RSRQ) for E-UTRAN FDD CSG cells, 0, 6, … , 36, (never). Default value = same as the value of E-UTRAN_FDD_REPORTING_THRESHOLD if signalled and if E-UTRAN_REP_QUANT = 1; otherwise 3 (-11.0 dB ≤ RSRQ < -10.5 dB) . |  |  |  |  | 0-7 |  |  |  |  | 3 |  |  |  |  | PACCH D/L (**) |  |  |  |
| E-UTRAN_CSG_FDD_REPORTING_THRESHOLD_2 |  |  |  |  | Reporting threshold for the non-reported value (RSRP) for E-UTRAN FDD CSG cells. Default value = same as the value of E-UTRAN_FDD_REPORTING_THRESHOLD_2 if signalled and if E-UTRAN_REP_QUANT = 1; otherwise 11 (-120 dBm ≤ RSRP < -118 dBm). |  |  |  |  | 0-63 |  |  |  |  | 6 |  |  |  |  | PACCH D/L (**) |  |  |  |
| E-UTRAN_CSG_TDD_REPORTING_THRESHOLD |  |  |  |  | Reporting threshold for the reported value (RSRQ) for E-UTRAN TDD CSG cells, 0, 6, … , 36, (never). Default value = same as the value of E-UTRAN_TDD_REPORTING_THRESHOLD if signalled and if E-UTRAN_REP_QUANT = 1; otherwise 3 (-11.0 dB ≤ RSRQ < -10.5 dB). |  |  |  |  | 0-7 |  |  |  |  | 3 |  |  |  |  | PACCH D/L (**) |  |  |  |
| E-UTRAN_CSG_TDD_REPORTING_THRESHOLD_2 |  |  |  |  | Reporting threshold for the non-reported value (RSRP) for E-UTRAN TDD CSG cells, Default value = same as the value of E-UTRAN_TDD_REPORTING_THRESHOLD_2 if signalled and if E-UTRAN_REP_QUANT = 1; otherwise 11 (-120 dBm ≤ RSRP < -118 dBm). |  |  |  |  | 0-63 |  |  |  |  | 6 |  |  |  |  | PACCH D/L (**) |  |  |  |

NOTE:	(**) These parameters occur also on BCCH if PBCCH does not exist.

NOTE 1:	If PBCCH does not exist, the MS shall perform cell re-selection according to the idle mode procedures defined in clause 6 (i.e. use parameter Qsearch_I). In case parameter Qsearch_P is broadcast on BCCH it shall be used according to clause 10 only if GPRS cell re-selection parameters for one or more cells are provided to the MS in a Packet Cell Change Order or Packet Measurement Order message.

NOTE 2:	In case an optional parameter is not included in a point-to-point signalling message, the default value of that parameter shall replace any previously broadcast value, where applicable.

Table 3a: Conversion from idle mode to GPRS cell re-selection parameters

| GPRS Parameter name | Conversion from idle mode parameters |
| --- | --- |
| GPRS_RXLEV_ACCESS_MIN | RXLEV_ACCESS_MIN |
| GPRS_MS_TXPWR_MAX_CCH | MS_TXPWR_MAX_CCH |
| C31 | 0 |
| GPRS_RESELECT_OFFSET(n) | CELL_RESELECT_OFFSET(n) - CELL_RESELECT_OFFSET(s) |
| GPRS_TEMPORARY_OFFSET | TEMPORARY OFFSET |
| GPRS_PENALTY_TIME | PENALTY_TIME |
| PRIORITY_CLASS | 0 |
| C31_HYST | 0 |
| C32_QUAL | 0 |
| GPRS_CELL_RESELECT_HYSTERESIS | CELL_RESELECT_HYSTERESIS |
| RA_RESELECT_HYSTERESIS | CELL_RESELECT_HYSTERESIS |
| XXX_GPRS_Qoffset | XXX_Qoffset |
| CELL_BAR_ACCESS_2 | CELL_BAR_ACCESS |
| EXC_ACC | cell exclusive access support capability |

NOTE:	If PENALTY_TIME = 11111 for a cell, the sign of CELL_RESELECT_OFFSET shall be changed and TEMPORARY OFFSET set to 0 for that cell.

Table 3b: Parameters affecting cell priority for cell selection and re-selection

| CELL_BAR ACCESS_2 | Cell selection priority | Status for cell reselection |
| --- | --- | --- |
| 0 | Normal | Normal |
| 1 | Barred | Barred |

NOTE 1:	A low priority cell is only selected if there are no suitable cells of normal priority (see 3GPP TS 43.022).

The conversion of the PENALTY_TIME into the GPRS_PENALTY_TIME shall be done by use of the timer values (e.g. PENALTY_TIME = 40s -> GPRS_PENALTY_TIME 2 = 40s). Similarly, dB values shall be used for GPRS_RESELECT_OFFSET(n) conversion.

End of modifications