3GPP TSG-RAN WG6 meeting #14	R6-190084

# Electronic Agreement Meeting (30th Sep – 28th Oct, 2019)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.331 | CR | 5961 | rev | 1 | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of NR SRVCC to UTRAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SRVCC_NR_to_UMTS-Core |  |  |  |  |  | Date: |  |  | 2019-10-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the incoming LS R6-190076, RAN2 inform RAN6 of the progress on NR SRVCC to UTRAN. Some impacts on RAN6 are needed in order to complete the feature. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes are made: - Add 38.331 and 33.501 as new references - Add NR as new abbreviation - In inter-RAT handover to UTRAN sections, it is clarifed that SRVCC from NR is supported, and UE behaviours on key derivation description is also captured |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR SRVCC to UTRAN is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.2, 8.3.6.1, 8.3.6.2, 8.3.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 25.300 CR 0060 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | This CR is for Rel-16 feature NR SRVCC to UTRAN. It is based on 15.4.0 as Rel-16 version is not available. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 25.301: "Radio Interface Protocol Architecture".

[3]	3GPP TS 25.303: "Interlayer Procedures in Connected Mode".

[4]	3GPP TS 25.304: "UE Procedures in Idle Mode and Procedures for Cell Reselection in Connected Mode".

[5]	3GPP TS 24.008: "Mobile radio interface layer 3 specification; Core Network Protocols; Stage 3".

[6]	3GPP TS 25.103: "RF parameters in support of RRM".

[7]	3GPP TS 25.215: "Physical layer – Measurements (FDD)".

[8]	3GPP TS 25.225: "Physical layer – Measurements (TDD)".

[9]	3GPP TS 25.401: "UTRAN overall description".

[10]	3GPP TS 25.402: "Synchronization in UTRAN; Stage 2".

[11]	3GPP TS 23.003: "Numbering, addressing and identification".

[12]	ICD-GPS-200: "Navstar GPS Space Segment/Navigation User Interface".

[13]	RTCM-SC104: "RTCM Recommended Standards for Differential GNSS Service (v.2.2)".

[14]	3GPP TR 25.921: "Guidelines and principles for protocol description and error handling".

[15]	3GPP TS 25.321: "Medium Access Control (MAC) protocol specification".

[16]	3GPP TS 25.322: "Radio Link Control (RLC) protocol specification".

[17]	3GPP TS 24.007: "Mobile radio interface signalling layer 3; General aspects".

[18]	3GPP TS 25.305: "Stage 2 Functional Specification of UE Positioning in UTRAN".

[19]	3GPP TS 25.133: "Requirements for Support of Radio Resource Management (FDD)".

[20]	3GPP TS 25.123: "Requirements for Support of Radio Resource Management (TDD)".

[21]	3GPP TS 25.101: "UE Radio Transmission and Reception (FDD)".

[22]	3GPP TS 25.102: "UE Radio Transmission and Reception (TDD)".

[23]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[24]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[25]	3GPP TS 23.122: "Non-Access-Stratum functions related to Mobile Station (MS) in idle mode".

[26]	3GPP TS 25.211: "Physical channels and mapping of transport channels onto physical channels (FDD)".

[27]	3GPP TS 25.212: "Multiplexing and channel coding (FDD)".

[28]	3GPP TS 25.213: "Spreading and modulation (FDD)".

[29]	3GPP TS 25.214: "Physical layer procedures (FDD)".

[30]	3GPP TS 25.221: "Physical channels and mapping of transport channels onto physical channels (TDD)".

[31]	3GPP TS 25.222: "Multiplexing and channel coding (TDD)".

[32]	3GPP TS 25.223: "Spreading and modulation (TDD)".

[33]	3GPP TS 25.224: "Physical Layer Procedures (TDD)".

[34]	3GPP TS 25.302: "Services provided by the physical layer ".

[35]	3GPP TS 25.306 "UE Radio Access Capabilities".

[36]	3GPP TS 25.323: "Packet Data Convergence Protocol (PDCP) Specification".

[37]	3GPP TS 25.324: "Broadcast/Multicast Control BMC".

[38]	3GPP TR 25.922: "Radio resource management strategies".

[39]	3GPP TR 25.925: "Radio interface for broadcast/multicast services".

[40]	3GPP TS 33.102: "3G Security; Security Architecture".

[41]	3GPP TS 34.108: "Common Test Environments for User Equipment (UE) Conformance Testing".

[42]	3GPP TS 34.123-2: "User Equipment (UE) conformance specification; Part 2: Implementation Conformance Statement (ICS) proforma specification".

[43]	3GPP TS 44.018: "Mobile radio interface layer 3 specification; Radio Resource Control Protocol".

[44]	3GPP TS 44.060: "General Packet Radio Service (GPRS); Mobile Station (MS) - Base Station System (BSS) interface; Radio Link Control/Medium Access Control (RLC/MAC) protocol".

[45]	3GPP TS 45.005: "Radio transmission and reception".

[46]	3GPP TS 45.008: "Radio subsystem link control".

[47]	ITU-T Recommendation X.680 (07/2002) "Information Technology - Abstract Syntax Notation One (ASN.1): Specification of basic notation".

[48]	ITU-T Recommendation X.681 (07/2002) "Information Technology - Abstract Syntax Notation One (ASN.1): Information object specification".

[49]	ITU-T Recommendation X.691 (07/2002) "Information technology - ASN.1 encoding rules: Specification of Packed Encoding Rules (PER)".

[50]	3GPP TS 31.102: "Characteristics of the USIM Application".

[51]	3GPP TS 25.308: "High Speed Downlink Packet Access (HSDPA): Overall Description; Stage 2".

[52]	IANA ROHC profile identifier definition (http://www.iana.org/assignments/rohc-pro-ids).

[53]	Void.

[54]	3GPP TS 23.246: "Multimedia Broadcast Multicast Service; Architecture and Functional Description".

[55]	3GPP TS 25.346: " Introduction of the Multimedia Broadcast Multicast Service (MBMS) in the Radio Access Network (Stage-2)".

[56]	3GPP TR 25.992: "Multimedia Broadcast Multicast Service (MBMS); UTRAN/GERAN Requirements".

[57]	3GPP TS 25.413: "UTRAN Iu Interface RANAP Signalling".

[58]	3GPP TS 25.309: "FDD Enhanced Uplink; Overall Description; Stage 2".

[59]	3GPP TS 25.319: "Enhanced Uplink; Overall Description; Stage 2".

[60]	3GPP TR 25.827: "1.28Mcps TDD Enhanced Uplink; Physical Layer Aspects".

[61]	Galileo OS Signal in Space ICD (OS SIS ICD), Issue 1.2,  February 2014, European Union.

[62]	3GPP TS 26.101: "Adaptive Multi-Rate (AMR) speech codec frame structure".

[63]	3GPP TS 26.201: "Adaptive Multi Rate – Wideband (AMR-WB) speech codec frame structure".

[64]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

[65]	3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception".

[66]	3GPP TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode".

[67]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol Specification".

[68]	IS-GPS-200, Revision D, Navstar GPS Space Segment/Navigation User Interfaces, March 7th, 2006.

[69]	IS-GPS-705, Navstar GPS Space Segment/User Segment L5 Interfaces, September 22, 2005.

[70]	IS-GPS-800, Navstar GPS Space Segment/User Segment L1C Interfaces, March 31, 2008.

[71]	Specification for the Wide Area Augmentation System (WAAS), US Department of Transportation, Federal Aviation Administration, DTFA01-96-C-00025, 2001.

[72]	IS-QZSS, Quasi Zenith Satellite System Navigation Service Interface Specifications for QZSS, Ver.1.0, June 17, 2008.

[73]	Global Navigation Satellite System GLONASS Interface Control Document, Version 5.1, 2008.

[74]	3GPP TS 36.133: "Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management".

[75]	3GPP TS 22.011: "Service accessibility".

[76]	3GPP TS 23.003: "Numbering, addressing and identification".

[77]	3GPP TS 23.041: "Technical realization of Cell Broadcast Service (CBS)".

[78]	3GPP TS 33.401: "3GPP System Architecture Evolution (SAE): Security architecture".

[79]	3GPP TS 24.301: " Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS)".

[80]	3GPP TS 37.320: "Universal Terrestrial Radio Access (UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRA); Radio measurement collection for Minimization of Drive Tests (MDT); Overall description; Stage 2".

[81]	3GPP TS 32.422: "Telecommunication management; Subsriber and equipment trace; Trace control and confiuration management".

[82]	3GPP TS 22.368: "Service Requirements for Machine Type Communications; Stage 1".

[83]	RFC 3095: "RObust Header Compression (ROHC): Framework and four profiles".

[84]	RFC 4815: "RObust Header Compression (ROHC): Corrections and Clarifications to RFC 3095".

[85]	3GPP TS 25.307: "Requirement on User Equipments (UEs) supporting a release-independent frequency band".

[86]	BDS-SIS-ICD-2.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal (Version 2.0), December 2013".

[87]	IEEE 802.11-2012, Part 11: "Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) specifications, IEEE Std.".

[88]	3GPP TS 24.312: "Access Network Discovery and Selection Function (ANDSF) Management Object (MO)".

[89]	ATIS-0500027: "Recommendations for Establishing Wide Scale Indoor Location Performance,"  May 2015.

[90]	Bluetooth Special Interest Group: "Bluetooth Core Specification v4.2", December 2014.

[91]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[xx]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[xy]	3GPP TS 33.501: "Security Architecture and Procedures for 5G System".

3	Definitions and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in [1] apply, together as:

Activated uplink frequency (FDD only): For a specific UE, an uplink frequency is said to be activated if the UE is allowed to transmit on that frequency. The primary uplink frequency is always activated when configured while a secondary uplink frequency has to be activated by means of an HS-SCCH order in order to become activated.

Adjacent Frequency: A frequency whose centre is within 5 MHz of the centre of the currently used frequency and belongs to the same frequency band as that of the currently used frequency.

Assisting serving HS-DSCH Cell: In addition to the serving HS-DSCH cell, a cell in the same frequency, where the UE is configured to simultaneously monitor a HS-SCCH set and receive HS-DSCH if it is scheduled in that cell.

Assisting secondary serving HS-DSCH Cell: In addition to the serving HS-DSCH cell, a cell in the secondary downlink frequency, where the UE is configured to simultaneously monitor a HS-SCCH set and receive HS-DSCH if it is scheduled in that cell.

Blind HARQ retransmissions for HSDPA (FDD only): A HARQ operation where a UE in CELL_DCH state supports reception of  MAC-hs/ehs PDU within 5 sub-frames from the reception of the previous MAC-hs/ehs PDU intended for the same HARQ process.

Common E-DCH resource: In FDD, common E-DCH resources are under direct control of the Node B and are shared by UEs in CELL_FACH state and IDLE mode. In 1.28 Mcps TDD, common E-DCH resources used by UEs in CELL_FACH and IDLE mode are under direct control of Node B and are shared by UEs in the CELL_DCH state.

Common E-RGCH RL: Cell which is not the serving E-DCH radio link and from which the UE can receive one Relative Grant in CELL_FACH state. The UE can have zero, one or up to 3 Common E-RGCH RL(s).

Configured Uplink Frequency (FDD only): For a specific UE, an uplink frequency is said to be configured if the UE has received all relevant information from RRC in order to perform transmission on that frequency.

CSG member cell: a cell broadcasting the identity of the selected PLMN, registered PLMN or equivalent PLMN and for which the CSG whitelist of the UE includes an entry comprising the cell’s CSG ID and the respective PLMN identity.

Current Frequency (FDD only): The frequency that is currently used by the UE. In case of dual cell or dual band configured, the serving HS-DSCH cell frequency is treated as current frequency.

Data Description Indicator (DDI): MAC-e header field used to identify the logical channel, MAC-d flow and the size of the MAC-d PDUs concatenated into a MAC-es PDU.

Designated non-serving HS-DSCH cell: The non-serving HS-DSCH cell from which the UE estimates the F-DPCH quality for TPC derivation, when the UE is configured with DPCCH2 in the uplink or the Serving E-DCH cell decoupling is configured. This TPC is transmitted on UL DPCCH.

E-DCH active set (FDD only): The set of cells on the primary uplink frequency which carry the E-DCH for one UE. Only radio links for which an E-HICH configuration is stored are considered part of the E-DCH active set.

Enhanced Uplink in CELL_FACH state and Idle mode: In FDD, combines the REL99 random access power ramping phase with E-DCH transmission. The procedure can be started in idle mode and RRC Connected mode, but not in CELL_DCH state. In 1.28 Mcps TDD, the REL7 enhanced random access procedure for E-DCH in CELL_DCH is used in idle mode and CELL_FACH state.

HARQ profile: One HARQ profile consists of a power offset attribute and maximum number of transmissions.

Implicit Grant handling: A scheduling scheme where a UE’s Scheduling Grant on the Secondary Serving E-DCH cell may be revoked by means of an Absolute Grant addressed to another UE.

MBMS activated services: the MBMS multicast services the UE has joined as well as the broadcast services the UE is interested in.

MCCH acquisition: the procedure for acquiring all MCCH information relevant for the UE ie. includes reception of the RB information for the UE’s MBMS activated services.

MBMS controlling cell: the cell from which the UE receives MCCH.

MBMS notification: a notification provided by UTRAN indicating a change in the provision of one or more MBMS services.

MBMS Selected Services: a subset of the MBMS activated services of the Broadcast type for which the UE applies RRC procedures to inform UTRAN that the service has been selected (by upper layers).

MBMS transmission: a possibly repeated session of an MBMS service. An MBMS transmission is uniquely identified by the combination of the MBMS service identity and the MBMS session identity.

MBSFN cluster: Set of cells operating in MBSFN mode providing only MBMS service in PtM mode and seen as one cell by a UE.

MBSFN mode: Transmission mode where a set of synchronized cells transmit exactly the same data for provision of MBMS service.

Multiflow (FDD only):  A mode of operation with two simultaneous HS-DSCH transport channels per carrier frequency, where the HS-DSCH transport channels may belong to the same or different Node Bs.

NodeB triggered HS-DPCCH transmission: Transmission on a common E-DCH resource to provide HS-DPCCH feedback in CELL_FACH state, which is triggered by the reception of an HS-SCCH order as defined in [16].

Primary Absolute Grant (FDD only): Absolute Grant received with the primary E-RNTI.

Primary uplink frequency (FDD only): If a single uplink frequency is configured for the UE, then it is the primary uplink frequency. In case more than one uplink frequencies are configured for the UE, then the primary uplink frequency is the frequency on which E-DCH corresponding to the serving E-DCH cell associated with  the serving HS-DSCH cell is transmitted. The association between a pair of uplink and downlink frequencies is indicated by higher layers.

Radio Links without DPCH/F-DPCH operation (FDD only): An operation mode in which UE supports to not receive both DPCH and F-DPCH downlink channels from the indicated Non-serving E-DCH cell(s).

Secondary Absolute Grant (FDD only): Absolute Grant received with the secondary E-RNTI.

Secondary E-DCH Active Set (FDD only): The set of cells on the secondary uplink frequency where E-DCH is carried for one UE. Only radio links for which an E-HICH configuration is stored are considered part of the secondary E-DCH active set. The secondary E-DCH active set and the active set on the secondary uplink frequency are always the same set of cells.

Secondary Serving E-DCH cell (FDD only): Cell from which the UE receives Absolute Grants from the Node-B scheduler on the downlink frequency associated with secondary uplink frequency. A UE has one Secondary Serving E-DCH cell on the secondary uplink frequency.

Secondary Serving E-DCH RLS or Secondary Serving RLS (FDD only): In Dual Cell E-DCH operation, the set of cells which contains at least the Secondary Serving E-DCH cell and from which the UE can receive and combine one Relative Grant. A UE can have zero or one Secondary Serving E-DCH RLS.

Secondary serving HS-DSCH cell(s): In addition to the serving HS-DSCH cell, the set of cells where the UE is configured to simultaneously monitor an HS-SCCH set and receive the HS-DSCH if it is scheduled in that cell. There can be up to 7 secondary serving HS-DSCH cells. If the UE is configured with two uplink frequencies, the first secondary serving HS-DSCH cell is the secondary serving HS-DSCH cell that is associated with the secondary uplink frequency.

Secondary Non-serving E-DCH RL or Secondary Non-serving RL (FDD only): In Dual Cell E-DCH operation, the cell which belongs to the Secondary E-DCH active set but does not belong to the Secondary Serving E-DCH RLS and from which the UE in CELL_DCH can receive one Relative Grant. The UE can have zero, one or several Secondary Non-serving E-DCH RL(s).

Secondary uplink frequency (FDD only): A secondary uplink frequency is a frequency on which an E-DCH corresponding to a  serving E-DCH cell associated with a secondary serving HS-DSCH cell is transmitted. The association between a pair of uplink and downlink frequencies is indicated by higher layers.

Serving E-DCH cell: Cell from which the UE receives Absolute Grants from the Node-B scheduler.  In TDD, a UE has one Serving E-DCH cell. In FDD, a UE has one Serving E-DCH cell on the primary uplink frequency.

Serving E-DCH cell decoupling (FDD only): An E-DCH operation mode in which the Serving HS-DSCH cell and the Serving E-DCH cell are different.

Serving E-DCH RLS or Serving RLS (FDD only): Set of cells which contains at least the Serving E-DCH cell and from which the UE can receive and combine one Relative Grant. In TDD, the UE has only one Serving E-DCH RLS. In FDD, a UE has one Serving E-DCH cell on the primary uplink frequency.

Non-serving E-DCH RL or Non-serving RL (FDD only): Cell which belongs to the E-DCH active set but does not belong to the Serving E-DCH RLS and from which the UE in CELL_DCH can receive one Relative Grant. In TDD, the UE can have zero, one or several Non-serving E-DCH RL(s). In FDD, the UE can have zero, one or several Non-serving E-DCH RL(s) on the primary uplink frequency.

SR-VCC: Single Radio Voice Call Continuity, Voice call or Voice and Video call continuity between IMS over PS access and CS access for calls that are anchored in IMS when the UE is capable of transmitting/receiving on only one of those access networks at a given time. Single Radio Voice Call Continuity between CS access and IMS over PS access is referred to as Reverse SRVCC (rSR-VCC).

Target Cell Pre-Configuration (FDD only): HS-DSCH Cell configuration provided to UE in Active Set Update for which HS-DSCH serving Cell Change may be initiated by HS-SCCH order sent from target cell.

Target cell HS-SCCH order: A HS-DSCH serving cell change command signaled to an UE by using HS-SCCH order in the target cell:

- for which a measurement report was trigged by an Event 1d  intra frequency event; or

- for which a measurement report was trigged by an Event 1c intra frequency event on the primary frequency, which indicates the current serving HS-DSCH cell as the only active CPICH triggering this event 1c report, and the target cell is the best cell in the current active set reported in this Event 1c measurement report. If more than one cell in the active set share the same quality, the target cell will be the first one listed in the in "Intra-frequency measured results list" in the Event 1c measurement report.

Primary E-DCH carrier (1.28Mcps TDD only): If a single E-DCH carrier is configured for the UE, then it is the primary E-DCH carrier. In case more than one E-DCH carriers are configured for the UE, then the primary E-DCH carrier is the carrier on which the E-RUCCH will be transmitted.

Additional E-DCH carrier (1.28Mcps TDD only): An additional E-DCH carrier is an E-DCH carrier other than the primary E-DCH carrier . The UE can have zero, one or several additional E-DCH carrier(s).

3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

ACDC	Application specific Congestion control for Data Communication

ACK	Acknowledgement

AG	Absolute Grant

AICH	Acquisition Indicator CHannel

AM	Acknowledged Mode

ANDSF	Access Network Discovery and Selection Function

ANR	Automatic Neighbour Relation

AP	Access Point

AS	Access Stratum

ASC	Access Service Class

ASN.1	Abstract Syntax Notation.1

BCCH	Broadcast Control Channel

BCD	Binary Coded Decimal

BCFE	Broadcast Control Functional Entity

BDS	BeiDou Navigation Satellite System

BER	Bit Error Rate

BLER	BLock Error Rate

BSS	Base Station Sub-system

CCCH	Common Control Channel

CCPCH	Common Control Physical CHannel

CH	Conditional on history

CLTD	Closed Loop Transmit Diversity

CM	Connection Management

CN	Core Network

C-RNTI	Cell RNTI

CSFB	CS Fallback

CSG	Closed Subscriber Group

CTCH	Common Traffic CHannel

CTFC	Calculated Transport Format Combination

CV	Conditional on value

DBDS	Differential BDS

DCA	Dynamic Channel Allocation

DCCH	Dedicated Control Channel

DCFE	Dedicated Control Functional Entity

DCH	Dedicated Channel

DCN	Dedicated Core Networks

DC-SAP	Dedicated Control SAP

DDI	Data Description Indicator

DGANSS	Differential GANSS

DGPS	Differential Global Positioning System

DL	Downlink

DPCCH2	Dedicated Physical Control Channel 2

DSCH	Downlink Shared Channel

DTCH	Dedicated Traffic Channel

DTM	Dual Transfer Mode

EAB	Extended Access Barring

E-AGCH	E-DCH Absolute Grant Channel

ECEF	Earth-Centered, Earth-Fixed

ECI	Earth-Centered-Inertial

E-DCH	Enhanced uplink DCH

E-DPCCH	E-DCH Dedicated Physical Control Channel (FDD Only)

E-DPDCH	E-DCH Dedicated Physical Data Channel (FDD Only)

EGNOS	European Geostationary Navigation Overlay Service

E-HICH	E-DCH HARQ Acknowledgement Indicator Channel

E-PUCH	Enhanced Uplink Physical Channel (TDD only)

E-RGCH	E-DCH Relative Grant Channel (FDD only)

E-RNTI	E-DCH RNTI

E-ROCH	E-DCH Rank and Offset Channel (FDD only)

E-RUCCH	E-DCH Random Access Uplink Control Channel (TDD only)

E-TFCI	E-DCH Transport Format Combination Indicator

ETWS	Earthquake and Tsunami Warning System

E-UCCH	E-DCH Uplink Control Channel (TDD only)

E-UTRA	Evolved Universal Terrestrial Radio Access

E-UTRAN	Evolved Universal Terrestrial Radio Access Network

FACH	Forward Access Channel

FDD	Frequency Division Duplex

F-DPCH	Fractional DPCH

F-TPICH	Fractional Transmitted Precoding Indicator Channel

GAGAN	GPS Aided Geo Augmented Navigation

GANSS	Galileo and Additional Navigation Satellite Systems

GC-SAP	General Control SAP

GERAN	GSM/EDGE Radio Access Network

GLONASS	GLObal'naya NAvigatsionnaya Sputnikovaya Sistema (Engl.: Global Navigation Satellite System)

GNSS	Global Navigation Satellite System

GRA	GERAN Registration Area

G-RNTI	Geran Radio Network Temporary Identity

HARQ	Hybrid Automatic Repeat Request

HCS	Hierarchical Cell Structure

HFN	Hyper Frame Number

H-RNTI	HS-DSCH RNTI

HS-DSCH	High Speed Downlink Shared Channel

ICD	Interface Control Document

ID	Identifier

IDNNS	Intra Domain NAS Node Selector

IE	Information element

IETF	Internet Engineering Task Force

IMB	Integrated Mobile Broadcast

IMEI	International Mobile Equipment Identity

IMSI	International Mobile Subscriber Identity

IP	Internet Protocol

ISCP	Interference on Signal Code Power

L1	Layer 1

L2	Layer 2

L3	Layer 3

LAI	Location Area Identity

MAC	Media Access Control

MBS	Metropolitan Beacon System

MBMS	Multimedia Broadcast Multicast Service

MBSFN	MBMS over a Single Frequency Network

MCC	Mobile Country Code

MCCH	MBMS point-to-multipoint Control Channel

MD	Mandatory default

MDT	Minimization of Drive Tests

MICH	MBMS notification Indicator Channel

MM	Mobility Management

MNC	Mobile Network Code

MP	Mandatory present

MTCH		MBMS point-to-multipoint Traffic Channel

MSAS	Multi-functional Satellite Augmentation System

MSCH	MBMS point-to-multipoint Scheduling Channel

NACC	Network Assisted Cell Change

NAS	Non Access Stratum

NR	NR Radio Access

Nt-SAP	Notification SAP

NW	Network

OLTD	Open Loop Transmit Diversity

OP	Optional

PCCH	Paging Control Channel

PCH	Paging Channel

PDCP	Packet Data Convergence Protocol

PDSCH	Physical Downlink Shared Channel

PDU	Protocol Data Unit

PLMN	Public Land Mobile Network

PNFE	Paging and Notification Control Functional Entity

PRACH	Physical Random Access CHannel

PRN	Pseudo-Random Noise

PSI	Packet System Information

p-t-m	Point-to-Multipoint

P-TMSI	Packet Temporary Mobile Subscriber Identity

p-t-p	Point-to-Point

PUSCH	Physical Uplink Shared Channel

QoE	Quality of Experience

QoS	Quality of Service

QZSS	Quasi-Zenith Satellite System

RAB	Radio access bearer

RACH	Random Access CHannel

RAI	Routing Area Identity

RAT	Radio Access Technology

RB	Radio Bearer

RFE	Routing Functional Entity

RG	Relative Grant

RL	Radio Link

RLC	Radio Link Control

RLS	Radio Link Set

RNC	Radio Network Controller

ROHC	RObust Header Compression

RNTI	Radio Network Temporary Identifier

RRC	Radio Resource Control

RSCP	Received Signal Code Power

RSN	Retransmission Sequence Number

RSRP	Reference Signal Received Power

RSRQ	Reference Signal Received Quality

RSSI	Received Signal Strength Indicator

SAP	Service Access Point

SBAS	Satellite Based Augmentation System

SCFE	Shared Control Function Entity

SCTD	Space Code Transmit Diversity

SCTO	Soft Combining Timing Offset (MBMS)

S-DPCCH	Secondary Dedicated Physical Control Channel

S-E-DPCCH	Secondary E-DPCCH (FDD only)

S-E-DPDCH	Secondary E-DPDCH (FDD only)

SF	Spreading Factor

SG	Serving grant

SHCCH	Shared Control Channel

SI	System Information

SIR	Signal to Interference Ratio

SPS	Semi-Persistent Scheduling

S-RNTI	SRNC - RNTI

SV		Space Vehicle

TDD	Time Division Duplex

TF	Transport Format

TFCS	Transport Format Combination Set

TFS	Transport Format Set

TM	Transparent Mode

TME	Transfer Mode Entity

TMSI	Temporary Mobile Subscriber Identity

Tr	Transparent

TSN	Transmission Sequence Number

Tx	Transmission

UE	User Equipment

UL	Uplink

UM	Unacknowledged Mode

URA	UTRAN Registration Area

U-RNTI	UTRAN-RNTI

USCH	Uplink Shared Channel

UTC	Universal Coordinated Time

UTRAN	Universal Terrestrial Radio Access Network

WAAS	Wide Area Augmentation System

WGS-84	World Geodetic System 1984

WLAN	Wireless Local Area Network

<Next modification>

8.3.6	Inter-RAT handover to UTRAN

Figure 8.3.6-1: Inter-RAT handover to UTRAN, successful case

8.3.6.1	General

The purpose of the inter-RAT handover procedure is to, under the control of the network, transfer a connection between the UE and another radio access technology (e.g. GSM ,E-UTRA or NR) to UTRAN.

This procedure may be used to perform CS handover which applies when there is a signalling connection to the CS domain.

When the UE supports Inter-RAT PS handover, this procedure may be used to perform PS handover which applies when there is only a signalling connection to the PS domain.

This procedure may be used to perform simultaneous CS and PS handover (i.e. DTM Handover) which applies when there are signalling connections to both CS and PS domains.

This procedure may be used to perform PS to CS handover (i.e. SR-VCC Handover) which applies when there is a signalling connection in E-UTRA or NR PS domain to be transferred to UTRA CS domain. For E-UTRA, SR-VCC handover may be performed with a simultaneous PS handover. For NR, SR-VCC handover can only be performed without a simultaneous PS handover.

This procedure may be used to perform CS to PS handover (i.e. rSR-VCC Handover) which applies when there is a signalling connection in GSM CS domain to be transferred to UTRA PS domain. The rSR-VCC Handover may be performed with a simultaneous PS handover.

8.3.6.2	Initiation

The procedure is initiated when a radio access technology other than UTRAN, e.g. GSM ,E-UTRA or NR, using radio access technology-specific procedures, orders the UE to make a handover to UTRAN.

A Handover to UTRAN command message is sent to the UE via the radio access technology from which inter- RAT handover is performed.

In case UTRAN decides to use a predefined or default radio configuration that is stored in the UE, it should include the following information in the Handover to UTRAN command message.

-	the IE "New U-RNTI" to be assigned;

-	the IE "Predefined configuration identity", to indicate which pre-defined configuration of RB, transport channel and physical channel parameters shall be used; or

-	the IE "Default configuration mode" and IE "Default configuration identity", to indicate which default configuration of RB, transport channel and physical channel parameters shall be used;

-	PhyCH information elements.

NOTE 1:	When using a predefined or default configuration during handover to UTRAN, UTRAN can only assign values of IEs "New U-RNTI" and "scrambling code" that are within the special subranges defined exclusively for this procedure. UTRAN may re- assign other values after completion of the handover procedure.

NOTE 2:	When using a predefined or default configuration during handover to UTRAN, fewer IEs are signalled; when using this signalling option some parameters e.g. concerning compressed mode, DSCH can not be configured. In this case, the corresponding functionality can not be activated immediately.

NOTE 3:	When using a predefined or default configuration, the HANDOVER TO UTRAN COMMAND should not include more than one radio link. If UTRAN includes more than one radio link in the HANDOVER TO UTRAN COMMAND using a predefined or default configuration, the UE behaviour is unspecified.

In case UTRAN does not use a predefined radio configuration that is stored in the UE, it should include the following information in the Handover to UTRAN command message.

-	the IE "New U-RNTI" to be assigned;

-	the complete set of RB, TrCH and PhyCH information elements to be used.

8.3.6.3	Reception of Handover to UTRAN command message by the UE

The UE shall be able to receive a HANDOVER TO UTRAN COMMAND message and perform an inter-RAT handover, even if no prior UE measurements have been performed on the target UTRAN cell and/or frequency.

The UE shall act upon all received information elements as specified in subclause 8.6, unless specified otherwise in the following.

The UE may:

1>	maintain a list of the set of cells to which the UE has Radio Links if the IE "Cell ID" is present.

The UE shall:

1>	store a U-RNTI value (32 bits), which is derived by the IEs "SRNC identity" and "S-RNTI 2" included in IE "U-RNTI-short". In order to produce a full size U-RNTI value, a full size "S-RNTI" (20 bits) shall be derived by padding the IE "S-RNTI 2" with 10 zero bits in the most significant positions; and

1>	if the HANDOVER TO UTRAN COMMAND message is used to perform anything other than CS handover from GERAN and the message does not contain at least one instance of the IE "RAB Info";

2>	the UE behaviour is unspecified.

1>	initialise the variable ESTABLISHED_SIGNALLING_CONNECTIONS with the signalling connections that remains after the handover according to the specifications of the source RAT;

NOTE 0:	The IE "CN domain identity" in IE "RAB Info" in the HANDOVER TO UTRAN COMMAND message determines towards which CN domain  signalling connections are established. The HANDOVER TO UTRAN COMMAND message contains at least one IE "RAB Info" except in the CS domain handover from GERAN case where it may not be present, in which case the UE behaviour is the same as if "CN domain identity" had been received and set to "CS Domain".

1>	initialise the variable UE_CAPABILITIES_TRANSFERRED with the UE capabilities that have been transferred to the network up to the point prior to the handover, if any;

1>	initialise the variable TIMERS_AND_CONSTANTS to the default values and start to use those timer and constants values;

1>	if IE "Specification mode" is set to "Preconfiguration" and IE "Preconfiguration mode" is set to "Predefined configuration":

2>	initiate the radio bearer and transport channel configuration in accordance with the predefined parameters identified by the IE "Predefined configuration identity";

2>	initiate the physical channels in accordance with the predefined parameters identified by the IE "Predefined radio configuration identity" and the received physical channel information elements;

2>	store information about the established radio access bearers and radio bearers according to the IE "Predefined configuration identity"; and

2>	set the IE "RAB Info Post" in the variable ESTABLISHED_RABS and the IE "Re-establishment timer" in the IE "RAB Info" in the variable ESTABLISHED_RABS to "useT314".

1>	if IE "Specification mode" is set to "Preconfiguration" and IE "Preconfiguration mode" is set to "Default configuration":

2>	initiate the radio bearer and transport channel configuration in accordance with the default parameters identified by the IE "Default configuration mode" and IE "Default configuration identity";

2>	initiate the physical channels in accordance with the default parameters identified by the IE "Default configuration mode" and IE "Default configuration identity" and the received physical channel information elements;

NOTE 1:	IE "Default configuration mode" specifies whether the FDD or TDD version of the default configuration shall be used.

2>	set the IE "RAB Info Post" in the variable ESTABLISHED_RABS and the IE "Re-establishment timer" in the IE "RAB Info" in the variable ESTABLISHED_RABS to "useT314".

1>	if IE "Specification mode" is set to "Preconfiguration":

2>	use the following values for parameters that are neither signalled within the HANDOVER TO UTRAN COMMAND message nor included within pre-defined or default configuration:

3>	0 dB for the power offset P Pilot-DPDCH bearer in FDD;

3>	calculate the Default DPCH Offset Value using the following formula:

3>	in FDD:

Default DPCH Offset Value = (SRNTI 2 mod 600) * 512

3>	in TDD:

Default DPCH Offset Value = (SRNTI 2 mod 7)

3>	handle the above Default DPCH Offset Value as if an IE with that value was included in the message, as specified in subclause 8.6.6.14.

1>	if IE "Specification mode" is set to "Complete specification":

2>	initiate the radio bearer, transport channel and physical channel configuration in accordance with the received radio bearer, transport channel and physical channel information elements.

1>	if IE "Default configuration for CELL_FACH" is set:

2>	act in accordance with the default parameters according to section 13.8.

1>	perform an open loop estimation to determine the UL transmission power according to subclause 8.5.3;

1>	set the IE "START" for each CN domain, in the IE "START list" in the HANDOVER TO UTRAN COMPLETE message equal to the START value for each CN domain as stored in the volatile memory of the ME for each CN domain if the USIM or the SIM is present;

NOTE 2:	Keys received while in another RAT (GERAN) or generated when coming from another RAT (EUTRAN) are not regarded as "new" (i.e.do not trigger the actions in subclause 8.1.12.3.1) in a subsequent security mode control procedure in UTRAN, irrespective of whether the keys are already being used in the other RAT or not. If the UE has received new keys in the other RAT before handover, then the START values in the USIM (sent in the HANDOVER TO UTRAN COMPLETE message and in the INTER_RAT_HANDOVER_INFO sent while in the other RAT) will not reflect the receipt of these new keys
If the source RAT is not E-UTRAN and the procedure is not an rSR-VCC handover: If ciphering has been active for the CS domain in the source RAT, then during the first security mode control procedure following the Inter-RAT handover to UTRAN procedure, UE activates integrity protection using the integrity key of the key set used. The term "key set used" denotes the key set that was used in the last successfully completed RRC Security Mode Control (UTRAN), RR Cipher Mode Control procedure or GMM Authentication and Ciphering procedure (GERAN) after entering connected mode in UTRAN or GERAN. The term "if ciphering has been active for the CS domain" is defined by a successfully completed RRC Security Mode Control procedure with "Ciphering Mode Info" (UTRAN) or RR Cipher Mode Control  procedure (GERAN) for the CS domain. More specifically the state of ciphering refers to the establishment of a security context where the key is loaded from the (U)SIM to the ME even if the security command indicates NULL ciphering ("no ciphering" in GERAN or UEA0 in UTRAN).
If the source RAT is GERAN: Upon performing rSR-VCC(CS to PS), then during the first security mode control procedure following the Inter-RAT handover to UTRAN procedure, UE activates integrity protection using the integrity key of the key set as defined in [40].
If the source RAT is E-UTRAN: Upon performing SR-VCC (PS to CS) or PS handover, then during the first security mode control procedure following the Inter-RAT handover to UTRAN procedure, UE activates integrity protection using the integrity key of the key set used (see 8.1.12.2.2.). The term "key set used" denotes the CS (SR-VCC) or PS (PS handover) key set (KSI, CK, IK)  which was mapped using the current SECURITY CONTEXT in EUTRAN as defined in [78] and [79].
If the SR-VCC is from NR (see [xx]), UE derives security keys (e.g. CKSRVCC and IKSRVCC) based on the configurations received from UTRAN command from NR, as specified in [xy].

1>	set the value of "THRESHOLD" in the variable "START_THRESHOLD" to the 20 MSBs of the value stored in the USIM [50] for the maximum value of START for each CN Domain, or to the default value in [40] if the SIM is present;

1>	if ciphering is indicated in the HANDOVER TO UTRAN COMMAND by the presence of the IE "Ciphering Algorithm":

2>	for the handover of CS domain, indicated by the IE "CN domain identity" of the IE "RAB info" of the IE "RAB information to setup" set to "CS domain" or absence of this IE, if ciphering was not active in the radio access technology from which inter-RAT handover is performed:

3>	the UE behaviour is unspecified.

NOTE 2a:	The handover of the CS domain from GERAN to UTRAN is particular because CS ciphering must be maintained across the inter-RAT handover procedure. For other inter-RAT handover procedures (SR-VCC, rSR-VCC and PS handover) ciphering in UTRAN is not dependent on previous ciphering status in the source radio access technology.

2>	if the handover is from EUTRAN and there is no current SECURITY CONTEXT in EUTRAN:

3>	the UE behaviour is unspecified.

NOTE 2b:	The current EUTRAN security context is defined in [78] and [79].  For EUTRA to UTRA key mapping, this context consists of a minimum of the current KASME, the associated key set identifier (KSI) and the associated downlink NAS COUNT values.

2>	set the variable LATEST_CONFIGURED_CN_DOMAIN to the value indicated in the IE "CN domain identity" of the IE "RAB info" of the IE "RAB information to setup" if all instances of the IE indicate the same CN domain, or to the CS domain when this IE is either not present or different instances indicate different CN domains;

2>	for the CN domain in variable LATEST_CONFIGURED_CN_DOMAIN set the IE "Status" in the variable CIPHERING_STATUS to "Started";

2>	if the variable LATEST_CONFIGURED_CN_DOMAIN is set to "CS domain":

3>	set the 20 MSB of the HFN component of the COUNT-C variable for all signalling radio bearers to the "START" value from the IE "UE security information" in the variable INTER_RAT_HANDOVER_INFO_TRANSFERRED.

2>	else if the variable LATEST_CONFIGURED_CN_DOMAIN is set to "PS domain":

3>	set the 20 MSB of the HFN component of the COUNT-C variable for all signalling radio bearers to the "START" value from the IE "UE security information2" in the variable INTER_RAT_HANDOVER_INFO_TRANSFERRED.

2>	if the CN domain indicated in the IE "CN domain identity" of any instance of the IE "RAB info" in the IE "RAB information to setup" is "CS domain":

3>	set the 20 MSB of the HFN component of the COUNT-C variable for all CS domain radio bearers to the "START" value included in the IE "UE security information" in the variable INTER_RAT_HANDOVER_INFO_TRANSFERRED;.

3>	set the IE "Status" in the variable CIPHERING_STATUS to "Started".

2>	if the CN domain indicated in the IE "CN domain identity" of any instance of the IE "RAB info" in the IE "RAB information to setup" is "PS domain":

3>	set the 20 MSB of the HFN component of the COUNT-C variable for all PS domain radio bearers to the "START" value included in the IE "UE security information2" in the variable INTER_RAT_HANDOVER_INFO_TRANSFERRED;

3>	set the IE "Status" in the variable CIPHERING_STATUS to "Started".

2>	set the remaining LSBs of the HFN component of COUNT-C for all radio bearers to zero;

2>	not increment the HFN component of COUNT-C for radio bearers using RLC-TM, i.e. keep the HFN value fixed without incrementing every CFN cycle;

2>	set the CFN component of the COUNT-C variable for radio bearers using RLC-TM to the value of the CFN as calculated in subclause 8.5.15;

2>	for all user radio bearers belonging to the "CS domain" and for signalling radio bearers when the variable LATEST_CONFIGURED_CN_DOMAIN is set to "CS domain":

3>	apply the algorithm according to IE "Ciphering Algorithm" with the ciphering key of the key set used and apply ciphering immediately upon reception of the HANDOVER TO UTRAN COMMAND.

2>	for all user radio bearers belonging to the "PS domain" and for signalling radio bearers when the variable LATEST_CONFIGURED_CN_DOMAIN is set to "PS domain":

3>	If the source RAT is E-UTRA; or

3> if the source RAT is GERAN and the procedure is an rSR-VCC handover:

4>	apply the algorithm according to IE "Ciphering Algorithm" with the ciphering key of the PS key set used and apply ciphering immediately upon reception of the HANDOVER TO UTRAN COMMAND.

3>	Otherwise:

4>	apply the algorithm according to IE "Ciphering Algorithm" with the ciphering key stored in the USIM/SIM and apply ciphering immediately upon reception of the HANDOVER TO UTRAN COMMAND.

NOTE 3:	If ciphering is indicated in the HANDOVER TO UTRAN COMMAND, UTRAN should not include the IE "Ciphering mode info" in the SECURITY MODE COMMAND message that starts Integrity protection. Upon performing simultaneous CS and PS handover (i.e. DTM Handover or SR-VCC with a simultaneous PS handover), UTRAN should start Integrity protection for the CS domain first then the PS domain. UTRAN should not include the IE "Ciphering mode info" in the SECURITY MODE COMMAND message that starts Integrity protection for the CS domain and UTRAN should include the IE "Ciphering mode info" in the SECURITY MODE COMMAND message that starts Integrity protection for the PS domain.

NOTE 4:	DTM Handover is not possible if the IE "specification mode" is set to "Preconfiguration" due to the existence of a single IE "RAB Info".

1>	if ciphering is not indicated in the HANDOVER TO UTRAN COMMAND:

2>	for the CN domain(s) included in the IE "CN domain identity" which is included in the IE "RAB info" of the IE "RAB information to setup", or the CS domain when these IEs are not present:

3>	set the IE "Status" in the variable CIPHERING_STATUS to "Not Started".

If the UE succeeds in establishing the connection to UTRAN, it shall:

1>	if "RAB Info" list contained only one CN domain identity in IE "CN domain identity":

2>	if CN domain is set to CS domain only indicate to upper layers that no CN system information is available for any domain other than the CS domain (see NOTE 5);

2>	if CN domain is set to PS domain only indicate to upper layers that no CN system information is available for any domain other than the PS domain (see NOTE 6).

1>	if "RAB Info" list contained RABs for the PS and CS domain:

2>	indicate to upper layers that CN system information is available for the PS and CS domain (see NOTE 7).

NOTE 5:	After CS handover or SR-VCC handover without a simultaneous PS handover has been completed, the UTRAN should provide the UE with the CN system information of the PS domain as soon as possible, in order not to delay access to the PS domain.

NOTE 6:	After PS handover or rSR-VCC handover has been completed, the UTRAN should provide the UE with the CN system information of the CS and PS domain as soon as possible in order not to delay access to the CS domain and trigger a NAS procedure for the PS domain.

NOTE 7:	After DTM handover or SR-VCC handover with a simultaneous PS handover has been completed, the UTRAN should provide the UE with the CN system information of the PS domain as soon as possible in order to trigger a NAS procedure for the PS domain.

1>	if the IE "Status" in the variable CIPHERING_STATUS of a CN domain is set to "Started" and transparent mode radio bearers have been established by this procedure for that CN domain:

2>	include the IE "COUNT-C activation time" in the response message and specify a CFN value for this IE other than the default, "Now", that is a multiple of 8 frames (CFN mod 8 =0) and lies at least 200 frames ahead of the CFN in which the response message is first transmitted;

2>	at the CFN value as indicated in the response message in the IE "COUNT-C activation time" for radio bearers using RLC-TM:

3>	set the 20 MSB of the HFN component of the COUNT-C variable common for all transparent mode radio bearers of this CN domain to the START value as indicated in the IE "START list" of the response message for the relevant CN domain; and

3>	set the remaining LSBs of the HFN component of COUNT-C to zero;

3>	increment the HFN component of the COUNT-C variable by one even if the "COUNT-C activation time" is equal to zero;

3>	set the CFN component of the COUNT-C to the value of the IE "COUNT-C activation time" of the response message. The HFN component and the CFN component completely initialise the COUNT-C variable;

3>	step the COUNT-C variable, as normal, at each CFN value. The HFN component is no longer fixed in value but incremented at each CFN cycle.

1>	if the IE "Status" in the variable CIPHERING_STATUS of a CN domain is set to "Not Started" and transparent mode radio bearers have been established by this procedure for that CN domain:

2>	initialise the 20 MSB of the HFN component of COUNT-C common for all transparent mode radio bearers of this CN domain with the START value as indicated in the IE "START list" of the response message for the relevant CN domain;

2>	set the remaining LSBs of the HFN component of COUNT-C to zero;

2>	do not increment the COUNT-C value common for all transparent mode radio bearers for this CN domain.

1>	for 3.84/7.68 Mcps TDD, if the HANDOVER TO UTRAN COMMAND provides for E-DCH/HS-DSCH operation without an uplink DPCH:

2>	the UE shall obtain timing advance for the 3.84/7.68 Mcps TDD cell according to [15] before transmitting on the uplink DCCH.

1>	if an IE "Logged Measurement Info-FDD" or "Logged Measurement Info-TDD" in variable LOGGED_MEAS_REPORT_VARIABLE is present and registered PLMN is present in the IE "PLMN Identity List" stored in variable LOGGED_MEAS_REPORT_VARIABLE:

2>	include IE "Logged Meas Available".

1>	if an IE "Logged Connection Establishment Failure Info-FDD" or "Logged Connection Establishment Failure Info-TDD" in variable LOGGED_CONNECTION_ESTABLISHMENT_FAILURE is present and the current Registered PLMN is the same as the PLMN in IE "PLMN Identity" stored in variable LOGGED_CONNECTION_ESTABLISHMENT_FAILURE:

2>	include IE "Connection Establishment Failure Info Available".

1>	transmit a HANDOVER TO UTRAN COMPLETE message on the uplink DCCH, using, if ciphering has been started, the new ciphering configuration;

1>	when the HANDOVER TO UTRAN COMPLETE message has been submitted to lower layers for transmission:

2>	if the source RAT was E-UTRA:

3>	store the CS "key set used" including the mapped KSI to the USIM CS security key if SR-VCC was performed;

3>	store the PS "key set used" including the mapped KSI to the USIM PS security key if PS handover was performed;

2>	if the source RAT was GERAN:

3>	store the PS "key set used" including the mapped KSI to the USIM PS security key if rSR-VCC was performed;

2>	if a SR-VCC handover is performed and the source RAT was NR:

3>	store the CS "key set used" including the mapped KSI to the USIM CS security key if SR-VCC was performed;

3>	indicate to the upper layer that SR-VCC handover from NR was completed successfully.

2>	if the UE supports voice and video SRVCC handover, and a SR-VCC handover is performed:

3>	if radio bearer configurations contain Conversational / speech CS RAB:

4>	indicate to the upper layer that voice-only SRVCC handover was completed successfully.

3>	if radio bearer configurations contain Conversational / unknown / UL:64 DL:64 kbps CS RAB:

4>	indicate to the upper layer that voice and video SRVCC handover was completed successfully.

2> if the UE supports rSRVCC handover, and an rSR-VCC handover is performed;

3>	indicate to the upper layer that rSR-VCC handover was completed successfully.

2>	enter UTRA RRC connected mode in state CELL_DCH;

2>	initialise variables upon entering UTRA RRC connected mode as specified in subclause 13.4;

2>	update the variable UE_CAPABILITY_TRANSFERRED with the UE capabilities stored in the variable INTER_RAT_HANDOVER_INFO_TRANSFERRED.

1>	and the procedure ends.

8.3.6.4	Invalid Handover to UTRAN command message

If the UE receives a Handover to UTRAN command message, which contains a protocol error causing the variable PROTOCOL_ERROR_REJECT to be set to TRUE according to clause 9, the UE shall perform procedure specific error handling according to the source radio access technology. The UE shall:

1>	if allowed by the source RAT:

2>	transmit an RRC FAILURE INFO message to the source radio access technology; and

2>	include the IE "Protocol error information" with contents set to the value of the variable PROTOCOL_ERROR_INFORMATION;

1>	Other details may be provided in the specifications related to the source radio access technology.

NOTE:	The other RAT may include the above diagnostics information in a subsequent handover request towards the same RNC.

8.3.6.4a	Unsupported configuration in HANDOVER TO UTRAN COMMAND message

If the UE does not support the configuration included in the HANDOVER TO UTRAN COMMAND message, e.g., the message includes a pre-defined configuration that the UE has not stored, the UE shall:

1>	continue the connection using the other radio access technology; and

1>	indicate the failure to the other radio access technology.

8.3.6.5	UE fails to perform handover

If the UE does not succeed in establishing the connection to UTRAN, it shall:

1>	terminate the procedure including release of the associated resources;

1>	resume the connection used before the handover; and

1>	indicate the failure to the other radio access technology.

Upon receiving an indication about the failure from the other radio access technology, UTRAN should release the associated resources and the context information concerning this UE.

8.3.6.6	Reception of message HANDOVER TO UTRAN COMPLETE by the UTRAN

Upon receiving a HANDOVER TO UTRAN COMPLETE message, UTRAN should consider the inter-RAT handover procedure as having been completed successfully and indicate this to the Core Network.