3GPP TSG-SA WG6 Meeting #62	S6-243224

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243xxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0077 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for SEALDD enabled L4S congestion control |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD_Ph2 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For the clause 9.8.2.2 about SEALDD enabled L4S congestion control, there are two options to perform congestion control: Option#1: the SEALDD server performs the rate adapation for multiple VAL applications by itself. Option#2: the SEALDD server calculates the congestion level for multiple VAL applications and feedbacks to VAL server. In current specification, this part is not clear to capture these two options, therefore, how to SEALDD enabled congestion control will cause misunderstanding. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This paper is proposed to add some text corrections to avoid the misunderstanding for SEALDD enabled L4S congestion control. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How to implement SEALDD enabled L4S congestion control is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.8.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.8.2.2	SEALDD enabled congestion control for VAL applications

Based on the congestion information exposed by 5GS (i.e., by using ECN marking for L4S), the SEALDD server can perform the differentiated congestion control for multiple VAL applications, after receiving the L4S feedback from the SEALDD client. Figure 9.8.2.2-1 illustrates the procedure for SEALDD enabled congestion control for VAL applications.

Pre-conditions:

1.	The VAL server has discovered and selected the SEALDD server by CAPIF functions as specified in clause 9.4.2.

Figure 9.8.2.2-1: SEALDD enabled congestion control procedure

1-6. Same as clause 9.2.2.2 step 1 to step 7 with difference that in step 5 the SEALDD client includes also L4S feedback capability in Sdd_RegularTransmissionConnection_Establish request.7.	If the L4S feedback capability is received in step 5, the SEALDD server can request 5GC to perform the ECN marking for L4S for the required VAL applications by utilizing the AF session with required QoS procedure in clause 4.15.6.6 of TS 23.502 [6].

8.	Same as step 9 of clause 9.2.2.2.

9.	For downlink traffic, if ECN marking is identified, the L4S feedback is performed in transport layer.

10.	After receiving the L4S feedback in step 9, the SEALDD server may perform the congestion mitigation for downlink traffics from multiple VAL applications. For the same UE, the SEALDD server can determine differentiated congestion control/rate control with the calculated congestion level or the calculated transmission rate for multiple VAL applications based on internal policy considering the received QoS requirements of different VAL applications identified by VAL service ID in step 1.

11.	[Optional] The SEALDD server sends Sdd_connection status notification to VAL server with the calculated congestion level for the VAL service.

NOTE 1: 	According to the congestion level provided by SEALDD server, the VAL server supporting media can perform congestion control, e.g. by adjusting the encoding scheme, and rendering scheme, etc.

NOTE 2: 	Step 11 is performed if the SEALDD server cannot perform transmission rate adaptation to mitigate the congestion by itself.