|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2022-2024 |  | SG2-TD539R2/PLEN |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | STUDY GROUP 2 |  |  |
|  |  |  | Original: English |  |  |
| Question(s): |  | 6/2 |  | Geneva, 19-28 June 2024 |  |
| TD |  |  |  |  |  |
| Source: |  | Editor |  |  |  |
| Title: |  | Revised baseline text for M.3388(ex. M.eiil-AITOM): "Effectiveness indicators of intelligence level for AI enhanced telecom operation and management" (for consent) |  |  |  |
| Contact: |  | Rongrong Guo China Telecom P.R. China |  |  | Tel: +86-13391821560 Fax: +86(10)-50902230 E-mail: guorr@chinatelecom.cn |
| Contact: |  | Yanqin Wu China Telecom P.R. China |  |  | Tel: +86(10)-50902892 Fax: +86(10)-50902230 E-mail: wuyq@chinatelecom.cn |

| Abstract: | The document provides new revised baseline text for M.3388(ex. M.eiil-AITOM): “Effectiveness indicators of intelligence level for AI enhanced telecom operation and management” based on SG2-C208 and comments of SG2 meeting during 19-28 June 2024. |
| --- | --- |

# Summary

The document provides new revised baseline text for M.3388(ex. M.eiil-AITOM): “Effectiveness indicators of intelligence level for AI enhanced telecom operation and management”(for consent) based on SG2-C208 and comments of SG2 meeting during 19-28 June 2024.

Draft Recommendation ITU-T M.3388(ex. M.eiil-AITOM)

Effectiveness indicators of intelligence level for AI enhanced telecom operation and management

Summary

The effectiveness indicators for AI enhanced telecom operation and management aim to evaluate intelligence level with quantitative methods and focus on showing effects and benefits of applying AI in terms of operation optimization, cost reduction, service fulfillment, customer experience upgrade, etc.

This draft Recommendation provides the principle, classification, definition and method of effectiveness indicators to evaluate intelligence level of AI enhanced telecom operation and management.

Keywords

Effectiveness indicators, Intelligence level, AITOM

Table of Contents

1. Scope	4

2. References	4

3. Definitions	4

3.1 Terms defined elsewhere	4

3.2 Terms defined in this Recommendation	4

4. Abbreviations and acronyms	4

5. Conventions	5

6. Introduction	5

7. Principles for determination of effectiveness indicators	5

8. Classification of effectiveness indicators	5

8.1 Dimensions to evaluate intelligence level for AITOM	5

8.2 Sub-dimension of customer experience	6

8.3 Sub-dimension of service operation	6

8.4 Sub-dimension of network management	7

9. Common effectiveness indicators	7

9.1 Effectiveness indicators to evaluate customer experience	7

9.2 Effectiveness indicators to evaluate service operation	8

9.3 Effectiveness indicators to evaluate network management	8

10. Method of effectiveness evaluation	9

Bibliography	10

Draft Recommendation ITU-T M.3388(ex. M.eiil-AITOM)

Effectiveness indicators of intelligence level for AI enhanced telecom operation and management

- Scope

This Recommendation provides the effectiveness indicators to evaluate the intelligence level of systems which follow the framework of AI enhanced telecom operation and management (AITOM). It aims to evaluate effectiveness with quantitative methods and could be used as a guideline to provide more evaluation dimensions on the effects and benefits of applying AI in terms of operation optimization, cost reduction, service fulfillment, customer experience upgrade, etc.

The scope of this Recommendation includes:

- Principle for determination of effectiveness indicators

- Classification of effectiveness indicators

- Common effectiveness indicators

- Method of effectiveness evaluation

- References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T M.3080]	Recommendation ITU-T M.3080 (2021), Framework of artificial intelligence enhanced telecom operation and management (AITOM)

[ITU-T M.3384] 	Recommendation ITU-T M.3384 (2023), Intelligence levels of AI enhanced Telecom Operation and Management.

[ITU-T M.3385] 	Recommendation ITU-T M.3385 (2023), Intelligence levels evaluation frame of AI enhanced Telecom Operation and Management.

- Definitions

## 3.1 Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	artificial intelligence (AI) [b-ITU-T F.749.13]: An interdisciplinary field, usually regarded as a branch of computer science, dealing with models and systems for the performance of functions generally associated with human intelligence, such as reasoning and learning.

## 3.2 Terms defined in this Recommendation

None.

- Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| AI | Artificial Intelligence |
| --- | --- |
| AITOM | AI enhanced Telecom Operation and Management |
| SLA | Service Level Agreement |

- Conventions

In this Recommendation:

The keywords "is required to" indicate a requirement which must be strictly followed and from which no deviation is permitted, if conformance to this Recommendation is to be claimed.

The keywords "is recommended" indicate a requirement which is recommended but which is not absolutely required. Thus, this requirement need not be present to claim conformance.

- Introduction

The Recommendation [ITU-T M.3384] specifies and classifies intelligence levels applied to a system based on the framework of AITOM [ITU-T M.3080] (AITOM-based system). The Recommendation [ITU-T M.3385] further defines qualitative evaluation methods with evaluation criteria. The characteristics of criteria for intelligence level are described based on whether or to what extend AI capabilities are applied into systems and particular operation tasks. These Recommendations are helpful to identify the shortcomings and guide the intelligent evolution of systems.

Besides, intelligence levels evaluation of systems which follow the framework of AITOM needs further support with additional quantitative based on effectiveness indicators. The results of the evaluation are required to be value-oriented and provide more information on the effects and benefits of applying AI in terms of operation optimization, cost reduction, service fulfillment, customer experience upgrade, etc.

- Principle for determination of effectiveness indicators

The effectiveness indicators of intelligence level for AITOM are required to follow the principles:

- Quantifiability. The effectiveness indicators are required to have clear definitions and calculation methods and be measured or quantified using specific data or numerical values.

- Objectivity. The effectiveness indicators are required to be based on actual telecom operation and management data generated by the systems during the daily operation and management processes.

- Reliability. The effectiveness indicators are required to keep consistent and stable in a specific period.

- Value orientation. The effectiveness indicators are required to consider practical benefits involving cost reduction, efficiency improvement, quality promotion, and revenue increase in telecom operation and management activities.

- Classification of effectiveness indicators

## 8.1 Dimensions to evaluate intelligence level for AITOM

To evaluate intelligence level for AITOM-based systems, it is required to identify effectiveness indicators from dimensions of customer experience, service operation and network management. According to different focuses, above three dimensions are further divided into 12 sub-dimensions and each sub-dimension is recommended to involve one or more specific indicators to evaluate the AITOM-based systems. Overview of effectiveness indicators classification is depicted in Figure 1.

Figure 1. Classification of effectiveness indicator of intelligence level for AI enhanced telecom operation and management

## 8.2 Sub-dimension of customer experience

To evaluate the effectiveness of customer experience, this dimension is further required to be broken down into four sub-dimensions, including customer self-service capability, customer response speed, customer service availability and customer perception prediction. It is required to construct an indicator set for each sub-dimension to measure the intelligence effectiveness of AITOM-based systems in this dimension.

- Customer self-service capability indicator set. The evaluation indicators are required to represent effectiveness of end-to-end online self-service capabilities including self-ordering, self-change, self-maintenance and self-management.

- Customer response speed indicator set. The evaluation indicators are required to represent effectiveness of customer-oriented rapid response capabilities including real-time service provision and rapid troubleshooting.

- Customer service availability indicator set. The evaluation indicators are required to represent effectiveness of ensuring high availability of service.

- Customer experience perception indicator set. The evaluation indicators are required to represent effectiveness of customer experience quality.

## 8.3 Sub-dimension of service operation

To evaluate the effectiveness of service operation, this dimension is further required to be broken down into four sub-dimensions, including service loading, service marketing, service provision, and service quality. It is required to construct an indicator set for each sub-dimension to measure the intelligence effectiveness of AITOM-based systems in this dimension.

- Service loading indicator set. The evaluation indicators are required to represent effectiveness of quick service launch capability from design to loading completion.

- Service marketing indicator set. The evaluation indicators are required to represent effectiveness of intelligent capability to realize accurate service recommendation and other marketing events.

- Service provision indicator set. The evaluation indicators are required to represent effectiveness of intelligent capability of service delivery including the whole process of resource exploration, service orchestration and service activation, etc.

- Service quality indicator set. The evaluation indicators are required to represent effectiveness of service quality based on SLA.

## 8.4 Sub-dimension of network management

To evaluate the effectiveness of network management, this dimension is further required to be broken down into four sub-dimensions, including network planning, network optimization, network maintenance and network recovery. It is required to construct an indicator set for each sub-dimension to measure the intelligence effectiveness of AITOM-based systems in this dimension.

- Network planning indicator set. The evaluation indicators are required to represent effectiveness of timely, accurate and dynamic resource planning and coverage to meet service and customer demand.

- Network optimization indicator set. The evaluation indicators are required to represent effectiveness of intelligent network optimization capabilities involving improving stability and energy efficiency of network management.

- Network maintenance indicator set. The evaluation indicators are required to represent effectiveness of automated network maintenance capabilities involving network patrol inspection and cutover.

- Network recovery indicator set. The evaluation indicators are required to represent effectiveness of the intelligent network recovery capabilities to timely discover, prevent, automatically locate and repair troubles.

- Common effectiveness indicators

## 9.1 Effectiveness indicators to evaluate customer experience

To evaluate the effectiveness of customer experience, it is recommended to set the following 5 effectiveness indicators:

- Customer self-service rate is recommended to evaluate customer self-service capability, which is defined as within the effective statistical period, the proportion of the number of customer self-support service to the total number of service provided to customers.

- Timeliness rate of service provision is recommended to evaluate customer response speed, which is defined as within the effective statistical period, the proportion of the number of service provided within the promised time limit to the total number of service.

- Timeliness rate of reported faults handling is also recommended to evaluate customer response speed, which is defined as within the effective statistical period, the proportion of the number of customer reported faults handled completely within the repair deadline to the total number of reported faults.

- Customer service availability is recommended to evaluate customer service availability, which is defined as within the effective statistical period, the proportion of normal service using time provided to customers according to promised performance to the total service time.

- Customer perceived degradation rate is recommended to evaluate customer perception quality, which is defined as within the effective statistical period, the ratio of the monitoring customer experience quality degradation count of customer perception to total monitoring counts.

## 9.2 Effectiveness indicators to evaluate service operation

To evaluate the effectiveness of service operation, it is recommended to set the following 4 effectiveness indicators:

- Service automatic loading rate is recommended to evaluate service loading effectiveness, which is defined as within the effective statistical period, the proportion of the number of service automatically loaded and launched by the system to the total number of newly developed service.

- Marketing accuracy rate is recommended to evaluate service marketing effectiveness, which is defined as within the effective statistical period, the proportion of the number of customers who have been contacted and successfully converted into orders through intelligent marketing methods to the number of customers involved in marketing campaign.

- Success rate of automatic service provision is recommended to evaluate service provision effectiveness, which is defined as within the effective statistical period, the proportion of the number of service successfully provided to the total number of provided service.

- SLA compliance rate is recommended to evaluate service quality effectiveness, which is defined as within the effective statistical period, the proportion of the number of provided service meeting promising SLA to the total number of provided service.

## 9.3 Effectiveness indicators to evaluate network management

To evaluate the effectiveness of network management, it is recommended to set the following 8 effectiveness indicators:

- Network resource pre-coverage rate is recommended to evaluate network planning effectiveness, which is defined as within the effective statistical period, the proportion of the number of service satisfied the service provision need with planned pre-coverage resource to the total number of service.

- Network cutover success rate is recommended to evaluate network maintenance effectiveness, which is defined as within the effective statistical period, the proportion of the number of tasks automatically completed in network cutover to the total number of network cutover tasks.

- Network inspection automation rate is recommended to evaluate network maintenance effectiveness, which is defined as within the effective statistical period, the proportion of the number of items that are automatically executed by network equipment for inspection to the total number of items that are inspected by network equipment.

- Network fault localization accuracy rate is recommended to evaluate network recovery effectiveness, which is defined as within the effective statistical period, the proportion of the number of accurately located faults to the total number of network faults.

- Network fault automatic detection rate is recommended to evaluate network recovery effectiveness, which is defined as within the effective statistical period, the proportion of the number of automatically discovered faults to the total number of network faults.

- Mean time to repair is recommended to evaluate network recovery effectiveness, which is defined as within the effective statistical period, the ratio of total network faults recovery time to the number of faults.

- Network optimization success rate is recommended to evaluate network optimization effectiveness, which is defined as within the effective statistical period, the proportion of the number of network quality optimization to the total number of network quality problems.

- Device power-saving ratio is recommended to evaluate network optimization effectiveness, which is defined as within the effective statistical period, the ratio of power consumption when the device power saving mode is enabled to consumption when the device power saving mode is disabled.

- Method of effectiveness evaluation

The method of effectiveness evaluation includes the following three steps:

- Step 1: Determine the benchmark values of the effectiveness indicators for each level which could keep in line with level classification in [ITU-T M.3385]. The benchmark values are determined on the basis of the difficulty and the urgency of technical enhancement for network operation and management.

- Step 2: Automatically calculate and access the true values of the effectiveness indicators based on indicators definition and calculation method. Each factor in effectiveness indicator is automatically counted by telecom operation and management data generated during the daily operation and management processes.

- Step 3: Obtain final real effectiveness evaluation level based on a comparative relationship between true value and the benchmark value of effectiveness indicators. If the real level of the effectiveness indicators does not reach the benchmark level, the output will optimize the prompt information of factors in effectiveness indicator , so as to constantly level up to form a closed loop.

# Bibliography

[b-ITU-T F.749.13]	Recommendation ITU-T F.749.13 (2021), Framework and requirements for civilian unmanned aerial vehicle flight c

_______________________