3GPP TSG-SA5 Meeting #122	S5-187399

# Spokane,US, 12-16 November 2018 												Revision of S5-1870108

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.290 | CR | 0023 | rev | 1 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add description for Charging Notification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SBI_CH |  |  |  |  |  | Date: |  |  | 2018-11-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The message content associated to the Notify Operation of the Nchf-ConvergedCharging service has no stage 2 description The chapter 5.3.2.4 Charging Notify is not part of the "Contents" paragraph due to style issue |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the message content description for Notify operation Correct style issue for 5.3.2.4 title |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No stage 2 description |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2.4, 5.4.1, 7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Baseline from ts_132290v150100p.docx to get tables with correct style. |  |  |  |  |  |  |  |  |

First change

#### 5.3.2.4	Charging notification

The CHF can in Converged Session based Charging provide notifications to the NF (CTF), the NF (CTF) implicitly subscribes to these when it sends a Charging Data Request [Initial], i.e. there is no separate subscription request from the NF for notification.

Figure 5.3.2.4-1 shows a scenario for Session based charging with a notification from the CHF triggering a Charging Data Request [Update].

Figure 5.3.2.x.1: Session based charging – Notification with Re-authorization

1)	Session based charging ongoing: there is a session based charging ongoing and there have at least been a Charging Data Request [Initial] sent from the NF (CTF) to the CHF, and the CHF have opened a CDR.

2)	Event triggering notification: an event is detected in the CHF that requires a notification to be sent to the NF (CTF). In this scenario a request for triggering a Charging Data Request [Update, Quota Request] is sent, but also requests for Charging Data Request [Update] (without request for quota) is possible.

3)	Charging Notify Request [Re-authorization]: the CHF sends the request to the NF (CTF), for a triggering of a Charging Data Request [Update, Quota Request] i.e. Re-authorization.

4)	Charging Notify Response: the NF (CTF) acknowledges the request by sending a response.

5)	Charging Data Request [Update, Quota Requested]: the NF (CTF) sends the request to the CHF, to be granted with more unit for the service to continue, and also for reporting the used units.

6)	Account, Rating, Reservation Control: the CHF performs the process related to the reported usage and the requested reservation, involving rating entity and user's account balance.

7)	 Update CDR: based on policies, the CHF updates the CDR with charging data related to the service.

8)	Charging Data Response [Update, Quota Granted]: the CHF grants quota to NF (CTF) for the service to continue, with the reserved number of units.

Figure 5.3.2.4.2 shows a scenario for Session based charging with a notification from the CHF triggering a Charging Data Request [Termination].

Figure 5.3.2.4.2: Session based charging – Notification with termination

1)	Session based charging ongoing: there is a session based charging ongoing and there have at least been a Charging Data Request [Initial] sent from the NF (CTF) to the CHF, and the CHF have opened a CDR.

2)	Event triggering notification: an event is detected in the CHF that requires a notification to be sent to the NF (CTF). In this scenario a request for triggering a Charging Data Request [Termination] is sent.

3)	Charging Notify Request [Terminate]: the CHF sends the request to the NF (CTF), for a triggering of a Charging Data Request [Termination] i.e. the termination of the charging session.

4)	Charging Notify Response: the NF (CTF) acknowledges the request by sending a response.

5)	Charging Data Request [Termination]: the NF (CTF) sends the request to the CHF, for charging data related to the service termination with the final consumed units.

6)	Account, Rating Control: the CHF performs the process related to the reported usage, involving rating entity and user's account balance.

7)	Close CDR: based on policies, the CHF closes the CDR with charging data related to the service.

8)	Charging Data Response [Termination]: The CHF informs the NF (CTF) on the result of the request.

Next change

### 5.4.1	Re-authorization

The CHF (NF Service Producer) may trigger a re-authorization request and the NF Service Consumer shall report quota usage. The reason for the quota being reported shall be notified to the CHF (NF Service Producer). This is described under charging notification procedure in clause 5.3.2.4.

Next change

## 7	Message contents

Converged charging is performed by NF (CTF) consuming service operations exposed by CHF, achieved using Charging Data Request and Charging Data Response.

The information structure used for these services operations is composed of two parts:

-	Common structures specified in the present document.

-	NF (CTF) consumer specific structures specified in the middle tier TSs.

Table 7.1 describes the data structure which is common to operations in request semantics.

Table 7.1: Common Data structure of Charging Data Request

| Information Element | Category | Description |  |
| --- | --- | --- | --- |
| Session Identifier | OC | This field identifies the charging session. |  |
| Subscriber Identifier | OM | This field contains the identification of the subscriber that uses the requested service. |  |
| NF Consumer Identification | M | This is a grouped field which contains a set of information identifying the NF consumer of the charging service. |  |
| NF Functionality | M | This field contains the function of the node. |  |
| NF Name | M | This fields holds the name of the NF consumer |  |
| NF Address | M | This field holds the IP-address of NF consumer |  |
| NF PLMN ID | OC | This field holds the PLMN ID of the network the NF consumer belongs to. |  |
| Invocation Timestamp | M | This field holds the timestamp of the charging service invocation by the NF consumer |  |
| Invocation Sequence Number | M | This field contains the sequence number of the charging service invocation by the NF consumer. |  |
| Notify URI | Oc | This field contains URI of Notifications sent by the CHF. |  |
| Triggers | OC | This field identifies the event(s) triggering the request and is common to all Multiple Unit Usage occurences. |  |
| Multiple Unit Usage | OC | This field contains the parameters for the quota management request and/or usage reporting. It may have multiple occurences. |  |
| Rating Group | M | This field holds the identifier of a rating group. |  |
| Requested Unit | OC | This field contains the amount of requested service units for a particular category or an indication that units are needed for a particular category. |  |
| Time | OC | This field holds the amount of requested time. |
| Total Volume | OC | This field holds the amount of requested volume in both uplink and downlink directions. |
| Uplink Volume | OC | This field holds the amount of requested volume in uplink direction. |
| Downlink Volume | OC | This field holds the amount of requested volume in downlink direction. |
| Service Specific Units | OC | This field holds the amount of requested service specific units. |
| Used Unit Container | OC | This field contains the amount of used non-monetary service units measured. up to the triggers and trigger timestamp. It may have multiple occurrences. |  |
| Service Identifier | OC | This field holds the Service Identifier. |  |
| Quota management Indicator | Oc | This field holds an indicator on whether the reported used units are with or without quota management control. If the field is not present, it indicates the used unit is without quota management applied. |  |
| Triggers | OC | This field holds reason for charging information reporting or closing for the used unit container. |  |
| Trigger Timestamp | Oc | This field holds the timestamp of the trigger. |  |
| Time | OC | This field holds the amount of used time. |  |
| Total Volume | OC | This field holds the amount of used volume in both uplink and downlink directions. |  |
| Uplink Volume | OC | This field holds the amount of used volume in uplink direction. |  |
| Downlink Volume | OC | This field holds the amount of used volume in downlink direction. |  |
| Service Specific Unit | Oc | This field holds the amount of used service specific units. |  |
| Event Time Stamps | OC | This field holds the timestamps of the event reported in the Service Specific Units, if the reported units are event based. |  |
| Local Sequence Number | OM | This field holds the container sequence number. |  |

Table 7.2 describes the data structure which is common to operations in response semantics.

Table 7.2: Common Data structure of Charging Data Response

| Information Element | Category | Description |
| --- | --- | --- |
| Session Identifier | OC | This field identifies the charging session. |
| Invocation Timestamp | M | This field holds the timestamp of the charging service response from the CHF. |  |
| Invocation Result | M | This field holds the result of the charging service invocation by the NF consumer |  |
| Result code | M | This field contains the result code: success or failure code. |  |
| Failed parameter | OC | This field holds missing and/or unsupported parameter that caused the failure. |  |
| Failure Handling | OC | This field holds the failure handling to be performed by the NF consumer when failure. |  |
| Invocation Sequence Number | M | This field holds the sequence number of the charging service invocation by the NF consumer. |  |
| Session Failover | OC | This field indicates whether alternative CHF is supported for ongoing charging service failover handling by NF consumer. |  |
| Triggers | OC | This field holds the triggers supplied from the CHF for the charging session that are independent of rating group. |  |
| Multiple Unit Information | OC | This field holds the parameters for the quota management information. It may have multiple occurrences. |  |
| Result Code | OC | This field contains the result of the Rating Group quota allocation. |  |
| Rating Group | OM | The identifier of a rating group. |  |
| Granted Unit | OC | This field holds the granted quota. |  |
| Tariff Time Change | OC | This field contains the switch time when the tariff will be changed. |  |
| Time | OC | This field holds the amount of granted time. |  |
| Total Volume | OC | This field holds the amount of granted volume in both uplink and downlink directions. |  |
| Uplink Volume | OC | This field holds the amount of granted volume in uplink direction. |  |
| Downlink Volume | OC | This field holds the amount of granted volume in downlink direction. |  |
| Service Specific Units | OC | This field holds the amount of granted requested service specific units. |  |
| Validity Time | OC | This field defines the time in order to limit the validity of the granted quota for a given category instance. |  |
| Final Unit Indication | OC | This field indicates the granted final units for the service. |  |
| Time Quota Threshold | OC | This field indicates the threshold in seconds when the granted quota is time |  |
| Volume Quota Threshold | OC | This field indicates the threshold in octets when the granted quota is volume |  |
| Unit Quota Threshold | OC | This field indicates the threshold in service specific units, that are defined in the service specific documents, when the granted quota is service specific |  |
| Quota Holding Time | OC | This field holds the quota holding time in seconds. |  |
| Triggers | OC | This field holds triggers for usage reporting associated to the rating group, which is supplied from the CHF. |  |

The CTF NF consumer specific structures which are specified in the middle tier TSs, are defined as extensions of:

-	common part structure of Charging Data Request and Charging Data Response.

-	structure of Multiple Quota Usage.

-	structure of Multiple Quota Information.

Table 7.x describes the data structure which is common to Charging Notify Request.

Table 7.x: Common Data structure of Charging Notify Request

| Information Element | Category | Description |
| --- | --- | --- |
| Notify URI | M | This field holds the URI previously supplied by the CHF for notifications associated to the charging session. |
| Notification type | M | This field holds the type of notification indicating re-authorization or termination. |
| Reauthorization Details | OC | This field holds the details of re-authorization. It’s only present when type of notification is re-authorization.If not present and type of notification is re-authoriziation, the re-authorization notification applies to all units. |
| Service Identifier | OC | This field holds the Service Identifier to which re-authorization notification applies. If present, the rating group shall also be present. If not present the re-authorization notification applies to all service identifiers. |
| Rating Group | OC | This field holds the rating group to which re-authorization notification applies. If not present the re-authorization notification applies to all rating groups. |
| Quota management Indicator | Oc | This field holds an indicator on whether the re-authorization notification is for quota management control or not. If not present the re-authorization notification applies to both units with and without quota management. |

End of changes