SA WG2 Meeting #133 	S2-1906778

# 13 - 17 May 2019, Reno, NV, USA								(revision of S2-1904740/05356)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.725 | CR | 0034 | rev | 5 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Simplified version of solution #24 QoS monitoring for URLLC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_URLLC |  |  |  |  |  | Date: |  |  | 2019-05-06 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This change request proposes a simplified version of solution #24 to key issues #4 using S5-193383 |  |  |  |  |  |  |  |  |
| Summary of change: |  | This solution provides means for QoS monitoring for URLLC on the GTP based interaces, i.e. on N3 interface between UPF and NG-RAN as well as on the N9 interface between the UPF (PSA) and I-UPF. This solution is based on the specification of the measurements specified for GTP based N3 interface in TS 28.552 [1]. This measurement provides the average (arithmetic mean) GTP packet delay on the N3 interface. The measurement is optionally split into subcounters per QoS level (5QI). - This measurement is obtained as: the time when receiving a GTP Echo Response message from the gNB at the egress GTP termination, minus time when sending the GTP Echo Request to gNB at the GTP ingress termination, obtained result is divided by two. OAM adjusts the N3 delay measurements by removal of the internal delay time in gNB [note: measurements defined in 28.552 [1] already specify gNB-CU internal delays, hence the internal delay at gNB-CU is available]. - Each measurement is an integer representing the mean delay in microseconds. The number of measurements is equal to one. - Measurement taken at UPF. - Measurements are provided to OAM. QoS monitoring within NG-RAN and between NG-RAN and UE (Uu) will be specified by RAN WGs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary complexity and in-band signaling violating system architecture. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.24 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**********************First Change ***********************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for next generation new services and markets; Stage 1".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.203: "Policies and Charging control architecture; Stage 2".

[5]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS); Stage 2".

[6]	3GPP TS 24.011: "Point-to-Point (PP) Short Message Service (SMS) support on mobile radio interface: Stage 3".

[7]	IETF RFC 7157: "IPv6 Multihoming without Network Address Translation".

[8]	IETF RFC 4191: "Default Router Preferences and More-Specific Routes".

[9]	IETF RFC 2131: "Dynamic Host Configuration Protocol".

[10]	IETF RFC 4862: "IPv6 Stateless Address Autoconfiguration".

[11]	ITU-T Recommendation I.130: "Method for the characterization of telecommunication services supported by an ISDN and network capabilities of an ISDN".

[12]	ITU-T Recommendation Q.65: "The unified functional methodology for the characterization of services and network capabilities".

[13]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS): Stage 3".

[14]	IETF RFC 3736: "Stateless DHCP Service for IPv6".

[15]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[16]	3GPP TS 22.173: "IMS Multimedia Telephony Service and supplementary services; Stage 1".

[17]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station in idle mode".

[18]	3GPP TS 23.167: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; IP Multimedia Subsystem (IMS) emergency sessions".

[19]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[20]	IETF RFC 7542: "The Network Access Identifier".

[21]	3GPP TS 23.002: "Network Architecture".

[22]	3GPP TS 23.335: "User Data Convergence (UDC); Technical realization and information flows; Stage 2".

[23]	3GPP TS 23.221: "Architectural requirements".

[24]	3GPP TS 22.153: "Multimedia priority service".

[25]	3GPP TS 22.011: "Service Accessibility".

[26]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[27]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description".

[28]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol Specification".

[29]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[30]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[31]	3GPP TS 37.340: "Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Stage 2".

[32]	3GPP TS 23.214: "Architecture enhancements for control and user plane separation of EPC nodes; Stage 2".

[33]	3GPP TS 22.101: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; Service aspects; Service principles".

[34]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[35]	3GPP TS 33.126: "Lawful Interception Requirements".

[36]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[37]	3GPP TS 22.280: "Mission Critical Services Common Requirements (MCCoRe); Stage 1".

[38]	3GPP TS 23.379: "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2".

[39]	3GPP TS 23.281: "Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2".

[40]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

[41]	3GPP TS 32.240: "Charging management; Charging architecture and principles".

[42]	3GPP TS 38.401: "NG-RAN Architecture description".

[43]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[44]	IETF RFC 4960: "Stream Control Transmission Protocol".

[45]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[46]	3GPP TS 23.041: "Public Warning System".

[47]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[48]	3GPP TS 24.502: "Access to the 5G System (5GS) via non-3GPP access networks; Stage 3".

[49]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[50]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in idle mode".

[51]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[52]	3GPP TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode".

[53]	Void.

[54]	IETF RFC 4861: "Neighbor Discovery for IP version 6 (IPv6)".

[55]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[56]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[57]	IETF RFC 4555: "IKEv2 Mobility and Multihoming Protocol (MOBIKE)".

[58]	3GPP TS 29.510: "5G System: Network function repository services; Stage 3".

[59]	3GPP TS 29.502: "5G System: Session Management Services: Stage 3".

[60]	IETF RFC 7296: "Internet Key Exchange Protocol Version 2 (IKEv2) ".

[61]	3GPP TS 23.380: "IMS Restoration Procedures".

[62]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[63]	3GPP TS 23.292: "IP Multimedia Subsystem (IMS) centralized services; Stage 2".

[64]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs".

[65]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane Nodes; Stage 3".

[66]	3GPP TS 32.421: "Telecommunication management; Subscriber and equipment trace; Trace concepts and requirements".

[67]	3GPP TS 32.290: "5G system; Services, operations and procedures of charging using Service Based Interface (SBI)".

[68]	3GPP TS 32.255: "5G Data connectivity domain charging; Stage 2".

[69]	TS 38.306: "NR; User Equipment -UE) radio access capabilities".

[70]	TS 36.306: "Evolved Universal Terrestrial Radio Access -E-UTRA); User Equipment -UE) radio access capabilities".

[71]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[72]	3GPP TS 23.285: "Architecture enhancements for V2X services".

[73]	IETF RFC 2865: "Remote Authentication Dial In User Service (RADIUS)".

[74]	IETF RFC 3162: "RADIUS and IPv6".

[75]	3GPP TS 29.281: "General Packet Radio System (GPRS) Tunnelling Protocol User Plane (GTPv1-U)".

[76]	3GPP TS 26.238: "Uplink streaming".

[77]	3GPP TR 26.939: "Guidelines on the Framework for Live Uplink Streaming (FLUS)".

[78]	International Telecommunication Union (ITU), Standardization Bureau (TSB): "Operational Bulletin No. 1156"; http://handle.itu.int/11.1002/pub/810cad63-en (retrieved October 5, 2018).

[79]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[80]	3GPP TS 24.250: "Protocol for Reliable Data Service between UE and SCEF; Stage 3".

[81]	draft-ietf-mptcp-rfc6824bis-12: "TCP Extensions for Multipath Operation with Multiple Addresses".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[82]	draft-ietf-tcpm-converters-05: "0-RTT TCP Convert Protocol".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[83]	IEEE 802.1CB-2017: "IEEE Standard for Local and metropolitan area networks-Frame Replication and Elimination for Reliability".

[84]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[85]	WiFi Alliance Technical Committee, Hotspot 2.0 Technical Task Group: "Hotspot 2.0 (Release 2) Technical Specification".

[86]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[87]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[88]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SRVCC); Stage 2".

[89]	CableLabs DOCSIS MULPI: "Data-Over-Cable Service Interface Specifications DOCSIS 3.1, MAC and Upper Layer Protocols Interface Specification".

[90]	BBF TR-124 issue 5: "Functional Requirements for Broadband Residential Gateway Devices".

[91]	BBF TR-101 issue 2: "Migration to Ethernet-Based Broadband Aggregation".

[92]	BBF TR-178 issue 1: "Multi-service Broadband Network Architecture and Nodal Requirements".

[93]	BBF WT-456: "AGF Functional Requirements".

[94]	BBF WT-457: "FMIF Functional Requirements".

Editor's note:	The references to BBF WT-456 and WT-457 will be revised when finalized by BBF.

[95]	IEEE P802.1Qcc: "Standard for Local and metropolitan area networks - Bridges and Bridged Networks - Amendment: Stream Reservation Protocol (SRP) Enhancements and Performance Improvements".

[96]	IEEE 802.1Qbv-2015: "IEEE Standard for Local and metropolitan area networks -- Bridges and Bridged Networks - Amendment 25: Enhancements for Scheduled Traffic".

[xx]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[xy]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

*********************Second Change (All new text) ***********************

## 6.24	Solution #24 for Key Issues #4 and #6: Accumulated packet delay estimation for QoS monitoring and division of PDB

## 6.24.0 	General

## This solution uses monitoring of packets on the GTP-U to determine the delay budget between gNB and UPF.

## Alternative 1:

This alternative solution proposes enhancements GTP-U protocol and GTP-U end-points functionalities for the purposes of QoS monitoring and end-to-end packet delay budget division on user plane path in uplink and downlink. In this solution, a GTP-U sender performs an estimation of RTT to the next hop GTP-U receiver by sending Echo messages and measuring time that elapses between the transmission of Echo Request message and the reception of Echo Response message. A GTP-U sender is also expected to estimate its user plane processing time as a general value or a user plane processing time related to a particular QoS requirement of existing QoS flow. A GTP-U sender computes an estimated accumulated packet delay by adding RTT/2, the processing time and, if available, an estimated accumulated packet delay from an upstream GTP-U sender (i.e. an immediately preceding GTP-U sender in user plane path) thus the estimated accumulated delay represents an estimated elapsed time since a user plane packet entered 3GPP domain. More specifically, the first GTP-U sender in the uplink direction (i.e. gNB-DU or gNB) computes the estimated packet delay by adding RTT/2, its estimated processing time and estimated/measured Uu delay. In the downlink direction, the first GTP-U sender is PSA UPF for which the estimated packet delay is the sum of RTT/2 and its processing delay.

It is expected that a GTP-U sender estimates RTT periodically in order to detect changes in transport delays.

A GTP-U sender may send the estimated packet delay that it accumulated to the next hop GTP-U receiver

-	in every header of GTP-U PDU (e.g. G-PDU);

-	in a header of one or more GTP-U PDUs (e.g. G-PDU) and a GTP-U receiver is assumed to store the received value;

-	in a header of one or more GTP-U PDUs that are sent first to a GTP-U receiver within a given QoS flow; or

-	in a GTP-U signalling message.

QoS monitoring is performed by network nodes that receive and store QoS with a requested packet delay budget for QoS flow by comparing a received accumulated packet delay with a requested packet delay budget possibly also taking into the account the estimated/measured nest hop transmission delay and node's processing time. If the node determines that the packet delay exceeds the requested packet delay budget then the node triggers QoS monitoring alert signalling to a control plane network function, e.g. SMF.

Echo Request message and Echo Response message are sent outside GTP-U tunnels (the messages are using TEID set to 0). If underlaying transport is using QoS differentiation (e.g. IP DiffServ) then it is up to the implementation to ensure that the Echo messages are classified correctly and receive similar treatment by the underlaying transport as GTP-U G-PDUs carrying QoS flows (user data).

Alternative 2:

This alternative solution provides means for QoS monitoring for URLLC on the N3 interface between the UPF and NG-RAN as well as on the N9 interface between the UPF (PSA) and the I-UPF. The solution is based on the specification of the measurements specified for GTP based N3 interface in TS 28.552 [xx].

NOTE:  Measurements defined in TS 28.552 [xx] includes estimated gNB internal delays which is provided to the OAM and as such is included to the measurement data.

This measurement provides the average (arithmetic mean) GTP packet delay on the N3 and N9 interface. The measurement is optionally split into subcounters per QoS level (5QI). In the below sequence, when the measurement is to be obtained for the N9 interface, the gNB is replaced by I-UPF and the UPF is replaced by UPF (PSA).

- This measurement is obtained as: the time when receiving a GTP Echo Response message from the gNB at the egress GTP termination, minus time when sending the GTP Echo Request to gNB at the GTP ingress termination and divides it by two. OAM adjusts the N3 delay measurements by removal of the internal delay time in gNB as defined in 28.552 [xx].

- Each measurement is an integer representing the mean delay in microseconds. The number of measurements is equal to one. If the optional QoS level measurement is perfomed, the number of measurements is equal to the number of QoS level parameters.

- Measurement is taken at UPF.

As in alternative 1, GTP Echo Request message and GTP Echo Response message are sent outside GTP-U tunnels (the messages are using TEID set to 0). If underlaying transport is using QoS differentiation (e.g. IP DiffServ) and the 5QIs associated with the relevant QoS Flows are provided to the UPF over N4 then it is up to the implementation in UPF to ensure that the Echo messages are classified correctly and receive similar treatment by the underlaying transport as GTP-U G-PDUs carrying QoS flows (user data).

### 6.24.2	Procedures

### Alternative 1:

Figure 6.24.2.1: Accumulated packet delay estimation for QoS monitoring and division of PDB in downlink

NOTE:	I-UPF is shown in the figure just to show irrespective of number of UPFs in the path, the same approach can be used however presence of I-UPF is not a pre-requisite for the packet delay estimation.

The procedure of accumulated packet delay estimation for QoS monitoring and division of PDB in downlink is illustrated in Figure 6.24.2.1.

1.	PSA UPF sends one or more Echo Request messages to measure RTT to I-UPF.

2.	PSA UPF computes an accumulated packet delay by adding the half of measured RTT and estimated user plane processing time at PSA UPF that may be QoS flow specific.

3.	PSA UPF sends the accumulated packet delay to I-UPF in GTP-U PDU (e.g. G-PDU or a signalling message).

4.	I-UPF measures RTT to NG-RAN by sending one or more Echo Request messages.

5.	An accumulated packet delay at I-UPF is computed by adding the value of accumulated packet delay received from PSA UPF, the half of RTT between I-UPF and NG-RAN and estimated user plane processing time at I-UPF.

6.	I-UPF sends the accumulated packet delay computed at I-UPF to NG-RAN.

7.	As an option, NG-RAN can determine the remaining available PDB by subtracting the accumulated packet delay from the allowed PDB for the given 5QI. The available PDB defines the PDB split between RAN and 5GC.

8.	NG-RAN can measure delay encountered in the air interface (Uu) based on "RAN centric data collection and utilization for LTE and NR SI"

The above call flow is an illustration to perform QoS monitoring. This can also be performed in the Uplink direction with the RAN performing one-way delay estimation over Uu, then delay estimation over N3 and N9 can be performed by I-UPF/PSA UPF. In this case, the PSA UPF can perform total accumulated delay estimation (one-way delay of Uu, RTT for N3, N9 user plane path).

Alternative 2:

This measurement provides the average (arithmetic mean) GTP packet delay on the N3 and N9 interface. In the below sequence, when the measurement is to be obtained for the N9 interface, the gNB is replaced by I-UPF and the UPF is replaced by UPF (PSA).

- This measurement is obtained as: the time when receiving a GTP Echo Response message from the gNB at the egress GTP termination, minus time when sending the GTP Echo Request to gNB at the GTP ingress termination.

- Each measurement is an integer representing the mean delay in microseconds. The number of measurements is equal to one.

- Measurement is taken at UPF.

- UPF provides the measurement results to OAM. OAM adjusts the N3 delay measurements by removal of the internal delay time in gNB as defined in 28.552.

NOTE 1:  Measurements defined in TS 28.552 [xx] includes estimated gNB internal delays which is provided to the OAM and as such is included to the measurement data.

NOTE 2: Providing Delay measurement from UPF to OAM requires SA5 involvement.

NOTE 3: QoS monitoring within NG-RAN and between NG-RAN and UE (Uu) will be specified by RAN WGs.

GTP Echo Request message and GTP Echo Response message are sent outside GTP-U tunnels (the messages are using TEID set to 0). If underlaying transport is using QoS differentiation (e.g. IP DiffServ) and the 5QIs associated with the relevant QoS Flows are provided to the UPF over N4, then it is up to the implementation to ensure that the Echo messages are classified correctly and receive similar treatment by the underlaying transport as GTP-U G-PDUs carrying QoS flows (user data).

### 6.24.3	Impacts on Existing Nodes and Functionality

### Alternative 1:

The transmission of Echo Request message is implementation specific. However, TS 29.281 currently imposes a restriction on the frequency with which Echo Request messages may be sent. An Echo Request shall not be sent more often than every 60 seconds on each path. This doesn't prevent resending an Echo Request with the same sequence number according to the T3-RESPONSE timer. It should be noted that an Echo Request with the same sequence number should not be resend if an Echo Request is used to measure RTT. More frequent sending of Echo Request may be necessary with a possible exceptional treatment of T3-RESPONSE timer.

This solution requires definition of either a new GTP-U extension header or a signalling message.

Alternative 2:

The gNB (I-UPF) provides GTP Echo Response to the UPF. The UPF clalculates the GTP packet delay and makes it available to OAM.

To provide 5QI level of measurement, the 5QIs associated with the relevant QoS Flows (QFIs) are provided over N4 to UPF at relevant procedures.

### 6.24.4	Solution Evaluation

### Alternative 1:

This solution provides a method that allows the system to determine PDB split dynamically and perform QoS monitoring with minimum changes to GTP-U protocol. Moreover, this solution does not require precise time synchronization amongst system entities.

This solution performs packet delay estimation without considering 5QI for a given QoS flow rather it performs Echo Request / Echo Response in the corresponding user plane path outside GTP-U tunnels.

As an implementation option, GTP-U receiver can also use this method to estimate packet delay and determine the remaining PDB available in user plane path (e.g. 5G-AN part of PDB). Remaining PDB is determined by subtracting the accumulated packet delay from the allowed PDB for the given 5QI that corresponds to the QoS flow.

Alternative 2:

This solution provides a method that allows the system to perform QoS monitoring for URLLC with minimum changes applying already specified measurements for GTP based N3 interface in TS 28.552 [xx]. This solution does not require precise synchronization among system entities.  This solution does not support standardized means of AF enabled dynamic QoS monitoring for URLLC.

************ End Changes ************