3GPP TSG-SA5 Meeting #139-e 	S5-215119

# e-meeting, 11 - 20 October 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | 0189 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-16 CR TS 28.532 Fix the incorrect reference of File data reporting service to TS 32.158 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2021-09-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In Cluase 12.6.1.3.2.1.2, the reference for “MnSRoot” and “MnSVersion” is clause 4.4.2 of TS 32.158, however, clause 4.4.2 of TS 32.158 is URI structure for resources representing managed object instances, and clause 4.4.3 is the URI structure for resources not representing managed object instances. So the reference should be corrected to clause 4.4.3 of TS 32.158. Same issue in clause 12.6.1.3.2.2.2, 12.6.1.3.2.3.2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the reference to TS 32.158 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The reference to TS 32.158 is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.6.1.3.2.1.2, 12.6.1.3.2.2.2, 12.6.1.3.2.3.2, A.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge link to be added |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

12.6.1.3	Resources

12.6.1.3.1	Resource structure

12.6.1.3.1.1	Resource structure on the MnS producer

Figure 12.6.1.3.1.1-1 shows the resource structure of the File Data Reporting MnS on the MnS producer.

Figure 12.6.1.3.1.1-1: Resource URI structure of the File Data Reporting MnS on the MnS producer

Table 12.2.1.3.1.1-1 provides an overview of the resources and applicable HTTP methods.

Table 12.2.1.3.1.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| Files | …/files | GET | Retrieve the information of the available files |
| Subscriptions | …/subscriptions | POST | Create a subscription |
| Subscription | …/subscriptions/{subscriptionId} | DELETE | Delete a single subscription |
| Notification Target | {notificationTarget} | POST | Send a notification to the notification target |

12.6.1.3.1.2	Resource structure on the MnS consumer

Figure 12.6.1.3.1.2-1 shows the resource structure of the File Data Reporting MnS on the MnS consumer.

Figure 12.6.1.3.1.2-1: Resource URI structure of the File Data Reporting MnS on the MnS consumer

Table 12.6.1.3.1.2-1 provides an overview of the resources and applicable HTTP methods.

Table 12.6.1.3.1.2-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| Notification Target | {notificationTarget} | POST | Send a notification to the notification target |

12.6.1.3.2	Resource definitions

12.6.1.3.2.1	Resource "…/files"

12.6.1.3.2.1.1	Description

This resource represents the information about a collection of available files.

12.6.1.3.2.1.2	URI

Resource URI = {MnSRoot}/FileDataReportingMnS/{MnSVersion}/files

The resource URI variables are defined in table 12.6.1.3.2.1.1-1.

Table 12.6.1.3.2.1.1-1: URI variables

| Name | Definition |
| --- | --- |
| MnSRoot | See clause 4.4.3 of TS 32.158 [15] |
| MnSVersion | See clause 4.4.3 of TS 32.158 [15] |

12.6.1.3.2.1.3	HTTP methods

12.6.1.3.2.1.3.1	HTTP GET

This method shall support the URI query parameters specified in the following table.

Table 12.6.1.3.2.1.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | Description | S |
| --- | --- | --- | --- |
| fileDataType | FileDataType | Selects files based on the file data type. | M |
| beginTime | DateTime | Selects files based on the earliest time they became available | M |
| endTime | DateTime | Selects files based on the latest time they became available | M |

This method shall support the request data structures, the response data structures and response codes specified in the following tables.

Table 12.6.1.3.2.1.3.1-2: Data structures supported by the GET request body on this resource

| Data type | Description | S |
| --- | --- | --- |
| n/a | n/a | n/a |

Table 12.6.1.3.2.1.3.1-3: Data structures supported by the GET response body on this resource

| Data type | Response codes | Description | S |
| --- | --- | --- | --- |
| array(FileInfo) | 200 OK | Information about the files identified in the request | M |
| ErrorResponse | 4xx/5xx | Returned in case of an error | M |

12.6.1.3.2.2	Resource "…/subscriptions"

12.6.1.3.2.2.1	Description

This resource is a container resource for individual subscriptions.

12.6.1.3.2.2.2	URI

Resource URI: {MnSRoot}/FileDataReportingMnS/{MnSVersion}/subscriptions

The resource URI variables are defined in table 12.6.1.3.3.2.2.2-1:

Table 12.6.1.3.3.2.2.2-1: URI variables

| Name | Definition |
| --- | --- |
| MnSRoot | See clause 4.4.3 of TS 32.158 [15] |
| MnSVersion | See clause 4.4.3 of TS 32.158 [15] |

12.6.1.3.2.2.3	HTTP methods

12.6.1.3.2.2.3.1	POST

This method shall support the URI query parameters specified in table 12.6.1.3.2.2.3.1-1.

Table 12.6.1.3.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | Description | S |
| --- | --- | --- | --- |
| n/a | n/a | n/a | n/a |

This method shall support the request data structures specified in table 12.6.1.3.2.2.3.1-2 and the response data structures and response codes specified in table 12.6.1.3.2.2.3.1-3.

Table 12.6.1.3.2.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | Description | S |
| --- | --- | --- |
| Subscription | Details of the subscription to be created | M |

Table 12.6.1.3.2.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | Response codes | Description | S |
| --- | --- | --- | --- |
| Subscription | 201 Created | In case of success the representation of the created subscription is returned. | M |
| ErrorResponse | 4xx/5xx | In case of failure the error object is returned. | M |

12.6.1.3.2.2.3.2	Void

12.6.1.3.2.3	Resource ".../subscriptions/{subscriptionId}"

12.6.1.3.2.3.1	Description

This resource represents a subscription.

12.6.1.3.2.3.2	URI

Resource URI: {MnSRoot}/FileDataReportingMnS/{MnSVersion}/subscriptions/{subscriptionId}

The resource URI variables are defined in table 12.6.1.3.2.3.2-1.

Table 12.6.1.3.2.3.2-1: URI variables

| Name | Definition |
| --- | --- |
| MnSRoot | See clause 4.4.3 of TS 32.158 [15] |
| MnSVersion | See clause 4.4.3 of TS 32.158 [15] |
| subscriptionId | Subscription identifier |

12.6.1.3.2.3.3	HTTP methods

12.6.1.3.2.3.3.1	DELETE

This method shall support the URI query parameters specified in table 12.6.1.3.2.3.3-1.

Table 12.6.1.3.2.3.3-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | Description | S |
| --- | --- | --- | --- |
| n/a | n/a | n/a | n/a |

This method shall support the request data structures specified in table 12.6.1.3.2.3.3-2 and the response data structures and response codes specified in table 12.6.1.3.2.3.3-3.

Table 12.6.1.3.2.3.3-2: Data structures supported by the DELETE Request Body on this resource

| Data type | Description | S |
| --- | --- | --- |
| n/a | n/a | n/a |

Table 12.6.1.3.2.3.3-3: Data structures supported by the DELETE Response Body on this resource

| Data type | Response codes | Description | S |
| --- | --- | --- | --- |
| n/a | 204 No Content | In case of success no message body is returned | M |
| ErrorResponse | 4xx/5xx | In case of failure the error object is returned. | M |

2nd Change

A.7.2	OpenAPI document "fileDataReportingMnS.yaml"

openapi: 3.0.1

info:

title: File Data Reporting MnS

version: 16.10.0

description: >-

OAS 3.0.1 definition of the File Data Reporting MnS

© 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.532; Generic management services

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.532/

servers:

- url: '{MnSRoot}/fileDataReportingMnS/{MnSVersion}'

variables:

MnSRoot:

description: See clause 4.4.3 of TS 32.158

default: http://example.com/3GPPManagement

MnSVersion:

description: Version number of the OpenAPI definition

default: XXX

paths:

/files:

get:

summary: Read information about available files

description: >-

Information about available files is read with HTTP GET. The files for

which information shall be returned are identified with the path

component (base resource) and the query component (fileDataType, beginTime,

endTime) of the URI.

parameters:

- name: fileDataType

in: query

description: >-

This parameter selects files based on the file data type.

required: true

schema:

$ref: '#/components/schemas/FileDataType'

- name: beginTime

in: query

description: >-

This parameter selects files based on the earliest time they

became available

required: false

schema:

$ref: 'comDefs.yaml#/components/schemas/DateTime'

- name: endTime

in: query

description: >-

This parameter selects files based on the latest time they

became available

required: false

schema:

$ref: 'comDefs.yaml#/components/schemas/DateTime'

responses:

'200':

description: >-

'Success case ("200 OK").

The resources identified in the request for retrieval are returned

in the response message body.'

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/FileInfo'

default:

description: Error case.

content:

application/json:

schema:

$ref: 'comDefs.yaml#/components/schemas/ErrorResponse'

/subscriptions:

post:

summary: Create a subscription

description: >-

To create a subscription the representation of the subscription is

POSTed on the /subscriptions collection resource.

requestBody:

required: true

content:

application/json:

schema:

$ref: 'faultMnS.yaml#/components/schemas/Subscription'

responses:

'201':

description: >-

Success case ("201 Created").

The representation of the newly created subscription resource shall

be returned.

content:

application/json:

schema:

$ref: 'faultMnS.yaml#/components/schemas/Subscription'

headers:

Location:

description: URI of the newly created subscription resource

required: true

schema:

type: string

default:

description: Error case.

content:

application/json:

schema:

$ref: 'comDefs.yaml#/components/schemas/ErrorResponse'

callbacks:

notifyFileReady:

'{request.body#/consumerReference}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NotifyFileReady'

responses:

'204':

description: >-

Success case ("204 No Content").

The notification is successfully delivered. The response message

body is absent.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'comDefs.yaml#/components/schemas/ErrorResponse'

notifyFilePreparationError:

'{request.body#/consumerReference}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NotifyFilePreparationError'

responses:

'204':

description: >-

Success case ("204 No Content").

The notification is successfully delivered. The response message

body is absent.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'comDefs.yaml#/components/schemas/ErrorResponse'

/subscriptions/{subscriptionId}:

delete:

summary: Delete a subscription

description: >-

The subscription is deleted by deleting the corresponding subscription

resource. The resource to be deleted is identified with the path

component of the URI.

parameters:

- name: subscriptionId

in: path

description: Identifies the subscription to be deleted.

required: true

schema:

type: string

responses:

'204':

description: >-

Success case ("204 No Content").

The subscription resource has been deleted. The response message body

is absent.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'comDefs.yaml#/components/schemas/ErrorResponse'

components:

schemas:

FileDataType:

type: string

enum:

- Performance

- Trace

- Anatytics

- Proprietary

FileNotificationTypes:

type: string

enum:

- notifyFileReady

- notifyFilePreparationError

FileInfo:

type: object

properties:

fileLocation:

$ref: 'comDefs.yaml#/components/schemas/Uri'

fileSize:

type: integer

fileReadyTime:

$ref: 'comDefs.yaml#/components/schemas/DateTime'

fileExpirationTime:

$ref: 'comDefs.yaml#/components/schemas/DateTime'

fileCompression:

type: string

fileFormat:

type: string

fileDataType:

$ref: '#/components/schemas/FileDataType'

NotifyFileReady:

allOf:

- $ref: 'comDefs.yaml#/components/schemas/NotificationHeader'

- type: object

properties:

fileInfoList:

type: array

items:

$ref: '#/components/schemas/FileInfo'

additionalText:

type: string

NotifyFilePreparationError:

allOf:

- $ref: 'comDefs.yaml#/components/schemas/NotificationHeader'

- type: object

properties:

fileInfoList:

type: array

items:

$ref: '#/components/schemas/FileInfo'

reason:

type: string

additionalText:

type: string

End of Change