3GPP TSG-SA5 Meeting #136-e	S5-212367

# Online,  1st Mar 2021 - 9th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0291 | rev |  | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 CR 28.552 Add measurements to cover all accessibility types |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | ePM_KPI_5G |  |  |  |  |  | Date: |  |  | 2021-02-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The current measurements defined and used for the KPI DRB Accessibility for UE services are not covering the accesses that are made using the RRC Resume procedure and the accesses made using the procedures of fallback to RRC Setup from RRC Resume or RRC Re-establishment procedures. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the following measurements related to support the updated KPI DRB Accessibility for UE services: - Number of RRC connection re-establishment attempts followed by RRC Setup - Number of RRC connection resuming attempts followed by RRC Setup |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will not be possible to define an accessibility KPI covering all possible accesses. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.17.x (new), 5.1.1.18.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The proposed measurements was Endorsed in S5-211093 in SA5#135e. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st modified section

##### 5.1.1.17.x	Number of RRC connection re-establishment attempts followed by RRC Setup

a)	This measurement provides the number of RRC connection re-establishment attempts where no UE context could be retrieved and therefore fallback to RRC Setup procedure was attempted.

b)	CC.

c)	On transmission of RRCSetup message to UE, after first having received RRCReestablishmentRequest message from that UE (see TS 38.331[20]).

d)	Each measurement is an integer value.

e)	The measurement name has the form RRC.ReEstabFallbackToSetupAtt.

f)	NRCellCU.

g)	Valid for packet switching.

h)	5GS.

Next modified section

##### 5.1.1.18.x	Number of RRC connection resuming attempts followed by RRC Setup

a)	This measurement provides the number of RRC connection resuming attempts where no UE context could be retrieved and therefore fallback to RRC Setup procedure was attempted.

b)	CC.

c)	On transmission of RRCSetup message to UE, after first having received RRCResumeRequest message or RRCResumeRequest1 from UE, the relevant subcounter per resume cause is stepped.

d)	Each subcounter is an integer value.

e)	The measurement name has the form RRC.ResumeFallbackToSetupAtt.cause.

Where cause indicates the RRC resume cause defined in clause 6.2.2 of TS 38.331 [20].

f)	NRCellCU.

g)	Valid for packet switching.

h)	5GS

End of modified section