### PS Domain feature list evaluation for release 2000.

### Key to Table

E = Essential for release 00, launch of a commercial all-IP network is not viable with these missing or required in R00 terminal specifications to enable forward compatibility to future releases

D= Desirable for R00, important features to enable a competitive and successful service launch, but could be slipped to R01

R01+ = could wait for these features, but hooks are required in R00 to enable them to be added later

No = Not needed at PS domain

Note:	CS Domain services are not considered at this document. S1 has agreed to maintain the  existing rel 99 requirements at rel 2000, allowing full service continuity.

| #### Feature Name | Short Description | ## R99 | ## E R00 | ## D R00 | ## R01+ | ## No | ## Comments/Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ###### DTAP CC |  |  |  |  |  |  | i.e. 04.08 based CC. Seamless support for existing GSM services |
| Rel 99 CS terminal support fully by PS domain. |  |  |  |  |  |  | Terminology need to be clarified with S2. |
| IP CC |  |  |  |  |  |  | i.e. H.323 / SIP related IP multimedia services. |
| #### Basic teleservices (PS Domain) |  |  |  |  |  |  |  |
| Multimedia IP Call | Includes IP telephony, all real time calls single and multi-media, processed by IP CC (e.g. H.323 or SIP) |  |  |  |  |  |  |
| Emergency Voice Call |  |  |  |  |  |  | This must use the existing emergency numbering schemes. Must be compliant with FCC mandates |
| Short message service (CB) | SMS CB as specified by 23.041. |  |  |  |  |  |  |
| Short message service (PP) | SMS as specified by 23.040. |  |  |  |  |  | Same reqs as Rel 99 GPRS(?) |
| #### Basic bearer services (PS Domain) |  |  |  |  |  |  |  |
| IPv4 connectivity |  |  |  |  |  |  | Same reqs as Rel 99 GPRS |
| IPv6 connectivity |  |  |  |  |  |  | Same reqs as Rel 99 GPRS(?) |
| #### Interworking |  |  |  |  |  |  |  |
| Speech to/from PSTN / ISDN / 2G mobile / 3G mobile |  |  |  |  |  |  |  |
| Multimedia to/from Internet |  |  |  |  |  |  |  |
| Modem and ISDN interworking | Access to PSTN / ISDN dial up –data services |  |  |  |  |  |  |
| Roaming |  |  |  |  |  |  |  |
| GSM to All IP |  |  |  |  |  |  | (U)SIM Roaming not terminal roaming |
| All IP to GSM |  |  |  |  |  |  | (U)SIM Roaming not terminal roaming |
| ANSI-41 to All IP |  |  |  |  |  |  |  |
| All IP to ANSI-41 |  |  |  |  |  |  |  |
| Handover |  |  |  |  |  |  |  |
| Speech from PS Domain to CS-GSM /UMTS |  |  |  |  |  |  |  |
| #### Supplementary Services (PS Domain) |  |  |  |  |  |  | Multimedia (/single media) IP Telephony aware supplementary services. To be considered only from end-user need view point. Standardization, service capabilities and implementation FFS |
| Multimedia Call Barring | Enables mobile subscriber to have barring of certain categories of outgoing multimedia calls. |  |  |  |  |  | Also includes incoming multimedia calls and barring when roaming. |
| Multimedia Call Forwarding | Forwarding of multimedia call triggered by conditions of Unconditional, Busy, No Reply, and Not Reachable |  |  |  |  |  |  |
| Multimedia Call Transfer | Enables served mobile subscriber who has two multimedia calls, each of which can be an incoming or outgoing multimedia call, to connect the other parties in the two multimedia calls and release the served mobile subscriber’s own connection. |  |  |  |  |  |  |
| Multimedia Call Deflection Service | Enables the served mobile subscriber to respond to an incoming multimedia call offered by the network by requesting redirection of this multimedia call to another address or location. |  |  |  |  |  |  |
| Multimedia Call Holding | Allows served mobile subscriber to interrupt communication on an existing active multimedia call and then subsequently re-establish communication |  |  |  |  |  |  |
| Multimedia Call Waiting | Permits mobile subscriber to be notified of an incoming multimedia call while the mobile subscriber is engaged in an active or held multimedia call. Subscriber can either accept, reject, ignore, or deflect the incoming multimedia call. |  |  |  |  |  |  |
| Advice of Charge | Supply user sufficient information to allow real-time estimate of the bill that will be eventually levied in the home PLMN. |  |  |  |  |  |  |
| Caller Identification and restriction | Similar to CLIP, CLIR, CNAP and CNAR. |  |  |  |  |  | Could include additional IP related information such as IP address. |
| Multimedia conferencing | Similar to Multiparty but is applicable to multimedia calls. |  |  |  |  |  |  |
| #### Network Services |  |  |  |  |  |  |  |
| Operator Determined Barring (ODB) | Allows service providers to regulate subscriber access to services by the barring of certain categories of outgoing or incoming multimedia calls and packet services. ODB could terminate ongoing multimedia calls and could bar future multimedia calls and packet services. |  |  |  |  |  |  |
| Intelligent Service (IN) Support for Multimedia Services | Provides mechanisms to support multimedia services consistently & independently of the serving network. |  |  |  |  |  | Similar to WIN and CAMEL but needs to support multimedia calls. Implementation is FFS. |
| (U)SIM Toolkit | Feature provides a set of facilities which allow the (U)SIM to interact with external entities (e.g. the network, the Mobile Equipment, or the user) to enable value-added multimedia applications to exist in the (U)SIM. |  |  |  |  |  |  |
| OSA for new elements | Provision of an API for controlled, secure and accountable access to multimedia service capability features by applications, based on user subscription profile |  |  |  |  |  |  |
| LCS for GPRS/PS domain | Support of LCS on the PS domain required to meet regulatory and commercial requirements (e.g., 3GPP 22.071) |  |  |  |  |  | Exact work required is for FFS. Must be compliant with FCC mandates. |
| SoLSA | SoLSA shall facilitate user-dependent radio resource selection based on LSA (e.g. when user is located at his office, radio coverage provided with indoor radio solutions should be preferred). |  |  |  |  |  |  |
| Lawful Surveillance / Intercept |  |  |  |  |  |  | Must be compliant with FCC mandates. |
| Number Portability | Ability for subscriber to change service providers while retaining the original directory number. Includes mobile to mobile, mobile to landline, & landline to mobile number portability scenarios. |  |  |  |  |  |  |
| Mobile Station Application Execution Environment (MExE) | Provides standardized execution environment in an MS, and an ability to negotiate its supported capabilities with a MExE service provider, allowing applications to be developed independently of any MS platform. |  |  |  |  |  |  |
| Personalization of Mobile Equipment (ME) | Storage of information in the ME which limits the SIMs which will operate with the ME. |  |  |  |  |  |  |
| Pre-Paid Multi-media Services | Basic and supplemental services for multi-media communications provided on a pre-paid basis. Includes real-time decrement of account balance with notification to subscriber at low and depleted balance. |  |  |  |  |  | Support of roaming subscribers is required. |
| Advanced Addressing | Support of symbolic and advanced addressing |  |  |  |  |  |  |
| System Selection | Ability for the mobile equipment to choose a preferred service provider, based upon geographic location, frequency band preferences, available operators, etc. Also the ability to force a mobile station to "disallow" service from a "forbidden" service provider, and to force a mobile station to use "home" services. Service provider lists must be downloadable over-the-air. |  |  |  |  |  | Similar to ANSI-136 Intelligent Roaming. |
| Over-the-Air Service Provisioning | Ability to download parameters to either the SIM or ME for provisioning of services. This includes both subscription parameters as well as operator-specific parameters. |  |  |  |  |  | Similar to ANSI-136 OTASP and OTAPA. |