TSG-SA Working Group 1 (Services) meeting #5	TSG S1 (99)888

San Diego, USA 29th Nov – 3rd Dec 1999	Agenda Item: 6.6

# TSG CN WG2	Tdoc 3GPP N2-99 J87

Phoenix, ARIZONA

15 - 19 November 1999

From:	TSG CN WG2

To:	TSG SA WG1, TSG SA WG2

Title:	LS on Dialled Services in CAMEL Phase 3

TSG CN WG2 has further progressed the Work on CAMEL dialled services within the CAMEL Phase 3 package. The work was based on the requirements given in the CAMEL Stage 1 specification 3G 22.078, Version 3.1.0.

The Stage 1 specification introduces the concepts of network and subscribed dialled services. According to the service requirements the CSE may only allow further call processing either unchanged or with modified information for both types of services. In order to have a more flexible handling TSG CN WG2 have agreed to enhance the capabilities of dialled services by allowing the CSE to bar the call and to perform charging activities, i.e. to add Free Format Data to the Call Data Record.

TSG SA WG1 is asked to confirm these new service capabilies for being part of CAMEL Phase 3 and to update the Stage 1 specification accordingly.

According to the service requirements it is possible to have for mobile originated and forwarding call handling a subscribed CAMEL service process, a subscribed dialled service process and a network dialled service process in sequence! In all three CAMEL relationships a CSE may request the serving MSC to add Free Format Data parameters to the Call Data Record. This data shall be treated transparently and independently in the serving network, i.e. a CAMEL Service Logic shall not be able to modify or overwrite the Free Format Data provided by another Service Logic within the same call.

TSG SA WG5 is asked to enhance the Release 99 description of mobile originated and call forwarding Call Data Records to cope with these requirements.