TSG-SA Working Group 1 (Services) meeting #6	TSG S1 (99) 971

San Diego, USA 29th Nov – 3rd Dec 1999	Agenda Item: 6.2

Title:			Cell Broadcast Service (CBS) & GPRS

Source:		Mannesmann Mobilfunk

The Cell Broadcast Service has become a very important service for many mobile operators. It is used for various local information services and telematic services and the usage is increasing. Mannesmann Mobilfunk has studied the impacts on CBS with the introduction of GPRS and identified serious problems.

In particular GPRS class B and C terminals seem not be prepared to receive cell broadcast messages when attached to GPRS. This limitation may hinder the introduction of GPRS because service continuity for existing services can not be guaranteed. From our perspective there seems to be no technical restriction for a MS to listen to the CBCH (Cell Broadcast Channel) when camping on the PCCCH (Pack Common Control Channel).

We consider the necessary CR’s as clarification. CR’s shall become at least part of Release ’99.

As agreed on SMG meeting Mannesmann Mobilfunk is asking S1 to support the requirement for GPRS class B and C mobiles to support CBS when attached to the GPRS service and to task STC’s to draft necessary changes in the respective standards (02.60 / 22.060 & 03.64 / 23.064).

GSM 03.64-800, section 6.1.4 on CBS: "An MS attached to GPRS shall not be required to monitor the CBCH channel if a PCCCH exists. [EDITOR'S NOTE: Service is not specified]"