TSG-SA Working Group 1 (Services) meeting #5	TSG S1 (99) 980

San Diego, USA 29th Nov – 3rd Dec 1999	Agenda Item: 4.3.3

3GPP TSGN WG1 meeting #8	Tdoc N1-99D27

25-29 October 1999, Kobe, Japan	(Revision of Tdoc N1-99C83)

To	S2, R2, SMG2 WPA, S1

- Source: 	CN1

Title: 	Response on LS on Information about current status on UE idle mode operation

- CN1 thanks R2 for the LS (R2-99D29) (attached).

CN1 agrees that UMTS PLMN search procedures should be specified. There could be two possibilities :

- A: write CRs against  23.022 PLMN selection part

- B: draft a new TS under N1 responsibility for this purpose

In order to align with R2 work, since 23.022 Cell Selection part seems not to be modified for UMTS, CN1 would rather propose B.

Then, in order to tackle R2 detailed proposal, the question is asked to S1 whether there is a stage one requirement on this list of Access Systems and to S2 whether it should be managed by MM or RR.

Depending on several requirements  (is this list on the SIM, can it be modified by SATK, is it broadcast on the BCCH, is it handled by Access Stratum procedure results ? by Non Access Stratum  procedures ? is it “transparent” for the Non Access Stratum as part of the Access Stratum services ?…), it seems not so obvious for N1 that this list should be managed in N1 group or in RAN group.

As an example, the PLMN list is managed by MM mainly because MM procedures make an intensive use of this list of this list (PLMN search, location registration procedures).

- CN1 kindly asks R2 , SMG2WPA, S1, and S2 their opinion on this issue, before starting to work on it.