TSG-SA Working Group 1 meeting #7	TSG S1 (00)049

Sophia Antipolis, France	Agenda Item: 5.14

| CHANGE REQUEST No : |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification / Report UMTS |  |  |  | 22.121 |  | Version: |  |  | 3.1.0 |  |  |  |
| Submitted to TSG_SA | 7 |  | for approval |  | X |  | without presentation ("non-strategic") |  |  |  |  |  |
| list TSG plenary meeting no. here  |  | for information |  |  |  |  | with presentation ("strategic") |  |  |  | X |  |
| PT SMG CR cover form is available from: http://docbox.etsi.org/tech-org/smg/Document/smg/tools/CR_form/crf28_1.zip |  |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | USIM |  | TE |  | Network | x |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | Virtual Home Environment |
| --- | --- |

| Source: | Siemens/Ericsson | Date: | 22/1/00 |
| --- | --- | --- | --- |

| Subject: | Modification of sections 10.2.1, 10.2.7 and 10.2.8 |
| --- | --- |

| Category: | F Correction |  | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 |  |
| And one release | C Functional modification of feature | X |  | Release 98 |  |
| Only shall be | D Editorial modification |  |  | UMTS 99 | X |
| Marked with an X) |  |  |  |  |  |

| Reason for change: | There have been long discussions around the issue "Message Transfer SCF" and "Data download". Especially, it has been considered as problematic that the to be changed sections imply the possibility to download data to the application server. This contradicts the view of the contributors that the application server establishes only a controlling relationship but not a bearer channel to the Service Capability Servers and the underlying network (via the SCS). The proposed changes try to clarify this point by - removing the term "message transfer" and replacing it by "information transfer". - introduction of the appropriate functions - deleting the section data download completely since it is already covered by the "information transfer" |
| --- | --- |

| Clauses affected: | 10.2.1, 10.2.7 & 10.2.8 |
| --- | --- |

| Other specs | Other releases of same spec |  |  List of CRs: |  |
| --- | --- | --- | --- | --- |
| Affected: | Other core specifications | X |  List of CRs: | 23.127 |
|  | MS test specifications / TBRs |  |  List of CRs: |  |
|  | BSS test specifications |  |  List of CRs: |  |
|  | O&M specifications |  |  List of CRs: |  |

| Other comments: |  |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

### 10.2.1	Session Control service capability features

This section details the Session Control related service capability features.  Session Control service capability features shall offer the functionality to establish, maintain, modify and release bearers to/from other parties or entities.

Herein, the term “session” can mean anything from a simple voice call to a complex multimedia “call” (including exchange of non delay-sensitive data). To define the necessary service capability features it is proposed to use a generic model (including the "session party handling").

For example, the following Session Control service capability features shall be provided (the list is not exhaustive):

- initiate and create session (e.g. used to set-up a Telephony session “out of the blue”)

- allow the session to continue with modified information (e.g. changed destination number)

- release the session (i.e. removing all parties from the session)

- add bearer to the session

- remove bearer from the session

- resume bearer to the session (i.e. move party from “on-hold” into Telephony Session)

- suspend bearer from the session  (i.e. move party from Telephony Session to “on hold”)

- request session  information (i.e. information like session duration, session end time)

- supervise session (e.g. monitor for session duration or data volume, tariff switching moments and changes in QoS)

- presentation of, or restriction of, information associated with a party involved in a session  (e.g. calling line ID, calling name)

- collect information from user (i.e the application shall be able to collect a information from the user. For example, the user might enter a PIN code)

For each session it shall be possible to specify:

- the desired media type (e.g. video, voice, non-real time data etc.)

- the events on which monitoring is required ([3])

Note: the mapping to service capabilities is for further study. (It shall be investigated to which extend the requirements above fit to CAMEL, MEXE and other service capabilities.)

### 10.2.7	Information Transfer service capability feature

The Information Transfer service capability feature shall enable an application to inform an end user about the presence of existing information for her.  Physically, this information is then sent by the underlying network . Information for the end user may be available e.g. as an announcement or any data to be requested by the end user. The indication from the application may be received by the user as e.g. a SMS or USSD message. The Information Transfer service capability feature provides the means to inform the underlying network that an indication shall be sent to the user containing sufficient reference which form of information is available and where to be found, e.g. announcement ID, URL, a string, etc.

Furthermore, the Information Transfer service capability features shall provide the means to initiate the collection of information from the user, e.g. certain digits a string, etc.

The following service capability feature shall be supported:

- Send information notification

The Send information notification service capability feature provides the means to inform the underlying network that an indication shall be sent to the user containing sufficient reference which form of information is available and where to be found, e.g. announcement ID, URL a string, etc.