3GPP TSG-SA WG1 	TSG S1 (00) 224

Beijing, China,	 Agenda Item:

10th – 14th April 2000

| CHANGE REQUEST No : |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification / Report UMTS |  |  |  | 22.082 |  | Version: |  |  | 3.0.1 |  |  |  |
| Submitted to TSG_SA | 8 |  | for approval |  | X |  | without presentation ("non-strategic") |  |  |  | X |  |
| list TSG plenary meeting no. here  |  | for information |  |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form is available from: http://docbox.etsi.org/tech-org/smg/Document/smg/tools/CR_form/crf28_1.zip |  |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | USIM |  | TE |  | Network | x |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | Call Forwarding Enhancements |
| --- | --- |

| Source: | Vodafone Airtouch | Date: | 27/03/00 |
| --- | --- | --- | --- |

| Subject: | Support for long forwarded-to numbers |
| --- | --- |

| Category: | F Correction | X | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 |  |
| And one release | C Functional modification of feature |  |  | Release 98 |  |
| Only shall be | D Editorial modification |  |  | UMTS 99 | X |
| Marked with an X) |  |  |  |  |  |

| Reason for change: | To provide a consistent set of documents for release 99 it is necessary to align stage 1 with stage 2 . |
| --- | --- |

| Clauses affected: | 1.2.3,1.4.6,1.4.7,2.2.3,2.4.4,2.4.6,2.4.7,3.2.3,3.4.4,3.4.6,3.4.7,4.2.3,4.4.6,4.4.7 |
| --- | --- |

| Other specs | Other releases of same spec |  |  List of CRs: |  |
| --- | --- | --- | --- | --- |
| Affected: | Other core specifications |  |  List of CRs: |  |
|  | MS test specifications / TBRs |  |  List of CRs: |  |
|  | BSS test specifications |  |  List of CRs: |  |
|  | O&M specifications |  |  List of CRs: |  |

| Other comments: |  |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

### 1.2.3	Terminology

A served mobile subscriber is a mobile subscriber of a particular PLMN access who is requesting that calls to her number be forwarded. This subscriber may also be referred to as the forwarding subscriber or the called subscriber.

A forwarded-to subscriber is a subscriber to whom the call shall be forwarded. A long forwarded-to number is a forwarded-to number of length greater than 15 digits and less or equal to 28 digits.

## 1.3	Normal procedures with successful outcome

### 1.3.1	Provision

The supplementary service will be provisioned for all Basic Services subscribed to and to which it is applicable.

The service can be offered with several subscription options. Options apply to all Basic services subscribed to. For each subscription option, only one value can be selected.

Subscription options	Value

Calling subscriber receives 	- No

notification the call has been forwarded	- Yes

MSISDN of the served subscriber can be	- No

presented to the forwarded-to subscriber	- Yes

### 1.3.2	Withdrawal

The service will be withdrawn at the subscriber's request or for administrative reasons.

### 1.3.3	Registration

The following information has to be registered in the network:

1)	the forwarded-to number, which may be accompanied by a forwarded-to sub-address;

2)	information as to whether all calls or all calls of one (or several) specific Basic service groups should be forwarded.

If no Basic Service code is inserted by the user this is interpreted as All Basic Services.

Registration can take place either by the service provider or with an appropriate control procedure by the subscriber.

Verification, where possible, of the forwarded-to number should be accomplished before accepting the call forwarding request. This verification is done by a simple check of the forwarded-to number to see if the number is within the allowed number range.

When the mobile subscriber so registers call forwarding unconditional, the network will return notification of acceptance or rejection of the request. This notification will include the forwarded-to number to which call forwarding unconditional is registered.

### 1.3.4	Erasure

A previous registration can be erased in either of three ways:

Firstly, the subscriber can specifically erase a previous registration with an appropriate control procedure.

### 1.4.6	Invocation

Within a PLMN or different PLMNs the number of tandem forwarding connections should be limited. The maximum number of tandem forwarding connections should be limited to a value between 1 and 5. This is to prevent infinite looping.

If the limit of successive forwardings of a call has already been reached, an unsuccessful call set up indication is sent backwards.

If the forwarded call cannot be completed to the forwarded-to destination, then the network will clear the forwarded part of the call and an unsuccessful call set up indication is sent backwards.

If the HPLMN receives a request for routeing information from an IPLMN which does not support long forwarded-to numbers and the destination subscriber has registered a long forwarded-to number, the HPLMN shall:

- Supply an alternative forwarded-to number of less than 16 digits, or

- Allow the call to continue to the called subscriber (i.e. suppress the invocation of CFU), or

- Instruct the IPLMN to release the call with a suitable error code, e.g. forwarding violation

NOTE:	Call forwarding unconditional applies only to the Basic services subscribed to. Calls to a directory number requesting a Basic service which is not subscribed to will never be forwarded.

### 1.4.7 Interrogation

It is possible that a user has registered a long forwarded-to number but has an MS that does not support long forwarded-to numbers. For this case if the user interrogates call forwarding then the service state shall be indicated without the number being displayed.

## 1.5	Alternate procedures

None identified.

## 1.6	Interactions with other supplementary services

#### 1.6.81.1	Calling line identification presentation

See GSM 02.81.

#### 1.6.81.2	Calling line identification restriction

See GSM 02.81.

#### 1.6.81.3	Connected line identification presentation

See GSM 02.81.

#### 1.6.81.4	Connected line identification restriction

See GSM 02.81.

#### 1.6.82.2	Call forwarding on mobile subscriber busy

If Call forwarding unconditional is active, activation of Call forwarding on mobile subscriber busy is rejected. The served mobile subscriber shall be informed of this supplementary service incompatibility.

### 2.2.2	Applicability to telecommunication services

The applicability of this supplementary service is defined in TS 22.004.

### 2.2.3	Terminology

A served mobile subscriber is a mobile subscriber of a particular PLMN access who is requesting that calls to her number be forwarded. This subscriber may also be referred to as the forwarding subscriber or the called subscriber.

A forwarded-to subscriber is a subscriber to whom the call shall be forwarded. . A long forwarded-to number is a forwarded-to number of length greater than 15 digits and less or equal to 28 digits.

## 2.3	Normal procedures with successful outcome

### 2.3.1	Provision

The supplementary service will be provisioned for all Basic Services subscribed to and to which it is applicable.

The service can be offered with several subscription options. Options apply to all Basic services subscribed to. For each subscription option, only one value can be selected.

Subscription options are summarized below:

Subscription options	Value

Forwarding subscriber receives 	- No

notification that the call has been forwarded	- Yes

Calling subscriber receives 	- No

notification that the call has been forwarded	- Yes

MSISDN of the served subscriber can be	- No

presented to the forwarded-to subscriber	- Yes

In case of User Determined User Busy (UDUB) a notification towards the forwarding subscriber will not be given.

### 2.3.2	Withdrawal

The service will be withdrawn at the subscriber's request or for administrative reasons.

### 2.3.3	Registration

The following information has to be registered in the network:

1)	the forwarded-to number, which may be accompanied by a forwarded-to sub-address;

2)	information as to whether all calls or all calls of one (or several) specific Basic service group(s) should be forwarded.

If no Basic Service Code is inserted by the user this is interpreted as All Basic Services.

Registration can take place either by the service provider or with an appropriate control procedure by the subscriber. Verification, where possible, of the forwarded-to number should be accomplished before accepting the call forwarding

-

### 2.4.4	Activation

If there is no CFB forwarded-to-number registered against a Basic Service group, the activation request for that Basic Service group shall be denied.

Examples of causes for a not accepted activation request:

-	service not provisioned;

-	conflicting situation with other supplementary services;

- service not registered.

If a user with a long forwarded-to number registered roams to a visited network not supporting long forwarded-to numbers then the home network shall:

- Supply an alternative forwarded-to number of less than 16 digits, or

- Make CFB quiescent

### 2.4.5	Deactivation

If the network cannot accept a deactivation request, an appropriate cause will be returned to the user, e.g. Service not Active.

### 2.4.6	Invocation

Within a PLMN or different PLMNs the number of tandem forwarding connections should be limited. The maximum number of tandem forwarding connections should be limited to a value between 1 and 5. This is to prevent infinite looping.

If the limit of successive forwardings of a call has already been reached, an unsuccessful call set up indication is sent backwards.

If the forwarded call cannot be completed to the forwarded-to destination, then the network will clear the forwarded part of the call and an unsuccessful call set up indication is sent backwards.

If the IPLMN does not support long forwarded-to numbers and the served mobile subscriber has registered a long forwarded-to number for CFB, then optimal routeing of late call forwarding shall not be invoked.

### 2.4.7 Interrogation

It is possible that a user has registered a long forwarded-to number but has an MS that does not support long forwarded-to numbers. For this case if the user interrogates call forwarding then the service state shall be indicated without the number being displayed.

## 3.1	Definition

This service permits a called mobile subscriber to have the network send all incoming calls, or just those associated with a specific Basic service group, addressed to the called mobile subscriber's directory number and which meet no reply to another directory number. The ability of the served mobile subscriber to originate calls is unaffected. If this service is activated, a call is forwarded only if the call meets no reply.

## 3.2	Description

### 3.2.1	Description

The served mobile subscriber can request a different forwarded-to number for each Basic service group containing a basic service to which she has subscribed.

### 3.2.2	Applicability to telecommunication services

The applicability of this supplementary service is defined in TS 22.004.

### 3.2.3	Terminology

A served mobile subscriber is a mobile subscriber of a particular PLMN access who is requesting that calls to her number be forwarded. This subscriber may also be referred to as the forwarding subscriber or the called subscriber.

A forwarded-to subscriber is a subscriber to whom the call shall be forwarded. A long forwarded-to number is a forwarded-to number of length greater than 15 digits and less or equal to 28 digits.

## 3.3	Normal procedures with successful outcome

### 3.3.1	Provision

The supplementary service will be provisioned for all Basic Services subscribed to and to which it is applicable.

The service can be offered with several subscription options. Options apply to all Basic services subscribed to. For each subscription option, only one value can be selected.

Subscription options are summarized below:

Subscription options	Value

Forwarding subscriber receives 	- No

notification that the call has been forwarded	- Yes

Calling subscriber receives 	- No

notification that the call has been forwarded	- Yes

MSISDN of the served subscriber can be	- No

presented to the forwarded-to subscriber	- Yes

### 3.3.2	Withdrawal

The service will be withdrawn at the subscriber's request or for administrative reasons.

### 3.4.4	Activation

If there is no CFNRy forwarded-to-number registered against a Basic Service group, the activation request for that Basic Service group shall be denied. [However, if there is CFNRy forwarded-to-numbers stored against other Basic Service groups contained in the same activation request, these will be activated.]

Examples of causes for a not accepted activation request:

-	service not provisioned;

-	conflicting situation with other supplementary services;

- service not registered.

If a user with a long forwarded-to number registered roams to a visited network not supporting long forwarded-to numbers then the home network shall:

- Supply an alternative forwarded-to number of less than 16 digits, or

- Make CFNRy quiescent

### 3.4.5	Deactivation

If the network cannot accept a deactivation request, an appropriate cause will be returned to the user, e.g. Service not Active.

### 3.4.6	Invocation

Within a PLMN or different PLMNs the number of tandem forwarding connections should be limited. The maximum number of tandem forwarding connections should be limited to a value between 1 and 5. This is to prevent infinite looping.

If the limit of successive forwardings of a call is reached and an attempt is made to forward the call an additional time, then the forwarded call shall be treated as in the following paragraph.

If the forwarded call cannot be completed to the forwarded-to destination, then the network will clear the call towards the calling user with an indication that the call cannot be completed. This indication shall not include information that the call has been forwarded.

If the IPLMN does not support long forwarded-to numbers and the served mobile subscriber has registered a long forwarded-to number for CFNRy, then optimal routeing of late call forwarding shall not be invoked.

NOTE 1:	The alerting of the forwarding user shall have ceased on invocation of the CFNRy Service.

NOTE 2:	The alerting of the calling subscriber should, if possible, not cease on invocation of CFNRy.

NOTE 3:	Call forwarding on no reply applies only to the Basic services subscribed to. Calls to a directory number requesting a Basic service which is not subscribed to will never be forwarded.

### 3.4.7 Interrogation

It is possible that a user has registered a long forwarded-to number but has an MS that does not support long forwarded-to numbers. For this case if the user interrogates call forwarding then the service state shall be indicated without the number being displayed.

outside the home PLMN country.

## 3.7	Interworking considerations

If the forwarded-to number is not a PLMN- or ISDN-number, then an interworking situation is said to exist.

If a forwarded call meets an interworking situation, then an interworking indication should be sent to the calling party. When interworking with non-PLMN or non-ISDN networks, tones and announcements will be required.

NOTE1:	The number of times a call has been forwarded once it has exited the PLMN or ISDN may not be limited.

NOTE2:	Tones and/or announcements should not be provided to the calling party if the information transfer capability is set to UDI.

# 4	Call Forwarding on Mobile Subscriber Not Reachable

## 4.1	Definition

This service permits a called mobile subscriber to have the network send all incoming calls, or just those associated with a specific Basic service group, addressed to the called mobile subscriber's directory number, but which is not reachable, to another directory number. The ability of the served mobile subscriber to originate calls is principally unaffected, but practically it is affected if the mobile subscriber is de-registered, if there is radio congestion or if the mobile subscriber for example is being out of radio coverage. If this service is activated, a call is forwarded only if the mobile subscriber is not reachable.

## 4.2	Description

### 4.2.1	Description

The served mobile subscriber can request a different forwarded-to number for each Basic service group containing a basic service to which he has subscribed.

### 4.2.2	Applicability to telecommunication services

The applicability of this supplementary service is defined in TS 22.004.

### 4.2.3	Terminology

A served mobile subscriber is a mobile subscriber of a particular PLMN access who is requesting that calls to his number be forwarded. This subscriber may also be referred to as the forwarding subscriber or the called subscriber. A forwarded-to subscriber is a subscriber to whom the call shall be forwarded. . A long forwarded-to number is a forwarded-to number of length greater than 15 digits and less or equal to 28 digits.

### 4.4.4	Activation

If there is no CFNRc forwarded-to-number registered against a Basic Service group, the activation request for that Basic Service group shall be denied.

Examples of causes for a not accepted activation request:

-	service not provisioned;

-	conflicting situation with other supplementary services;

-	service not registered.

If a user with a long forwarded-to number registered roams to a visited network not supporting long forwarded-to numbers then the home network shall:

- Supply an alternative forwarded-to number of less than 16 digits, or

- Make CFNRc quiescent

### 4.4.5	Deactivation

If the network cannot accept a deactivation request, an appropriate cause will be returned to the user, e.g. Service not Active.

### 4.4.6	Invocation

Within a PLMN or different PLMNs the number of tandem forwarding connections should be limited. The maximum number of tandem forwarding connections should be limited to a value between 1 and 5. This is to prevent infinite looping.

If the limit of successive forwardings of a call has already been reached, an unsuccessful call set up indication is sent backwards.

If the forwarded call cannot be completed to the forwarded-to destination, then the network will clear the forwarded part of the call and an unsuccessful call set up indication is sent backwards.

NOTE:	Call forwarding on mobile subscriber not reachable applies only to the Basic services subscribed to. Calls to a directory number requesting a Basic service which is not subscribed to will never be forwarded.

If the HPLMN receives a request for routeing information from an IPLMN which does not support long forwarded-to numbers and the destination subscriber has registered a long forwarded-to number and the HPLMN determines that the called mobile subscriber is not reachable, the HPLMN shall:

- Supply an alternative forwarded-to number of less than 16 digits, or

- Allow the call to continue to the called subscriber (i.e. suppress the invocation of CFNRc), or

- Instruct the IPLMN to release the call with a suitable error code, e.g. forwarding violation

If the IPLMN does not support long forwarded-to numbers and the served mobile subscriber has registered a long forwarded-to number for CFNRc, then optimal routeing of late call forwarding shall not be invoked.

### 4.4.7 Interrogation

It is possible that a user has registered a long forwarded-to number but has an MS that does not support long forwarded-to numbers. For this case if the user interrogates call forwarding then the service state shall be indicated without the number being displayed.