S1 Release 00 Ad-Hoc					Tdoc: S1R2KRH_002

Cyber Meeting on Roaming & Handover

30-31 March 2000

Source: AT&T

| Title: | GERAN Handover Requirements from AT&T |
| --- | --- |

Document for:	Discussion and Acceptance

# Introduction

In GSM/EDGE Radio Access Network, mobility is maintained through two different processes named as Cell Re-election and Handover, which can be applied to different applications based on the nature of required service.

Cell Reselection procedure is a mobile controlled procedure for re-searching a serving cell site that a mobile can camp on with better link quality, when existing radio link quality is deteriorated. The network in certain circumstances can also activate the cell selection/reselection procedure to force a mobile moving to a different serving BTS, such as the case of cell barred or network forced cell reselection.

Handover procedure is a full network controlled activity/process in order to provide with a continuous connectivity at radio link layer, when the quality of an existing radio link is deteriorated lower than the minimum requirement of the link quality for the purpose of the communications. A new radio link with the best available link quality will be established with the assistance of mobile station for the continuous communications.

# Input for TR22.976

Based upon the work of SMG2, it is recommended that the following tables be added to section 7.4 Roaming Requirements of TR22.976:

Table 1. Required Handover Scenarios from GERAN to Other Systems.

| To | GSM CS CN + GSM BSS | GSM PS CN+ GSM BSS | ANSI-136 | UTRAN PS +UMTS CN R99 | UTRAN CS +UMTS CN R99 | GERAN PS +UMTS CN R00 | GERAN CS +UMTS CN R00 | UTRAN PS +UMTS CN R00 | UTRAN CS +UMTS CN R00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| From |  |  |  |  |  |  |  |  |  |
| GERAN PS +UMTS CN R00 | FFS (HO) | CRS | NA | FFS (HO/CRS) | HO Post R00 | HO/CRS | HO Post R00 | HO/CRS | HO Post R00 |
| GERAN CS +UMTS CN R00 | HO | NA | FFS (HO) | NA | HO | HO Post R00 | HO | HO Post R00 | HO |

Table 2 Required Handover Scenarios from Other Systems to GERAN.

| From | GSM CS CN + GSM BSS | GSM PS CN+ GSM BSS | ANSI-136 | UTRAN PS +UMTS CN R99 | UTRAN CS +UMTS CN R99 | GERAN PS +UMTS CN R00 | GERAN CS +UMTS CN R00 | UTRAN PS +UMTS CN R00 | UTRAN CS +UMTS CN R00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| To |  |  |  |  |  |  |  |  |  |
| GERAN PS +UMTS CN R00 | FFS (HO) | CRS | NA | FFS (HO/CRS) | HO Post R00 | HO/CRS | HO Post R00 | HO/CRS | HO Post R00 |
| GERAN CS +UMTS CN R00 | HO | NA | FFS (HO) | NA | HO | HO Post R00 | HO | HO Post R00 | HO |

FFS = 	For Further Study

HO = 	Supported by HandOver

CRS = 	Supported by Cell Re-Selection

HO/CRS = Supported by both HandOver and Cell ReSelection

N/A= 	Not applicable to GERAN R00 requirements

N = 	Not support in GERAN R00 Requirement

PLMN Category Definitions

GERAN PS +UMTS CN R00: 	PS domain service including both best effort data and real time service, but excluding legacy GSM PS (GPRS) service

GERAN CS +UMTS CN R00:	CS domain services excluding existing legacy GSM CS (voice) service

GSM CS CN + GSM BSS:	GSM CS voices service only

GSM PS CN+ GSM BSS:	GSM PS data service only (GPRS best effort data)

ANSI-136:	ANSI-136 TDMA voice service only

UTRAN PS +UMTS CN R99/00:	All PS domain services in Release 99/00

UTRAN CS +UMTS CN R99/00:	All CS domain services in Release 99/00