3GPP TSG SA WG 1 Meeting #100 	S1-223029

Toulouse, France, 14 – 18 November 2022

Source:	MediaTek Inc.

Title:	Discussion of SENSE applicability to PLMNs

Agenda item:	6.1

Document for:	Discussion

Contact:	Carlson Lin

carlson dot lin at mediatek dot com

Abstract: CT1 sent SA1 an LS on SENSE for home PLMN and disaster roaming PLMN (C1-226124). This discussion paper considers how SA1 modifies current SPEC and how SA1 replies to CT1.

1. Introduction

CT1 sent SA1 an LS on SENSE for home PLMN and disaster roaming PLMN (C1-226124) This discussion paper considers how SA1 modifies current SPEC and how SA1 replies to CT1.

2. CT1 questions

CT1 wrote in C1-226124

1. Overall Description:

CT1 is working based on the requirements for SENSE feature in TS 22.011 CR#0322 (S1-213329) and would like to clarify the applicability of the SENSE feature for different PLMNs. Considering the "The signal threshold is specific for a certain Access Technology and shall apply to all PLMNs with the corresponding access technology combinations." as specified in CR#0322, the following questions need to be clarified:

Question 1: if a UE supports SENSE, the UE can register to a VPLMN (non-HPLMN/EHPLMN) depending on signal quality of this VPLMN compared to HPLMN/EHPLMN at the time of PLMN selection. Up until now when both HPLMN/EHPLMN and VPLMN are available, the UE selects the former not the latter. Can SA1 confirm the UE behavior and usage of the threshold in this case?

Question 2: CT1 has identified conflicting requirements for the combination of SENSE and MINT in the Stage-1 specifications. If a UE for which SENSE and MINT are both supported and enabled, detects a disaster condition and during the subsequent PLMN selection(s) finds an allowable PLMN with a signal quality below the SENSE threshold and a forbidden PLMN offering disaster roaming for which the conditions for disaster roaming according to 23.122 are fulfilled with a signal level above the SENSE threshold, shall the UE prioritise and select the forbidden PLMN, even if an allowable PLMN is available? Current definitions in TS 22.011 seems to imply that the forbidden PLMN is chosen in the given example, while TS 22.261 requirement "The 3GPP system shall be able to provide means to enable a UE to access PLMNs in a forbidden PLMN list if a Disaster condition applies and no other PLMN is available except for PLMNs in the forbidden PLMN list." seems to imply that the allowable PLMN is chosen.

CT1 asks SA1

ACTION: 	CT1 would like to kindly ask SA1 to provide answers for CT1’s questions above.

3. Discussions

3.1. General Background

From the SENSE WID (SP-215025) clause 3, we can see that the intention of this WI was mainly for solving a problem regarding stationary UE doing VPLMN/roaming network selection in permanent roaming environment, however in the TS 22.011 CR (S1-213329), the requirements cover not just VPLMN but all PLMNs (including RPLMN, HPLMN, Disaster PLMN…etc), we can see below:

Based on the TS 22.011 CR (S1-213329), the priority order of the network selection becomes as below illustration:

>= Operator controlled signal threshold

Priority 1:	RPLMN >= signal threshold

Priority 2:	(Eq)HomePLMNs >= signal threshold

Priority 3:	UserCtrledPLMNs >= signal threshold

Priority 4:	OpCtrledPLMNs >= signal threshold

Priority 5:	High quality Other PLMNs >= signal threshold

Priority 6:	Low quality Other PLMNs >= signal threshold

Priority 7:	Disaster PLMNs >= signal threshold

< Operator controlled signal threshold

Priority 8:	RPLMN < signal threshold

Priority 9:	(Eq)HomePLMNs < signal threshold

Priority 10:	UserCtrledPLMNs < signal threshold

Priority 11:	OpCtrledPLMNs < signal threshold

Priority 12:	Low quality Other PLMNs: < signal threshold

Priority 13:	Disaster PLMNs < signal threshold

3.2. Discussions of SENSE and Registered PLMN (RPLMN)

Figure 3.3.1 UE in a location where only a VPLMN < Threshold available

UE prefers RPLMN in legacy PLMN selection is a very fast and power-efficient way of getting service. Upon power-on or recovery of coverage, if the UE has RPLMN then the UE usually only need scan a few stored/remembered ARFCNs to detect the RPLMN, once detected the UE can select the RPLMN directly because it is the highest priority candidate. However this good design is not maintained after the SENSE is introduced.

Apply SENSE to RPLMN is not a good idea, especially when UE is stationary:

-	take the scenario of Figure 3.3.1 as example, if a RPLMN < threshold unfortunately was selected by the UE after first power-on (probabily due to zero > threshold PLMN candidate available in the environment), if the UE does not treat RPLMN as highest priority candidate in later power-ons/recoverys, then before the UE can select the RPLMN < threshold candidate (only one candidate in the environment) the UE needs to waste its battery power and time (check a lot of ARFCNs) to confirm there is zero > threshold PLMN candidate in the environment.

Apply SENSE to RPLMN is not needed as there is already remedy procedure to take UE to better signal quality PLMN (also introduced by SENSE):

-	There is already higher priority PLMN search precedure regularly triggered (and when SENSE is used the UE also take signal strength into account in higher priority PLMN search precedure). Thus we see no reason to apply SENSE to RPLMN "At switch-on or recovery from lack of coverage", doing so will only dramatically change the old perfect functioning code logic, make network selection much longer, make RPLMN become unstable.

3.3. Discussions of SENSE and Home PLMN (HPLMN)

Figure 3.2.1 UE in a location where only a VPLMN > Threshold and HPLMN < Threshold available

If in a location (in home country or visited country) the UE finds ONLY two candidates:

>= Operator controlled signal threshold

Priority 6:	Low quality Other PLMNs >= signal threshold

< Operator controlled signal threshol

Priority 9:	 (Eq)HomePLMNs < signal threshold

Based on TS 22.011 clause 3.2.2.2 the UE will select the Priority 6 candidate, not the Priority 9 candidate. Please note that priority 9 candidate is Home network but it is not selected, this means:

-	the UE need to pay high roaming charging fee even when the HPLMN is available (and satisfy cell selection prirotiy, i.e., a HPLMN suitble cell exists) at the UE location;

-	considering the common deployment scenario, usually the Home network can provide more services than VPLMN. The UE select the VPLMN, instead of the HPLMN, means that the UE get better signal quality at the cost of losing some kinds of services; and

-	The principle of "treating Home PLMN as highest priority" is broken;

In our view, in this scenario the UE selects HPLMN.

3.4. Discussions of SENSE and Disaster Roaming PLMN

Figure 3.4.1 UE in a location where only a Disaster PLMN > Threshold and allowable PLMN < Threshold available

As pointed out by CT1 in the LS, SENSE should not be applicable to Disaster PLMNs.

4. Conclusion

Based on above discussion (clause 3.2, 3.3 and 3.4) and according to SENSE WID (SP-215025) clause 3 (WI was mainly for solving a problem regarding stationary UE doing VPLMN/roaming network selection in permanent roaming environment), the signal threshold need not to be applied to RPLMN, (Eq)HomePLMNs and Disaster PLMNs. Besides,  due to TS 22.011 "NOTE 2: The allowed range of the Operator controlled signal threshold per access technology is between the cell selection criterion and the high quality signal as defined in TS 23.122 [3]", the signal threshold is not applied to High quality Other PLMNs by definition. Therefore, SENSE signal threshold is only applicable to UserCtrledPLMNs (bullet ii), OpCtrledPLMNs (bullet iii), and Low quality Other PLMNs (bullet v) so the "ideal" priority order of the network selection should be:

Priority 1:	RPLMN, regardless of its signal level

Priority 2:	(Eq)HomePLMNs, regardless of its signal level

>= Operator controlled signal threshold

Priority 3:	UserCtrledPLMNs >= signal threshold

Priority 4:	OpCtrledPLMNs >= signal threshold

Priority 5:	High quality Other PLMNs, by definition >= signal threshold

Priority 6:	Low quality Other PLMNs >= signal threshold

< Operator controlled signal threshold

Priority 7:	UserCtrledPLMNs < signal threshold

Priority 8:	OpCtrledPLMNs < signal threshold

Priority 9:	Low quality Other PLMNs: < signal threshold

Priority 10:	Disaster PLMNs, regardless of its signal level

5. Proposal

Propose SA1 to agree the S1-223030 and send a reply LS to CT1 to clarify that SENSE signal threshold is

-	applicable to

-	UserCtrledPLMNs (bullet ii),

-	OpCtrledPLMNs (bullet iii), and

-	Low quality Other PLMNs (bullet v).

-	not applicable to

-	RPLMN,

-	(Eq)HomePLMNs (bullet i), and

-	Disaster PLMNs.

6. Reference Information

SENSE WID (SP-215025) clause 3:

3	Justification

The number of IoT devices is growing exponentially and with that also the number of stationary UEs. Such UEs … could be located … in places that cannot be reached easily after the deployment or deep indoor.

…the UEs do not move and they are often in a permanent roaming situation, either because the modules were deployed in other countries than that of the provided USIMs or because of the use of Global USIMs for IoT use cases.

While most of the UEs work fine, a small percentage of UEs experience unstable conditions. This means, sometimes they select a VPLMN and stay on that VPLMN, where they … attach, and … setting up a data bearer occasionally or nearly always fails. …

…

The reason for this kind of problems is related to the current way of how VPLMNs are selected today. …, the signal level of available cells is not taken into account, solely the cell selection criteria as broadcast by the PLMN and the priority of networks.

This results in UEs selecting or staying on a network of which the coverage on that particular place is poor, because the PLMN has higher priority, while other PLMNs of lower priority would be available with much better local coverage. For typical consumer UEs this is no problem – due to mobility the conditions are changing quickly … But for stationary devices without supervision by a user it can be a problem.

Therefore, an improvement is needed that allows to take the signal level into account….

TS 22.011 CR (S1-213329) highlight:

3.2.2		Procedures

3.2.2.1	General

…the 5G system shall support a mechanism to have an Operator controlled signal threshold per access technology on the USIM to be used for network selection. The signal threshold is specific for a certain Access Technology and shall apply to all PLMNs with the corresponding access technology combinations.

NOTE 1: 	The use of the Operator controlled signal threshold per access technology is intended for IoT stationary devices, without user interaction.

NOTE 2:	The allowed range of the Operator controlled signal threshold per access technology is between the cell selection criterion and the high quality signal as defined in TS 23.122 [3].

…

3.2.2.2	At switch-on or recovery from lack of coverage

If the Operator controlled signal threshold per access technology is set on the USIM, it shall be used … In particular, if the Operator controlled signal threshold per access technology is set on the USIM:

i) the UE shall only select a network if

- 	the network selection conditions as described below are met and

-	the received signal quality of the candidate PLMN/access technology combination is equal to or higher than the Operator controlled signal threshold per access technology.

ii) if no candidate PLMN/access technology combination fulfils the Operator controlled signal threshold criteria, the UE shall repeat the network selection procedure without applying the Operator controlled signal threshold per access technology.

[network selection procedure Begin]

…

At switch on, when in coverage of the last registered PLMN as stored in the SIM/USIM, the UE will attach to that network.

…

A)	Automatic network selection mode

The UE shall select and attempt registration on other PLMNs,…, in the following order:

i)	An EHPLMN if the EHPLMN list is present or the HPLMN (derived from the IMSI) if the EHPLMN list is not present for preferred access technologies in the order specified. In the case that there are multiple EHPLMNs present then the highest priority EHPLMN shall be selected. It shall be possible to configure a voice capable UE so that it shall not attempt registration on a PLMN if all cells identified as belonging to the PLMN do not support the corresponding voice service;

ii)	each entry in the "User Controlled PLMN Selector with Access Technology" data field in the SIM/USIM (in priority order). It shall be possible to configure a voice capable UE so that it shall not attempt registration on a PLMN if all cells identified as belonging to the PLMN do not support the corresponding voice service;

iii)	each entry in the "Operator Controlled PLMN Selector with Access Technology" data field in the SIM/USIM (in priority order). It shall be possible to configure a voice capable UE so that it shall not attempt registration on a PLMN if all cells identified as belonging to the PLMN do not support the corresponding voice service;

iv)	other PLMN/access technology combinations with sufficient received signal quality (see 3GPP TS 23.122 [3]) in random order. It shall be possible to configure a voice capable UE so that it shall not attempt registration on a PLMN if all cells identified as belonging to the PLMN do not support the corresponding voice service;

v)	all other PLMN/access technology combinations in order of decreasing signal quality. It shall be possible to configure a voice capable UE so that it shall not attempt registration on a PLMN if all cells identified as belonging to the PLMN do not support the corresponding voice service.

… When there is no available PLMN except for PLMNs in the "Forbidden PLMN" data field in the SIM/USIM, and the available PLMNs indicate that Disaster Condition applies, this PLMN shall be considered allowable for registration to the UE while the Disaster Condition is applicable.

…

[network selection procedure End]