3GPP TSG SA WG 1 Meeting #100 	S1-223061

Toulouse, France, 14 - 18 November 2022	(revision of S1-223zzz)

Source:	Samsung

pCR Title:	22.837 pCR: Update on 5.4, Transparent Sensing use case

Draft Spec:	3GPP TR 22.837-020

Agenda item:	7.1

Document for:	Approval

Contact:	Erik Guttman

erik.guttman@samsung.com

Abstract: This use case presents proposes to remove an EN.

1. Introduction

There is an Editor's Note that can be resolved as there is a new use case in S1-223zzz.

2. Reason for Change

It is important to remove Editor's Notes to progress the study.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.837-020

First Change

## 5.4	Use Case on Transparent Sensing Use Case

### 5.4.1	Description

In general, a UE senses using either or combination of the sensors such as camera, NR-based sensing and Lidars, Radars. In NR-based sensing, the UE and BS senses for stationary and moving Objects around UE – using time-difference-of-arrival (TDoA), angle-of-arrival (AoA), angle-of-departure (AoD) measurements, RSSI etc. as shown in Figure 5.Q.1-1 [14]Transparent sensing is a use case in which sensing data is captured by the UE and communicated so that the 5GS is aware of the sensing information. From this information, service enablers can be defined. One example of such information is location data, whose corresponding service enabler is Location Based Services.

Figure 5.Q.1-1: BS and UE sensing Objects

In this use case, sensing data is made available to the 5GS, and the requirements for this exposure are considered. The data so obtained can be used for diverse purposes. One such purpose is Localization (identifying both a three dimensional position and orientation.) Transparent Sensing data used for Localization is described in TR 22.856 [11]

Figure 5.Q.1-2: Opaque and Transparent Sensing Data

The distinguishing characteristic of this use case is that the sensing data is provided to the 5GS itself, not merely by the 5GS to an AF out of scope of 3GPP standardization.

### 5.4.2	Pre-conditions

A UE has access to one or more sensors. In this use case. the UE has access to four sensors: NR-based sensing, 3D LiDAR, an RGB Camera and a Smart Phone Camera. The sensors' physical configuration is known (e.g. the cameras are 10 cm apart). The NR-based sensing capabilities of the UE and its connected BS are used to capture information about the nearby environment by the UE.

A mobile network MN supports the acquisition of sensing data. We term this support by the network a 'sensing data consuming service.'

### 5.4.3	Service Flows

The user U activates a mechanism to enable NR-based sensing at UE and MN and provide sensing measurement data to the 5GS. This process is analogous to activating or enabling a location tracking service.

MN acquires sensing measurement data provided by U's UE, for a period of time.

The user U deactivates the mechanism to provide sensing measurement to the 5GS.

Editor's Note: Additional details on the use of LiDAR, cameras and/or spatial localization need to be added.

### 5.4.4	Post-conditions

The sensing data acquired by the 5GS is processed in order to enable other services. The processed information can for example provide 'Spatial Localization' information that can be exposed to authorized third parties, as discussed in 22.856 [11]. "Spatial Localization Use Case".

### 5.4.5	Existing feature partly or fully covering use case functionality

Positioning in 5G Networks been proposed in 3GPP release-16, it specifies positioning signals and measurements for the 5G NR. In release-16, 5G Positioning architecture extends 4G positioning architecture by adding Location Management Function (LMF) and Transmission reception points (TRP).  5GS provides new positioning methods based on multi-cell round-trip time measurements, multiple antenna beam measurements, multiple to enable downlink angle of departure (DL-AoD) and uplink angle of arrival (UL-AoA) [15][16]. The Rel-17 5G system supports positioning of the device-based but not device-free – objects that do not radiate EM signals. [14][15][16]

The 5GS already supports transport of sensor data. The table below provides indicative performance requirements for media used for sensor information communication.

| Sensor Type | Uplink KPI | Remarks |
| --- | --- | --- |
| 3D Lidar | 30 Mbps | An example 3D LiDAR: 16 channel, 0.3M data points, dual return mode 2 bytes distance, 1byte [13] |
| Industrial RGB Camera | 16 ~ 800 Mbps | 2,592 x 2,048 x 10bits x 2.5 Hz x 6 EA, compression ratio 2% |
| Smart Phone Camera | 4 ~ 200 Mbps | 2,160 x 2,880 x 8bits x 1 Hz x 4 EA, compression ratio 2% |

Figure 5.4.5-1: Performance Requirements (already possible to fulfill with the 5GS)

### 5.4.6	Potential New Requirements needed to support the use case

[PR.5.4.6 - 001] Subject to user consent and national or regional regulation, based on operator policy, the 5GS shall support a mechanism to receive uplink sensing measurements from a UE that is authorized to provide uplink sensing measurements to the network.

NOTE: 	This requirement assumes there is some functionality in the 5GS to discern and interpret the acquired sensing measurements.

[PR.5.4.6 - 002] Subject to user consent and national or regional regulation, based on operator policy, the 5GS shall support a mechanism to expose sensing result to authorized third parties.

[PR.5.4.6 - 003] Subject to user consent, network operator policy and national or regional regulation, the 5GS shall support a mechanism to enable RAN entities and UEs to acquire NR-based sensing measurement data to capture information about the nearby environment.

Editor's Note: It is FFS if explicit requirements for LiDAR, cameras and other sensors need to be added.Editor's Note:	Using existing E-UTRA measurements is FFS.

End of Changes