3GPP SA WG1 Meeting #100 	S1-223085

November 14 – 18 2022, Toulouse, France

Source: Intel, Cisco

Title:	New WID on Introduction to Support WBA OpenRoaming Framework for the Interconnect between SNPN & Credentials Holder (CH)

Document for: Approval

Agenda Item: DUMMY

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Introduction to Support WBA OpenRoaming Framework for the Interconnect between SNPN & Credentials Holder (CH)

Acronym: WBAOpenRoamingSNPN

Unique identifier:

Potential target Release:	Rel-19

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No | X |  |  |  |  |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A |  |  |  |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  |  | {optional free text} |

Dependency on non-3GPP (draft) specification: None

# 3	Justification

N32 reference point is currently designed to support tens of millions of subscribers using a long-lived connection between VPLMN and HPLMN. With the introduction of SNPN, the requirements on the N32 reference points are dramatically increased due to a couple of complementary factors: 1)  the need to support thousand times more networks and 2) subscribers use short lived forwarding connections between VPLMNS and HPLMNs due to much smaller scale of the conventional public networks.
In response to this scalability issue, the potential enhancements and security aspects for the N32 reference point that facilitate deployments by SNPN and Credentials Holder (CH) shall address the following objectives:

1. How to enable support for N32 functionality between SNPN and CH (which can be either a PLMN or another SNPN or AAA server) that does not require long-lived persistent connectivity on N32.

2. How to enable support for N32 functionality that facilitates firewall and border gateway configuration on the SNPN side of N32.

3. How to scale the N32 design to support a three order of magnitude increase in the number of subscribers.

# 4	Objective

The objective is to define normative stage 1 requirements to support WBA OpenRoaming Framework for the interconnect between SNPN & Credentials Holder (CH) using N32.

The Service requirements for the 5G system (TS 22.261, Rel-19) scope of WBA OpenRoaming Framework for the interconnect between SNPN & CH are:

- Interconnection of Standalone non-public network (SNPN) with a large number of entities that authenticate and authorize access to the non-public network.

- The authorizing entities could operate behind a NAT and use dynamic IP addresses whose information may not be available in advance of signaling establishment.

NOTE:    Infosec policies of the enterprise operating the 5G system for standalone operation of a non-public network can prohibit accepting inbound socket connections from unknown IP addresses.

NOTE:    Infosec policies of the enterprise operating the 5G system for standalone operation of a non-public network can prohibit accepting inbound socket connections from unknown IP addresses.

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 22.261 | Text modifications and addition of new requirements on Diverse Mobility Management – Description and Service Continuity Requirement | SA#98-e |  |

# 6	Work item Rapporteur(s)

Suresh Srinivasan suresh.srinivasan@intel.com

# 7	Work item leadership

SA1

# 8	Aspects that involve other WGs

None.

# 9	Supporting Individual Members

Supporting IM name

Intel

Cisco