3GPP TSG SA WG 1 Meeting #100 	S1-223118

Toulouse, France, 14 – 18 November 2022	(revision of S1-22xxxx)

Source:	CATT

pCR Title:	Use case on service differentiation for UEs via satellite access

Draft Spec:	3GPP TR 22.865 v0.1.0

Agenda item:	7.7

Document for:	Approval

Contact:	Qing Wan wanqing1@cictmobile.com

Abstract: The contribution proposes a use case about UEs with different GNSS capabilities connecting to the network via satellite access and describes potential requirements for TR22.865.

1. Introduction

Introduce a use case about service differentiation for UE with different GNSS capabilities connecting to 5G network via satellite access.

2. Reason for Change

Service requirements about GNSS independent operation shall be addressed to Section 5 “Use Cases” of TR 22.865.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree on the following changes to 3GPP TR22.865 v0.1.0

* * * First Change * * * *

5 Use cases

5. X	Use case on service differentiation for UEs via satellite access

5.X.1	Description

Satellite network has been introduced to 5G system to improve the service availability and reliability since 3GPP Rel-15. In parallel, various UE models with different capabilities (e.g. eMTC UE, CPE) are defined to serve the vertical needs. How to facilitate different types of UEs to benefit from satellite network is worthwhile to study.

The current assumption of 3GPP normative work is, UE shall be capable of GNSS positioning to determine the location for obtaining 5G services via satellite access, which has excluded the possibility to provide 5G services to UEs without GNSS receiver, or unable to determine the location with GNSS receiver. In fact, some services such as broadcast or multicast service, public safety associated services are not highly sensitive to the precise location. Moreover, UEs in stationary mobility type such as for home access, for metering have fixed location to support the position relevant operation during satellite access. The use case illustrates how the UEs with different capabilities obtain 5G services via satellite access.

5.X.2	Pre-conditions

It is assumed that network operator has deployed NGSO (e.g. LEO) satellite enabled NG-RAN to provide 5G network PLMN#X in the area Area#A, where have sparse population and no coverage of terrestrial access network as a result.

All UEs support 5G satellite RAT but with different subscription and positioning capabilities.

Table 5.X.2-1: UEs with different capabilities

|  | Subscription | Positioning Capability | Mobility Type |
| --- | --- | --- | --- |
| UE1 | Subscriber of PLMN#X for eMBB services | No GNSS capability; Support other 3GPP positioning technologies | Full mobility |
| UE2 | Subscriber of other PLMN with roaming agreement to PLMN_X, for eMBB services | No GNSS capability; Support other 3GPP positioning technologies | Full mobility |
| UE3 | Subscriber of PLMN#X for MIoT services | No GNSS capability; Not support any 3GPP positioning technologies | Full mobility. |
| UE4 | Subscriber of PLMN#X for eMBB services | No GNSS capability; Not support any 3GPP positioning technologies | Stationary |
| UE5 | Subscriber of PLMN#X for eMBB services | No GNSS capability; Not support any 3GPP positioning technologies | unknown |

5.X.3	Service Flows

1. All UEs intend to register to PLMN_X but can’t provide the location during the registration.

2. After UE authentication and authorization, the network accepts the registration request and determines the available services based on the subscription, mobility types:

- UE1: allow the broadband broadcast service, public safety related services and emergency services regarding the operator’s policy. When the network acquires a valid and precise UE location via other means, allow all the subscribed services.

- UE2:  allow public safety related services and emergency services regarding the roaming agreement and the operator’s policy.

- UE3: allow only the broadcast message service (e.g. PWS message) regarding the operator’s policy.

- UE4: allow subscribed eMBB services based on UE’s registered location.

- UE5: the network detects UE5 is a dedicated user of digital broadband broadcast application, and fetch the location from the application platform. Then allow subscribed eMBB services based on UE’s location.

Figure 5.X.2-1: Service differentiation for UEs via satellite access

5.X.4	Post-conditions

All UEs can successfully register to PLMN_X to get full or part of subscribed services.

5.X.5	Existing features partly or fully covering use case functionality

SA1 has introduced several requirements about satellite access in TS 22.261[2].

Clause 6.3.2.3 describes basic requirements about satellite access for 5G system and UE.

The 5G system shall be able to provide services using satellite access.

A UE supporting satellite access shall be able to provide or assist in providing its location to the 5G network.

A 5G system with satellite access shall be able to determine a UE's location in order to provide service (e.g. route traffic, support emergency calls) in accordance with the governing national or regional regulatory requirements applicable to that UE.

The 5G system with satellite access shall be able to support low power MIoT type of communications.

Regarding the above requirements, UE shall have the ability to provide or assist in providing the location for obtaining the services from 5G system. The restriction of UE’s positioning capability has limited the potential users of 5G system using satellite access, which expect to be served by 5G network.

5.X.6	Potential New Requirements needed to support the use case

[PR 5.X.6-001] Subject to the regulatory requirements and operator’s policy, the 5G system shall be able to provide services to UEs with or without GNSS capability using only satellite access.

[PR 5.X.6-002] Subject to the regulatory requirements and operator’s policy, the 5G system shall be able to differentiate service and QoS control for authorized UEs with or without GNSS capability using only satellite access.

[PR 5.X.6-003] Subject to the agreement, a 5G system with satellite access shall be able to determine a UE's location based on the information from trusted 3rd party in order to provide services to UE only using satellite access.

* * * End of Changes * * * *