3GPP TSG SA WG 1 Meeting #100 	S1-223125

Toulouse, France, 14 – 18 November 2022

Source:	Huawei

pCR Title:	Update of KPI table for railway intrusion detection

Draft Spec:	3GPP TR 22.837 v0.2.0

Agenda item:	7.2

Document for:	Approval

Contact:	< Fangyuan Zhu (zhufangyuan@huawei.com)>

Abstract: This document proposes to update the KPI table for the use case of railway intrusion detection.

1. Introduction

It is proposed to update the Clause 5.7: use case of railway intrusion detection.

2. Reason for Change

Some KPI values proposed in this UC are referred from the document “Rail Industry Safety Induction Handbook: https://railsafe.org.au/__data/assets/pdf_file/0009/32022/Rail-Industry-Safety-Induction-RISI-Handbook-V5.1.pdf”.  In order to better understand the determination of KPI value, the related descriptions from this refernece are listed in the following.

Confidence level

The confidence level for this UC is estimated as 95%.

Positioning accuracy by sensing

According to this reference, the width of the danger zone horizontally from the nearest track is within 3m. Therefore, the horizontal positioning accuracy by sensing is estimated as ≤1.5m. The vertical positioning accuracy by sensing is not applicable.

The two major components of the Rail Corridor are the Danger Zone and safe places. The Danger Zone The Danger Zone is anywhere within 3 metres horizontally from the nearest track and any distance above or below this 3 metres, unless constantly in a safe place. A safe place A safe place is a place where a person and their equipment cannot be struck by rail traffic.

Max sensing service latency

In order to ensure passenger safety if any hazardous situation is detected, the latency for 5G system to sense the intrusion and report the sensing information on the presence of an intrusion (e.g. pedestrian/animal) to railway management departments is recommended as ≤1500ms.

Refreshing rate

The refreshing rate for sensing in this UC is recommended as ≤0.1s.

Missed detection

The value of missed detection is recommended as ≤0.1%.

False alarm

The value of missed detection is recommended as ≤0.1%.

3. Conclusions

It is proposed to determine the KPI value for this UC based on the above reference.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.837 V0.2.0.

* * * First Change * * * *

5.7.6	Potential New Requirements needed to support the use case

[PR. 5.7.6 - 001] Subject to operator policy, the 5G system shall enable the core network to collect and aggregate sensing measurement data from base stations.

[PR. 5.7.6 - 002] Subject to operator policy, the 5G system shall enable the core network to expose a suitable API to provide the information regarding sensing results to authorized third parties.

[PR. 5.7.6 - 002] The 5G system shall be able to support the following KPIs:

Table 5.x.6-1: Performance requirements for railway intrusion detection

| Scenario | Sensing service area | Confidence level [%] | Positioning accuracy by sensing (for a target confidence level) |  | Velocity estimation accuracy by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| Intrusion detection on a railway | Outdoor (Along railway) | 95% | ≤1.5m (Note 1) | N/A | N/A | N/A | N/A | N/A | ˂1500ms (Note 2) | ≤ 0.1s | ≤0.1% | ≤0.1% |
| Note 1: the width of the danger zone horizontally from the nearest track is within 3m [20]. Note 2: in order to ensure passenger safety if any hazardous situation is detected, this is the latency for 5G system to sense the intrusion and report the sensing information on the presence of an intrusion (e.g. pedestrian/animal) to railway management departments. |  |  |  |  |  |  |  |  |  |  |  |  |

* * * End of Change * * * *