3GPP TSG SA WG 1 Meeting #100 	S1-223159

Toulouse, France, 14 – 18 November 2022	 (revision of S1-22xxxx)

Source:	vivo

pCR Title:	Use case on roaming for sensing service of sports monitoring

Draft Spec:	3GPP TR22.837

Agenda item:	7.2

Document for:	Approval

Contact:	Amy Zhang, amy.zhang@vivo.com

Abstract: This document provides a text proposal for the use of communication-assisted sports monitoring when people is traveling in another country, i.e. roaming.

1. Introduction

Introduce a use case of how to enable roaming regarding sensing service into the sensing TR.

2. Reason for Change

Adding a use case with some functional requirements and KPIs to the TR.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR22.837 v0.2.0.

* * * First Change * * * *

# 2	References

[x]	Li S, Li X, Lv Q, et al. WiFit: Ubiquitous bodyweight exercise monitoring with commodity wi-fi devices, 2018 IEEE SmartWorld, Ubiquitous Intelligence & Computing, Advanced & Trusted Computing, Scalable Computing & Communications, Cloud & Big Data Computing, Internet of People and Smart City Innovation, IEEE, 2018: 530-537.

* * * Next Change * * * *

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

sensing measurement data: data collected about radio/wireless signals impacted (e.g. reflected, refracted, diffracted) by an object or environment of interest for sensing purposes.

sensing measurement process: process of collecting sensing measurement data.

sensing result: the information derived from processing sensing measurement data.

NOTE: Examples of sensing result are characteristics of an object or environment, etc.

Sensing service area location:  an area location whether with or without obstacle, the 5G system can provide sensing service with certain quality.

transparent sensing: sensing measurements are communicated such that they can be discerned and interpreted by the 5GS, e.g. the data is communicated using a standard protocol to an interface defined by the 5GS.

Motion rate accuracy: describes the closeness of the measured magnitude of the target object’s vibration/rotation frequency caused by part(s) of the target object to the true magnitude of the target object’s vibration/rotation frequency.

* * * Next Change * * * *

## 5.x	Use case on roaming for sensing service of sports monitoring

### 5.x.1	Description

Sports monitoring application describes the case of a human being monitored when doing exercise via utilizing wireless signals instead of cameras, or wearable devices. With enhanced privacy preservation, wireless signals that propagated in the 5G system (e.g. between 5G UE and 5G UE, and between the radio access network and device) can be further reused and processed to retrieve the target sensing object’s characteristics [x]. In a sports monitoring situation, the target object is human and the target object’s characteristic is human body gesture. By comparing the detected body gesture with the correct body gesture when people are doing exercises like sit-ups, and push-ups, this sport monitoring application will give feedback, for example, it can count the number of the exercise e.g. sit-ups, and calculate calories.

### 5.x.2	Pre-conditions

The sports monitoring application provider has a service agreement with mobile operator A in country X and mobile operator B in country Y.

Mobile operator A in country X and Mobile operator B in country Y has roaming agreements.

Bob installs a sports monitoring application on his mobile phone and subscribes to the sensing service with mobile operator A.

### 5.x.3	Service Flows

Step 1: Bob is a sports fan and does exercise everyday. He travels to country Y and gets accommodation in hotel M, where mobile operator B’s sensing service is available. Bob triggers the application and selects his favorite sport, sit-up.

Step 2: The sensing request is received by Mobile operator B and Mobile operator B then authenticates whether the sensing request can be satisfied, e.g. by verifying the sensing system availability (infrastructure, sensing modalities), the roaming agreement with Bob’s home mobile operator A in country X, the identity of Bob and etc.

Step 3: if the authentication succeeds, mobile operator B authorizes and provides such 5G sensing service to Bob with required performance targets and requirements and Bob pays for the sensing service to mobile operator B.

- Executes sensing measurement process (e.g. micro doppler shift) with the sensing entities (e.g. RAN entities and the roaming UE, or CPE and the roaming UE) to obtain sensing measurement data

- Processes the sensing measurement data to derive sensing result and expose it to the application server.

Step 4: if the authentication fails, mobile operator B does not provide such 5G sensing service to Bob and potentially, Bob or the sports monitoring application server will receive the reason of why mobile operator B cannot provide such sensing service.

### 5.x.4	Post-conditions

Bob can enjoy the sports monitoring application even when he travels in another country.

### 5.x.5	Existing features partly or fully covering the use case functionality

None

### 5.x.6	Potential New Requirements needed to support the use case

[PR 5.x.6-1] Based on operator policy, the 5G System shall be able to provide the 5G wireless sensing services in case of roaming.

[PR 5.x.6-2] The 5G system shall support means to perform sensing between RAN entities and roaming UE.

[PR 5.x.6-3] The 5G system shall enable a third-party service provider to be able to request a PLMN to perform sensing service authentication of a UE based on 3GPP credentials.

[PR 5.x.6-4] The 5G system shall allow the operator to authorize/de-authorize a UE for using 5G wireless sensing service in case of roaming.

[PR 5.x.6-5] The 5G system shall support charging for 5G wireless sensing service/applications in case of roaming.

[PR.5.x.6-6] The 5G system shall be able to provide 5G wireless sensing service with the following KPIs:

| Scenario | Sensing service area | Confidence level [%] | Position accuracy by sensing |  | Velocity accuracy by sensing |  | Motion rate accuracy | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal[m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] |  | Range resolution (horizontal/ vertical) [m x m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| Sports Monitoring | Indoor (Livingroom) | 95% | - | - | - | - | 3 times/min [0.05Hz] (NOTE 1) 4 times/min [0.07Hz] (NOTE 2) | - | - | 60s | 1 min | - | - |
| NOTE 1: we take sit-up rate = 30 times/min as reference NOTE 2: we take push-up rate = 40 times/min as reference |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

* * * End of Change * * * *

<Proposed change in revision marks>