3GPP TSG-SA WG1 Meeting #100	S1- 223203

F2F Meeting, 14 November – 18 November 2022

Title:	Use Case on inter PLMN or PLMN-SNPN scenario for URLLC service

Agenda item:	7.10

Source:	Futurewei

Document for:	Approval

Contact:	Amanda Xiang 
amanda.xiang@futurewei.com

Abstract: This paper proposes a new use case to be captured in TR 22.841 v.0.1.0.

---------- [All new Text] ----------

# 5.x  Use case on inter PLMN or PLMN- SNPN scenario for URLLC service

### 5.x.1	Description

This example scenario refers to a stadium, served by ad-hoc/in-venue 5G NR deployment (high-capacity) from one SNPN or PLMN network (MNO-A), plus 5G NR coverage, from inside and outside the stadium, by another PLMN (MNO-B). There is no NW sharing in place between the two 5G networks.

Fig. 5.x-1 Stadium scenario

The above example may similarly apply to other local premises, e.g., campus, enterprise, mall, factory.

MNO-A has business and roaming partnership with MNO-B, including the agreement to lease extra capacity (from MNO-B) to provide ultra-broadband experience to some of MNO-A “golden” customers. The agreement between MNO-A&B entails the ability for those MNO-A users to get higher data rate connection, by allowing their data traffic to use an extra NR connectivity link from PLMN-B’s (with anchor and aggregation in their HPLMN’ CN). Such functionality is assumed to be supported by both PLMNs’ CNs, UEs (of golden MNO-A users), and is associated with a set of traffic policies and conditions, negotiated by the MNOs, which the HPLMN can control and provision (to UEs and CN entities).

### 5.x.2	Pre-conditions

Tom and Jacob are at the stadium, for a football game. Tom is a golden subscriber of MNO-A, Jacob is a “normal” MNO-A customer.

Both their UEs are camped and registered on MNO-A network, in Idle.

Tom’s UE is dual (NR) radio capable. Tom also brings an AR glass to allow him to participate an immersive service provided by MNO-A.  This service allows AR glass to send real time video stream to the server which is hosted in MNO-A inside the stadium, and the server analyses video and renders with other information and sent information back to the AR glass, so the user can have fully immersive experience.  This AR glass is connected with Tom’s UE using Bluetooth.

The specific traffic policy for Tom, part of MNO-A “golden user” agreement, includes the use of dual access (connecting to both MNO-A and MNO-B) for certain URLLC service traffic, e.g., for high-throughput, low latency, high reliability immersive service., otherwise MNO-A single access/link should be used. The specific traffic policy also includes the restriction such as when the dual access will not be available, such as time and location restriction.  For this immersive game service, per MNO-A policy and agreement between MNO-A and MNO-B, Tom’s UE cannot connect to MNO-B for dual access after game is finished.

### 5.x.3	Service Flows

1) Before the game starts, both Tom and Jacob make a voice call to their best friends, describing the atmosphere at the stadium. Both use single 3GPP access connectivity via MNO-A network.

2) After a while, when the game starts, Tom wears his AR and starts its immersive service. Tom’s UE registers to both MNO-A and MNO- B and establishes a dual access connection across the 2 networks. Per the policy defined by MNO-A and the SLA between MNO-A and MNO-B, Tom’s 80% of the immersive service traffic are going through MNO_A network while rest of being steered to MNO-B network. 
Jacob’s UE continues to use single access via MNO-A network to stream video his friend.

3) During the game, because many MNO-A users are also sharing the video and pictures, the MNO-A is getting congested. In order to have good user experience for the golden customer, after detecting connection performance is deteriorating, Tom’s UE and MNO-A CN steer more immersive traffic to MNO-B to avoid service disruption based on MNO-A’s policy.  After MNO-A network connection performance is getting better, Tom’s UE and MNO-A CN steer more immersive traffic back to MNO-A network.

4) One hour after the game is finished, the immersive game service is terminated. Tom’s UE is not able to register to MNO-B network and it can only use single access with MNO-A network for its communication,

### 5.x.4	Post-conditions

### Tom is enjoying the game with that immersive service without service interruption or degradation.

### 5.x.5	Existing features partly or fully covering the use case functionality

A) Stage-2&3 include a feature called ATSSS (Access Traffic Steering, Switching, Splitting), e.g., ref to TS 25.301 sec. 5.3.2, which supports functionalities similar to those described in this use case, but limited to 3GPP access plus non-3GPP dual access.

B) Existing service requirements, e.g., from TS 22.261 sec. 6.18 and 6.41, capture some general multi-NW connectivity requirements, which do not fully cover or satisfy the specific target use case and functionalities. See some extracts (not exhaustive) listed below:

[Sec. 6.18: Multi-network connectivity and service delivery across operators]

The 5G system shall enable users to obtain services from more than one network simultaneously on an on-demand basis.

For a user with a single operator subscription, the use of multiple serving networks operated by different operators shall be under the control of the home operator.

When a service is offered by multiple operators, the 5G system shall be able to maintain service continuity with minimum service interruption when the serving network is changed to a different serving network operated by a different operator.

NOTE 1: A business agreement is required between the network operators.

In the event of the same service being offered by multiple operators, unless directed by the home operator's network, the UE shall be prioritized to receive subscribed services from the home operator's network.

NOTE 2: If the service is unavailable (e.g., due to lack of network coverage) from the home operator's network, the UE may be able to receive the service from another operator's network.

NOTE 3: QoS provided by the partner operator's network for the same service will be based on the agreement between the two operators and could be different than that provided by the home operator's network.

[Sec. 6.41: PALS]

Based on localized service agreements, the hosting network shall be able to provide required connectivity and QoS for a UE simultaneously connected to the hosting network for localized services and its home network for home network services.

### 5.x.6	Potential New Requirements needed to support the use case

[PR 5.x.6-001] The 5G system shall be able to support mechanisms to enable dynamically steering, split and switch of UE’s user plane traffic of one data session across two 5G networks concurrently (e.g., both using NR access) belonging to two different PLMN operators (one of which is HPLMN), or between the HPLMN and a SNPN based on the connection status, in order to meet the QoS requirement of the data session .

NOTE1: It is assumed that the HPLMN subscription is used to access both networks, data traffic is anchored in the HPLMN and a proper business agreement is in place among the two network operators, including specific traffic routing policies, e.g., based on geographical location, subscription, traffic type.

NOTE2: Data routing can be based on traffic policies under home network operator control, e.g., depending on QoS requirements as well as the business agreeemnt between home network operator and other network operator.

### [PR 5.x.6-002] The 5G system shall be able to support mechanisms to allow home operator to restrict the usage of steering, splitting and switching  of UE’s user plane traffic of one data session across two 5G networks (e.g., both using NR access) belonging to two different PLMN operators (one of which is HPLMN), or between the HPLMN under certain conditions, such as location, time, connectivity conditions.