3GPP TSG-SA WG1 Meeting #100 	   S1-223204

Electronic Meeting, 14 November – 18 November 2022

Title:	New use case – Metaverse Multi Access Scenario

Agenda item:	7.4

Source:	CableLabs, Charter Communications, Comcast

Document Type:   pCR

Document for:	Approval

Contact:	Omkar Dharmadhikari
o.dharmadhikari@cablelabs.com

Abstract: This paper proposes a new use case to be captured in TR 22.856 v.0.2.0.

---------- [All new Text] ----------

## 5.X	Use case on Enabling Metaverse services to users via multiple access connections

### 5.X.1	Description

The metaverse enables immersive virtual media, 3D avatar and holographic communications for realizing use cases such as interactive gaming, virtualized shared workspaces, and immersive conference rooms for remote collaboration, etc. The goal is to create a virtual world we can work in, interact with, and even escape to. Majority of the Metaverse use cases are applicable to indoor and/or localized areas such as home, offices, stadiums, shopping malls, movie theatres, theme parks, hospitals, universities, concert halls, etc. Even though Metaverse goes beyond reality into virtual worlds that seem to be distant, the use cases are tied to a single physical location which is mostly indoors and serving a localized area. Such physical locations often prefer non-3GPP (trusted, untrusted or wireline) access.

Metaverse applications often require more bandwidth and lower latencies which can be challenging to meet. Although major improvements to satisfy these requirements of uninterrupted, lag-free, immersive metaverse experience have been made such as:

- incorporation of 1200 MHz of new spectrum in the 6 GHz band with Wi-Fi 6E enabling bigger channel sizes up to 160 MHz

- support up to 1024 QAM with Wi-Fi 6 and 6E and Wi-Fi 7 aiming to support up to 4096 QAM

- doubling maximum channel bandwidth available to each device to 320MHz in the 6GHz band with Wi-Fi 7

- incorporation of High Band Simultaneous (HBS) Multi-Link Operation (MLO) in 802.11be that aggregates two simultaneous 160 MHz channels (four streams) in 5 GHz and 6 GHz bands reducing latency to < 2msec

In case of converged or hybrid network architecture, a single metaverse user can access Metaverse application via 5GS using both 3GPP and non-3GPP accesses simultaneously. Seamless access to metaverse services using different digital representations and exchange of related KPIs across accesses between the 5GS and metaverse application server would be critical from a user perspective.

### 5.X.2	Pre-conditions

Mark is enjoying a weekend at home where he has access to both residential broadband and cellular network from his service provider. Mark is using a pair of VR glasses to see a live concert and at the same time he is using tactile gloves for virtual painting using his residential broadband network. The two activities are running simultaneously on the same access network from the service provider.

### 5.X.3	Service Flows

1. Mark is simultaneously using VR glasses to see a live concert and tactile gloves for virtual painting via a 3rd party metaverse application service provider on his residential broadband access network (non-3GPP access).

2. Metaverse has a policy for the network to apply the same QoS level for the live concert and for virtual painting and informs the network on this decision.

3. The non-3GPP access network now starts getting congested as the children start streaming movies and wife is on the video call with her parents.

4. Now the service provider using non-3GPP access alone cannot provide the expected QoS level for Mark. The network informs the Metaverse application about the access network(s) availability and KPI information e.g., latency, throughput, connection density for each of the available access network(s).

The service provider begins using both 3GPP and non-3GPP access networks simultaneously for metaverse traffic meeting the required QoS requirements for Mark using existing traffic steering and splitting mechanisms.

5. Based on the KPI and network availability information shared by the network, the metaverse application server performs coordination for the metaverse traffic (between video flow and audio flow for the concert and the video flow and tactile flow for virtual painting) across the networks and shares the co-ordination information with the network for necessary policy modification.

### 5.X.4	Post-conditions

Mark can seamlessly access Metaverse applications via 5GS using both 3GPP and non-3GPP access networks.

### 5.X.5	Potential New Requirements

[P.R.-5.1.6-1] The 5GS shall enable third party Metaverse applications to be aware of access network(s) availability and their key performance indicators (KPIs), e.g., latency, throughput, connection density from 3GPP and non-3GPP networks.

[P.R.-5.1.6-2] Subject to operator policy, if metaverse traffic traverses across different access networks, the 5GS shall use the coordination information from the Metaverse applications to dynamically perform policy modifications for a user across networks (two 3GPP networks, or 3GPP and non-3GPP networks).