3GPP TSG-SA WG1 Meeting #100 	S1-223227

Toulouse, France, 14– 18 November 2022	(revision of S1-22xxxx)

Title:	Sensing for parking space determination

Agenda Item:	7.1

Source:	Ericsson

Contact:	Daniel,lonnblad at ericsson.com

Abstract: This document proposes a new use case about sensing for parking space determination for FS_Sensing.

To be able to write sensing contributions without selecting a specific implementation (which is outside the scope of SA1) we propose to use neutral terms. By introducing Sensing transmitter and sensing receiver we can describe specific requirement for the components without alluding to specific implementation.

Sensing transmitter: A sensing transmitter is the entity that sends out the sensing signal which the sensing service will use in its operation. A sensing transmitter can be a RAN node or a UE. A sensing transmitter can be located in same or different entity as the sensing receiver.

Sensing Receiver: A sensing Receiver is an entity that receives the sensing signal which the sensing service will use in its operation. A sensing receiver can be a RAN node or a UE. A sensing Receiver can be located in same or different entity as the sensing receiver.

**************** First Change ******************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	W. Favoreel, "Pedestrian sensing for increased traffic safety and efficiency at signalized intersections," 2011 8th IEEE International Conference on Advanced Video and Signal Based Surveillance (AVSS), 2011, pp. 539-542, doi: 10.1109/AVSS.2011.6027406.

[3]	Advances in Wildlife Crossing Technologies: https://highways.dot.gov/public-roads/septoct-2009/advances-wildlife-crossing-technologies.

[4]	Protection Detection: Making Roads Safe for Drivers and Wildlife: https://onlinepubs.trb.org/onlinepubs/webinars/201118.pdf.

[5]	F. Liu et al., "Integrated Sensing and Communications: Towards Dual-functional Wireless Networks for 6G and Beyond," in IEEE Journal on Selected Areas in Communications, doi: 10.1109/JSAC.2022.3156632.

[6]	T. S. Rappaport, G. R. MacCartney, M. K. Samimi and S. Sun, "Wideband Millimeter-Wave Propagation Measurements and Channel Models for Future Wireless Communication System Design," in IEEE Transactions on Communications, vol. 63, no. 9, pp. 3029-3056, Sept. 2015, doi: 10.1109/TCOMM.2015.2434384.

[7]	C. Han, Y. Bi, S. Duan and G. Lu, "Rain Rate Retrieval Test From 25-GHz, 28-GHz, and 38-GHz Millimeter-Wave Link Measurement in Beijing," in IEEE Journal of Selected Topics in Applied Earth Observations and Remote Sensing, vol. 12, no. 8, pp. 2835-2847, Aug. 2019, doi: 10.1109/JSTARS.2019.2918507.

[8]	IEEE 802.11-18/0611r16: “Wireless LANs, WiFi Sensing Uses Cases”

[9]	TEM STANDARDS TEM STANDARDS AND RECOMMENDED PRACTICE: https://unece.org/fileadmin/DAM/trans/main/tem/temdocs/TEM-Std-Ed3.pdf

[10]	ETSI TS 103 300-2: "Intelligent Transport Systems (ITS); Vulnerable Road Users (VRU) awareness; Part 2: Functional Architecture and Requirements definition; Release 2".

[11]	3GPP TR 22.856, "Localized Mobile Metaverse Services".

[12]	3GPP TR 29.238, "Uplink Streaming".

[13]	“Velodyne™ LiDAR VPL-16 User Manual,” 63-9243 Rev. E, Velodyne™ LiDAR, https://velodynelidar.com/wp-content/uploads/2019/12/63-9243-Rev-E-VLP-16-User-Manual.pdf  <accessed 9.8.22>

[14] 		Liu, A., Huang, Z., Li, M., Wan, Y., Li, W., Han, T.X., Liu, C., Du, R., Tan, D.K.P., Lu, J. and Shen, Y., 2022. A survey on fundamental limits of integrated sensing and communication. IEEE Communications Surveys & Tutorials, 24(2), pp.994-1034.

[15] 		https://medium.com/desn325-emergentdesign/s-l-a-m-and-optical-tracking-for-xr-cfabb7dd536f

[16] 		Dwivedi, S., Shreevastav, R., Munier, F., Nygren, J., Siomina, I., Lyazidi, Y., Shrestha, D., Lindmark, G., Ernström, P., Stare, E. and Razavi, S.M., 2021. Positioning in 5G networks. IEEE Communications Magazine, 59(11), pp.38-44.

[17]	T. Murakami et al, “Wildlife Detection System Using Wireless LAN Signal,” in NTT Technical Review vol.17, No.6, pp. 45-48, June 20019, https://www.ntt-review.jp/archive/ntttechnical.php?contents=ntr201906fa13.pdf&mode=show_pdf

[18]	Eradication of elephant mortality and injury due to railway accidents through automatic tracking and alert system in IEEE Conference Publication, IEEE Xplore

[19]	Impact of wild animals (deer and bears) on train operations 210616_KO_Animal2.pdf (jrhokkaido.co.jp) (in Japanese) [z]	Rail Industry Safety Induction Handbook: https://railsafe.org.au/__data/assets/pdf_file/0009/32022/Rail-Industry-Safety-Induction-RISI-Handbook-V5.1.pdf

[20]	Rail Industry Safety Induction Handbook: https://railsafe.org.au/__data/assets/pdf_file/0009/32022/Rail-Industry-Safety-Induction-RISI-Handbook-V5.1.pdf

[21]	Society of Automotive Engineers (SAE), “Taxonomy and definition for terms related to Driving automation systems for on-Road Motor Vehicles”, https://www.sae.org/standards/content/j3016_202104/

[22]	Census of Fatal Occupational Injuries Summary, 2020   https://www.bls.gov/news.release/cfoi.nr0.htm

[23]	Javed MA, Muram FU, Hansson H, Punnekkat S, Thane H. Towards dynamic safety assurance for Industry 4.0. Journal of Systems Architecture. 2021 Mar 1; 114:101914.

[24]	American National Standards Institute/Industrial Truck Safety Development Foundation, Safety standard for driverless, automatic guided industrial vehicles and automated functions of manned industrial vehicles, December 2019, 2019, [Online] http://www.itsdf.org.

[25]	Moore, Erik George, "Radar Detection, Tracking and Identification for UAV Sense and Avoid Applications" (2019). Electronic Theses and Dissertations. 1544.

[26] 	I. Sobron, I. Landa, I. Eizmendi and M. Velez, "Adaptive TOA Estimation with Imperfect LOS and NLOS Knowledge in UWB Positioning Systems," 2020 IEEE SENSORS, 2020, pp. 1-4. available at: https://sci-hub.wf/10.1109/sensors47125.2020.9278589

[27] 	L. Wang, Z. Zhang, X. Di and J. Tian, "A Roadside Camera-Radar Sensing Fusion System for Intelligent Transportation," 2020 17th European Radar Conference (EuRAD), 2021, pp. 282-285.

[28]	5GAA_White_Paper_C-V2X Use Cases Volume II: Examples and Service Level Requirements

[29]	Moore, Erik George, "Radar Detection, Tracking and Identification for U AV Sense and Avoid Applications" (2019). Electronic Theses and Dissertations. 1544.

[30]	Moore, Erik George, "Radar Detection, Tracking and Identification for U AV Sense and Avoid Applications" (2019). Electronic Theses and Dissertations. 1544

[31]	X. Liu, J. Cao, S. Tang and J. Wen, "Wi-Sleep: Contactless Sleep Monitoring via WiFi Signals," 2014 IEEE Real-Time Systems Symposium, 2014, pp. 346-355, doi: 10.1109/RTSS.2014.30.

[32]	Chen V C. The micro-Doppler effect in radar. Artech house, 2019.

[33]	3GPP TS 22.261: “Service requirements for the 5G system”.

[34]	A. Chebrolu, "FallWatch: A Novel Approach for Through-Wall Fall Detection in Real-Time for the Elderly Using Artificial Intelligence", 2021 Third International Conference on Transdisciplinary AI (TransAI), 2021, pp. 57-63, doi: 10.1109/TransAI51903.2021.00018, https://ieeexplore.ieee.org/document/9565618

[35]	B. A. Alsaify et al., “A CSI-Based Multi-Environment Human Activity Recognition Framework” Applied Sciences 12, no. 2: 930, 2022. https://doi.org/10.3390/app12020930

[36]	U. Saeed U et al., "Discrete Human Activity Recognition and Fall Detection by Combining FMCW RADAR Data of Heterogeneous Environments for Independent Assistive Living", Electronics 10(18):2237, 2021. https://doi.org/10.3390/electronics10182237

[37]	C. Dou, H. Huan, "Full Respiration Rate Monitoring Exploiting Doppler Information with Commodity Wi-Fi Devices". Sensors 21, 3505, 2021. https://doi.org/10.3390/s21103505

[38]	J. Pu, H. Zhang, "RF-Heartbeat: Robust and Contactless Heartbeat Monitoring Based on FMCW Radar", 2021. TechRxiv Preprint. https://doi.org/10.36227/techrxiv.15021645.v2

[39]	 Report on Automated Valet Parking: technology assessment and use case implementation description – 5G Automotive Association (5gaa.org)

************* End of First Change ***************

**************** Second Change ******************

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

sensing measurement data: data collected about radio/wireless signals impacted (e.g. reflected, refracted, diffracted) by an object or environment of interest for sensing purposes.

sensing measurement process: process of collecting sensing measurement data.

sensing result: the information derived from processing sensing measurement data.

NOTE: Examples of sensing result are characteristics of an object or environment, etc.

Sensing service area location:  an area location whether with or without obstacle, the 5G system can provide sensing service with certain quality.

transparent sensing: sensing measurements are communicated such that they can be discerned and interpreted by the 5GS, e.g. the data is communicated using a standard protocol to an interface defined by the 5GS.

Sensing transmitter: A sensing transmitter is the entity that sends out the sensing signal which the sensing service will use in its operation. A sensing transmitter can be a RAN node or a UE. A sensing transmitter can be located in same or different entity as the sensing receiver.

Sensing Receiver: A sensing Receiver is an entity that receives the sensing signal which the sensing service will use in its operation. A sensing receiver can be a RAN node or a UE. A sensing Receiver can be located in same or different entity as the sensing receiver.

************* End of Second Change ***************

**************** Third Change ******************

# 5	Use cases

## 5.x	Use case of Sensing for Parking Space Determination

### 5.x.1	Description

The use case to find parking place would be greatly improved by being digitalized compared to the current method, which is to drive around and search for an empty spot. When 3GPP networks introduces sensing service, one option to determine available parking spaces will be to use this 3GPP sensing technology.

Another related use case is automated parking e.g. AVP (Automated Valet Parking) and AFP (Automatic Factory Parking) [39] where cars are provided with drive-path information to do automated parking in a given parking lot facility. Connectivity is an important component in automatic parking, and the 3GPP sensing technology can serve as the way to determine available parking spaces and the best route for a car to reach it.

The coverage could be either a public network or a private network specifically for the parking garage.

There are multiple methods to achieve the sensing service, but these are better discussed during the solution phase, and are therefore not included in the use case or service requirement. A crucial requirement in this use case is that the sensing resolution need to be good enough to determine that there is an available parking lot. The service area is another KPI to be derived from the use case.

### 5.x.2	Pre-conditions

A multi- story parking garage has installed a 3GPP sensing service to help vehicles to find empty parking spaces within the garage. This parking garage can provide the information using the service “find a parking lot” to vehicles about the availability of the parking space.

Consider an example scenario shown in Fig. 5.x.2-1. A typical parking space is with length 5 m and width 2.5 m. The sensing service needs to identify cars parked at the different parking spaces, and the resolution therefore requires to be able to separate different parking spaces. The results can be aggregated, and the state of each parking place can be updated in seconds, which helps avoid a wasting of time for the entering vehicles.

Figure 5.x.2-1 example of sensing scenario

### 5.x.3	Service Flows

- Ella wants to park her car at the parking garage. Her vehicle, on entering the parking garage, queries the service application for availability of a parking bay.

- The service application uses the 3GPP sensing system to find the best available parking bay and can immediately answer to the request.

- With the sensing results, the service application can send the results back with either a dynamic map showing how to drive to the parking bay or instruct the car’s self-driving system how to maneuvers to get to the spot

### 5.x.4	Post-conditions

Thanks to the sensing system Ella has found a parking space without issue, making life easier during daily travel.

### 5.x.5	Existing features partly or fully covering the use case functionality

None.

### 5.x.6	Potential New Requirements needed to support the use case

[PR 5.x.6-1] The 5G system shall be able to provide sensing services for licensed and unlicensed spectrum.

[PR 5.x.6-2] The 5G system shall be able to authorize sensing receiver(s) and sensing transmitter(s) to participate in a sensing session.

[PR 5.x.6-3] Based on operator’s policy, the 5G system shall expose a suitable API to a trusted third party to authorize sensing services.

[PR 5.x.6-4] Based on operator’s policy, the 5G system shall expose a suitable API to a trusted third party to request the activation of the sensing service with specific KPI requirement, as well as deactivation of the same service.

[PR 5.x.6-5] The 5G system shall be able to support charging for the sensing services (e.g. considering service type, sensing accuracy, target area, duration).

[PR 5.x.6-6] The 5G system shall be able to provide sensing with following KPIs.

Table 5.x.6-1: Performance requirements for the use of sensing in the Parking space determination

| Scenario | Distance Accuracy (confidence level 95%) |  | Velocity Accuracy (confidence level 95%) |  | Resolution |  | Sensing service latency | Refreshing rate |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Horizontal Accuracy | Vertical Accuracy | Horizontal Accuracy | Vertical Accuracy | Horizontal Resolution | Vertical Resolution |  |  |
| Parking space determination | 0.5m | 0.5m | 0.1m/s | - | 2m | - | 1s | - |

************* End of Third Change ***************